3GPP TSG-RAN2 Meeting #Adhoc 	R2-1700166

Spokane, Washington, USA, 17th - 19th January 2017

Agenda item:	3.3.1.1.1

Source: 	Huawei, HiSilicon

Title: 	Measurement and Mobility in high frequency

Document for:	Discussion

# Introduction

Major progress has been made in last RAN2#96 meeting in respect of mobility with RRC involvement, and the following FFSs are left for further discussion [1].

Agreements for connected active

1: 	RRM measurement for cell level mobility should be performed based on a common framework regardless of network beam configurations (e.g., number of beams) and the UE beam configuration.

FFS: Which beams the UE selects from the detected beams in order to derive a cell level quality. Options to be studied:

a/ best beam,

b/ N best beams,

c/ all detected beams

d/ beams above a threshold.

Other options are not precluded

Agreements

1	At least events like LTE A1-A6 and periodic will be supported for NR (modification to the events may be considered)

FFS other events may be studied

2	Measurement report will contain cell measurements

FFS whether the measurement report contains beams measurements

Agreements

1	At least cell id and all information required to access the target cell will be included in the HO command.

2	For at least some cases information required for contention based and contention free access can be included in the HO command

3	To be studied what beam related information of the target cell may be required.

4	Study the possibility of handover where a condition configured by the gNB is used by the UE to determine when it executes the handover.

In this paper, we present our views on these left issues, including beam based measurement mechanism, cell quality derivation, measurement configuration and measurement events design.

# Discussion

## Beam based Measurement

Beamforming technique is used to compensate large path loss in high frequency system, and the narrower the beam is the more beamforming gain it can provide. To derive a reasonable coverage the beam has to be narrow enough, and at the same time the NR cell has to be covered by many narrow beams. For DL-based mobility in RRC connected mode UE should perform RRM measurement which includes L1 filtering and L3 filtering at least. When the cell is covered by many beams spatially or temporally multiplexed, for example, UE may detect several serving beams simultaneously or over time. The measurement filtering mechanism should also be improved regarding this. Besides L1 filtering is implementation dependant, the behaviour of the L3 filtering is standardised and the configuration of the layer 3 filtering is provided by RRC signalling.

In a high frequency cell, for each of UE detected beam, the UE can derive a L1 filtering result. There are two options to handle L1 filtering results of all detected beams described as follows.

Option 1: each beam has a dedicated L3 filtering result.

After L1 filtering result is ready, it will be input into the L3 filter to obtain a L3 filtering result immediately. Then UE gets the RRM measurement result of each beam. In this way UE can report beam level quality of every detected beam, or  it can perform combination of beams to report a cell level quality.

Fig. 1 beam level L3 filtering

Option 2: each beam only has a dedicated L1 filtering result, and L3 filtering is based on L1-level combing.

In high frequency, channel variation is very large so one beam may not be detected by UE all the time due to beam blockage. In this circumstance for one blocked beam L3 filtering could not work well due to lack of L1 filtering result. So only L1 filtering is performed and L3 filtering is based on the combination result of L1 filtering results of beams. In consequence UE can only report cell level quality to gNB.

Fig. 2 L3 filtering based on combination of L1 filtering results of beams

If the high frequency CH stays stable which means at least part of beams can be detected all the time, the option 1 can be used. And if high frequency channel varies fast, then the option 2 may be better.

In addition, there could be multiple receiving beams at the UE side. Then the input to the Layer 1 filtering may be the measurements (samples) of multiple beam pairs (a beam pair consists of a Tx beam and a Rx beam). The Layer 1 filtering, Layer 3 filtering and/or the measurement results combination can be performed on the basis of the results of these beam pairs.

Proposal 1: if high frequency channel is stable beam level L3 filtering can be performed, and if high frequency channel changes fast L3 filtering should be performed based on combination of L1 filtering results of beams (pairs).

Proposal 2: cell quality can be derived based on combination of L3 filtering results of beams (pairs), and it can also be derived based on L3 filtering result of the combination result of L1 filtering results of beams (pairs).

The combination method should be common no matter combination is performed based on L1 filtering results and L3 filtering results. It is about how to derive a cell quality based on measurements from individual beams exactly. Actually it depends on the beam tracking ability and accuracy.

There are several options for beam level combination:

- UE only uses best beam quality instead of combining individual beams. This option is suitable if accurate beam tracking can be implemented, i.e. UE can always choose the best beam as serving beam.

- UE combines part of beams, e.g. beams whose RSRP is more than a threshold, or UE combines finite number of beams, e.g. best N beams. This option can be applied if beam tracking is implemented among candidate serving beams. The combining algorithm may simply be averaging.

- UE combines all simultaneously detected (or eligible serving) beams to derive a cell quality if beam varies fast so that beam tracking has to be performed in all detected beams. The combining algorithm may simply be averaging.

Proposal 3: different cell quality evaluating methods combining the best beam/part of beams/all detected beams should all be supported according to different beam tracking performance.

## Measurement configuration and report

Different from LTE, beam sweeping is used in NR and there is no always existing BRS for measurement. Also the measured beams transmit signal in one narrow direction and UE must use the corresponding Rx beam to perform the measurement. Therefore, considering the UE power saving and design complexity, the gNB should inform the UE which beams should be measured and when and in which direction the measured beam will be transmitted.

Proposal 4: gNB configures UE a beam measurement set (serving and candidate beams) and beam specific information including DMTC for RRM measurement.

In order to obtain the beam information of neighbour gNB for measurement for inter-gNB handover, gNBs should exchange their beam configuration. The configuration may include beam width, directions, angular, single beam sweeping or multiple beam sweeping, etc.

Proposal 5: neighbour NBs exchange their beam information, including beam width, directions, angular, single beam sweeping or multiple beam sweeping etc. so that it can configure DMTC for RRM measurement of neighbouring cells.

After UE detects multiple beams in NR cells from both of serving cells and neighbour cells, for RRM measurement reporting, there are two options:

- UE reports individual beam (pair) measurement result.

- UE reports combined quality of multiple beams.

With option 1, gNB can derive all beams’ information if all the detected beams are reported, which generates more system overhead; with option 2, smaller system overhead is needed to report a combined quality.

Considering PHY with or without MAC may be responsible for handling beam tracking in the serving cell based on PHY-level beam measurements, a combined beam quality is more suitable to be reported in L3 which can present a long term cell quality. For neighbour cells a combined beam quality should also be used for evaluation.  And for performing handover between gNBs the beam information of target cell is really useful to choose a suitable beam for better handover performance.

So we have the following proposal:

Proposal 6: UE reports the combined beam quality of serving cell and of neighbour cells for cell level quality evaluation for handover. It can also report beam level measurement results according to gNB configuration.

Proposal 7: besides cell-specific measurement value, beam (pair)-specific measurement value could also be included in measurement report which can help for better handover performance.

## Measurement events

### Traditional measurement events

In high frequency system, traditional measurement events can be kept but the mobility scenario can be different, which involves intra-cell beam switching, inter-cell beam switching within a TRPG, as well as inter-cell beam switching among TRPGs. Such mobility scenario calls for the corresponding measurement report. To cater for such different level or hierarchical measurement reporting, different measurement trigger thresholds could be issued by taking into account the intra/inter TRP/TRPG cases that are related to different level of signaling. Typically, inter-TRPG shall request higher threshold since RRC link transfer is required for this cell level mobility while intra-TRPG shall request lower threshold since no RRC link transfer is required for this beam level mobility. Moreover, it could be considered that during the measurement events configuration, gNB could indicate whether the UEs compare the cell-specific measurement result or beam-specific measurement result from serving cell and/or neighbouring cell, and further indicate whether the UEs report which kind of measurement result when measurement events are satisfied.

Proposal 8: with respsect to the multiple level mobility scenario and the corresponding hierarchical measurement reporting, different thresholds could be issued by taking into account the intra/inter TRP/TRPG cases that are related to different level of signaling.

Proposal 9: during the measurement events configuration, gNB could indicate whether the UEs compare the cell-specific measurement result or beam-specific measurement result from serving cell and/or neighbouring cell, and further indicate whether the UEs report which kind of measurement result.

### Beam related measurement events

Measurement events can be regarded as a kind of measurement result with fulfilling specific condition. It provides the abstraction of UE radio environment to gNB with an event identification. So if UE can add beam related measurement information in measurement report, it is natural to abstract these measurement results to different events which can reduce system overhead for measurement report. Furthermore if beam level measurement events can trigger a measurement report can be a FFS, and if it can be used for beam management heavily depends on RAN1. To clarify the meaning of the events agreed, example event definitions are given below:

Event D1:  beam quality becomes better than threshold

Entering condition:

Leaving condition:

The variables in the formula are defined as follows:

- Mcp is the measurement result of the target beam resource (dBm for beam-RSRP).

- Hys is the hysteresis parameter for this event (dB)

- Thresh is the offset parameter for this event (dB)

Event D2: beam quality becomes offset better than the best beam quality

Entering condition:

Leaving condition:

The variables in the formula are defined as follows:

- Mcp is the measurement result of the target beam resource (dBm for BEAM-RSRP).

- Mrp is the measurement result of the strongest beam resource (dBm for BEAM-RSRP).

- Hys is the hysteresis parameter for this event (dB)

- Off is the offset parameter for this event (dB)

Event D3: beam quality becomes offset better than a configured reference beam quality

Event D4: beam quality becomes worse than threshold

Event D5: best N beam quality are changed, N is a preconfigured number

Event D6: The measurement result of a candidate beam quality becomes offset better than that of the worst reference beam quality

Proposal 10: the need and exact formulation of new events for beam specific measurement can be studied in RAN2.

# Conclusion

In this paper, we discussed the beam based measurement mechanism and how to derive cell quality from individual beams, and we have the following proposals:

Proposal 1: if high frequency channel is stable beam level L3 filtering can be performed, and if high frequency channel changes fast L3 filtering should be performed based on combination of L1 filtering results of beams (pairs).

Proposal 2: cell quality can be derived based on combination of L3 filtering results of beams (pairs), and it can also be derived based on L3 filtering result of the combination result of L1 filtering results of beams (pairs).

Proposal 3: different cell quality evaluating methods combining the best beam/part of beams/all detected beams should all be supported according to different beam tracking performance.

Proposal 4: gNB configures UE a beam measurement set (serving and candidate beams) and beam specific information including DMTC for RRM measurement.

Proposal 5: neighbour NBs exchange their beam information, including beam width, directions, angular, single beam sweeping or multiple beam sweeping etc. so that it can configure DMTC for RRM measurement of neighbouring cells.

Proposal 6: UE reports the combined beam quality of serving cell and of neighbour cells for cell level quality evaluation for handover. It can also report beam level measurement results according to gNB configuration.

Proposal 7: besides cell-specific measurement value, beam (pair)-specific measurement value could also be included in measurement report which can help for better handover performance.

Proposal 8: with respsect to the multiple level mobility scenario and the corresponding hierarchical measurement reporting, different thresholds could be issued by taking into account the intra/inter TRP/TRPG cases that are related to different level of signaling.

Proposal 9: during the measurement events configuration, gNB could indicate whether the UEs compare the cell-specific measurement result or beam-specific measurement result from serving cell and/or neighbouring cell, and further indicate whether the UEs report which kind of measurement result.

Proposal 10: the need and exact formulation of new events for beam specific measurement can be studied in RAN2.

# Reference

- RAN2#96 Chairman notes