3GPP TSG-RAN WG2 NR Ad Hoc	R2-1700504

Spokane, USA, 17 – 19 January 2017

Agenda item:	3.3.1.1.2

Source:	Samsung

Title:	Mobility Enhancements in NR phase 1 (Release 15)

Document for:	Discussion

- 1	Introduction

NR SI is approaching to the completion and NR phase 1 WI will commence in March 2017. The time schedule for NR phase 1 is 15 months which is very tight comparing to LTE.

Figure 1

Mobility is an essential feature of mobile communication; many solutions to enhance mobility performance have been submitted for NR SI. Considering the limited time schedule for NR phase 1, we don’t afford to discuss and standardize all of them in the first release.

This paper aims to trigger the discussion on the scope of mobility enhancements for NR phase 1.

- 2	Baseline mobility for NR

Last meeting RAN2 made following agreements.

<Table 1>

Agreements on basic HO mechanism 1: NR shall support HO as part of the NR mobility procedures. 2: Network based mobility shall reuse the same principles as LTE (Rel-13) and for inter gNB HO consisting of at least: - Source gNB initiates the HO over the Xn interface via a HO request - Target gNB performs admission control and provides the RRC configuration as part of the HO acknowledgement - Source gNB provides the configuration to the UE including the HO command via RRC - The UE moves the connection to the target gNB via RRC Further enhancements/modifications can be considered

According to the agreements NR will support LTE-like HO as a baseline for intra-NR mobility, which would have following signalling sequence.

Figure 2

The question we like to discuss in this paper is what enhancements on top of the baseline mechanism shall be standardized during Rel-15/NR phase 1.

- 3. Mobility Enhancements for NR phase 1

- 3.1 Potential enhancements having discussed so far

Table 1 summarizes the solutions having been submitted in the last RAN2 meetings.

<Table 1>

| Solutions | Description | Interruption | Reliability |
| --- | --- | --- | --- |
| LTE baseline HO | Conventional NW conventional HO | Baseline ( ~ 50ms)* | Baseline |
| Make Before Break [1] | Source link is maintained until RACH preamble transmission in the target starts | Better than baseline (~ 15ms)* | Same as baseline |
| RACH-less HO [1] | HO without RACH | Better than baseline (~ 30ms)* | Same as baseline |
| MBB + RACH-less [1] | Source link is maintained until the first PUSCH transmission starts at target | Better than baseline (0 ~ 5ms)* | Same as baseline |
| UE based mobility/ Delayed HO [2][3] | HO command is provided earlier (when source channel quality is still good) and UE decides when to perform HO | Same as baseline | Better than baseline |
| RRC diversity | Mobility related RRC messages are transmitted multiple times via different connections | Same as baseline | Better than baseline |
| Dual-connection based mobility (e.g. DC with MeNB/SeNB change) [4] [5] | Simultaneous connections with source cell and target cell are maintained. | Better than baseline (0ms possible) | Better than baseline |

* Estimation from LTE release 14 eMOB WI

In general, mobility performance has two aspects; 1) how reliably mobility is executed (e.g. how infrequently mobility failure occurs) and 2) how long interruption is incurred during the mobility. MBB, RACH-less HO, combined solution and Dual-connection based mobility are solutions targeting interruption reduction. UE based mobility and RRC diversity are solutions to enhance the reliability.

Table 2 lists the expected UE complexity and standardization effort of each solution which should also be considered in evaluation.

<Table 2>

| Solutions | UE complexity | Standardization effort |
| --- | --- | --- |
| Make Before Break | 1 Rx/ 1 Tx | LTE solution can be largely reused. |
| RACH-less HO | 1 Rx/ 1 Tx | LTE solution can be largely reused. |
| MBB + RACH-less | 1 Rx/ 1 Tx | LTE solution can be largely reused. |
| UE based mobility/ Delayed HO | 1 Rx/ 1 Tx | New solution. Main work will be done by RAN2 |
| RRC diversity | 2 Rx/ 2 Tx | New solution; built on top of NR-NR dual connectivity. Multiple WGs would be involved |
| Dual-connection based mobility (e.g. DC with MeNB/SeNB change) | 2 Rx/ 2 Tx (at least in LF) | New solution; built on top of NR-NR dual connectivity. Multiple WGs would be involved. |

- 3.2 Enhancements on interruption

Given the tight schedule of Release 15, the basic principle should be that already existing enhancements are prioritized. Release 14 LTE eMOB WI is standardizing Make-Before-Break, RACH-less handover and combined solution, which will be finalized this March and could be largely reused in NR.

Proposal 1:  In Release 15 work item phase, for NR mobility enhancement on interruption, RAN2 standardize solutions that have already been specified in LTE (i.e. make-before-break, RACH less handover and combined solution)

Specifying Dual-connection based mobility in release 15 has following implications.

- It requires multiple Rx/Tx chains which wouldn’t be widely used in the initial UEs of the first release.

- It will be built on top of dual connectivity. Real discussion will be possible after dual connectivity is fully standardized.

- It is a new solution that hasn’t been discussed in LTE. Feasibility study should be performed first. For example, two independent key sets would be used simultaneously, which has no precedence in LTE (key set used in SeNB is derived from KeNB of MeNB). It may have impacts on SA3 discussion and/or UE requirements.

Based on above considerations, we propose;

Proposal 2: Feasibility of dual connection solution is studied first in Release 15 timeframe.

One can argue that LTE based solutions do not achieve zero mobility interruption time. We believe combined solution of MBB and RACH-less HO can achieve almost zero mobility interruption time [6]. It should also be pointed out that the limited timeframe may not allow achieving all KPIs in the first release anyway.

- 3.3 Enhancements on reliability

Handover reliability (i.e. HO failure rate and HO ping-pong rate) is an important factor in mobile communication system in a sense that handover failure is easily noticed by users and degrades QoE significantly. As pointed out in [7], achieving decent handover reliability is a challenging task especially in HF system. With the above considerations we believe NR system shall aim to provide reliable handover from the first release.

Proposal 3: RAN2 aims to provide means to enhance handover reliability in release 15.

Unfortunately, there are no existing solutions to be reused for NR; neither UE based mobility nor RRC diversity has been fully studied in LTE. To make it in Release 15, RAN2 can study the feasibility of the solutions during the remaining period of SI and the initial period of WI when the details of the baseline HO are discussed.

Proposal 4: RAN2 discusses the feasibilities of solutions on handover reliability during the remaining period of SI and the initial period of Phase 1 WI.

- 3 Conclusion

This paper discuss the scope of mobility enhancement in NR phase 1 and put forward following four proposals;

Proposal 1:  In Release 15 work item phase, for NR mobility enhancement on interruption, RAN2 standardize solutions that have already been specified in LTE (i.e. make-before-break, RACH less handover and combined solution)

Proposal 2: Feasibility of dual connection solution is studied first in Release 15 timeframe.

Proposal 3: RAN2 aims to provide means to enhance handover reliability in release 15.

Proposal 4: RAN2 discusses the feasibilities of solutions on handover reliability during the remaining period of SI and the initial period of Phase 1 WI.

- 4 References

[1]	RP-160921	Further mobility enhancements in LTE	ZTE, Intel, Samsung, China Telecom

[2]	R2-168513	Single-connect mobility in NR	Intel Corporation

[3]	R2-168597	RRC procedures for network based and UE based mobility	Qualcomm Incorporated, Convida Wireless

[4]	R2-168299	How to realize zero ms UP interruption in NR	Ericsson

[5]	R2-168564	0ms mobility interruption in NR	Huawei, HiSilicon

[6]	R2-1700504	How to define zero mobility interruption time	Samsung

[7]	R2-168852	Evaluation and Design Aspects for NR Mobility Enhancement	Samsung