3GPP TSG-RAN WG2 - AH	Tdoc R2-1700536

# Spokane, Washington, U.S., 17th – 19th January 2017

Agenda Item:	3.2.2.6

Source:	Ericsson

Title:	Signalling flows for paging and resume for RRC_INACTIVE

Document for:	Discussions & Approval

# 1	Introduction

This contribution is a revision of R2-168712 submitted to the last RAN2 meeting. It has been agreed in SA2 that NR should support an efficient inactive “state” (with similar battery savings LTE’s IDLE) both in CN connected and in CN idle. The detailed characteristics of the CN connected “state” (called RRC_INACTIVE in RAN2) and if it is actually a separate protocol state or not has not yet been concluded.

Some assumption has however been agreed in RAN2:

- The CN/RAN connection should be maintained for the UE in this “state”

- The UE is allowed to move around within and area and the RAN should manage this area and be made aware when UE move outside this area

- It is the responsibility of the RAN to reach (notify) the UE when DL data arrives

So far there has been no agreement on how RAN areas will be defined. Independent of this there needs to be signalling flows supporting RRC_INACTIVE and RAN areas also considering the case when there is no Xn connectivity. This contribution proposes some signalling flows for discussion.

# 2	Discussion

This section provides example calls flows on how the UE is moved to inactive mode, context retrieval, paging etc. which are common regardless of which notification area concept is adopted.

### 2.1	Moving UEs to Inactive Mode

Figure 2.2.1-1: Moving UEs to Inactive Mode

1.	Before the UE is moved to Inactive mode, the UE is in RRC_CONNECTED/CN-CONNECTED, i.e registered at the Serving CP CN node most likely assigned a (list of) TA(s) with PDU Session contexts activated, a UE-associated signalling connection is established on NG-C, and bearer contexts are established at the UE, the Serving RAN node and the Serving UP CN node.

2.	The UE is moved to RRC_ INACTIVE. The Serving RAN node provides a UE Context identification (ResumeIdentity). It is proposed to call the message ordering the UE to RRC_INACTIVE for RRCConnectionSuspend.

Note:	Dependent on the chosen approach additional information may provided in this message (e.g. a list of RAN Paging Area IDs, a list of Cell IDs) (FFS).

3.	When the UE was successfully moved to Inactive mode, it is regarded by the CN as being in CN-CONNECTED, it has changed to RRC_ INACTIVE, it is still registered at the Serving CP CN node in a (list of) TA(s), the UE-associated signalling connection is established towards the Serving RAN node on NG-C, NG-U UP resources are kept established but DRBs are released.

### 2.2	Context Retrieval via Xn (Example based on Rel-13 X2 functions)

This section outlines a stage-2 like description of Xn Context Retrieval. It shows Xn Context Retrieval in the context of RAN Paging, which is not detailed in this section. Further data forwarding and forwarding of NAS related signalling is described.

Figure 2.2.2-1: Context Retrieval via Xn

1.	In case of a mobile terminating traffic, Paging is triggered by the Old Serving RAN node, which may involve, if necessary, other (neighbouring) RAN nodes and, probably dependent on the approach, the CN.

2.	The UE performs the RRC connection establishment procedure. If it has received a Resume ID from the old serving RAN node it provides this ID in the RRCConnectionResumeRequest message. Figure 2.2.2-1 shows the case where the UE performs RRC connection establishment with a New Serving RAN node which is Xn connected to the Old Serving RAN node.

3.	The New Serving RAN node attempts to retrieve the UE Context by triggering the Xn Retrieve UE Context procedure.

4.	If the Old Serving RAN node is able to resolve the Resume ID it provides UE Context data to the New Serving RAN node.

Note:	It is for further study whether it is necessary that the response message should also foresee means to forward NAS PDUs.

5.&6. The New Serving RAN node, after having performed admission control, triggers Path Switch Request towards the CN.

7. The New Serving RAN node requests the Old Serving RAN node to release the UE Context. This message may contain DL forwarding addresses.

8.	Forwarding of pending DL user data is performed.

### 2.3	Context Retrieval via CN (within the same pool of CP CN nodes)

This section provides description on how a UE Context can be retrieved from the Old Serving RAN node via the CN, which becomes necessary if the New Serving RAN node realises that it has no Xn connection to the eNB indicated in the Resume ID.

The main principle of this scheme would be to retrieve the UE Context via the same CN pool by means of connection less signalling, as the New Serving RAN node would not know which CP CN node in the pool actually holds the UE Context. The alternative of performing Context Retrieval in a connection oriented fashion would require to subsequently tear down the (short-lived) signalling connection unnecessarily and to keep a state-logic in a CP CN node that is not concerned with the UE.

Figure 2.2.3-1: UE Context Retrieval via the CN

1.	In case of a mobile terminating traffic, Paging is triggered by the Old Serving RAN node, which may involve, if necessary, other (neighbouring) RAN nodes and, probably dependent on the approach, the CN.

2.	RRC connection is established, the UE provides the Resume ID in the RRCConnectionResumeRequest message.

3.	The New Serving RAN node realises that it has no Xn connection to any RAN node with the identity indicated in the Resume ID and requests a CP CN node within a pool to which it is connected, to resolve the RAN node identity. This CP CN node doesn’t necessarily need to be the CP CN node serving the UE. Respective signalling is performed in a connection-less and (for the CP CN node) stateless manner.

4.	If the CP CN node is able to resolve the address of the indicated RAN node identity, it forwards the request to that node.

5.	The Old Serving RAN node provides UE Context Data.

Note:	It is for further study whether it is necessary that the response message should also foresee means to forward NAS PDUs.

6.	The CP CN node forwards the information received in step 5 to the New Serving RAN node.

7.	The New Serving RAN node triggers the path switch procedure (as if it would have received UE context data via Xn) and indicates that no Xn connection is available towards the Old Serving RAN node. The message also contains forwarding addresses.

8.	The CP CN node response to the Path Switch Request.

9.	The CP CN node triggers the UE Context Release procedure (as the New Serving RAN node is not able to trigger it via Xn) and provides forwarding addresses.

10.	If necessary, forwarding of user data takes place. Dependent on network configuration and deployment, either direct or indirect forwarding is applied.

### 2.4	Context Retrieval failed (either on Xn or via NG-C)

This section provides description of how a failed context retrieval is handled, also containing details of data forwarding of pending DL user data in the Old Serving RAN node.

Figure 2.2.4-1: Failed Context retrieval.

1.&2: The UE is paged and establishes the RRC connection, first trying to provide the Resume ID.

3.	Retrieving the UE Context, either by Xn or NG-C signalling fails. The RRC connection establishment commences without the Resume ID.

4.	The New Serving RAN node establishes an NG-C UE-associated signalling connection.

If it is required to allow data forwarding also in this, the following scheme is proposed: In order to avoid lengthy additional signalling, the New Serving RAN node should provide forwarding addresses to the Old Serving RAN node in steps 4 and 6.

5.	The CP CN node detects that 2 signalling connections exist for the same UE and releases the one towards the Old Serving RAN node.

6.	Forwarding addresses are provided from the CN as received by the New Serving RAN node.

7.	If necessary, forwarding of user data takes place. Dependent on network configuration and deployment, either direct or indirect forwarding is applied.

### 2.5	Xn Paging

This section outlines a stage-2 like description of the Xn Paging procedure.

Figure 2.2.5-1: Xn Paging

0.	The UE is in RRC_IDLE/INACTIVE and the Serving CP CN node regards the UE being in ECM-CONNECTED, as described for the post conditions in section 2.3.1.

1.	A DL user data packet arrives at the Serving RAN node

2.	The Serving RAN node, based on available statistics, decides to page within its own cells first.

3.&4. The first paging attempt is not successful.

5.	The Serving RAN node, based on available statistics, decides to expand the paging scope to its direct X2 neighbours (expanding paging scope over whole RAN area can also be performed at step 2).

6.	The Serving RAN node sends a Paging Request to the selected Xn neighbours, including assistance data (Paging Attempt Count, Recommended Cell List, etc.)

7.	The neighbour nodes page the UE.

### 2.6	CN Assisted RAN Paging

CN Assisted RAN Paging is needed if the Serving RAN node is not able to reach the UE by paging in its own and its Xn connected neighbour eNBs’ cells.

A NG-C message is introduced for trigger the CN to distribute pages into an area indicate by the serving RAN node.

Note, that the nature of the UE Paging Identity is FFS in RAN2, it could be provided by the CN (~S-TMSI) or by the RAN (e.g. Resume ID).

The RAN Paging Area and cell-list approaches heavily rely on Xn connectivity, which is not always given. If paging assistance data would indicate cells/nodes outside the RAN serving node’s Xn connectivity, CN assisted RAN paging would need to be applied also for those approaches.

Figure 2.3.1-1: CN assisted paging

0.-7. These steps are described in section 2.2.5.

8.	The UE hasn’t been reachable the previous paging attempts in cells of the Serving RAN node and its X2-connected neighbours.

9.	The Serving RAN node decides to request assistance from the CN.

10.	The Serving RAN node indicates the paging scope by introducing a (list of) TA(s) in the NG-C Paging Assistance Request.

11.	As the Serving RAN node is in charge of the overall paging process and the CN is only supposed to distribute S1 Paging messages to the eNBs within the indicate paging scope, the Serving RAN node would need to be provided by the Serving CP CN node with respective paging parameters within NG-C Context Setup and Modification signalling. Examples for parameters are: The TAI(s) the UE is registered within the Serving CN CP node, (e)DRX related parameters, etc.

12a. The Serving CP CN node doesn’t mirror back the Paging Assistance Request to the Serving RAN node which performs another paging attempt within its cells.

12b.	Paging is distributed to eNBs that might have been previously requested via Xn to page the UE.

12c. Paging is distributed to eNBs that are out of Xn-reach for the Serving RAN node.

# 3	Conclusion

Regardless which solution is adopted for the “RAN notification areas” there is most likely a need to support the procedures described in this contribution.

Proposal 1: Consider the procedure discussed in this contribution in the work on defining the solution for RAN paging etc. in CN connected inactive “state” (RRC_INACTIVE).