3GPP TSG-RAN WG2 #AH	Tdoc R2-1700659

# Spokane, Washington, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.2

Source:	Ericsson (Rapporteur)

Title:	TP on QoS and new protocol layer

# Introduction

This paper provides a text proposal to TR 38.804 on QoS for NR and new protocol layer according to the below email discussion and agreements.

[NR-AH#03][NR] QoS TP (Ericsson)

To update TP from 427 to make clear distinction between AS and NAS level reflective QoS and add figure from 621 including updates to show the flow IDs.

-	To also include capturing agreements on new protocol layer.

Intended outcome: Agreed TP

Deadline: Thursday 26/01/2017

Agreements:

R2-1700427	Email discussion (29) report for TP on QoS	Ericsson	report		Rel-14		FS_NR_newRAT

=>	Endorsed as baseline.

=>	To be updated to make a clear distinction between AS and NAS level reflective QoS

R2-1700621	Text Proposal to TR 38.804 on QoS architecture for NR	NTT DOCOMO INC.	discussion		Rel-14		FS_NR_newRAT

=>	Try to find a way to include the flow ID in the figure.

Agreements

1: A new user plane AS protocol layer (e.g. PDAP) above PDCP should be introduced to accommodate all the functions introduced in AS for the new QoS framework, including:

-	QOS flow->DRB routing;

-	QoS-flow-id marking in DL packets;

-	QoS-flow-id marking in UL packets;.

2	The new protocol layer is applicable for all cases connecting to the 5G-CN

3:	Single protocol entity is configured for each individual PDU session.

# TP on QoS for TR

# 8	QoS control

## 8.1	QoS architecture in NR and NextGen Core

Editor’s note:	 intended to capture overall architecture in NR and NextGen Core something like sub-clause 13.1 in TS 36.300.

The QoS architecture in NR and NextGen Core is depicted in the Figure 8.1-1 and described in the following:

-	For each UE, the 5G-CN establishes one or more PDU Sessions.

-	For each UE, the RAN establishes one or more Data Radio Bearers per PDU Session. The RAN maps packets belonging to different PDU sessions to different DRBs. Hence, the RAN establishes at least one default DRB for each PDU Session indicated by the CN upon PDU Session establishment.

-	NAS level packet filters in the UE and in the 5G-CN associate UL and DL packets with QoS Flows.

-	AS-level mapping in the UE and in the RAN associate UL and DL QoS Flows with Data Radio Bearers (DRB).

Figure 8.1-1: QoS architecture in NR and NextGen Core

5G-CN and RAN ensure quality of service (e.g. reliability and target delay) by mapping packets to appropriate QoS Flows and DRBs. Hence there is a 2-step mapping of IP-flows to QoS flows (NAS) and from QoS flows to DRBs (Access Stratum).

In NR, the data radio bearer (DRB) defines the packet treatment on the radio interface (Uu).

A DRB serves packets with the same packet forwarding treatment. Separate DRBs may be established for QoS flows requiring different packet forwarding treatment.

In the downlink, the RAN maps QoS Flows to DRBs based on NG3 marking (QoS Flow ID) and the associated QoS profiles.

In the uplink, the UE marks uplink packets over Uu with the QoS flow ID for the purposes of marking forwarded packets to the CN

In the uplink, the RAN may control the mapping of QoS Flows to DRB in two different ways:

- Reflective mapping: for each DRB, the UE monitors the QoS flow ID(s) of the downlink packets and applies the same mapping in the uplink; that is, for a DRB, the UE maps the uplink packets belonging to the QoS flows(s) corresponding to the QoS flow ID(s) and PDU Session observed in the downlink packets for that DRB. To enable this reflective mapping, the RAN marks downlink packets over Uu with QoS flow ID.;

Editor’s Note: It is FFS whether the marking with a QoS flow ID can be semi-statically configured (to not include the QOS flow ID when not needed).

- Explicit Configuration: besides the reflective mapping, the RAN may configure by RRC an uplink “QoS Flow to DRB mapping”.

Editor’s Note: The precedence of the RRC configured mapping and reflective QoS is FFS (can reflective QoS update and thereby override an RRC configured mapping? Or does a configured QoS Flow ID to DRB mapping always take precedence over a reflective mapping?)

[Working assumption:] If an incoming UL packet matches neither an RRC configured nor a reflective “QoS Flow ID to DRB mapping”, the UE shall map that packet to the default DRB of the PDU session.

Within each PDU session, is up to RAN how to map multiple QoS flows to a DRB. The RAN may map a GBR flow and a non-GBR flow, or more than one GBR flow to the same DRB, but mechanisms to optimise these cases are not within the scope of standardization.

A new user plane Access Stratum (AS) protocol layer above PDCP is responsible for the functions introduced in the UE and RAN for the new QoS framework, including:

- QoS flow to DRB mapping;

- QoS-flow-id marking in DL packets;

- QoS-flow-id marking in UL packets.

The new user plane protocol layer is applicable for connections to the 5G-CN.

A single protocol entity of the new user plane protocol layer is configured for each individual PDU session.

--- End Text Proposal –