3GPP TSG-RAN WG2 NR Ad Hoc	Tdoc R2-1700665

# Spokane, USA, 17th – 19th January 2017

Agenda Item:	3.2.2.6

Source:	Ericsson

Title:	Text proposal to TR 38.804 capturing the agreements on NR access control

Document for:	Decision

# Introduction

This paper provides a text proposal to TR 38.804 on Access control for NR:

[NR-AH1#07][NR]  TP on Access Control (Ericsson)

Intended outcome: Agreed TP

Deadline: Thursday 26/01/2017

# History of RAN2 Agreements:

RAN2#NR AH:

Agreements

1:	NR system should support overload/access control functionality of RACH backoff, RRC Connection Reject, RRC Connection Release and UE based access barring mechanisms.

2:	RAN2 should aim to specify one unified access barring mechanism for NR that can address all the use cases and scenarios defined in LTE.

3:	The unified access barring mechanism needs to be forward compatible in order to cope with future use cases/scenarios.

4:	RAN2 should aim to specify an access barring mechanism for NR that is applicable for all RRC states in NR (RRC_IDLE, RRC_CONNECTED and RRC_INACTIVE). [FFS whether it will be possible for the mechanism to be completely common between the states]

5	Study whether it is possible to specify the unified access barring mechanism fully inside the 3GPP WGs.

# TP on Access control for TR

# 5.5.X	Access control

The NR system should support overload and access control functionality such as RACH backoff, RRC Connection Reject, RRC Connection Release and UE based access barring mechanisms.

One unified access barring mechanism for NR should be introduced to address all the use cases and scenarios that LTE addressed with different specialized mechanisms. The unified access barring mechanism should be forward compatible in order to cope with future use cases/scenarios.

In NR, the unified access barring mechanism should be applicable for all RRC states in NR (RRC_IDLE, RRC_CONNECTED and RRC_INACTIVE).

Editor’s note: It is FFS whether it will be possible for the mechanism to be completely common between the states

Editor’s note: It is FFS if it is possible to specify the unified access barring mechanism fully inside the 3GPP WGs.

# Conclusion

Adopt the text proposal in section 3.