3GPP TSG-RAN WG2 NR#2	              R2-1706581

Qingdao, China, 27 – 29 June2017

Title:	Draft LS to RAN1 on RRM Measurement Configuration in NR

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Mediatek (To be RAN2)

To:	RAN1

CC:                             RAN4

Contact Person:

Name:	               Yuanyuan Zhang

E-mail Address:  yuany.zhang@mediatek.com

1. Overall Description:

RAN2 has discussed RRM measurement configuration considering both NR-SS and CSI-RS can be used for RRM Measurement.

RAN2 agreed that in NR a measurement object is corresponding to one carrier frequency. As part of the Measurement Object it is possible to configure a list of CSI-RS resource specific configurations for RRM measurement. In LTE, the carrier center frequency and the SS center frequency is the same. So the ARFCN-ValueEUTRA is configured as the carrier frequency in the measurement object. While in NR, the carrier center frequency may be different from the SS center frequency. With respect to the measurement object, RAN2 would like to ask RAN1:

Q1: Whether the ARFCN-ValueNR represents the carrier center frequency or the SS center frequency?

RAN2 agreed that for NR-SS based RRM measurement, SS burst set periodicity can be configured, which can be frequency-specific. NR-SS configuration for CONNECTED UE can be provided by dedicated signaling. FFS whether NR-SS configuration acquired in SI can be reused in CONNECTED. With respect to measurement configuration for NR-SS, RAN2 would like to ask RAN1:

Q2: What other SS related information besides SS burst periodicity need to be configured for a carrier frequency and which parameters related to NR-SS can be cell-specific?

RAN2 agreed that for CSI-RS based RRM measurement, CSI-RS measurement configuration is provided only via dedicated signaling. An ID e.g. CSI-RS identifier is needed for each CSI-RS resource configured for RRM measurement. With respect to measurement configuration for CSI-RS, RAN2 would like to ask RAN1:

Q3: What parameters can be configured for CSI-RS based RRM measurement?

Q4: Whether can UE get the CSI-RS identifier implicitly but not explicitly informed via RRC signaling?

2. Actions:

To RAN1 group.

ACTION: 	RAN2 respectfully asks RAN1 to provide answers to the questions listed above.

3. Dates of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #99 	21 - 25 Aug 2017 		 Berlin, Germany

TSG-RAN WG2 Meeting #99bis	9 - 13 Oct 2017 		 Prague, Czech