3GPP TSG-RAN WG2 #AH	Tdoc R2-1707176

Qingdao, P.R. of China, 26th – 30th June 2017	Update of R2-1704684

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Enhanced HARQ feedback mode in SPS

Document for:	Discussion, Decision

# Introduction

In RAN2#98, the following agreements were made in the chairman’s notes for SPS [1]:

In NR, when the UE is configured with SPS, the UE should always skip SPS grant if there is no data to transmit, i.e., skipping SPS grant is mandated in NR regardless of SPS periodicity.

In this paper, we discuss a reliability enhancement feedback mode for NR SPS. General HARQ aspects of NR SPS is provided in [2], some further SPS enhancements are in [3] and our views on grant-free are in [4].

# Discussion

For URLLC data both latency and reliability needs to be considered. Error cases that can be tolerated for eMBB operation can be severe for URLLC services. For example, RLC re-transmissions can reach reliability requirement, but the latency is too large for URLLC. Optimizations to resolve some error cases is therefore motivated.

SPS UL is highly relevant for URLLC service. The main reason is that very low UL latency can be enabled when the SR step is skipped. Supporting also reliable retransmissions and automatic repetitions may provide higher reliability on top of the low latency.

## Reliability-enhancement feedback mode

In the companion paper [2], we propose that if no retransmission UL-grant is received within the configurable time T, then UE considers the transmission successful and data buffer can be overwritten by a new transmission.

Note that it has been agreed that when the UE is configured with SPS, the UE should always skip UL transmission if there is no data. Hence, gNB does not expect a transmission in every resource specified by the SPS UL grant, and therefore the absence of a retransmission UL grant (i.e., no feedback) from the network can also imply that the transmission was not detected at all in gNB, or that UE missed the PDCCH. Such error cases might be acceptable for eMBB data, since it can be recovered by higher layers at the cost of increased latency. But it is undesirable for data with high reliability requirement, such as URLLC data.

One mitigation mechanism is to enforce the transmission of a PDDCH message that indicates ACK feedback for each UL data transmission. Consequently, if no such PDDCH is received within the configurable feedback time T, then UE considers that the transmission is not successful and UE should retransmit the packet. Note that this does not preclude retransmission based on the UL grant as proposed in paper [2].

This mode of feedback operation can be configured by the network. As one example, the UL grant with NDI not-toggled can be used to indicate ACK on PDCCH.

- The retransmission of the packet can be on the SPS UL resources. Since the frequency resources, MCS etc. are already configured the retransmission is done according to the SPS grant and is thereby non-adaptive. Moreover, since the initial transmission might be missed by the gNB we should keep the same redundant version for the retransmission. This is aligned with SPS UL operation in LTE.

A reliability-enhanced feedback mode is configurable for SPS UL

If the higher reliability is configured for the UE:

If a PDDCH indicating ACK is received within the time T, then UE considers the transmission successful;  if no PDDCH indicate ACK or no UL grant for retransmisison is received within the time T, UE retransmits the packet on a SPS resource with the same RV as the initial transmission.

This mode of feedback for SPS UL may lead to increased PDCCH load. For instance, ACK feedback would need to be sent for most of the UL transmissions since the BLER is low. However, this might be needed for packets that requires a very high reliability. In addition, as agreed in RAN2#98, “skipping SPS grant is mandated in NR regardless of SPS periodicity”. Thus, the total impact may be low if the traffic rate with this service requirement is lower in general.

## HARQ for SPS with repetition

- During the study item phase of NR, it has been agreed in RAN1 that for an UL transmission scheme with/without grant, K repetitions including initial transmission (K>=1) for the same transport block are supported. The main use case is data with ultra-low latency and high-reliabilty requirement. It can also potentially be used for coverage enhancement where the number of K is large.

- The feedback mechanisms discussed above for SPS UL should be possible to configure also with repetition. Also the maximum feedback time T should function in the same way.

- Observation 1	A reliability-enhanced feedback mode including a maximum feedback time, should be configurable for SPS UL together with repetions.

- Regarding the number of PDCCH messages to send as feedback, either the network sends one per repetition copy, or one for the entire sequence. In either case the HARQ process and the resource should be clear for the UE. This should be up to the network to decide.

- Observation 2	The network can send feedback for every repetition or per sequence.

# Conclusion

In section 2 we made the following observations and proposals:

- A reliability-enhanced feedback mode is configurable for SPS UL

If the higher reliability is configured for the UE:

- Proposal 2	If a PDDCH indicating ACK is received within the time T, then UE considers the transmission successful;  if no PDDCH indicate ACK or no UL grant for retransmisison is received within the time T, UE retransmits the packet on a SPS resource with the same RV as the initial transmission.

- Observation 1	A reliability-enhanced feedback mode including a maximum feedback time, should be configurable for SPS UL together with repetions.

- Observation 2	The network can send feedback for every repetition or per sequence.

# References

RAN2-98, Hangzhou Chairman notes

R2-1707175, General HARQ aspects of SPS UL

R2-1707158, SPS enhancements in NR

R2-1707174, Grant Free and Semi-Persistent Scheduling in NR