3GPP TSG-RAN WG2 Ad Hoc on NR	Tdoc R2-1707277

Qingdao, China, June 27-29, 2017

Agenda Item:	10.2.8

Source:	Ericsson

Title:	Contents of Handover Request

Document for:	Discussion, Decision

# Introduction

On high level, the NR mobility procedure between two cells follows the LTE procedure to a large extent. The agreed procedure is captured in 38.300 in Figures 9.2.3-1 and 9.2.3.2.1-1.

Figure 9.2.3-1: Inter-gNB handover procedures.

Figure 9.2.3.2.1-1: Intra-AMF/UPF Handover

In general the mobility procedure is a joint responsibility between RAN2, RAN3 and SA2. However, the focus of this contribution is the information needed by the Target gNB to generate the transparent container forwarded to the UE, which is a pure RAN2 aspect, i.e. the third step (“3. Handover Request”) of the handover procedure).

# Discussion

First, we note that Figure 9.2.3-1 contains slightly confusing terminology by using Handover Command and Handover Complete as names of the RRC procedures for handover. It would be better to align this with the actual RRC messages, “RRCConnectionReconfiguration with MobilityControlInformation” and “RRCConnectionReconfiguration Complete”, which are more correct and more commonly used in 36.300 (even though one has to admit this is not fully consistently handled in 36.300).

Use “RRCConnectionReconfiguration with MobilityControlInformation” and “RRCConnectionReconfiguration Complete” as the names of the RRC messages in Figure 9.2.3-1 in 38.300.

The current 38.300 does not currently provide much details of the contents of the Handover Request (“passing necessary information to prepare the HO at the target side”), but based on the baseline LTE procedure, it is probably safe to assume that the information contained in current X2 HANDOVER REQUEST (Appendix A) and HandoverPreparationInformation (Appendix B) are a good starting point.

From RAN2 point of view, the main main information elements are contained in the HandOverPreparationInformation IE, and listed in the table below, together with some of the X2 HANDOVER REQUEST information elements (note that some of the RRC and X2 IEs related to e.g. D2D and MBMS have been omitted).

Table 1: Selected information elements from HandOverPreparationInformation and X2 HANDOVER REQUEST messages

| Information element | Contents | RRC/X2 | Comment |
| --- | --- | --- | --- |
| UE-CapabilityRAT |  | RRC | List of UE capabilities for different RATs. Useful for NR. |
| AS-Config | Measurement Config Radio Resource Config Source UE Identity Source MIB Source SIB1 Source SIB2 Antenna Info DL Carrier Frequency | RRC | Useful for NR. Useful for NR. C-RNTI. Useful for NR. Useful for NR. Useful for NR. Useful for NR. Useful for NR. Useful for NR. |
| RRM-Config | UE Inactive Time Candidate Cell List | RRC | Useful for NR. Useful for NR. |
| AS-Context | Re-establishment Info In-device coexistence Info Power Preference Indication | RRC | Further discussion on the NR re-establishment procedure needed. Unclear if needed. Unclear if needed. |
| ue-ConfigRelease |  | RRC | Indicates the version of the RRC used. Useful for NR. |
| ue-SupportedEARFCN |  | RRC | Used to indicate target cell frequency with it belongs to multiple bands. Probably useful for NR. |
| makeBeforeBreakReq |  | RRC | Unclear if needed |
| Target Cell ID | ECGI | X2 | Globally unique cell ID RAN3 decision. |
| UE security capabilities |  | X2 | RAN3 decision. |
| AS security info |  | X2 | RAN3 decision. |
| List of E-RABs |  | X2 | RAN3 decision. |
| UE history information |  | X2 | List of visited cells RAN3 decision. |

Based on the analysis in Table 1, we propose

NR HandOverPreparationInformation contains UE Capabilities for different RATs, AS configuration (including Measurement and Radio Resource configurations, Source cell C-RNTI, Source MIB and NR-SIB1, Antenna Info and DL Carrier Frequency), RRM Configuration, UE release indication and UE supported EARFCN

# Conclusion

Based on the analysis in section 2, we propose the following:

- Use “RRCConnectionReconfiguration with MobilityControlInformation” and “RRCConnectionReconfiguration Complete” as the names of the RRC messages in Figure 9.2.3-1 in 38.300.

- NR HandOverPreparationInformation contains UE Capabilities for different RATs, AS configuration (including Measurement and Radio Resource configurations, Source cell C-RNTI, Source MIB and NR-SIB1, Antenna Info and DL Carrier Frequency), RRM Configuration, UE release indication and UE supported EARFCN

# Appendix A: X2 HANDOVER REQUEST

This message is sent by the source eNB to the target eNB to request the preparation of resources for a handover.

Direction: source eNB  target eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| Old eNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the source eNB | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| Target Cell ID | M |  | ECGI 9.2.14 |  | YES | reject |
| GUMMEI | M |  | 9.2.16 |  | YES | reject |
| UE Context Information |  | 1 |  |  | YES | reject |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – | – |
| >UE Security Capabilities | M |  | 9.2.29 |  | – | – |
| >AS Security Information | M |  | 9.2.30 |  | – | – |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – | – |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – | – |
| >E-RABs To Be Setup List |  | 1 |  |  | – | – |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – | – |
| >>>DL Forwarding | O |  | 9.2.5 |  | – | – |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >>>Bearer Type | O |  | 9.2.92 |  | YES | reject |
| >RRC Context | M |  | OCTET STRING | Includes the RRC Handover Preparation Information message as defined in subclause 10.2.2 of TS 36.331 [9] | – | – |
| >Handover Restriction List | O |  | 9.2.3 |  | – | – |
| >Location Reporting Information | O |  | 9.2.21 | Includes the necessary parameters for location reporting | – | – |
| >Management Based MDT Allowed | O |  | 9.2.59 |  | YES | ignore |
| >Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.64 |  | YES | ignore |
| >UE Sidelink Aggregate Maximum Bit Rate | O |  | 9.2.97 | This IE applies only if the UE is authorized for V2X services. | YES | Ignore |
| UE History Information | M |  | 9.2.38 | Same definition as in TS 36.413 [4] | YES | ignore |
| Trace Activation | O |  | 9.2.2 |  | YES | ignore |
| SRVCC Operation Possible | O |  | 9.2.33 |  | YES | ignore |
| CSG Membership Status | O |  | 9.2.52 |  | YES | reject |
| Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to the handover; the source eNB provides it in order to enable later analysis of the conditions that led to a wrong HO. | YES | ignore |
| Masked IMEISV | O |  | 9.2.69 |  | YES | ignore |
| UE History Information from the UE | O |  | OCTET STRING | VisitedCellInfoList contained in the UEInformationResponse message (TS 36.331 [9]) | YES | ignore |
| Expected UE Behaviour | O |  | 9.2.70 |  | YES | ignore |
| ProSe Authorized | O |  | 9.2.78 |  | YES | ignore |
| UE Context Reference at the SeNB | O |  |  |  | YES | ignore |
| >Global SeNB ID | M |  | Global eNB ID 9.2.22 |  |  |  |
| >SeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the SeNB |  |  |
| >SeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the SeNB |  |  |
| Old eNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the source eNB | YES | reject |
| V2X Services Authorized | O |  | 9.2.93 |  | YES | ignore |
| UE Context Reference at the WT | O |  |  |  | YES | ignore |
| >WT ID | M |  | 9.2.95 |  |  |  |
| >WT UE XwAP ID | M |  | 9.2.96 |  |  |  |

# Appendix B: LTE HandoverPreparationInformation

This message is used to transfer the E-UTRA RRC information used by the target eNB during handover preparation, including UE capability information.

Direction: source eNB/ source RAN to target eNB

HandoverPreparationInformation message

-- ASN1START

HandoverPreparationInformation ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverPreparationInformation-r8	HandoverPreparationInformation-r8-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverPreparationInformation-r8-IEs ::= SEQUENCE {

ue-RadioAccessCapabilityInfo		UE-CapabilityRAT-ContainerList,

as-Config							AS-Config					OPTIONAL, 		-- Cond HO

rrm-Config							RRM-Config					OPTIONAL,

as-Context							AS-Context				OPTIONAL, 		-- Cond HO

nonCriticalExtension				HandoverPreparationInformation-v920-IEs		OPTIONAL

}

HandoverPreparationInformation-v920-IEs	::= SEQUENCE {

ue-ConfigRelease-r9					ENUMERATED {

rel9, rel10, rel11, rel12, v10j0, v11e0,

v1280, rel13, ...} 			OPTIONAL, 	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v9d0-IEs		OPTIONAL

}

HandoverPreparationInformation-v9d0-IEs	::= SEQUENCE {

lateNonCriticalExtension			OCTET STRING (CONTAINING HandoverPreparationInformation-v9j0-IEs)	OPTIONAL,

nonCriticalExtension				HandoverPreparationInformation-v9e0-IEs			OPTIONAL

}

-- Late non-critical extensions:

HandoverPreparationInformation-v9j0-IEs ::= SEQUENCE {

-- Following field is only for pre REL-10 late non-critical extensions

lateNonCriticalExtension			OCTET STRING					OPTIONAL,

nonCriticalExtension				HandoverPreparationInformation-v10j0-IEs		OPTIONAL

}

HandoverPreparationInformation-v10j0-IEs ::= SEQUENCE {

as-Config-v10j0						AS-Config-v10j0			OPTIONAL,

-- Following field is only for late non-critical extensions from REL-10

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

-- Regular non-critical extensions:

HandoverPreparationInformation-v9e0-IEs	::= SEQUENCE {

as-Config-v9e0						AS-Config-v9e0					OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v1130-IEs		OPTIONAL

}

HandoverPreparationInformation-v1130-IEs	::= SEQUENCE {

as-Context-v1130					AS-Context-v1130				OPTIONAL,	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v1250-IEs						OPTIONAL

}

HandoverPreparationInformation-v1250-IEs ::= SEQUENCE {

ue-SupportedEARFCN-r12 				ARFCN-ValueEUTRA-r9				OPTIONAL,	-- Cond HO3

as-Config-v1250					AS-Config-v1250				OPTIONAL, 	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v1320-IEs						OPTIONAL

}

HandoverPreparationInformation-v1320-IEs ::= SEQUENCE {

as-Config-v1320						AS-Config-v1320					OPTIONAL, 	-- Cond HO2

as-Context-v1320					AS-Context-v1320				OPTIONAL, 	-- Cond HO2

nonCriticalExtension				HandoverPreparationInformation-v14x0-IEs						OPTIONAL

}

HandoverPreparationInformation-v14x0-IEs ::= SEQUENCE {

as-Config-v14x0					AS-Config-v14x0						OPTIONAL, 	-- Cond HO2

makeBeforeBreakReq-r14			ENUMERATED {true}				OPTIONAL, 	-- Cond HO2

nonCriticalExtension			SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Config The radio resource configuration. Applicable in case of intra-E-UTRA handover. If the target receives an incomplete MeasConfig and RadioResourceConfigDedicated in the as-Config, the target eNB may decide to apply the full configuration option based on the ue-ConfigRelease.

as-Context Local E-UTRAN context required by the target eNB.

makeBeforeBreakReq To request the target eNB to add the makeBeforeBreak indication in the mobilityControlInfo.

rrm-Config Local E-UTRAN context used depending on the target node’s implementation, which is mainly used for the RRM purpose.

ue-ConfigRelease Indicates the RRC protocol release or version applicable for the current UE configuration. This could be used by target eNB to decide if the full configuration approach should be used. If this field is not present, the target assumes that the current UE configuration is based on the release 8 version of RRC protocol. NOTE 1.

ue-RadioAccessCapabilityInfo For E-UTRA radio access capabilities, it is up to E-UTRA how the backward compatibility among supportedBandCombinationReduced, supportedBandCombination and supportedBandCombinationAdd is ensured. If supportedBandCombinationReduced and supportedBandCombination/supportedBandCombinationAdd are included into ueCapabilityRAT-Container, it can be assumed that the value of fields, requestedBands, reducedIntNonContCombRequested and requestedCCsXL are consistend with all supported band combination fields. NOTE 2

ue-SupportedEARFCN Includes UE supported EARFCN of the handover target E-UTRA cell if the target E-UTRA cell belongs to multiple frequency bands.