3GPP TSG-RAN WG2 #AH NR 1801	Tdoc R2-1800329

Vancouver, Canada, Jan 22nd – 26th, 2018

Title:	[DRAFT] LS on Activation and Deactivation time of Secondary Cells

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson (to be RAN2)

To:	RAN4, RAN1

Contact Person:

#### Name:	Mats Folke

Tel. Number:	+46761271385

E-mail 	Address:mats.folke@ericsson.com

1. Overall Description:

RAN2 has discussed the time to activate and deactivate the secondary cells. RAN2 concluded that the gNB can indicate the activation state (deactivated or activated) of an SCell at its configuration.

From a RAN2 perspective it is beneficial to minimize the time needed for activation, as this improves the throughput. Given the general improvements in processing time for NR compared to LTE, RAN2 assumes it should be possible to decrease the time needed to activate an SCell in NR.

From a RAN2 perspective it is beneficial to minimize the time needed for deactivation, as this improves the energy consumption in the UE and also enables the network to more quickly activate the SCell again, should the need arise. Given the general improvements in processing time for NR compared to LTE, RAN2 assumes it should be possible to decrease the time needed to deactivate an SCell in NR.

RAN2 therefore asks RAN4 to study the delay needed to activate and deactivate an SCell and take the improvements in processing time for NR into account in trying to minimize these delays.

Upon discussing the delays used for activating and deactivating secondary cells where TS 38.321 refer to TS 38.213 it was discovered that in sub-clause 4.3 TS 38.213 refer back to the RRC specification TS 38.331 for the definition of the delays.

RAN2 believe that this is an erroneous reference and that the correct reference should be TS 38.133. RAN2 does not intend to include any value for the delay in TS 38.331.

RAN2 therefore asks RAN1 to investigate and if needed, update the reference to the correct specification.

2. Actions:

To RAN4 group.

ACTION: 	RAN2 asks RAN4 to study the delay needed to activate and deactivate an SCell and take the general improvements in processing time for NR into account in trying to minimize these delays.

To RAN1 group.

ACTION:	RAN2 asks RAN1 to investigate and if needed, update the reference to the correct specification.

3. Date of Next RAN2 Meetings:

RAN2#101	26th February – 2nd March 2018	Athens, Greece

RAN2#101-Bis	16th – 20th April 2018	China