3GPP TSG-RAN WG2 #NR AH1801 	Tdoc R2-1800433

Vancouver, Canada, Jan 22nd – 26th, 2018 	(Revision of R2-173303)

Agenda Item:	10.4.1.7.4

Source:	Ericsson

Title:	Text proposal to 38.331 on RRC states

Document for:	Discussion, Decision

# Introduction

This contribution is a revision of Tdoc R2-173303.

At the RAN2#98 meeting a text proposal to 38.331 section 4.2 was agreed in R2-1706050.

At the last RAN plenary, it was agreed in RP-171432 to add the following to the “LTE connecting to 5GC” WI:

-	RRC_INACTIVE state for E-UTRA when connected to 5G-CN, with similar functionality as the RRC_INACTIVE state in NR;  CN aspects of the RRC-INACTIVE state are covered in 5GS-Ph1 (SP-160958) and are expected to be  the same for both NR and LTE from a CN standpoint

With this agreement, it is natural to allow mobility in RRC_INACTIVE between LTE and NR connected to 5GC when supported in the UE and network. This contribution proposes to add this state transition to RRC 38.331.

# Text proposal to 38.331

# 4	General

## 4.1	Introduction

## 4.2	Architecture

### 4.2.1	UE states and state transitions including inter RAT

A UE is either in RRC_CONNECTED state or in RRC_INACTIVE state when an RRC connection has been established. If this is not the case, i.e. no RRC connection is established, the UE is in RRC_IDLE state. The RRC states can further be characterised as follows:

-	RRC_IDLE:

-	A UE specific DRX may be configured by upper layers;

-	UE controlled mobility based on network configuration;

-	The UE:

-	Monitors a Paging channel;

-	Performs neighbouring cell measurements and cell (re-)selection;

-	Acquires system information.

-	RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer;

-	UE controlled mobility based on network configuration;

- 	The UE stores the AS context;

-	The UE:

-	Monitors a Paging channel;

-	Performs neighbouring cell measurements and cell (re-)selection;

- 	Performs RAN-based notification area updates when moving outside the RAN-based notification area;

FFS Whether a RAN-based notification area is always configured or not.

FFS UE behavior if it is decided that a RAN-based notification area is not always configured.

-	Acquires system information.

-	RRC_CONNECTED:

-	The UE stores the AS context.

-	Transfer of unicast data to/from UE.

-	At lower layers, the UE may be configured with a UE specific DRX.;

-	For UEs supporting CA, use of one or more SCells, aggregated with the PCell, for increased bandwidth;

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth;

-	Network controlled mobility, i.e. handover within NR and to/from E-UTRAN.;

-	The UE:

-	Monitors a Paging channel;

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it;

-	Provides channel quality and feedback information;

-	Performs neighbouring cell measurements and measurement reporting;

-	Acquires system information.

Figure 4.2.1-1 illustrates an overview of UE RRC state machine and state transitions in NR. A UE has only one RRC state in NR at one time.

Figure 4.2.1-1:	UE state machine and state transitions in NR

Figure 4.2.1-2 illustrates an overview of UE state machine and state transitions in NR as well as the mobility procedures supported between NR/5GC and E-UTRAN/EPC, and between NR/5GC and LTE/5GC.

Figure 4.2.1-2:	UE state machine and state transitions between NR/5GC and E-UTRAN/EPC, and between NR/5GC and LTE/5GC