3GPP TSG-RAN WG2 NR AH#1801	Tdoc R2-1800698

Vancouver, Canada, 22nd – 26th January 2018

Agenda Item:	10.3.4.2

Source:	Ericsson

Title:	SDAP Header Format

Document for:	Discussion, Decision

# Introduction

RAN2#100 meeting agreed following:

Agreements

=>	SDAP header remains fixed to 8 bits.   The details are FFS.

LS response from SA2[1] regarding to SDAP header format, submitted for RAN2#100:

S2-178056:

Size of the QFI field: SA2 has no strong preference regarding the exact size of the QFI field at the AS. However, SA2 would like to note that for non-GBR QoS flows, the 5QI value can be used as QFI of the QoS flow in case standardized or pre-configured 5QIs are used, as specified in TS 23.501. Currently, the standardized 5QI ranges from 1 to 79 in TS 23.501, hence suggesting at least a 7-bit QFI will be needed.

RQI bits: According to TS 23.501, (NAS) Reflective QoS is controlled on a per-packet basis i.e. for each packet subject to (NAS) Reflective QoS, a (NAS) Reflective QoS Indication shall be provided first on N3 by the UPF to the RAN and then on the radio interface by the RAN to the UE, unaltered vs. that on N3. Furthermore, the RAN shall not send a packet with a (NAS) Reflective QoS Indication to the UE on the radio interface unless the corresponding (NAS) Reflective QoS Indication was received from the UPF by RAN for this packet.

This contribution discusses about SDAP header format options taking into account the previous agreements and input from SA2. The topic of AS reflective QoS is discussed in [4]

# Discussion

QFI is always attached to N3 header when there are multiple flows in PDU session. In order to allow efficient implementation from gNB towards 5CN, SDAP header format should be aligned with the N3 (NG-U) header format. Having QFI aligned between SA2 and RAN allows implementations to copy the QFI to SDAP header and vice versa.

SDAP header format is aligned with SA2 to allow QFI and RQI copying behaviour from and to N3 header.

## Header format

RAN2-99 agreed to have SDAP header design where header length is 1-byte. RAN2#100 confirmed this and stated that the details of the format are FFS. It was also discussed whether 6-bit QFI field is sufficient for the system.

When considering 6-bit QFI values, the 5QI value range becomes a problem. Currently, the maximum defined value for standardized QFI in 5QI table is 79 [2]. Having value range that is able to indicate full value range of 5QI (from 0 to 79) is beneficial as it results that the standardized 5QI values can be directly signalled to the gNB and UE without need for intermediate mapping from 5QI value to RAN QFI value. One advantage of this behaviour is that the UE is able to immediately know the 5QI value from QFI when standardized QFI is used.

7-bit QFI allows direct use of standardized QFI value in UE

SA2 states in the LS reply that the QFI should be at least 7-bits and the RQI bit should be delivered to the UE unaltered. Clean header design that full-fills the SA2 requirements is possible to achieve with the header format that includes 7-bit QFI + 1-bit RQI for DL and 7-bit QFI + 1-bit R-field for UL. As the QFI information is only carried for DL, UL header has one spare bit (R) for later use. The figure 1 illustrates the format.

Figure 1: SDAP 1-byte header format options

Confirm that the agreed one-byte DL and UL SDAP header contains a 7-bit QFI field

There is only one QFI format and it is 7-bits

Confirm that the agreed one-byte DL header includes RQI bit, which is carried to the UE unaltered

The UL SDAP header has one spare bit (R) for later use.

Depending on the outcome of discussion [2] there may be a need to be able to indicate AS reflective QoS information. We think that simple, yet sufficient, solution is to bind the NAS and AS reflective mechanism and only use AS reflective QoS with NAS reflective QoS.

If AS reflective QFI to DRB mapping remains, AS reflective QoS is used only when NAS reflective QoS is updated on the same packet. RRC signaling is used in all other cases.

# Conclusion

In section 2 we made the following observations:

Observation 1	7-bit QFI allows direct use of standardized QFI value in UE

Based on the discussion in section 2 we propose the following:

Proposal 1	SDAP header format is aligned with SA2 to allow QFI and RQI copying behaviour from and to N3 header.

Proposal 2	Confirm that the agreed one-byte DL and UL SDAP header contains a 7-bit QFI field

Proposal 3	There is only one QFI format and it is 7-bits

Proposal 4	Confirm that the agreed one-byte DL header includes RQI bit, which is carried to the UE unaltered

Proposal 5	The UL SDAP header has one spare bit (R) for later use.

Proposal 6	If AS reflective QFI to DRB mapping remains, AS reflective QoS is used only when NAS reflective QoS is updated on the same packet. RRC signaling is used in all other cases.

# References

3GPP TS 23.501. System Architecture for the 5G System. V1.5.0 (2017-11)

R2-1712919 - Analysis of the QoS framework (Stage 2). Ericsson.  Vancouver, Canada, 22nd – 26th January 2018

S2-178056. Reply LS on SDAP header design. SA WG2. Oct 23 – 27, 2017, Ljubljana, Slovenia

R2-1800694. Analysis of the QoS Framework (Stage 2). Ericsson.  Vancouver, Canada, 22nd – 26th January 2018

R2-1713583 Considerations on Reflective QoS Qualcomm Incorporated. Reno, USA, 27th November – 1st December 2017