3GPP TSG-RAN WG2 NR Ad hoc 1801		R2-1800896

Vancouver, Canada, 22nd-26th January 2018

Source:	vivo

Title:	RAR reception for SRS only carrier in NR

Agenda Item:	10.3.1.4.3

Document for:	Discussion and Decision

- Introduction

In RAN2#100, an LS [1] was received for introducing the type 3 PHR.  As we known that introduction of type3 PHR is for SRS only carrier. In this contribution, we provide impact analysis for RAR reception for SRS only carrier in NR.

- Discussion

  - Ignore the UL grant in RAR

In NR, there is UL grant in RAR, only except for message 1 based SI request. From our understanding. The UE has to ignore the UL grant in RAR for SRS only carrier, because there is no PUSCH on SRS only carrier at all. It is captured in MAC specification for LTE.

TS 36.321[2] - if the SCell is configured with ul-Configuration-r14, ignore the received UL grant otherwise process the received UL grant value and indicate it to the lower layers;

For NR we should use the same metod.

Proposal1: Ignore the UL grant in RAR if the Scell  is configured as SRS only carrier Scell for NR, same as LTE.

  - Text proposal

5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

………..

4>	apply the following actions for the Serving Cell where the Random Access Preamble was transmitted:

5>	process the received Timing Advance Command (see subclause 5.2);

5>	indicate the ra-PreambleInitialReceivedTargetPower and the amount of power ramping applied to the latest preamble transmission to lower layers (i.e. (PREAMBLE_POWER_RAMPING_COUNTER – 1) * powerRampingStep);

5>	if the SCell is configured as the SRS only carrier Scell, ignore the received UL grant otherwise process the received UL grant value and indicate it to the lower layers;

- Conclusion

In this contribution, we provide impact analysis for RAR reception for SRS only carrier in NR and give our text proposal.

Proposal1: Ignore the UL grant in RAR if the Scell  is configured as SRS only carrier Scell for NR, same as LTE.

- References

- R2-1714189	 LS on SRS PHR reporting (R1-1721680; contact: Huawei)

- TS 36.321, “Medium Access Control (MAC) protocol specification”

- TS 38.321, “Medium Access Control (MAC) protocol specification”