3GPP TSG-RAN WG2 NR Ad-Hoc meeting	R2-1800951

# Vancouver, Canada, 22nd - 26th January 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0002 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on UE capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-01-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Update UE capabilities |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update max data rate formula based on RAN1 LS (R2-1800012) - Add clarification for the case when the UE supports multiple SCSs in 4.1.2 - Correct reference for channel BW in 4.1.2 - Update parameters with the table format - Add FDD-TDD difference, mandatory and information level into the table - Remove clause 7 mandatory features - Remove obvious and/or repeated editor’s note - Add some references in 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incorrect, incomplete and less readable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.1.2, 4.4, 4.5, 4.6, 4.7, 4.8, 4.9, 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

The First Change

# Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	UE radio access capability parameters	6

4.1	Supported max data rate 	6

4.1.1	General 	6

4.1.2	Max data rate without ue-CategoryDL and ue-CategoryUL	6

4.1.3	Max data rate with ue-CategoryDL and ue-CategoryUL	6

4.1.4	Total layer 2 buffer size	6

4.2	General parameters	7

4.3	SDAP Parameters	7

4.4	PDCP Parameters	7

4.5	RLC parameters	8

4.6	MAC parameters	9

4.7	Physical layer parameters	10

4.8	RF parameters	10

4.9	Measurement parameters	11

4.10	Inter-RAT parameters	12

4.10.1	eutraFDD	12

4.10.2	eutraTDD	12

5	Optional features without UE radio access capability parameters	12

6	Conditional mandatory features	12

7.	Capability coordination in MR-DC operation	13

Annex <A> (normative): <Normative annex title>	14

Annex <B> (informative): <Informative annex title>	14

Annex <X> (informative): Change history	15

The Next Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS38.101-1: “NR User Equipment (UE) radio transmission and reception Part 1: Range 1 Standalone”.

[3]	3GPP TS38.101-2: “NR User Equipment (UE) radio transmission and reception Part 2: Range 2 Standalone”.

[4]	3GPP TS 38.321: “NR Medium Access Control (MAC) protocol specificiation”.

The Next Change

### 4.1.2	Max data rate without ue-CategoryDL and ue-CategoryUL

The approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows.

wherein

J is the number of aggregated component carriers in a band or band combination

Rmax = 948/1024

For the j-th CC,

is the maximum number of layers

is the maximum modulation order

is the scaling factor

The scaling factor can at least take the values 1 and 0.75.

is signalled per band and per band per band combination

  - is the numerology (as defined in TS38.211)

  - is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed.

  - is the maximum RB allocation in bandwidth  with numerology , as defined in 5.3 TS38.101-1 [2] and 5.3 TS38.101-2 [3], where  is the UE supported maximum bandwidth in the given band or band combination

is the overhead and takes the following values

[0.14], for frequency range FR1 for DL

[0.18], for frequency range FR2 for DL

[0.08], for frequency range FR1 for UL

[0.10], for frequency range FR2 for UL

Note: Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL

The approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported band or band combinations. If the UE supports multiple numerologies, the approximate maximum data rate can be computed as the maximum of the approximate data rates computed using the above formula for each of the supported numerologies.

The Next Change

### 4.4	PDCP Parameters

| Definitions for parameters | Per | M | FDD-TDD diff |
| --- | --- | --- | --- |
| continueROHC-Context This field defines whether the UE supports ROHC context continuation operation where the UE does not reset the current ROHC context upon handover. | UE | TBD | TBD |
| dataRateDRB-IP This field defines the upper bound of the aggregated data rate of user plane integrity protected data in either UL or DL DRBs. Value 64kbps corresponds to the aggregated data rate of user plane integrity protected data in either UL or DL cannot exceed 64kbps and so on. It is not supported in this version and is targeted for completion in June 2018. | UE | TBD | No |
| maxNumberROHC-ContextSessions This field defines the maximum number of header compression context sessions supported by the UE, excluding context sessions that leave all headers uncompressed. | UE | TBD | No |
| outOfOrderDelivery This field indicates whether UE supports Out of order delivery of data to upper layers by PDCP. | UE | TBD | No |
| shortSN This field indicates whether the UE supports 12 bit length of PDCP sequence number. | UE | TBD | No |
| supportedROHC-Profiles This field defines which ROHC profiles from the list below are supported by the UE: - 0x0000 ROHC No compression (RFC 5795) - 0x0001 ROHC RTP/UDP/IP (RFC 3095, RFC 4815) - 0x0002 ROHC UDP/IP (RFC 3095, RFC 4815) - 0x0003 ROHC ESP/IP (RFC 3095, RFC 4815) - 0x0004 ROHC IP (RFC 3843, RFC 4815) - 0x0006 ROHC TCP/IP (RFC 6846) - 0x0101 ROHC RTP/UDP/IP (RFC 5225) - 0x0102 ROHC UDP/IP (RFC 5225) - 0x0103 ROHC ESP/IP (RFC 5225) - 0x0104 ROHC IP (RFC 5225) A UE that supports one or more of the listed ROHC profiles shall support ROHC profile 0x0000 ROHC uncompressed (RFC 5795). | UE | TBD | No |
| uplinkOnlyROHC-Profiles This field defines which ROHC profile(s) from the list below are supported in uplink-only ROHC operation by the UE. - 0x0006 ROHC TCP (RFC [6846]) A UE that supports uplink-only ROHC profile(s) shall support ROHC profile 0x0000 ROHC uncompressed (RFC 5795). | UE | TBD | No |
| volteOverNR-PDCP This field indicates whether the UE supports VoLTE over NR-PDCP. | UE | TBD | TBD |

### 4.5	RLC parameters

| Definitions for parameters | Per | M | FDD-TDD diff |
| --- | --- | --- | --- |
| amWithShortSN This field indicates whether the UE supports RLC AM with 12 bit length of RLC sequence number. | UE | TBD | No |
| umWIthLongSN This field indicates whether the UE supports RLC UM with 12 bit length of RLC sequence number. | UE | TBD | No |
| umWithShortSN This field indicates whether the UE supports RLC UM with 6 bit length of RLC sequence number. | UE | TBD | No |

### 4.6	MAC parameters

| Definitions for parameters | Per | M | FDD-TDD diff |
| --- | --- | --- | --- |
| lcp-Restriction This field indicates whether UE supports the selection of logical channels for each UL grant based on RRC configured restriction. | UE | TBD | No |
| logicalChannelSR-DelayTimer This field indicates whether the UE supports the logicalChannelSR-DelayTimer as specified in TS 38.321 [4] | UE | TBD | No |
| longDRX-Cycle This field indicates whether UE supports long DRX cycle as specified in TS 38.321 [4]. | UE | TBD | TBD |
| numberOfConfiguredGrantConfigurations This field indicates the maximum number of grant configurations supported per cell group. | UE | TBD | No |
| numberOfSR-Configurations This field indicates the maximum number of SR configurations supported per cell group. | UE | TBD | No |
| shortDRX-Cycle This field indicates whether UE supports short DRX cycle as specified in TS 38.321 [4]. | UE | TBD | TBD |
| skipUplinkTxDynamic This field indicates whether the UE supports skipping of UL transmission for an uplink grant indicated on PDCCH if no data is available for transmission as specified in TS 38.321 [4]. | UE | TBD | No |

### 4.7	Physical layer parameters

| Definitions for parameters | Per | M | FDD-TDD diff |
| --- | --- | --- | --- |
| modulationOrder This field defines the supported modulation scheme by the UE. | TBD | No | No |
| subCarrierSpacing This field defines the supported sub-carrier spacing by the UE. | TBD | No | No |
| supportedBasebandProcessingCombination This field defines the supported CA baseband processing combinations by the UE. Baseband combinations associated to supportedBandCombination are only included. A baseband combination is applicable to the band combination which consist of bands with equal ca-BandwidthClassDL, ca-BandwidthClassUL, and equal or higher supportedMIMO-CapabilityDL and supportedMIMO-Capability. | UE | No | No |
| supportedBasebandProcessingCombination-MRDC This field defines the supported MN and SN baseband processing combinations for MR-DC by the UE. How to map the supported baseband combinations to band combinations for MR-DC is FFS. | TBD | TBD | No |

### 4.8	RF parameters

| Definitions for parameters | Per | M | FDD-TDD diff |
| --- | --- | --- | --- |
| ca-BandwidthClassDL and ca-BandwidthClassUL This field defines for DL and UL, the class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE, as specified in TS36.101 [X]. | Band per BPC | No | No |
| intraBandAsyncFDD This field indicates whether the UE supports asynchronous FDD-FDD intra-band LTE-NR DC with MRTD and MTTD as specified in [X]. If it is not supported for FDD-FDD intra-band LTE-NR DC, the UE supports only synchronous FDD-FDD intra-band LTE-NR DC. | TBD | No | No |
| intraBandSimultaneousTxRx This field indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band LTE-NR DC operation. | TBD | No | No |
| multipleTimingAdvance This field indicates whether multiple timing advances are supported by the UE. For NR CA band combination, if the band combination comprised of more than one band entry (i.e., inter-band or intra-band non-contiguous band combination), the field indicates that different timing advances on different band entries are supported. For EN-DC band combination, this field is not presented and it is mandatory for the UE supporting EN-DC band combination. | BC | No | No |
| singleTx This field indicates that the UE does not supports simultaneous UL transmissions. The UE may only set this bit for a problematic band combination defined in TS38.101 [X]. If sets the bit it applies to all fallback BCs that are defined in TS38.101 [X] as problematic BCs. It does not apply to fallback BCs that are defined in TS38.101 [X] as not problematic BCs. | BC | No | No |
| supportedBandCombination This field defines the supported CA and/or MR-DC band combinations by the UE. For each band in a band combination the UE provides the supported CA bandwidth classes for DL and/or UL. | UE | No | No |
| supportedBandListNR This field includes the supported NR bands as defined in TS38.101 [X]. | UE | No | No |

### 4.9	Measurement parameters

| Definitions for parameters | Per | M | FDD-TDD diff |
| --- | --- | --- | --- |
| independentGapConfig This field indicates whether the UE supports two independent measurement gap configurations for FR1 and FR2 specified in [X]. | UE | TBD | TBD |
| intraCarrierConcurrentMeas This field indicates whether the UE supports concurrent intra-frequency measurement on serving cell or neighbouring cell and PDCCH or PDSCH reception from the serving cell with a different numerology. | UE | TBD | TBD |
| sstd-MeasType1 This field indicates whether the UE supports SSTD measurements between the PCell and a configured PSCell. | UE | TBD | TBD |

The Last Change

## 7	Capability coordination in MR-DC operation

In MR-DC operation, only two nodes (one EUTRA eNB and one NR gNB) need to be considered in the EUTRA/NR capability coordination. For capabilities for which coordination is needed, it is up to the MN to make the decision on how to resolve the dependency between MN and SN configurations. The MN provides the resulting UE capabilities usable for SCG configuration to the SN. The SN is allowed to initiate the re-negotiation of capability. For capabilities for which no coordination is needed, the SN specific capabilities are just forwarded by the MN to the SN.

Editor’s Note: Whether to include the list of capabilities which need the coordination or to refer the other specification needs to be discussed.