3GPP TSG-RAN WG2 AH1801 	R2-1801232

# Vancouver, Canada, 22th Jan – 26th Jan 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on SCell RLF handling in 38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-01-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To introduce SCell RLF handling in 38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new message, SCellRLFReport, as well as the corresponding procedures for handling SCellRLF reportings are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SCell RLF handling functionality that has been agreed will not be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.11.3, 5.7, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.11.3	Detection of radio link failure

The UE shall:

1>	upon T310 expiry, or

1>	upon random access problem indication from MCG MAC while T311 is not running, or

Editor’s Note: FFS: Under which condition physical layer problems detection is performed, e.g. neither T300, T301, T304 nor T311 is running. It’s subject to the harmonization of the RRC procedures for RRC Connection establishment/resume/ re-establishment and RRC connection reconfiguration.

1>	upon indication from MCG RLC that the maximum number of retransmissions has been reached:

Editor’s Note: FFS whether maximum ARQ retransmission is only criteria for RLC failure.

2>	consider radio link failure to be detected for the MCG i.e. RLF;

Editor’s Note: FFS Whether indications related to beam failure recovery may affect the declaration of RLF.

.

Editor’s Note: FFS: RLF related measurement reports e.g. VarRLF-Report is supported in NR.

2>	if AS security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in x.x.x, with release cause 'other';

2>	else:

3>	initiate the connection re-establishment procedure as specified in x.x.x.

The UE shall:

1>	upon T313 expiry, or

1>	upon random access problem indication from SCG MAC, or

- upon indication from SCG RLC that the maximum number of retransmissions has been reached:

2> if the RLC is associated with an SCell that is being used for CA level duplication;

3>	consider SCell radio link failure to be detected for the SCell i.e. SCell-RLF;

3> initiate the SCell RLF report procedure as specified in 5.7.N to report SCell failure;

2>	else:

3>	consider radio link failure to be detected for the SCG i.e. SCG-RLF;

3>	initiate the SCG failure information procedure as specified in 5.7.3.2 to report SCG radio link failure;

END OF CHANGES

START OF CHANGES

### 5.7.N	SCell RLF Report

#### 5.7.N.1	General

Figure 5.7.N.1-1: SCell RLF Report

The purpose of this procedure is to inform NR about an SCell failure the UE has experienced during CA level duplication, i.e., exceeding the maximum number of RLC retransmissions.

#### 5.7.N.2	Initiation

A UE initiates the procedure to report SCell RLF when SCell transmission is not suspended and when the following condition is met:

1> upon detecting that the maximum number of RLC retransmissions has been reached, in accordance with subclause 5.3.11.3, for an SCell that is being used for CA level duplication;

Upon initiating the procedure, the UE shall:

1>	deactivate the CA level duplication on the SCell where the RLF was detected;

1>	initiate transmission of the SCellRLFReport message as specified in 5.7.N.3;

#### 5.7.N.3	Actions related to transmission of SCellRLFReport message

The UE shall set the contents of the SCellRLFReport message as follows:

1>	if the UE initiates transmission of the SCellRLFReport message to provide SCell radio link failure information due to exceeding maximum of RLC retransmissions:

2> set the FailureReportSCell to include sCellIndex of the SCell where the RLF was detected;

The UE shall submit the SCellRLFReport message to lower layers for transmission.

END OF CHANGES

START OF CHANGES

### 6.2.2	Message definitions

#### –	SCellRLFReport

The SCellRLFReport message is used to provide information regarding failures detected by the UE for an SCell in CA level duplication.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to NR

SCellRLFReport message

-- ASN1START

SCellRLFReport-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scellRlfReport-r15 			SCellRLFReport-r15,

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCellRLFReport-r15 ::=	SEQUENCE {

failureReportSCell-r15				FailureReportSCell-r15

}

FailureReportSCell-r15 ::= 			SEQUENCE {

sCellIndex-r12						SCellIndex-r10

--FFS: If measurement report is to be included

}

-- ASN1STOP

END OF CHANGES