3GPP TSG-RAN WG2 Meeting NR AH1801		R2-1801303

Vancouver, Canada, 22nd – 26th January 2018                 	            Revision of R2-1708257

Agenda Item:	10.4.5.6

Source: 	MediaTek Inc.

Title:  	Overhead Reduction for Paging in Multi-beam Operation

Document for:	Discussion and decision

# Introduction

In RAN1 #87 meeting, the following agreements regarding paging in NR were made.

For paging in multi-beam operation, support beam sweeping for paging, and study the following methods: o Alt-1: Multiplexing paging with SS blocks § FFS: Details of paging o Alt-2: Adding another round of beam sweeping for paging § Note: Another round of beam sweeping is different from the beam sweeping of SS burst set o Other alternatives are not precluded o Companies report their assumption for paging

Moreover, the following agreements were made in RAN2 NR AH meeting in Jan. 2017.

1: UE in RRC_IDLE and RRC_INACTVE state monitors paging/notification every DRX cycle. 2: UE monitors one paging occasion per DRX cycle. Paging occasion is the time interval over which a paging message is transmitted by gNB. 3: The length of DRX cycle is configurable. A default DRX cycle length is provided in system information. Additionally, a UE specific DRX cycle length can also be provided to UE in dedicated signalling. 4: The number of paging occasions in the DRX cycle is configurable and provided in system information. 5: If multiple paging occasions are configured by network in the DRX cycle then UEs can be distributed to these paging occasions based on UE ID. 6: RAN2 understanding is that paging can be transmitted at least using beam sweeping (content of paging may be a paging indicator or the paging message, FFS) 7: Paging occasion can consists of multiple time slots (e.g. subframe or OFDM symbol). (Multiple time slots enables transmission of paging using a different set of DL TX beam(s) in each time slot, or could enable repetition - RAN1 decision). 8: The number of time slots in a paging occasion is provided in system information.

From the above agreements, we identify that the major concern of paging in multi-beam operation is that the transmission of beam-swept paging message for a group of UEs consumes large amount of radio resources. In this contribution, we discuss two different approaches for paging overhead reduction: (1) paging with localization and (2) response-driven paging.

# Discussion

## Paging with localization

### The concept

The overhead due to repeated paging records can be significantly reduced if the network knows (or at least can guess) which cell the UE is camping on, and the most suitable beams for the UE, and first try paging only on these beams for each UE.

Then instead of transmitting the paging message on all beams of all cells in the tracking area (TA) or RAN area (RA), a “smart” paging is possible, i.e., page first within a group of beams, then in a cell, and then many cells. The concept of paging with localization is illustrated in Figure 1, considering a moving UE. At the beginning, the network obtains UE position (beam or cell level) from the last RRC release or via TAU/PAU. The UE may be paged within a group of beams if it does not move far away. When UE moves further, the paging message needs to be transmitted in the whole cell. Then the UE moves to the coverage of neighbouring cell, and it can be paged via paging within a group of cells. If the UE cannot be found in this group of cells, the network will broadcast the paging message within the whole TA or RA. Finally, when the UE moves out of the TA/RA, it sends tracking area update (TAU) or RAN area update (RAU) indication to the network.

Figure 1.	Paging with localization

We have the following observation regarding paging with localization for UEs in multi-beam operation.

Observation 1:	Overhead for paging UEs in multi-beam operation can be reduced by supporting paging with localization for, i.e., page first in a group of beams, then in a cell, then in a group of cells, and finally in the whole TA/RA.

### Methods

To allow implementation of paging with localization, we need some methods for network to obtain information about UE location or distributions.

Method 1: UE location at last release or TAU/RAU

UE location can be recorded upon last RRC release, or passed to the network when UE performs TAU or RAU. In this way, no extra mechanism needs to be introduced. For UE in RRC_IDLE, however, the UE location hypothesis is imprecise since TA is usually large. For UE is RRC_INACTIVE, this method works if the RA is not configured too large.

Observation 2:	Network can estimate which cell a UE is camping on based on UE location at last RRC connection release or when UE performs tracking or RAN-based notification area updates.

Method 2: Preamble when camping on a different cell

In NR, due to higher carrier frequency, cells with smaller coverage but higher density may be deployed. There may be cells which no Idle/Inactive UE is camping on, and overhead can be reduced if paging is not transmitted on these cells. A UE may send a preamble whenever it enters the coverage of a new cell. Since UE sends a preamble rather than UE ID, the network does not obtain exact UE location information (which UE camps on which cell). Instead, the distribution of UEs can be estimated by the network. Paging message is first delivered on the cells that receive preamble within a configured duration. If the network does not receive expected paging response, it delivers paging in multiple cells. In this way, paging overhead can be reduced, even though paging delivery at cell-level precision is not achieved.

Proposal 1:	UE in RRC_IDLE or RRC_INACTIVE can be configured to send preamble upon entering the coverage of a different cell. The preamble is used for the network to estimate UE distributions and helps reduce paging overhead.

## Response-driven paging

As proposed in [2] [3], to reduce downlink overhead due to paging message transmission, the gNB may first transmit a paging indication for a group of UEs when at least one of the UE is to be paged. Then all UEs alerted by paging indication send random access request to the cells they camp on. The cell-level location and most suitable beam of each UE can be learned, and then the network can send paging messages more precisely, via unicast or multicast. The procedure is referred to as response-driven paging, and illustrated in the figure below.

Figure 2. Response-driven paging procedure

While the network needs not to repeat paging messages on different beams and thus the overhead in the downlink is reduced, overhead in the uplink increases. The trade-off involves parameters such as total number of UEs, number of UEs to be paged, and the number of beams to be swept. Overhead comparison for normal and response-driven paging is illustrated by an example shown in Figure 3.

Figure 3. Normal vs. response-driven paging

In this example, there are 40 UEs in a “group” monitoring the same PO, among them 3 UEs are to be paged, and there are 8 gNB beams to be swept through. Assume that a UE identity in paging message and the paging response of a UE both occupy one resource unit. For response-driven paging, totally 43 resource units are used for paging response and paging message transmission. In contrast, for normal paging, 24 resource units are used for paging message transmission in a cell, and the number is multiplied if the paging message is also transmitted in other cells in the same TA/PA.

We observe that in some cases, response-driven paging reduces paging overhead.

Observation 3: Response-driven paging brings lower overhead due to paging message compared to normal paging, if (1) there is a high percentage of UEs to be paged in a group, (2) there are many gNB beams to be swept through, or (3) the number of cells in the same TA/PA is large enough.

While response-driven paging may be beneficial from overhead point of view, there are still some concerns.

- UE power consumption increases significantly due to the transmission of paging response.

- It is unclear how multiple idle-mode UEs transmit paging response on the same gNB beam. The mechanism, e.g., via random access or pre-scheduling, can be complicated or further increases power consumption.

- Format of paging indicator and the content of paging response need to be defined.

Considering the above concerns and the fact that paging load is usually not very heavy, we suggest paging with localization for paging overhead reduction, and postponing the discussion on response-driven paging to next release.

Proposal 2:	Response-driven paging is deprioritized in this release.

# Conclusion

We have the following observation:

Observation 1:	Overhead for paging UEs in multi-beam operation can be reduced by supporting paging with localization for, i.e., page first in a group of beams, then in a cell, then in a group of cells, and finally in the whole TA/RA.

Observation 2:	Network can estimate which cell a UE is camping on based on UE location at last RRC connection release or when UE performs tracking or RAN-based notification area updates.

Observation 3: Response-driven paging brings lower overhead due to paging message compared to normal paging, if (1) there is a high percentage of UEs to be paged in a group, (2) there are many gNB beams to be swept through, or (3) the number of cells in the same TA/PA is large enough.

It is proposed to discuss and decide on the following proposals:

Proposal 1:	UE in RRC_IDLE or RRC_INACTIVE can be configured to send preamble upon entering the coverage of a different cell. The preamble is used for the network to estimate UE distributions and helps reduce paging overhead.

Proposal 2:	Response-driven paging is deprioritized in this release.

# Reference

- 3GPP TR 38.804, Study on New Radio Access Technology; Radio Interface Protocol Aspects

- R2-1702779, Configurable Paging Procedure for NR, Nokia, Alcatel-Lucent Shanghai Bell

- R2-1704763, Response-driven paging to reduce beam sweeping overhead in NR, Ericsson

- R2-1702892, Paging in NR – Beamforming Aspects, Samsung