3GPP TSG-RAN2 AH - 1801                                      R2-1801411

Vancouver, Canada, Jan 22 – Jan 26, 2018

Agenda item:	10.4.1.4.1

Source:	Samsung

Title:	Issues/Corrections: CSI-RS based MeasObjectNR configuration

Document for:	Discussion & Decision

- Introduction

In this contribution we discuss issues/corrections related to CSI-RS based MeasObjectNR configuration for RRM.

- Discussion

In RAN2 #100, CSI-RS based measurement objective configuration for RRM measurement and mobility was drafted based on LS from RAN1 which contains L1 parameters for RRC as in [1]. However some of the parameters captured in TS 38.331 is not aligned with the LS from RAN1 [1]. Moreover, although the L1 parameters were supposed to be aligned with agreements made from RAN1 [2], some of the parameters from [1] are not aligned with the agreements from [2].

In this contribution we discuss corrections and miss-alignments of the L1 parameters related with the CSI-RS based RRM mobility and propose appropriate ASN.1 architecture which appropriately represents the RAN1 agreements from [2].

  - Corrections

In this subsection, we propose corrections for some of the parameters of TS 38.331 which are not aligned with the LS from RAN1 [1].

The field associatedSSB corresponds to L1 parameter 'Associated-SSB' and this represents associated SSB for each CSI-RS. Each CSI-RS resource may be associated with at most one SSB to reduce UE complexity. Hence, the L1 parameters [1] indicates that this field associatedSSB shall be optional and configured per resource and it is also aligned with the RAN1 agreements [2]. However, the current TS 38.331 captured this field associatedSSB under CSI-RS-ResourceConfig-Mobility IE which contains MO specific values.

Proposal 1: For associated SSB indication of each CSI-RS resource, the filed associatedSSB shall be optional and configured per resource

Since the different CSI-RSs could be associated with different SSBs, the above proposal is correct. However, if some of the CSI-RSs are associated with the same SSB, then the same SSB index is transmitted repeatedly. In order for saving signaling overhead, we may consider introducing a kind of indication, one-bit, that indicates this CSI-RS is associated with the same SSB as the previous CSI-RS.

Changes for Proposal 1 : #### – MeasObjectNR -- ASN1START -- TAG-MEAS-OBJECT-NR-START ... CSI-RS-ResourceConfig-Mobility ::= SEQUENCE { -- MO specific values csi-rs-MeasurementBW SEQUENCE { -- Size of the measurement BW in PRBs -- Corresponds to L1 parameter 'CSI-RS-measurementBW-size' (see FFS_Spec, section FFS_Section) nrofPRBs ENUMERATED { size24, size48, size96, size192, size268}, -- Starting PRB index of the measurement bandwidth -- Corresponds to L1 parameter 'CSI-RS-measurement-BW-start' (see FFS_Spec, section FFS_Section) -- FFS_Value: Upper edge of value range unclear in RAN1 startPRB INTEGER(0..251), -- Each CSI-RS resource may be associated with one SSB. If such SSB is indicated, the NW also indicates whether the UE may assume -- quasi-colocation of this SSB with this CSI-RS reosurce. -- Corresponds to L1 parameter 'Associated-SSB' (see FFS_Spec, section FFS_Section) -- FFS: What does the UE do if it there is no such SSB-Index? isServingCellMO BOOLEAN }, -- subcarrier spacing of CSI-RS. It can take the same values available also for the data channels and for SSB subcarrierSpacing SubcarrierSpacing, -- FFS_Description. -- FFS_CHECK: Should this be in the resource-config (here) or in the resource (below)? -- Corresponds to L1 parameter 'Common-PRB-Grid-offset' (see FFS_Spec, section FFS_Section) prb-GridOffset INTEGER (maxNrofPhysicalResourceBlocksTimes4) OPTIONAL, -- List of resources csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility } CSI-RS-Resource-Mobility ::= SEQUENCE { csi-rs-ResourceId-RRM CSI-RS-ResourceId-RRM, cellId PhysicalCellId, -- FFS_CHECK whether the following fields are supposed to be per resource (here) or in the resource config (above) -- Contains periodicity and slot offset for periodic/semi-persistent CSI-RS (see 38.211, section x.x.x.x)FFS_Ref slotConfig CHOICE { ms5 INTEGER (0..4), ms10 INTEGER (0..9), ms20 INTEGER (0..19), ms40 INTEGER (0..39) }, associatedSSB SEQUENCE { -- FFS_Value: Check the value range ssb-Index SSB-Index OPTIONAL, -- The CSI-RS resource is either QCL’ed not QCL’ed with the associated SSB in spatial parameters -- Corresponds to L1 parameter 'QCLed-SSB' (see FFS_Spec, section FFS_Section) isQuasiColocated BOOLEAN }, -- Resource Element mapping pattern for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref resourceElementMappingPattern ENUMERATED {ffsTypeAndValue}, -- Sequence generation parameter for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref sequenceGenerationConfig INTEGER (0..1023), -- Frequency domain density for the 1-port CSI-RS for L3 mobility -- Corresponds to L1 parameter 'Density' (see FFS_Spec, section FFS_Section) density ENUMERATED {d1,d3} OPTIONAL, ... } CSI-RS-ResourceId-RRM ::= INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1) ... -- ASN1STOP

  - Cell-level CSI-RS configurations.

According to L1 parameter LS [1] and agreements of RAN1 [2], there are various parameters which are commented to be configured per cell. However, the current TS 38.331 CSI-RS resource configuration for RRM has only two levels: ResourceConfig level for MO specific parameters and Resource level for CSI-RS resource specific parameters.

In this subsection, we propose corrections for such parameters to be configured per cell, by introducing the cell level CSI-RS configuration.

The field cellId corresponds to L1 parameter ‘PCI’ which represents the physical cell ID for the CSI-RS, and a common value shall be assigned across all the resources configured for a cell. Hence, the L1 parameters [1] indicates that this field density shall be configured per cell and it is also aligned with the RAN1 agreements [2]. However, the current TS 38.331 captured this field density under CSI-RS-Resource-Mobility IE which contains CSI-RS resource specific values.

Proposal 2: For the physical cell ID for the CSI-RS, the filed cellId shall be configured per cell

The field density corresponds to L1 parameter ‘Density’ which represents the frequency domain density for the 1-port CSI-RS for L3 mobility, and a common value shall be assigned across all the resources configured for a cell. Hence, the L1 parameters [1] indicates that this field density shall be configured per cell and it is also aligned with the RAN1 agreements [2]. However, the current TS 38.331 captured this field density under CSI-RS-Resource-Mobility IE which contains CSI-RS resource specific values.

Proposal 3: For the frequency domain density of 1-port CSI-RS, the filed density shall be configured per cell

The above two parameters, cellId and density are clearly indicated to be configured per cell in both the L1 parameters [1] and agreements [2] so the above proposals 2 and 3 are reasonable corrections.

However, for the following parameters, there are miss-alignments among L1 parameters [1] and agreements [2].

The field measurement-BW corresponds to L1 parameter ‘Measurement-BW’ which represents the frequency bandwidth to measure the CSI-RS for L3 mobility. The L1 parameter [1] indicates that this field measurement-BW shall be configured per carrier, however, the RAN1 agreement [2] indicates that this field measurement-BW shall be configured per cell. Since we believe the agreement of the RAN1#91 meeting report represents accurate consensus among RAN1 companies, we propose that this miss-alignment shall be resolved and the field measurement-BW is configured per cell.

Proposal 4: The field measurement-BW shall be configured per cell according to the RAN1#91 agreement from [2]. Note that this field description in the L1 parameter [1] is not aligned with that of the agreement [2] and it needs to be checked.

The field prb-GridOffset corresponds to L1 parameter ‘Common-PRB-Grid-offset’ represents information to define common PRB grid for CSI-RS sequence generation. This field corresponds to an offset (in terms of number of subcarriers in CSI-RS numerology) between PRB 0 for common PRB indexing and a reference location which is the lowest PRB of the cell-defining SSB. The L1 parameter [1] indicates that this field measurement-BW shall be configured per resource, however, the RAN1 agreement [2] indicates that this field measurement-BW shall be configured per cell. And, the current TS 38.331 captured this parameter per MO. Since we believe the agreement of the RAN1#91 meeting report represents accurate consensus among RAN1 companies, we propose that this miss-alignment shall be resolved and the field measurement-BW is configured per cell.

Proposal 5: The field prb-GridOffset shall be configured per cell according to the RAN1#91 agreement from [2]. Note that this field description in the L1 parameter [1] is not aligned with that of the agreement [2] and it needs to be checked.

Therfore, in order to accommodate the above four parameters: cellId, density, measurement-BW, and prb-GridOffset, we propose the cell level CSI-RS configuration to be introduced as follows:

Proposal 6: In order to accommodate the parameters to be configured per cell: cellId, density, measurement-BW, and prb-GridOffset, we propose CSI-RS resource configuration for RRM to have the cell level configuration with the new IE named as CSI-RS-Cell-Mobility.

Changes for Proposal 2 ~ 6: #### – MeasObjectNR -- ASN1START -- TAG-MEAS-OBJECT-NR-START ... CSI-RS-ResourceConfig-Mobility ::= SEQUENCE { -- MO specific values isServingCellMO BOOLEAN, -- subcarrier spacing of CSI-RS. It can take the same values available also for the data channels and for SSB subcarrierSpacing SubcarrierSpacing, -- FFS_Description. OPTIONAL, -- List of cells CSI-rs-CellList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-CellsRRM)) OF CSI-RS-Cell-Mobility } CSI-RS-Cell-Mobility ::= SEQUENCE { cellId PhysicalCellId, -- Frequency domain density for the 1-port CSI-RS for L3 mobility -- Corresponds to L1 parameter 'Density' (see FFS_Spec, section FFS_Section) density ENUMERATED {d1,d3} OPTIONAL, csi-rs-MeasurementBW SEQUENCE { -- Size of the measurement BW in PRBs -- Corresponds to L1 parameter 'CSI-RS-measurementBW-size' (see FFS_Spec, section FFS_Section) nrofPRBs ENUMERATED { size24, size48, size96, size192, size268}, -- Starting PRB index of the measurement bandwidth -- Corresponds to L1 parameter 'CSI-RS-measurement-BW-start' (see FFS_Spec, section FFS_Section) -- FFS_Value: Upper edge of value range unclear in RAN1 startPRB INTEGER(0..251), -- Each CSI-RS resource may be associated with one SSB. If such SSB is indicated, the NW also indicates whether the UE may assume -- quasi-colocation of this SSB with this CSI-RS reosurce. -- Corresponds to L1 parameter 'Associated-SSB' (see FFS_Spec, section FFS_Section) -- FFS: What does the UE do if it there is no such SSB-Index? } -- FFS_CHECK: Should this be in the resource-config (here) or in the resource (below)? -- Corresponds to L1 parameter 'Common-PRB-Grid-offset' (see FFS_Spec, section FFS_Section) prb-GridOffset INTEGER (maxNrofPhysicalResourceBlocksTimes4), -- List of resources csi-rs-ResourceList-Mobility SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesRRM)) OF CSI-RS-Resource-Mobility } CSI-RS-Resource-Mobility ::= SEQUENCE { csi-rs-ResourceId-RRM CSI-RS-ResourceId-RRM, -- FFS_CHECK whether the following fields are supposed to be per resource (here) or in the resource config (above) -- Contains periodicity and slot offset for periodic/semi-persistent CSI-RS (see 38.211, section x.x.x.x)FFS_Ref slotConfig CHOICE { ms5 INTEGER (0..4), ms10 INTEGER (0..9), ms20 INTEGER (0..19), ms40 INTEGER (0..39) }, associatedSSB SEQUENCE { -- FFS_Value: Check the value range ssb-Index SSB-Index OPTIONAL, -- The CSI-RS resource is either QCL’ed not QCL’ed with the associated SSB in spatial parameters -- Corresponds to L1 parameter 'QCLed-SSB' (see FFS_Spec, section FFS_Section) isQuasiColocated BOOLEAN }, -- Resource Element mapping pattern for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref resourceElementMappingPattern ENUMERATED {ffsTypeAndValue}, -- Sequence generation parameter for CSI-RS (see 38.211, section x.x.x.x) FFS_Ref sequenceGenerationConfig INTEGER (0..1023), ... } CSI-RS-ResourceId-RRM ::= INTEGER (0..maxNrofCSI-RS-ResourcesRRM-1) ... -- ASN1STOP

- Conclusion

Based on the above, RAN2 is requested to discuss and if possible agree on the following proposals:

Proposal 1: For associated SSB indication of each CSI-RS resource, the filed associatedSSB shall be optional and configured per resource

Proposal 2: For the physical cell ID for the CSI-RS, the filed cellId shall be configured per cell

Proposal 3: For the frequency domain density of 1-port CSI-RS, the filed density shall be configured per cell

Proposal 4: The field measurement-BW shall be configured per cell according to the RAN1#91 agreement from [2]. Note that this field description in the L1 parameter [1] is not aligned with that of the agreement [2] and it needs to be checked.

Proposal 5: The field prb-GridOffset shall be configured per cell according to the RAN1#91 agreement from [2]. Note that this field description in the L1 parameter [1] is not aligned with that of the agreement [2] and it needs to be checked.

Proposal 6: In order to accommodate the parameters to be configured per cell: cellId, density, measurement-BW, and prb-GridOffset, we propose CSI-RS resource configuration for RRM to have the cell level configuration with the new IE named as CSI-RS-Cell-Mobility.

Proposal 7: Adopt the proposed changes in CR [3].

- References

- R2-1714164, LS on RRC parameters for NR (R1-1721616; contact: Ericsson)

- Draft Report of 3GPP TSG RAN WG1 #91 v0.2.0

- R2-1801412,  “Draft CR on corrections for CSI-RS based MeasObjectNR configuration”