3GPP TSG-RAN WG2 NR AH#3			     R2-1801457

Vancouver, Canada, 22nd – 26th January, 2018         (Resubmission of R2-1711748)

Agenda item:	   10.3.4.4 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 		Considerations on release of a mapping of QoS flow to DRB

Document for:	Discussion and Decision

1.	Introduction

This document discusses some issues regarding release of the DRB mapping for a QoS flow.

2.	Discussion

According to TS 38.300, gNB controls the mapping of QoS flow to DRB in two different ways: reflective mapping and explicit configuration. However, RAN2 have discussed about how to (re)map QoS flow to DRB, but have not discussed how to get rid of the mapping. The gNB may be expected to release DRB mapping for 1) some QoS flow(s) or 2) all QoS flow(s) associated with a DRB. Moreover, the latter can make the gNB trigger a release of the DRB. Therefore, we think that the latter can be initiated by either sending to UE each identity of the QoS flow(s) or releasing the DRB. In that case, the UE needs to release the DRB mapping for each QoS flow indicated by the received QoS flow’s identity or for all QoS flow(s) which is associated with the DRB to be released.

Proposal 1:	In order to initiate a release of DRB mapping for QoS flow(s), gNB may send to UE each identity of the QoS flow(s). The UE should release the DRB mapping for QoS flow indicated by the QoS flow's identity.

Proposal 2:	When receiving a release request for a DRB, UE should release the DRB mapping for all QoS flow(s) mapped to the DRB to be released.

3.	Proposal

In this document, we present our view on the release of the DRB mapping for a QoS flow. We have following proposals:

Proposal 1:	In order to initiate a release of DRB mapping for QoS flow(s), gNB may send to UE each identity of the QoS flow(s). The UE should release the DRB mapping for QoS flow indicated by the QoS flow's identity.

Proposal 2:	When receiving a release request for a DRB, UE should release the DRB mapping for all QoS flow(s) mapped to the DRB to be released.