3GPP TSG-RAN WG2 NR AH1807 Meeting	R2-1809601

# Montreal, Canada, 2nd - 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.1.0 (ASN.1 SA review baseline) |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.331 for handling of SCG failure on Split SRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the June specification of EN-DC, the configuration of UL split SRB where UL SRB can be configured in SCG only is supported. However, when UL path of a split SRB is configured as SCG only and the SCG failure occurs, the UE cannot send SCGFailureInformation message to the network because in the current specifications, the SCG is suspended and the MCG path is not allowed by configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the primary path of the SRB1 to MCG when SCGFailure occurs and split SRB1 is configured with UL path as SCG only. - Update the field description for the primaryPath in the PDCP-Config to remove that only MCG is supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SCGFailureInformation cannot be sent if SCG Failure occurs and UL SRB is configured as SCG only. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.3.2 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

### 5.7.3	SCG failure information

#### 5.7.3.1	General

Figure 5.7.3.1-1: SCG failure information

The purpose of this procedure is to inform EUTRAN or NR MN about an SCG failure the UE has experienced i.e. SCG radio link failure, e failure of SCG reconfiguration with sync, SCG configuration failure for RRC message on SRB3, SCG integrity check failure and exceeding the maximum uplink transmission timing difference.

Editor’s Note: SCG failure considers the case of exceeding the maximum uplink transmission timing difference if RAN1 decides that EN-DC supports the synchronised operation case. FFS how to capture

Editor’s Note: FFS whether to include the handling of SCell Failure in CA duplication case in SCGfailureinformation procedure and whether to rename SCGfailureinformation.

#### 5.7.3.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with subclause 5.3.10.3;

1>	upon reconfiguration with sync failure of the SCG, in accordance with subclause 5.3.5.8.3;

1>	upon SCG configuration failure, in accordance with subclause 5.3.5.8.2;

1>	upon integrity check failure indication from SCG lower layers, in accordance with subclause 5.3.5.8.1.

Upon initiating the procedure, the UE shall:

1>	suspend SCG transmission for all SRBs and DRBs;

1>	reset SCG-MAC;

1>	stop T304, if running;

1>	if the UE is operating in EN-DC:

2> if SRB1 is configured with moreThanOneRLC and the cellGroup for primaryPath is set to 1

3> change the cellGroup for primaryPath of SRB1 to 0

2>	initiate transmission of the SCGFailureInformationNR message as specified in TS 36.331 [10, 5.6.13a].

Editor’s Note: The section for transmission of SCGFailureInformation in NR RRC entity for SA is FFS_Standalone.

END OF FIRST CHANGE

START OF SECOND CHANGE

### 6.3.2	Radio resource control information elements

…

PDCP-Config field descriptions

cipheringDisabled If included, it indicates that cipherng is disabled for the DRB. The network does not configure this field for EN-DC.

discardTimer Value in ms of discardTimer specified in TS 38.323 [5]. Value ms50 corresponds to 50 ms, ms100 corresponds to 100 ms and so on.

drb-ContinueROHC Indicates whether the PDCP entity continues or resets the ROHC header compression protocol during PDCP re-establishment. This field is configured only in case of reconfiguration with sync where the PDCP termination point is not changed.

headerCompression If rohc is configured, the UE shall apply the configured ROHC profile(s) in both uplink and downlink. If uplinkOnlyROHC is configured, the UE shall apply the configure ROHC profile(s) in uplink (there is no header compression in downlink). ROHC can be configured for any bearer type. ROHC should be configured at reconfiguration involving PDCP re-establishment if the RB was previously configured with ROHC. Header compression should not be configured when out-of-order delivery is allowed for PDCP SDUs.

integrityProtection Indicates whether or not integrity protection is configured for this radio bearer. The value of integrityProtection for a DRB can only be changed using reconfiguration with sync. FFS: text to indicate where to find the key.

maxCID Indicates the value of the MAX_CID parameter as specified in TS 38.323 [5] FFS: need to specify something with respect to UE capabilities.

moreThanOneRLC FFS / TODO: Handle more than two secondary cell groups

outOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. Out-of-order delivery is configured only when the radio bearer is established

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates whether duplication is configured. The value of this field, when the field is present, indicates whether duplication is activated. The value of this field is always TRUE, when configured for a SRB.

pdcp-SN-Size PDCP sequence number size, 12 or 18 bits.

primaryPath Indicates the cell group ID and LCID of the primary RLC entity as specified in TS 38.323 clause 5.2.1 for UL data tranmission when more than one RLC entity is associated with the PDCP entity.

pdcp-SN-Size PDCP sequence number size, 12 or 18 bits.

statusReportRequired For AM DRBs, indicates whether the DRB is configured to send a PDCP status report in the uplink, as specified in TS 38.323 [5]. For UL DRBs, the value shall be ignored by the UE.

t-Reordering Value in ms of t-Reordering specified in TS 38.323 [5]. Value ms0 corresponds to 0ms, value ms20 corresponds to 20ms, value ms40 corresponds to 40ms, and so on. When the field is absent the UE applies the value infinity.

ul-DataSplitThreshold Parameter specified in TS 38.323 [5]. Value b0 corresponds to 0 bits, value b100 corresponds to 100 bits, value b200 corresponds to 200 bits, and so on.

END OF SECOND CHANGE