| 3GPP TS 25.425 V19.0.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; UTRAN Iur interface user plane protocols for Common Transport Channel data streams (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	7

1	Scope	8

2	References	8

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	9

3.4	Specification Notations	9

4	General Aspects	10

4.1	Common Transport Channel Data Streams User Plane Protocol Services	10

4.1.1	RACH Data Streams User Plane Protocol Services	10

4.1.2	FACH Data Streams User Plane Protocol Services	11

4.1.3	USCH/DSCH Data Streams User Plane Protocol Services [TDD]	11

4.1.4	HS-DSCH Data Streams User Plane Protocol Services	11

4.1.5	E-DCH for CELL_FACH and Idle Data Streams User Plane Protocol Services [FDD and 1.28Mcps TDD]	11

4.2	Services expected from the Data Transport Network layer	11

4.3	Protocol Version	11

5	Common Transport Channel Data Streams User Plane Procedures	11

5.1	Data Transfer	12

5.1.1	RACH Data Transfer	12

5.1.2	FACH Data Transfer	12

5.1.3	USCH Data Transfer [TDD]	13

5.1.4	DSCH Data Transfer [TDD]	13

5.1.5	HS-DSCH Data Transfer	14

5.1.6	E-DCH for CELL_FACH and Idle Data Transfer [FDD and 1.28Mcps TDD]	15

5.1.7	Indication of UE State Transition [1.28Mcps TDD]	15

5.1.8	HS-DSCH UL Synchronization Establishment Failure [1.28Mcps TDD]	15

5.2	Flow Control	16

5.2.1	FACH Flow Control	16

5.2.2	DSCH Capacity Request [TDD]	16

5.2.3	DSCH Capacity Allocation [TDD]	17

5.2.4	FACH Capacity Request	17

5.2.5	HS-DSCH Capacity Request	18

5.2.6	HS-DSCH Capacity Allocation	18

5.2A	Node Synchronisation	19

5.3	General	19

5.3.1	DSCH / USCH transport bearer replacement [TDD]	19

5.3.2	HS-DSCH Transport Bearer Replacement	20

5.4	HS-DSCH PDU DROP INDICATION	20

6	Frame Structure and Coding	21

6.1	General	21

6.2	Data Frame structure	22

6.2.1	RACH Channels	22

6.2.2	FACH Channels	23

6.2.3	USCH Channels [TDD]	24

6.2.4	DSCH Channels [TDD]	24

6.2.4A	HS-DSCH Channels	24

6.2.4B	E-DCH for CELL_FACH and Idle Channels [FDD and 1.28Mcps TDD]	27

6.2.5	Coding of Information Elements in DATA FRAMEs	29

6.2.5.1	Header CRC	29

6.2.5.2	Frame Type (FT)	29

6.2.5.3	DRNTI	29

6.2.5.4	S-RNTI	29

6.2.5.5	UE-ID Type Indicator (UE-ID Type)	29

6.2.5.6	S-CCPCH Indicator (S-CI)	29

6.2.5.7	Common Transport Channel Priority Indicator (CmCH-PI)	30

6.2.5.8	MAC-c/sh SDU Length	30

6.2.5.9	NumOfSDU	30

6.2.5.10	Propagation delay [FDD]	30

6.2.5.10A	Ext Propagation Delay [FDD]	30

6.2.5.11	Rx Timing Deviation [3.84Mcps TDD]	30

6.2.5.11A	Received SYNC UL Timing Deviation [1.28Mcps TDD]	31

6.2.5.11B	Rx Timing Deviation [7.68Mcps TDD]	31

6.2.5.12	User Buffer Size	31

6.2.5.13	MAC-c/sh SDU	31

6.2.5.14	Payload CRC	31

6.2.5.15	Spare Extension	31

6.2.5.16	MAC-d PDU Length	32

6.2.5.17	NumOfPDU	32

6.2.5.18	MAC-d PDU	32

6.2.5.19	Cell Portion ID [FDD]	32

6.2.5.20	New IE Flags	32

6.2.5.21	Flush	32

6.2.5.22	DRT (Delay Reference Time)	32

6.2.5.23	Frame Sequence Number	33

6.2.5.24	Logical Channel ID in block n	33

6.2.5.25	Total Number of PDU blocks	33

6.2.5.26	MAC-d PDU length in block n	33

6.2.5.27	Number of MAC-d PDUs in block n (#PDUs in block n)	33

6.2.5.28	DRT Indicator	33

6.2.5.29	FACH Indicator (FI) [FDD and 1.28Mcps TDD]	34

6.2.5.30	S-RNTI [FDD and 1.28Mcps TDD]	34

6.2.5.31	UE-ID Type Indicator (UE-ID Type)	34

6.2.5.32	FSN/DRT Reset	34

6.2.5.33	MAC-d Type 2 PDU	34

6.2.5.34	PCH Indicator (PI) [FDD]	34

6.2.5.35	DRX Cycle Length Coefficient [FDD]	34

6.2.5.36	Number of Subframes [FDD and 1.28Mcps TDD]	35

6.2.5.37	Number of MAC-is SDU in frame [FDD and 1.28Mcps TDD]	35

6.2.5.38	Number of MAC-is PDUs [FDD and 1.28Mcps TDD]	35

6.2.5.39	Subframe Number [FDD and 1.28Mcps TDD]	35

6.2.5.40	E-DCH User Buffer Size [FDD and 1.28Mcps TDD]	35

6.2.5.41	MAC-is PDU descriptor [FDD and 1.28Mcps TDD]	36

6.2.5.42	eDRX Indication [FDD]	36

6.2.5.43	Cell Portion LCR ID [1.28Mcps TDD]	36

6.2.5.44	TS0 Indication [1.28Mcps TDD]	36

6.2.5.45	MU-MIMO Indication(MI) [1.28Mcps TDD]	36

6.2.5.46	CPC Recovery (CPC Recov) [FDD]	36

6.2.5.47	FeDRX Indication [FDD]	36

6.2.5.48	NodeB Triggered HS-DPCCH Transmission(NTHT) [FDD]	36

6.2.5.49	TTI length [FDD]	37

6.2.5.50	SN, Frame Number	37

6.2.5.51	S/D, Store/Discard indicator	37

6.2.5.52	Connection Frame Number (CFN)	37

6.2.5.53	CR Ind (Cell Reselection Indication)	37

6.2.5.54	CRR Rec (Cell Reselection Request Received)	37

6.2.5.55	Extended S-RNTI	37

6.2.5.56	Operation Preference of Common E-DCH resource Indication (OPCE Ind) [FDD]	38

6.3	Control Frame structure	38

6.3.1	Introduction	38

6.3.2	Header structure of the control frames	38

6.3.2.1	Control frame CRC	38

6.3.2.2	Frame type (FT)	38

6.3.2.3	Control Frame Type	38

6.3.3	Payload structure and information elements	39

6.3.3.1	FACH FLOW CONTROL	39

6.3.3.1.1	S-RNTI	39

6.3.3.1.2	Common Transport Channel Priority Indicator (CmCH-PI)	39

6.3.3.1.3	Credits	39

6.3.3.1.4	Spare Extension	40

6.3.3.1.5	Extended S-RNTI	40

6.3.3.2	DSCH CAPACITY REQUEST [TDD]	40

6.3.3.2.1	Common Transport Channel Priority Indicator (CmCH-PI)	40

6.3.3.2.2	User Buffer Size	40

6.3.3.2.3	Spare Extension	40

6.3.3.3	DSCH CAPACITY ALLOCATION [TDD]	40

6.3.3.3.1	Common Transport Channel Priority Indicator (CmCH-PI)	41

6.3.3.3.2	Maximum MAC-c/sh SDU Length	41

6.3.3.3.3	Credits	41

6.3.3.3.4	Interval	41

6.3.3.3.5	Repetition Period	41

6.3.3.3.6	Spare Extension	41

6.3.3.4	FACH CAPACITY REQUEST	41

6.3.3.4.1	D-RNTI	42

6.3.3.4.2	Common Transport Channel Priority Indicator (CmCH-PI)	42

6.3.3.4.3	User Buffer Size	42

6.3.3.4.4	Spare extension	42

6.3.3.5	HS-DSCH CAPACITY REQUEST	42

6.3.3.5.1	Common Transport Channel Priority Indicator (CmCH-PI)	42

6.3.3.5.2	User Buffer Size	42

6.3.3.5.3	Spare Extension	42

6.3.3.5.4	DRNTI [FDD]	43

6.3.3.6	HS-DSCH CAPACITY ALLOCATION	43

6.3.3.6.1	Common Transport Channel Priority Indicator (CmCH-PI)	44

6.3.3.6.2	Maximum MAC-d PDU Length	44

6.3.3.6.3	HS-DSCH Credits	44

6.3.3.6.4	HS-DSCH Interval	44

6.3.3.6.5	HS-DSCH Repetition Period	44

6.3.3.6.8	Spare Extension	45

6.3.3.6.9	Congestion Status	45

6.3.3.6.10	SRNTI Indicator [FDD and 1.28Mcps TDD]	45

6.3.3.6.11	S-RNTI [FDD and 1.28Mcps TDD]	45

6.3.3.6.12	Maximum MAC-d PDU Length Type 2	45

6.3.3.6.13	Extended S-RNTI [FDD]	45

6.3.3.7	DL NODE SYNCHRONISATION	45

6.3.3.7.1	Payload structure	45

6.3.3.7.2	T1	46

6.3.3.7.3	Spare Extension	46

6.3.3.8	UL NODE SYNCHRONISATION	46

6.3.3.8.1	Payload structure	46

6.3.3.8.2	T1	46

6.3.3.8.3	T2	46

6.3.3.8.4	T3	47

6.3.3.8.5	Spare Extension	47

6.3.3.9	HS-DSCH UL SYNCHRONIZATION ESTABLISHMENT FAILURE [1.28Mcps TDD]	47

6.3.3.9.1	Payload structure	47

6.3.3.9.2	S-RNTI	47

6.3.3.10	HS-DSCH PDU DROP INDICATION	47

6.3.3.10.1	Payload structure	47

6.3.3.10.2	Drop Reason	47

6.3.3.10.3	PDU Index	48

7	Handling of Unknown, Unforeseen and Erroneous Protocol Data	48

7.1	General	48

Annex A (informative):	Change History	49

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document shall provide a description of the UTRAN RNS-RNS (Iur) interface user plane protocols for Common Transport Channel data streams as agreed within the TSG-RAN working group 3.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	Void

[3]	Void

[4]	Void

[5]	3GPP TS 25.401: "UTRAN overall description".

[6]	Void

[7]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[8]	3GPP TS 25.423: "UTRAN Iur Interface RNSAP Signalling".

[9]	3GPP TS 25.321: "Medium Access Control (MAC) protocol specification".

[10]	3GPP TS 25.224: "Physical layer procedures (TDD)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions in TS 25.401 [5] and the following apply:

Common Transport Channel: it is defined as a transport channel that is shared by several users i.e. DSCH [TDD], USCH [TDD], RACH, FACH [FDD and 1.28Mcps TDD -, E-DCH for CELL_FACH and Idle] and HS-DSCH.

Transport Connection: service provided by the transport layer and used by Frame Protocol for the delivery of FP PDU

## 3.2	Symbols

No special symbols are defined in the present document.

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CFN	Connection Frame Number

CmCH	Common Transport Channel

C-RNC	Controlling Radio Network Controller

CRC	Cyclic Redundancy Checksum

DCH	Dedicated Transport Channel

DL	Downlink

D-RNTI	Drift RNTI

DRT	Delay Reference Time

DSCH	Downlink Shared Channel

E-DCH	Enhanced Dedicated Transport Channel

Extended S-RNTI	Extended Serving RNTI

FACH	Forward Access Channel

FP	Frame Protocol

FT	Frame Type

FSN	Frame Sequence Number

HSDPA	High Speed Downlink Packet Access

HS-DSCH	High Speed Downlink Shared Channel

RACH	Random Access Channel

RNC	Radio Network Controller

RNTI	Radio Network Temporary Identity

SRNC	Serving Radio Network Controller

S-RNTI	Serving RNTI

SSCS	Service Specific Convergence Sublayer

TB	Transport Block

TBS	Transport Block Set

TFI	Transport Format Indicator

TNL	Transport Network Layer

ToA	Time of Arrival

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

U-RNTI	UTRAN RNTI

USCH	Uplink Shared Channel

## 3.4	Specification Notations

For the purposes of the present document, the following notations apply:

[FDD]	This tagging of a word indicates that the word preceding the tag "[FDD]" applies only to FDD. This tagging of a heading indicates that the heading preceding the tag "[FDD]" and the section following the heading applies only to FDD.

[TDD]	This tagging of a word indicates that the word preceding the tag "[TDD]" applies only to TDD, including 7.68Mcps TDD, 3.84Mcps TDD and 1.28Mcps TDD. This tagging of a heading indicates that the heading preceding the tag "[TDD]" and the section following the heading applies only to TDD, including 7.68Mcps TDD, 3.84Mcps TDD and 1.28Mcps TDD.

[7.68Mcps TDD]	This tagging of a word indicates that the word preceding the tag “[7.68Mcps TDD]” applies only to 7.68Mcps TDD. This tagging of a heading indicates that the heading preceding the tag “[7.68Mcps TDD]” and the section following the heading applies only to 7.68Mcps TDD.

[3.84Mcps TDD]	This tagging of a word indicates that the word preceding the tag “[3.84Mcps TDD]” applies only to 3.84Mcps TDD. This tagging of a heading indicates that the heading preceding the tag “[3.84Mcps TDD]” and the section following the heading applies only to 3.84Mcps TDD.

[1.28Mcps TDD]	This tagging of a word indicates that the word preceding the tag "[1.28Mcps TDD]" applies only to 1.28Mcps TDD. This tagging of a heading indicates that the heading preceding the tag "[1.28Mcps TDD]" and the section following the heading applies only to 1.28Mcps TDD.

[FDD - …]	This tagging indicates that the enclosed text following the "[FDD - " applies only to FDD. Multiple sequential paragraphs applying only to FDD are enclosed separately to enable insertion of TDD specific (or common) paragraphs between the FDD specific paragraphs.

[TDD - …]	This tagging indicates that the enclosed text following the "[TDD - " applies only to TDD including 7.68Mcps TDD, 3.84Mcps TDD and 1.28Mcps TDD. Multiple sequential paragraphs applying only to TDD are enclosed separately to enable insertion of FDD specific (or common) paragraphs between the TDD specific paragraphs.

[7.68Mcps TDD - …]	This tagging indicates that the enclosed text following the "[7.68Mcps TDD - " applies only to 7.68Mcps TDD. Multiple sequential paragraphs applying only to 7.68Mcps TDD are enclosed separately to enable insertion of FDD and TDD specific (or common) paragraphs between the 7.68Mcps TDD specific paragraphs.

[3.84Mcps TDD - …]	This tagging indicates that the enclosed text following the "[3.84Mcps TDD - " applies only to 3.84Mcps TDD. Multiple sequential paragraphs applying only to 3.84Mcps TDD are enclosed separately to enable insertion of FDD and TDD specific (or common) paragraphs between the 3.84Mcps TDD specific paragraphs.

[1.28Mcps TDD - …]	This tagging indicates that the enclosed text following the "[1.28Mcps TDD – " applies only to 1.28Mcps TDD. Multiple sequential paragraphs applying only to 1.28Mcps TDD are enclosed separately to enable insertion of FDD and TDD specific (or common) paragraphs between the 1.28Mcps TDD specific paragraphs.

Procedure	When referring to a procedure in the specification, the Procedure Name is written with the first letters in each word in upper case characters followed by the word  "procedure", e.g. FACH Data Transfer procedure.

Frame	When referring to a control or data frame in the specification, the CONTROL/DATA FRAME NAME is written with all letters in upper case characters followed by the words "control/data frame", e.g. FACH FLOW CONTROL control frame.

IE	When referring to an information element (IE) in the specification, the Information Element Name is written with the first letters in each word in upper case characters and all letters in Italic font followed by the abbreviation  "IE", e.g. Credits IE.

Value of an IE	When referring to the value of an information element (IE) in the specification, the "Value" is written as it is specified in subclause 6.2.5 or 6.3.3 enclosed by quotation marks, e.g. "0" or "255".

# 4	General Aspects

## 4.1	Common Transport Channel Data Streams User Plane Protocol Services

This subclause describes the services that the User Plane Protocols provide such as data transfer, flow control.

### 4.1.1	RACH Data Streams User Plane Protocol Services

RACH frame protocol provides the following services:

-	Transport of MAC-c/sh SDUs from the DRNC to the SRNC for RACH common transport channels.

### 4.1.2	FACH Data Streams User Plane Protocol Services

FACH frame protocol provides the following services:

-	Transport of MAC-c/sh SDUs from the SRNC to the DRNC for FACH common transport channel.

-	Flow Control between MAC-d and MAC-c/sh.

### 4.1.3	USCH/DSCH Data Streams User Plane Protocol Services [TDD]

[TDD USCH]/DSCH frame protocol provides the following services:

-	Transport of MAC-c/sh SDUs between the SRNC and the DRNC for [TDD USCH] and DSCH common transport channels.

-	Flow Control between MAC-d and MAC-c/sh.

### 4.1.4	HS-DSCH Data Streams User Plane Protocol Services

HS-DSCH frame protocol provides the following services:

-	Transport of MAC-d flows between the SRNC and the DRNC for HS-DSCH common transport channels.

-	Flow Control between MAC-d and CRNC HS-DSCH Flow Control.

### 4.1.5	E-DCH for CELL_FACH and Idle Data Streams User Plane Protocol Services [FDD and 1.28Mcps TDD]

E-DCH for CELL_FACH and Idle frame protocol provides the following services:

-	Transport of MAC-is PDUs in Cell_FACH state between the DRNC and the SRNC for E-DCH common transport channels.

## 4.2	Services expected from the Data Transport Network layer

The following services are expected from the transport layer:

-	Delivery of Frame Protocol PDUs.

In sequence delivery is not required. However, frequent out-of-sequence delivery may impact the performance and should be avoided.

## 4.3	Protocol Version

This revision of the specification specifies version 1 of the protocols.

# 5	Common Transport Channel Data Streams User Plane Procedures

This clause specifies the user plane procedures for Common Transport Channels data streams. Typical related scenarios at Iur interface should be described.

For the user plane of the radio network layer there are five Common Transport Channel frame handling protocols:

1.	Random Access Channel/Common Packet Channel [FDD] Frame Protocol (RACH FP) for transport of Iur data streams carried on RACH on the Uu-interface.

2.	Forward Access Channel Frame Protocol (FACH FP) for transport of Iur data streams carried on FACH on the Uu-interface.

3.	Downlink Shared Channel Frame Protocol ([TDD - DSCH FP]) for transport of Iur data streams carried on DSCH on the Uu-interface.

4.	Uplink Shared Channel Frame Protocol ([TDD - USCH] FP) for transport of Iur data streams carried on USCH on the Uu-interface.

5.	High Speed Downlink Shared Channel Frame Protocol (HS-DSCH FP) for transport of Iur data streams carried on HS-DSCH on the Uu-interface.

6.	[FDD and 1.28Mcps TDD - E-DCH for CELL_FACH and Idle Frame Protocol (E-DCH for CELL_FACH and Idle FP) for transport of Iur data streams carried on E-DCH in Cell_FACH and Idle on Uu-Interface.]

## 5.1	Data Transfer

### 5.1.1	RACH Data Transfer

Figure 1: RACH Data Transfer procedure

Data received on the RACH transport channel is transmitted from the DRNC to the SRNC using RACH DATA FRAMEs. The data is protected by a mandatory payload CRC. Multiple MAC-c/sh SDUs of same length may be transmitted in the same RACH DATA FRAME.

### 5.1.2	FACH Data Transfer

Figure 2: FACH Data Transfer procedure

Data to be transmitted on the FACH transport channel is transmitted from the SRNC to the DRNC using FACH DATA FRAMEs. Multiple MAC-c/sh SDUs of same length and same priority level (CmCH-PI) may be transmitted in the same FACH DATA FRAME. Within one priority and size the MAC-c/sh SDUs shall be transmitted by the DRNS on the Uu interface in the same order as they were received from the SRNC.

The UE-ID Type Indicator IE indicates which UE-ID type MAC-c/sh shall include in the MAC header.

### 5.1.3	USCH Data Transfer [TDD]

Figure 3: USCH Data Transfer procedure

Whenever there is USCH data in the DRNC, transfer is done immediately to the SRNC via the USCH Data Port using USCH DATA FRAMEs.

Data received on the USCH transport channel is transmitted from the DRNC to the SRNC using USCH DATA FRAMEs. The data is protected by a mandatory payload CRC. Multiple MAC-c/sh SDUs of same length may be transmitted in the same USCH DATA FRAME.

### 5.1.4	DSCH Data Transfer [TDD]

Figure 4: DSCH Data Transfer procedure

When the SRNC has been granted capacity by the DRNC via the DSCH CAPACITY ALLOCATION Control Frame or via the DSCH initial capacity allocation as described in TS 25.423 [8] and the SRNC has data waiting to be sent, then the DSCH DATA FRAME is used to transfer the data. If the SRNC has been granted capacity by the DRNC via the DSCH initial capacity allocation as described in TS 25.423 [8], this capacity is valid for only the first DSCH DATA FRAME transmission. When data is waiting to be transferred, and a CAPACITY ALLOCATION is received, a DATA FRAME will be transmitted immediately according to allocation received.

Multiple MAC-c/sh SDUs of same length and same priority level (CmCH-PI) may be transmitted in the same DSCH DATA FRAME.

The DSCH DATA FRAME includes a User Buffer Size IE to indicate the amount of data pending for the respective UE for the indicated priority level. Within one priority level and size the MAC-c/sh SDUs shall be transmitted by the DRNS on the Uu interface in the same order as they were received from the SRNC.

### 5.1.5	HS-DSCH Data Transfer

Figure 4AA: HS-DSCH Data Transfer procedure

The HS-DSCH Data Transfer procedure is used to transfer a HS-DSCH DATA FRAME (TYPE 1 or TYPE 2) from the SRNC to a DRNC. Two types of HS-DSCH Frame Protocols exist for HS-DSCH data transfer procedure, i.e., HS-DSCH Frame Protocol TYPE 1 (including HS-DSCH DATA FRAME TYPE 1 and HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame) and HS-DSCH Frame Protocol TYPE 2 (including HS-DSCH DATA FRAME TYPE 2 and HS-DSCH CAPACITY ALLOCATION TYPE 2 Control Frame).

HS-DSCH DATA FRAME TYPE 2 is selected if the IE HS-DSCH MAC-d PDU Size Format in RNSAP (TS 25.423 [8]) is present and set to ‘Flexible MAC-d PDU Size’ [FDD and 1.28Mcps TDD - or if the IE Enhanced FACH Support Indicator in RNSAP (TS 25.423 [8]) is present]. HS-DSCH DATA FRAME TYPE 1 is selected in any other case.

HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame shall be associated only with HS-DSCH DATA FRAME TYPE 1 while HS-DSCH CAPACITY ALLOCATION TYPE 2 Control Frame shall be associated only with HS-DSCH DATA FRAME TYPE 2. HS-DSCH CAPACITY REQUEST Control Frame shall be used for both of HS-DSCH Frame Protocols. HS-DSCH Frame Protocol TYPE 2 is used for Flexible MAC-d PDU Size [FDD and 1.28Mcps TDD - and Enhanced FACH] as described in RNSAP (TS 25.423 [8]).

When the SRNC has been granted capacity by the DRNC via the HS-DSCH CAPACITY ALLOCATION (TYPE 1 or TYPE 2) Control Frame or via the HS-DSCH initial capacity allocation as described in TS 25.423 [8] and the SRNC has data waiting to be sent, then the HS-DSCH DATA FRAME (TYPE 1 or TYPE 2) is used to transfer the data. If the SRNC has been granted capacity by the DRNC via the HS-DSCH initial capacity allocation as described in TS 25.423 [8], this capacity is valid for only the first HS-DSCH DATA FRAME (TYPE 1 or TYPE 2) transmission. If the HS-DSCH Frame Protocol TYPE 2 has been selected by the DRNC, the granted capacity shall be interpreted as the total number of octets which is retrieved by multiplying the maximum MAC-d PDU length (indicated by the Maximum MAC-d PDU Length Type 2 IE) with the number of MAC-d PDUs (indicated by the HS-DSCH Credits IE). When data is waiting to be transferred, and a CAPACITY ALLOCATION (TYPE 1 or TYPE 2) is received, a DATA FRAME (TYPE 1 or TYPE 2) will be transmitted immediately according to allocation received. If the allocation received is >0 but less than the data waiting to be transferred, then the RNC may send one HS-DSCH DATA FRAME TYPE 2 containing one MAC-d PDU  with a length up to the RNSAP Maximum MAC-d PDU Size Extended  IE value. In such a case, the amount of data exceeding the allocated capacity for the first HS-DSCH Interval shall be credited from the following HS-DSCH Intervals, if available.

In case of HS-DSCH Frame Protocol TYPE 1, multiple MAC-d PDUs of same length and same priority level (CmCH-PI) may be transmitted in one MAC-d flow in the same HS-DSCH DATA FRAME TYPE 1.

In case of HS-DSCH Frame Protocol TYPE 2, MAC-d PDUs with the same logical channel ID shall be associated to one unique priority level (CmCH-PI).

The HS-DSCH DATA FRAME (TYPE 1 and TYPE 2) includes a User Buffer Size IE to indicate the amount of data pending for the respective MAC-d flow for the indicated priority level. Within one priority level (CmCH-PI) and size the MAC-d PDUs shall be transmitted by the DRNS on the Uu interface in the same order as they were received from the SRNC.

If the Flush IE in the HS-DSCH DATA FRAME (TYPE 1 and TYPE 2) is set to "flush" the DRNS should remove all MAC-d PDUs from the corresponding MAC-hs Priority Queue that have been received prior to this data frame on the same transport bearer.

For the purpose of TNL Congestion Control on HSDPA, the Frame Sequence Number and the DRT IEs may be included by the SRNC depending on higher layer configuration.

### 5.1.6	E-DCH for CELL_FACH and Idle Data Transfer [FDD and 1.28Mcps TDD]

Figure 4AB: E-DCH for CELL_FACH and Idle Data Transfer procedure

The E-DCH for CELL_FACH and Idle Data Transfer procedure is used to transfer an E-DCH DATA FRAME from the DRNC to the SRNC.

### 5.1.7	Indication of UE State Transition [1.28Mcps TDD]

In 1.28Mcps TDD, for UE in CELL_PCH or URA_PCH states with stored H-RNTI, the network use the stored dedicated H-RNTI to inform the UE to establish the uplink synchronization via HS-SCCH before transmitting DCCH and DTCH to the UE. And when UE receives dedicated H-RNTI on HS-SCCH, the UE shall initiate the uplink synchronization procedure and transit to CELL_FACH state. The serving DRNC shall send the UE State Transition Indication to SRNC to inform that the UE’s state has been transited from CELL_PCH/URA_PCH to CELL_FACH.

The UE State Transition Indication is indicated in a user data frame with values set as follows:

-	The CFN and Subframe Number IE values shall reflect the time when the state transition was detected

-	The Number of MAC-is PDUs IE shall be set to zero. As a consequence there are no MAC-is PDU descriptor IE and  there are no MAC-is PDUs IEs in the payload part of the data frame related to UE state transition indication.

### 5.1.8	HS-DSCH UL Synchronization Establishment Failure [1.28Mcps TDD]

Figure 4AC: HS-DSCH UL Synchronization Establishment Failure procedure

In 1.28Mcps TDD, when the DRNC was notified by the Node B that the HS-DSCH data transimssion to the UE in CELL_FACH state is failed because of the uplink synchronization failure TS 25.224 [10], the DRNC could use HS-DSCH UL SYNCHRONIZATION ESTABLISHMENT FAILURE control frame to notify the SRNC to terminate the HS-DSCH data transimssion.

## 5.2	Flow Control

### 5.2.1	FACH Flow Control

Figure 4A: FACH Flow Control procedure

The FACH Flow Control procedure is used by the DRNC to control the user data flow. It may be generated in response to a FACH Capacity Request procedure or at any other time. The Credits IE indicates the number of MAC-c/sh SDUs the SRNC is allowed to transmit for the UE identified by the SRNTI IE and the associated priority level indicated by the Common Transport Channel Priority Indicator IE.

The Credits IE indicates the total amount of credits granted. Any credits previously granted are withdrawn.

If Credits IE = 0 (e.g. due to congestion in the DRNC), the SRNC shall immediately stop transmission of MAC-c/sh SDUs.

Credits IE = 'unlimited' indicates that the SRNC may transmit an unlimited number of MAC-c/sh SDUs.

### 5.2.2	DSCH Capacity Request [TDD]

Figure 5: DSCH Capacity Request procedure

The DSCH Capacity Request procedure provides means for the SRNC to request DSCH capacity by indicating the user buffer size in the SRNC for a given priority level.

The SRNC is allowed to reissue the DSCH Capacity Request if no CAPACITY ALLOCATION has been received within an appropriate time threshold.

### 5.2.3	DSCH Capacity Allocation [TDD]

Figure 6: DSCH Capacity Allocation procedure

DSCH Capacity Allocation procedure is generated within the DRNC. It may be generated either in response to a DSCH Capacity Request or at any other time.

The DRNC may use this message to modify the capacity at any time, irrespective of the reported user buffer status.

The DSCH CAPACITY ALLOCATION frame is used by the DRNC to control the user data flow. Credits IE indicates the number of MAC-c/sh SDUs that the SRNC is allowed to transmit for the UE and the associated priority level indicated by the Common Transport Channel Priority Indicator IE.

The Maximum. MAC- c/sh SDU length, Credits, Interval and Repetition Period IEs indicates the total amount of capacity granted. Any capacity previously granted is replaced.

If Credits IE = 0 (e.g. due to congestion in the DRNC), the SRNC shall immediately stop transmission of MAC-c/sh SDUs. If Credits IE = 255, the SRNC can transmit MAC-c/sh SDUs with unlimited capacity.

The IEs used in the DSCH CAPACITY ALLOCATION Control Frame are the Common Transport Channel Priority Indicator, Credits, Maximum MAC- c/sh SDU Length, Interval and the Repetition Period.

If the Repetition Period IE = 'unlimited repetition period' it indicates that the SRNC may transmit the specified number of MAC-c/sh SDUs for an unlimited period according to the bounds of Maximum MAC-c/sh SDU Length, Credits and Interval IEs.

### 5.2.4	FACH Capacity Request

Figure 6A: FACH Capacity Request procedure

The FACH Capacity Request procedure provides the means for the SRNC to notify the DRNC about the user buffer size for a given priority level. It may be sent if no FACH FLOW CONTROL frame has been received within an appropriate time threshold, or to signal an event such as data arrival or user buffer discard.

### 5.2.5	HS-DSCH Capacity Request

Figure 6B: HS-DSCH Capacity Request procedure

The HS-DSCH Capacity Request procedure provides means for the SRNC to request HS-DSCH capacity by indicating the user buffer size in the SRNC for a given priority level.

The SRNC is allowed to reissue the HS-DSCH Capacity Request if no CAPACITY ALLOCATION (TYPE 1 or TYPE 2) has been received within an appropriate time threshold.

### 5.2.6	HS-DSCH Capacity Allocation

Figure 6C: HS-DSCH Capacity Allocation procedure

HS-DSCH Capacity Allocation procedure is generated within the DRNC. It may be generated either in response to a HS-DSCH Capacity Request or at any other time.

The DRNC may use this message to modify the capacity at any time, irrespective of the reported user buffer status.

The HS-DSCH CAPACITY ALLOCATION (TYPE 1 and TYPE 2) Control Frame are used by the DRNC to control the user data flow. In case of HS-DSCH Frame Protocol TYPE 1, HS-DSCH Credits IE indicates the number of MAC-d PDUs that the SRNC is allowed to transmit for the MAC-d flow and the associated priority level indicated by the Common Transport Channel Priority Indicator IE. In case of HS-DSCH Frame Protocol TYPE 2, the HS-DSCH Credits IE multiplied by the Maximum MAC-d PDU Length Type 2 IE indicates the number of MAC-d PDU octets that the SRNC is allowed to transmit for the MAC-d flow and the associated priority level indicated by the Common Transport Channel Priority Indicator IE.

The Maximum MAC- d PDU length (in case of HS-DSCH Frame Protocol TYPE 1) or Maximum MAC- d PDU Length Type 2 (in case of HS-DSCH Frame Protocol TYPE 2), HS-DSCH Credits, HS-DSCH Interval and HS-DSCH Repetition Period IEs indicates the total amount of capacity granted. Any capacity previously granted is replaced.

If HS-DSCH Credits IE = 0 (e.g. due to congestion in the DRNC), the SRNC shall immediately stop transmission of MAC-d PDUs. If HS-DSCH Credits IE = 2047 in case of HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame or  65535 in case of HS-DSCH CAPACITY ALLOCATION TYPE 2 Control Frame, the SRNC can transmit MAC-d PDUs with unlimited capacity.

The IEs used in the HS-DSCH CAPACITY ALLOCATION (TYPE 1 and TYPE 2) Control Frame are the Common Transport Channel Priority Indicator, HS-DSCH Credits, Maximum MAC- d PDU Length (in case of HS-DSCH Frame Protocol TYPE 1) or Maximum MAC- d PDU Length Type 2 (in case of HS-DSCH Frame Protocol TYPE 2), HS-DSCH Interval and the HS-DSCH Repetition Period.

If the HS-DSCH Repetition Period IE = "unlimited repetition period" it indicates that the SRNC may transmit the amount of granted capacity for an unlimited period according to the bounds of Maximum MAC-d PDU Length IE (TYPE 1) or Maximum MAC- d PDU length Type 2 IE (TYPE 2), HS-DSCH Credits IE and HS-DSCH Interval IE.

## 5.2A	Node Synchronisation

The Node Synchronisation procedure is used by the SRNC to acquire information on the Node B timing.

The procedure is initiated by the SRNC by sending a DL NODE SYNCHRONISATION control frame containing the parameter T1 to the DRNS.

Upon reception of a DL NODE SYNCHRONISATION control frame, the DRNS shall respond with UL NODE SYNCHRONISATION control frame, including the parameters T2 and T3, as well as the T1 which was indicated in the initiating DL NODE SYNCHRONISATION control frame.

The T1, T2, T3 parameters are defined as:

-	T1: RNC specific frame number (RFN) that indicates the time when RNC sends the DL NODE SYNCHRONISATION control frame through the SAP to the transport layer.

-	T2: Node B specific frame number (BFN) that indicates the time when Node B receives the correspondent DL NODE SYNCHRONISZATION control frame through the SAP from the transport layer.

-	T3: Node B specific frame number (BFN) that indicates the time when Node B sends the UL NODE SYNCHRONISATION control frame through the SAP to the transport layer.

The general overview on the Node Synchronisation procedure is reported in TS 25.401 [5].

Figure 6D: Node Synchronisation procedure

## 5.3	General

### 5.3.1	DSCH / USCH transport bearer replacement [TDD]

As described in RNSAP (TS 25.423 [8]), transport bearer replacement can be achieved for a DSCH or USCH by using the Synchronised Radio Link Reconfiguration Preparation procedure in combination with the Synchronised Radio Link Reconfiguration Commit procedure. In both cases the following steps can be discerned:

1)	The new transport bearer is established after which 2 transport bearers exist in parallel.

2)	The transport channel(s) is/are switched to the new transport bearer.

3)	The old transport bearer is released.

In step 1), communication on the old transport bearer continues as normal.

In step 2), the moment of switching is determined as follows:

-	The DSCH DATA FRAMEs or USCH DATA FRAMEs shall be transported on the new transport bearer from the CFN indicated in the RADIO LINK RECONFIGURATION COMMIT message.

Starting from this CFN the RNCs shall support all the applicable Common Transport Channels frame protocol procedures on the new transport bearer and no requirements exist regarding support of Common Transport Channels frame protocol procedures on the old transport bearer.

Finally in step 3), the old transport bearer is released.

### 5.3.2	HS-DSCH Transport Bearer Replacement

As described in RNSAP (TS 25.423 [8]), transport bearer replacement can be achieved for a HS-DSCH MAC-d Flow by using the Synchronised Radio Link Reconfiguration Preparation procedure in combination with the Synchronised Radio Link Reconfiguration Commit procedure, or by using the Unsynchronised Radio Link Reconfiguration procedure. In both cases the following steps can be discerned:

1)	The new transport bearer is established after which 2 transport bearers exist in parallel.

2)	The HS-DSCH MAC-d Flow is switched to the new transport bearer.

3)	The old transport bearer is released.

HS-DSCH Transport Bearer Replacement, step 1:

Communication on the old transport bearer continues as normal. In addition, the DRNS shall support HS-DSCH DATA FRAMEs (TYPE 1 or TYPE 2), the HS-DSCH CAPACITY REQUEST Control Frame (see sub-clause 5.2.5) and may support the HS-DSCH CAPACITY ALLOCATION (TYPE 1 or TYPE 2) Control Frame (see sub-clause 5.2.6) on the new transport bearer. HS-DSCH DATA FRAMEs (TYPE 1 or TYPE 2) transported on the new transport bearer shall be transmitted on the Uu Interface in the same way as those received on the old transport bearer (see sub-clause 5.1.5).

The DRNS may use the old or the new transport bearer for the HS-DSCH CAPACITY ALLOCATION (TYPE 1 or TYPE 2) Control Frame. The HS-DSCH CAPACITY ALLOCATION (TYPE 1 or TYPE 2) Control Frame indicates the total amount of capacity granted for the MAC-d flow and the indicated priority level, irrespective of the transport bearer used. Any capacity previously granted is replaced.

The SRNC may use the old or the new transport bearer for the HS-DSCH CAPACITY REQUEST Control Frame. The rules for reissuing a HS-DSCH CAPACITY REQUEST Control Frame as outlined in sub-clause 5.2.5 still apply.

HS-DSCH Transport Bearer Replacement, step 2:

Regarding step 2), the moment of switching is determined as follows:

Starting from the CFN indicated in the RADIO LINK RECONFIGURATION COMMIT message – or directly when using the unsynchronised Radio Link Reconfiguration procedure, the DRNS shall support all applicable Common Transport Channels frame protocol procedures on the new transport bearer and no requirements exist regarding support of Common Transport Channels frame protocol procedures on the old transport bearer.

HS-DSCH Transport Bearer Replacement, step 3:

Finally in step 3), the old transport bearer is released.

## 5.4	HS-DSCH PDU DROP INDICATION

Figure 6E: HS-DSCH PDU DROP INDICATION procedure

The HS-DSCH PDU DROP INDICATION Control Frame is used by the DRNS to report to SRNC the PDU drop events. It is used when DRNS drops the PDUs. The content of the dropped PDU or Data Frame is available in the DRNS so in this case direct identification of the PDUs is possible.

# 6	Frame Structure and Coding

## 6.1	General

The general structure of a Common Transport Channel frame consists of a header and a payload. This structure is depicted in the figure 7.

| Header | Payload: Data or Control Information |
| --- | --- |

Figure 7: General Frame Structure

The header shall contain the frame type field and information related to the frame type.

There are two types of frames (indicated by the Frame Type field).

1.	Data frame.

2.	Control frame.

In the present document the structure of frames will be specified by using pictures similar to the following figure 8.

| 7 | 6 | 5 | 4 | 3 |  | 2 | 1 | 0 | Condition |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Field 1 |  |  |  |  |  |  | Field 2 |  |  | Byte 1 |
| Field 3 |  |  |  |  |  |  |  |  |  | Byte 2 |
| Field 3 (cont) |  |  |  |  | Field 4 |  |  |  |  | Byte 3 |
| Spare Extension |  |  |  |  |  |  |  |  |  |  |

Figure 8: Example frame structure

Unless otherwise indicated, fields which consist of multiple bits within a byte will have the more significant bit located at the higher bit position (indicated above frame in figure 8). In addition, if a field spans several bytes, more significant bits will be located in lower numbered bytes (right of frame in figure 8).

On the Iur interface, the frame will be transmitted starting from the lowest numbered byte. Within each byte, the bits are sent according decreasing bit position (bit position 7 first).

The Spare Extension indicates the location where new IEs can in the future be added in a backward compatible way.

The Spare Extension shall not be used by the transmitter and shall be ignored by the receiver.

Spare bits shall be set to 0 by the transmitter and shall be ignored by the receiver.

The parameters are specified giving the value range and the step (if not 1). The coding is done as follows (unless otherwise specified):

-	Lowest value (in the range) coded as a sequence of 0's;

-	Highest value in the range coded as a sequence of 1's.

## 6.2	Data Frame structure

### 6.2.1	RACH Channels

RACH Iur data stream corresponds to the data stream of one specific UE. The used transport bearer for the transport of FACH/RACH or FACH is bi-directional.

The RACH/FACH FP does not facilitate multiplexing of data streams from different UEs onto the same data frame, but does allow multiple UEs to share the same transport bearer.

The RACH DATA FRAME structure is defined as common for FDD and TDD with conditional fields.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header CRC |  |  |  |  |  |  | FT |  | Header |
| SRNTI |  |  |  |  |  |  |  |  |  |
| SRNTI (cont) |  |  |  |  |  |  |  |  |  |
| SRNTI (cont) |  |  |  | Spare bits 3-0 |  |  |  |  |  |
| Propagation Delay |  |  |  |  |  |  |  | Conditional FDD |  |
| Rx Timing Deviation |  |  |  |  |  |  |  | Conditional 3.84 Mcps and 7.68Mcps TDD |  |
| Received Synch Uplink Timing Deviation |  |  |  |  |  |  |  | Conditional 1.28Mcps TDD |  |
| MAC –c/sh SDU Length |  |  |  |  |  |  |  |  |  |
| MAC –c/sh SDU Length (cont) |  |  |  |  | Spare bits 2-0 |  |  |  |  |
| NoOfSDUs |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-c SDU-1 |  |  |  |  | Payload |
| MAC –c/sh SDU 1 (cont) |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-c SDU-n |  |  |  |  |  |
| MAC –c/sh SDU n (cont) |  |  |  |  |  |  |  |  |  |
| New IE fl 7(E) | New IE fl 6 | New IE fl 5 | New IE fl 4 | New IE fl 3 | New IE fl 2 | New IE fl 1 | New IE fl 0 |  |  |
| Spare bits7-6 |  | Cell Portion ID |  |  |  |  |  | Conditional FDD |  |
| Spare bits 7-1 for 3.84Mcps Spare bits 7-2 for 7.68Mcps |  |  |  |  |  | Rx Timing Deviation cont |  | Conditional 3.84 Mcps and 7.68Mcps TDD |  |
| Spare bits 7-2 |  |  |  |  |  | Ext Propagation Delay |  | Conditional FDD |  |
| Ext Propagation Delay |  |  |  |  |  |  |  | Conditional FDD |  |
| Cell Portion LCR ID |  |  |  |  |  |  |  | Conditional 1.28Mcps TDD |  |
| Extended S-RNTI |  |  |  |  |  |  |  | Conditional FDD |  |
| Extended S-RNTI (cont) |  |  |  |  |  |  |  |  |  |
| Extended S-RNTI (cont) |  |  |  |  |  | Spare bits 1-0 |  |  | Payload Tail |
| Spare Extension |  |  |  |  |  |  |  |  |  |
| Payload CRC |  |  |  |  |  |  |  |  |  |
| Payload CRC (cont) |  |  |  |  |  |  |  |  |  |

Figure 9: RACH DATA FRAME structure

Propagation delay is a conditional Information Element which is only present when the Cell supporting the RACH Transport Channel is a FDD Cell.

Rx Timing Deviation is a conditional Information Element which is only present when the Cell supporting the RACH Transport Channel is a 3.84 Mcps or 7.68 Mcps TDD Cell.

Received SYNC UL Timing Deviation is a conditional Information Element which is only present when the Cell supporting the RACH Transport Channel is a 1.28 Mcps TDD Cell.

With respect to new IEs, for which the presence is indicated by the New IE Flags IE, the Figure 9 is an example of how a frame is structured when all such new IEs are present. Note that non-presence of such a new IE changes the position of all subsequent IEs on octet level.

[FDD- Bit 0 of New IE Flags in RACH DATA FRAME indicates if a Cell Portion ID is present (1) or not present (0) in the byte (bits 0-5) following the New IE Flags IE.]

[FDD- Bit 1 of New IE Flags in RACH DATA FRAME indicates if the Ext Propagation Delay IE is present (1) or not present (0).]

[FDD - Bit 2 of New IE Flags in RACH DATA FRAME indicates if the Extended S-RNTI IE is present (1) or not present (0). If the Extended S-RNTI IE is included, the S-RNTI IE shall be igored by the receiver.]

[FDD - Bits 3 through 6 of New IE Flags in RACH DATA FRAME shall be set to 0.]

[FDD - Field length of Spare Extension IE in RACH DATA FRAME is 0-25 octets.]

[3.84 Mcps and 7.68 Mcps TDD - Bit 0 of New IE Flags in RACH DATA FRAME indicates if the extended bits of the Rx Timing Deviation are present (1) or not present (0) in the byte (bit 0 for 3.84 Mcps TDD, bits 0 and 1 for 7.68 Mcps TDD) following the New IE Flags IE. Bits 1 through 6 of New IE Flags in RACH DATA FRAME shall be set to 0. Field length of Spare Extension IE in RACH DATA FRAME is 0-30 octets.]

[1.28Mcps TDD - Bit 0 of New IE Flags in RACH DATA FRAME indicates if Cell Portion LCR IE is present (1) or not present (0)

[1.28Mcps TDD – Bits 1 through 6 of New IE Flags in RACH DATA FRAME shall be set to 0.]

[1.28Mcps TDD – Field length of Spare Extension IE in RACH DATA FRAME is 0-30 octets.]

### 6.2.2	FACH Channels

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header CRC |  |  |  |  |  |  | FT |  | Header |
| DRNTI |  |  |  |  |  |  |  |  |  |
| DRNTI (cont) |  |  |  |  |  |  |  |  |  |
| DRNTI (cont) |  |  |  | Cm CH-PI |  |  |  |  |  |
| Spare bits 7-6 |  | UE-ID Type | MAC –c/sh SDU Length |  |  |  |  |  |  |
| MAC –c/sh SDU Length (cont) |  |  |  |  |  |  |  |  |  |
| Num Of SDUs |  |  |  |  |  |  |  |  |  |
| User Buffer Size |  |  |  |  |  |  |  |  |  |
| User Buffer Size (cont) |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-c/sh SDU-1 |  |  |  |  | Payload |
| MAC –c/sh SDU 1 (cont) |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-c/sh SDU-n |  |  |  |  |  |
| MAC –c/sh SDU n (cont) |  |  |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |  |  |
| Payload CRC |  |  |  |  |  |  |  |  | Payload Tail |
| Payload CRC (cont) |  |  |  |  |  |  |  |  |  |

Figure 10: FACH DATA FRAME structure

### 6.2.3	USCH Channels [TDD]

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header CRC |  |  |  |  |  |  | FT |  | Header |
| MAC –c/sh SDU Length |  |  |  |  |  |  |  |  |  |
| MAC –c/sh SDU Length (cont) |  |  |  |  | Spare bits 2-0 |  |  |  |  |
| Num Of SDUs |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-c/sh SDU-1 |  |  |  |  | Payload |
| MAC –c/sh SDU 1 (cont) |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-c/sh SDU-n |  |  |  |  |  |
| MAC –c/sh SDU n (cont) |  |  |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |  |  |
| Payload CRC |  |  |  |  |  |  |  |  | Payload Tail |
| Payload CRC (cont) |  |  |  |  |  |  |  |  |  |

Figure 11: USCH DATA FRAME structure

### 6.2.4	DSCH Channels [TDD]

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header CRC |  |  |  |  |  |  | FT |  | Header |
| Spare bits 7-4 |  |  |  | CmCH-PI |  |  |  |  |  |
| MAC –c/sh SDU Length |  |  |  |  |  |  |  |  |  |
| MAC –c/sh SDU Length (cont) |  |  |  |  | Spare bits 2-0 |  |  |  |  |
| Num Of SDUs |  |  |  |  |  |  |  |  |  |
| User Buffer Size |  |  |  |  |  |  |  |  |  |
| User Buffer Size (cont) |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-c/sh SDU-1 |  |  |  |  | Payload |
| MAC –c/sh SDU 1 (cont) |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-c SDU-n |  |  |  |  |  |
| MAC –c/sh SDU n (cont) |  |  |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |  |  |
| Payload CRC |  |  |  |  |  |  |  |  | Payload Tail |
| Payload CRC (cont) |  |  |  |  |  |  |  |  |  |

Figure 12: DSCH DATA FRAME structure

### 6.2.4A	HS-DSCH Channels

Two types of HS-DSCH DATA FRAME exist for the HS-DSCH data transfer, i.e. HS-DSCH DATA FRAME TYPE 1 and HS-DSCH DATA FRAME TYPE 2.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header CRC |  |  |  |  |  |  | FT |  | Header |
| FSN |  |  |  | CmCH-PI |  |  |  |  |  |
| MAC –d PDU Length |  |  |  |  |  |  |  |  |  |
| MAC –c/sh SDU Length (cont) |  |  |  |  | Flush | FSN/DRT Reset | CPC Recov |  |  |
| Num Of PDUs |  |  |  |  |  |  |  |  |  |
| User Buffer Size |  |  |  |  |  |  |  |  |  |
| User Buffer Size (cont) |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | MAC-d PDU-1 |  |  |  |  | Payload |
| MAC –d PDU 1 (cont) |  |  |  |  |  |  | Pad |  |  |
| Spare bits 7-4 |  |  |  | MAC-d PDU-n |  |  |  |  |  |
| MAC –d PDU n (cont) |  |  |  |  |  |  | Pad |  |  |
| New IE fl 7(E) | New IE fl 6 | New IE fl 5 | New IE fl 4 | New IE fl 3 | New IE fl 2 | New IE fl 1 | New IE fl 0 |  |  |
| DRT |  |  |  |  |  |  |  |  |  |
| DRT (cont) |  |  |  |  |  |  |  |  |  |
| SN |  |  |  |  |  |  |  |  |  |
| SN(cont) |  |  |  |  |  |  | S/D |  |  |
| SN |  |  |  |  |  |  |  |  |  |
| SN(cont) |  |  |  |  |  |  | S/D |  |  |
| Spare Extension |  |  |  |  |  |  |  |  |  |
| Payload CRC |  |  |  |  |  |  |  |  | Payload Tail |
| Payload CRC (cont) |  |  |  |  |  |  |  |  |  |

Figure 12A: HS-DSCH DATA TYPE 1 FRAME structure

Bit 0 of New IE Flags in HS-DSCH DATA FRAME TYPE 1 indicates if a DRT is present (1) or not (0) in the 2 octets following the New IE Flags IE. Bit 1 of New IE Flags in HS-DSCH DATA FRAME TYPE1 indicates if SN is present (1) or not (0) in the third and the forth octets following the New IE Flags IE. Bit 0 in the forth octet is allocated for IE S/D. Bit 2 of New IE Flags in HS-DSCH DATA FRAME TYPE1 indicates if SN is present (1) or not (0) in the fifth and the sixth octets following the New IE Flags IE. Bit 0 in the sixth octet is allocated for IE S/D. Bits 3 through 6 of New IE Flags in HS-DSCH DATA FRAME TYPE 1 shall be set to 0.

Field length of Spare Extension IE in HS-DSCH DATA FRAME TYPE 1 is 0-25 octets.

| 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header CRC |  |  |  |  |  |  |  | FT |  | Header |
| Frame Seq Nr |  |  |  | CmCH-PI |  |  |  |  |  |  |
| Total Num of PDU blocks |  |  |  |  | Flush |  | FSN/DRT Reset | DRT Indicator |  |  |
| FI | PI | eDRX Indicator | TS0 Indicator | MI | CPC Recov |  | FeDRX Indicator | NTHT |  |  |
| User Buffer Size |  |  |  |  |  |  |  |  |  |  |
| User Buffer Size (cont) |  |  |  |  |  |  |  |  |  |  |
| MAC-d PDU length in block 1 |  |  |  |  |  |  |  |  |  |  |
| M len 1 (cont) |  |  | Spare | #PDUs in block 1 |  |  |  |  |  |  |
| Logical CH ID block1 |  |  |  | MAC-d PDU length in block 2 |  |  |  |  |  |  |
| M len 2 (cont) |  |  |  |  |  |  |  | Spare |  |  |
| #PDUs in block2 |  |  |  | Logical Ch ID in block2 |  |  |  |  |  |  |
| MAC –d PDU length in block n |  |  |  |  |  |  |  |  |  |  |
| M len n (cont) |  |  | Spare | #PDUs in block n |  |  |  |  |  |  |
| Logical CH ID block n |  |  |  | Padding if n is odd |  |  |  |  |  |  |
| DRT |  |  |  |  |  |  |  |  |  | Payload |
| DRT (cont) |  |  |  |  |  |  |  |  |  |  |
| D-RNTI |  |  |  |  |  |  |  |  |  |  |
| D-RNTI (cont) |  |  |  |  |  |  |  |  |  |  |
| D-RNTI (cont) |  |  |  | UE-ID Type | DRX Cycle Length Coefficient |  |  |  |  |  |
| MAC-d Type 2 PDU 1 |  |  |  |  |  |  |  |  | First block of PDUs with the same size |  |
| MAC-d Type 2 PDU 1 (cont) |  |  |  |  |  |  |  |  |  |  |
| MAC-d Type 2 PDU m1 |  |  |  |  |  |  |  |  |  |  |
| MAC-d Type 2 PDU m1 (cont) |  |  |  |  |  |  |  |  |  |  |
| MAC-d Type 2 PDU 1 |  |  |  |  |  |  |  |  | Nth block of PDUs with the same size |  |
| MAC-d Type 2 PDU mn (cont) |  |  |  |  |  |  |  |  |  |  |
| New IE fl 7(E) | New IE fl 6 | New IE fl 5 | New IE fl 4 | New IE fl 3 | New IE fl 2 |  | New IE fl 1 | New IE fl 0 |  |  |
| SN |  |  |  |  |  |  |  |  |  |  |
| SN (cont) |  |  |  |  |  |  |  | S/D |  |  |
| SN |  |  |  |  |  |  |  |  |  |  |
| SN (cont) |  |  |  |  |  |  |  | S/D |  |  |
| CFN |  |  |  |  |  |  |  |  |  |  |
| Spare bits 7-2 |  |  |  |  |  | OPCE Ind |  | CR Ind |  |  |
| Spare Extension |  |  |  |  |  |  |  |  |  |  |
| Payload CRC |  |  |  |  |  |  |  |  |  | Payload Tail |
| Payload CRC (cont) |  |  |  |  |  |  |  |  |  |  |

Figure 12B: HS-DSCH DATA FRAME TYPE 2 structure

Bit 0 of New IE Flags in HS-DSCH DATA FRAME TYPE 2 indicates if  SN is present (1) or not (0) in the following 2 octets following the New IE Flags IE. Bit 0 in the 2nd octet is allocated for IE S/D. Bit 1 of New IE Flags in HS-DSCH DATA FRAME TYPE2 indicates if SN is present (1) or not (0) in the 3rd and the 4th octets following the New IE Flags IE. Bit 0 in the 4th octet is allocated for IE S/D.

Bit 2 of New IE Flags in HS-DSCH DATA FRAME TYPE 2 indicates if a CFN is present (1) or not (0) in the fifth octet following the New IE Flags IE.

Bit 3 of New IE Flags in HS-DSCH DATA FRAME TYPE 2 indicates if a Cell Reselection Indication (CR Ind) IE is present (1) or not (0) in the sixth octet following the New IE Flags IE.

Bit4 of New IE Flags in HS-DSCH DATA FRAME TYPE 2 indicates if the Operation Preference of Common E-DCH resource Indication (OPCE Ind) is present (1) or not (0). Bits 5 through 6 of New IE Flags in HS-DSCH DATA FRAME TYPE 2 shall be set to 0.

Field length of Spare Extension IE in HS-DSCH DATA FRAME TYPE 2 is 0-26 octets.

### 6.2.4B	E-DCH for CELL_FACH and Idle Channels [FDD and 1.28Mcps TDD]

The structure of the E-DCH DATA FRAME is shown in Figure 12C. This frame structure is used for carrying MAC-is PDUs  in Cell_FACH and Idle state (TS 25.321 [9]) for one UE.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Header CRC |  |  |  |  |  |  | FT |  | Frame Header |
| Header CRC (cont) |  |  |  | FSN |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | Number of subframes |  |  |  |  |  |
| CFN |  |  |  |  |  |  |  |  |  |
| E-DCH User Buffer Size (eUB Size) |  |  |  |  |  |  |  |  |  |
| E-DCH User Buffer Size (eUB Size) |  |  |  |  |  |  |  |  |  |
| eUB Size |  | Spare bits 5-4 |  | Nr of MAC-is SDU in frame |  |  |  |  |  |
| Nr of MAC-is SDU in frame(cont) |  |  |  |  |  |  |  |  |  |
| Spare bits 7-4 |  |  |  | SRNTI |  |  |  |  |  |
| SRNTI (cont) |  |  |  |  |  |  |  |  |  |
| SRNTI (cont) |  |  |  |  |  |  |  |  |  |
| Spare | 1st Subframe number |  |  | N of MAC-is PDUs |  |  |  |  |  |
| Spare | 2nd Subframe number |  |  | N of MAC-is PDUs |  |  |  |  |  |
| Spare | Last Subframe number |  |  | N of MAC-is PDUs |  |  |  |  |  |
| MAC-is PDU descriptor of 1st MAC-is PDU of 1st subframe |  |  |  |  |  |  |  |  |  |
| MAC-is PDU descriptor of 2nd MAC-is PDU of 1st subframe |  |  |  |  |  |  |  |  |  |
| MAC-is PDU descriptor of last MAC-is PDU of 1st subframe |  |  |  |  |  |  |  |  |  |
| MAC-is PDU descriptor of 1st MAC-is PDU of last subframe |  |  |  |  |  |  |  |  |  |
| MAC-is PDU descriptor of last MAC-is PDU of last subframe |  |  |  |  |  |  |  |  |  |
| First MAC-is PDU of first subframe |  |  |  |  |  |  |  |  | Frame Payload |
| Last MAC-is PDU of first subframe |  |  |  |  |  |  |  |  |  |
| First MAC-is PDU of last subframe |  |  |  |  |  |  |  |  |  |
| Last MAC-is PDU of last subframe |  |  |  |  |  |  |  |  |  |
| New IE fl 7(E) | New IE fl 6 | New IE fl 5 | New IE fl 4 | New IE fl 3 | New IE fl 2 | New IE fl 1 | New IE fl 0 |  |  |
| Spare bits 7-2 |  |  |  |  |  | CRR Rec | TTI Length |  |  |
| Extended S-RNTI |  |  |  |  |  |  |  | Conditional FDD |  |
| Extended S-RNTI(cont) |  |  |  |  |  |  |  |  |  |
| Extended S-RNTI (cont) |  |  |  |  |  | Spare bits 1-0 |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |  | Optional |
| Payload CRC |  |  |  |  |  |  |  |  |  |
| Payload CRC (cont) |  |  |  |  |  |  |  |  |  |

Figure 12C: E-DCH DATA FRAME structure

For the description of the fields see subclause 6.2.5.

Bit 0 of New IE Flags in E-DCH DATA FRAME indicates if a TTI length is present (1) or not (0).

Bit 1 of New IE Flags in E-DCH DATA FRAME indicates if Cell Reselection Request Received (CRR Rec) IE is present (1) or not (0).

Bit 2 of New IE Flags in E-DCH DATA FRAME indicates if Extended S-RNTI IE is present (1) or not (0). If the Extended S-RNTI IE is included, the S-RNTI IE shall be igored by the receiver. Bits 3 through 6 of New IE Flags in E-DCH DATA FRAME shall be set to 0.

The Payload CRC IE is optional in frames that contain a Payload, i.e. the whole 2 bytes field may or may not be present in the frame structure (this is defined at the setup of the transport bearer). The Payload CRC IE may only be present if the E-DCH data frame contains payload.

### 6.2.5	Coding of Information Elements in DATA FRAMEs

#### 6.2.5.1	Header CRC

Description: Cyclic Redundancy Checksum calculated on the header of a data frame with polynomial X^7+X^6+X^2+1 for the 7 bit header CRC [FDD and 1.28Mcps TDD - and X^11+X^9+X^8+ X^2+X+1 for the 11 bit header CRC]. The CRC calculation shall cover all bits in the header, starting from bit 0 in the first byte (FT field) up to the end of the header.

Value range: {0-127} [FDD and 1.28Mcps TDD – , {0..2047} for E-DCH DATA FRAME].

Field length: 7 bits [FDD and 1.28Mcps TDD - , 11 bits for E-DCH DATA FRAME].

#### 6.2.5.2	Frame Type (FT)

Description: Describes if it is a control frame or a data frame.

Value range: {0=data, 1=control}.

Field Length: 1 bit.

#### 6.2.5.3	DRNTI

Description: Identifies the UE in the DRNC.

Value range: {0-1048575}.

Field length: 20 bits.

#### 6.2.5.4	S-RNTI

Description: S-RNTI is defined in TS 25.401 [5]. S-RNTI is used in UL control frames [FDD and 1.28Mcps TDD - and E-DCH data frames] to identify the UE context in the SRNC.

Value range: {0-1048575}.

Field length: 20 bits.

#### 6.2.5.5	UE-ID Type Indicator (UE-ID Type)

Description: Indicates the UE Identifier Type to be included by MAC-c/sh in the MAC header.

Value range: {0=U-RNTI, 1=C-RNTI}.

Field Length: 1 bit.

#### 6.2.5.6	S-CCPCH Indicator (S-CI)

Void.

#### 6.2.5.7	Common Transport Channel Priority Indicator (CmCH-PI)

Description: CmCH-PI is the relative priority of the data frame and the SDUs included.

Value range: {0-15, where 0=lowest priority, 15=highest priority}.

Field length: 4 bits.

#### 6.2.5.8	MAC-c/sh SDU Length

Description: The value of that field indicates the length of every MAC-c/sh SDU in the payload of the [TDD - DSCH, USCH and] FACH DATA FRAME in number of bits.

Value range: {0-5000}.

Field Length: 13 bits.

#### 6.2.5.9	NumOfSDU

Description: Indicates the number of MAC-c/sh SDUs in the payload.

Value range: {1-255}.

Field Length: 8 bits.

#### 6.2.5.10	Propagation delay [FDD]

Description: One-way radio interface delay as measured during RACH access. If the measured value exceeds the range of this information element, the information element shall be set to its maximum value, and the Ext Propagation Delay IE shall be used to represent the measured value, see subclause 6.2.5.10A.

Value range: {0 - 765 chips}.

Granularity: 3 chips.

Field length: 8 bits.

#### 6.2.5.10A	Ext Propagation Delay [FDD]

Description: One-way radio interface delay as measured during RACH access, extended value part. This IE shall be present only if the range of the Propagation Delay IE is insufficient to represent the measured value.

Value range: {0 - 3069 chips}.

Granularity: 3 chips.

Field length: 10 bits.

#### 6.2.5.11	Rx Timing Deviation [3.84Mcps TDD]

Description: Measured Rx Timing Deviation as a basis for timing advance.

Value range: {-1024, ..., +1023} chips

{N*4 - 256} chips  RxTiming Deviation < {(N+1)*4 - 256} chips

With N = 0, 1, ...,127

{(N-128)*4 - 1024} chips  Rx Timing Deviation < {(N-127)*4 - 1024} chips

With N = 128, 129, ...,319

{N*4 - 1024} chips  Rx Timing Deviation < {(N+1)*4 - 1024} chips

With N = 320, 321, ...,511

Granularity: 4 chips.

Field length: 9 bits. The least significant 8 bits are contained in the RX timing deviation field and the most significant bit is contained in the RX timing deviation (continuation) field.

#### 6.2.5.11A	Received SYNC UL Timing Deviation [1.28Mcps TDD]

Description: Measured Received SYNC UL Timing Deviation as a basis for propagation delay.

Value range: {0, ..., +255} chips

Granularity: 1 chip.

Field length: 8 bits.

#### 6.2.5.11B	Rx Timing Deviation [7.68Mcps TDD]

Description: Measured Rx Timing Deviation as a basis for timing advance.

Value range: {-2056, ..., +2055} chips

{N*4 - 2056 } chips  RxTiming Deviation < {(N+1)*4 - 2056 } chips

With N = 0, 1, ..., 1027

Granularity: 4 chips.

Field length: 10 bits. The least significant 8 bits are contained in the RX timing deviation field and the most significant 2 bits are contained in the RX timing deviation (continuation) field.

#### 6.2.5.12	User Buffer Size

Description: Indicates the users' buffer size (i.e. the amount of data in the buffer) in octets for a given Common Transport Channel Priority Indicator level.

Value range: {0-65535}.

Field length: 16 bits.

#### 6.2.5.13	MAC-c/sh SDU

Description: A MAC-c/sh SDU contains the C/T IE field (TS 25.321 [7]) of the MAC header followed by one RLC PDU.Field length: See the value of the MAC-c/sh SDU Length IE.

#### 6.2.5.14	Payload CRC

Description: Cyclic Redundancy Checksum calculated on the payload of a data frame with polynomial X^16+X^15+X^2+1. The CRC calculation shall cover all bits in the data frame payload, starting from bit 7 in the first byte up to bit 0 in the byte before the payload CRC.

Field length: 16 bits.

#### 6.2.5.15	Spare Extension

Description: Indicates the location where new IEs can in the future be added in a backward compatible way.

Field length: 0-32 octets.

#### 6.2.5.16	MAC-d PDU Length

Description: The value of that field indicates the length of every MAC-d PDU in the payload of the HS-DSCH DATA FRAME in number of bits.

Value range: {0-5000}.

Field Length: 13 bits.

#### 6.2.5.17	NumOfPDU

Description: Indicates the number of MAC-d PDUs in the payload.

Value range: {1-255}.

Field Length: 8 bits.

#### 6.2.5.18	MAC-d PDU

Description: A MAC-d PDU contains the MAC-d PDU as defined in TS 25.321 [9].

Field length: See the value of the MAC-d PDU Length IE.

#### 6.2.5.19	Cell Portion ID [FDD]

Description: Cell Portion ID indicates the cell portion with highest SIR during RACH access. Cell Portion ID is configured by O&M.

Value range: {0-63}.

Field Length: 6 bits.

#### 6.2.5.20	New IE Flags

Description: The New IE Flags IE is only present if at least one new IE is present. The New IE Flags IE contains flags indicating which new IEs that are present following the New IE Flags IE. The last bit position of the New IE Flags IE is used as the Extension Flag to allow the extension of the New IE Flags IE in the future. Extension octets of the New IE Flags IE shall follow directly after the first octet of the New IE Flags IE. When an extension octet of the New IE Flags IE is present, then all previous extension octets of the New IE Flags IE and the New IE Flags IE shall also be present, even if they have all their flag bits indicating no presence of their respective new IEs.

Value range:

Bit 0-6 of each octet:	Indicates if a new IE is present (1) or not present (0) in the bytes following the New IE Flags IE. The meaning of each bit is explained in the corresponding DATA FRAME subclause;

Bit 7 of each octet:	Indicates if an extension octet of the New IE Flags IE follows (1) or not (0).

Field length: 1 – 31 octets.

#### 6.2.5.21	Flush

Description: Indicates whether the DRNS should remove (1) or not (0) all the MAC-d PDUs from the corresponding MAC-hs Priority Queue that have been received prior to this HS-DSCH DATA FRAME on the same transport bearer.

Value range: {0 = no flush, 1 = flush}.

Field Length: 1 bit.

#### 6.2.5.22	DRT (Delay Reference Time)

Description: DRT is a 16-bit Delay Reference Time. DRT can be used for dynamic delay measurements. The DRT counter bridges the same time span as RFN and BFN. DRT is locked to RFN in SRNC and is a 40960 counter with 1 ms resolution.

Value range: {0..40959DEC ms (0..9FFFHEX ms)}.

Granularity: 1 ms.

Field length: 16 bits.

#### 6.2.5.23	Frame Sequence Number

Description: The 4-bit Frame Sequence Number (FSN) is incremented for each transmitted HS-DSCH data frame belonging to one MAC-d flow [FDD and 1.28Mcps TDD – or E-DCH DATA FRAME belonging to one E-DCH for CELL_FACH and Idle data stream]. At wraparound of the Frame Sequence Number, the value "0" shall not be used. Each flow generates its own Frame Sequence Number.

Value range:

0	is a special value and indicates that the Frame Sequence Number IE shall be treated as spare.

1 – 15	indicates the Frame Sequence Number.

Granularity: 1.

Field length: 4 bits.

#### 6.2.5.24	Logical Channel ID in block n

Description: This field provides identification of the logical channel instance associated with the PDUs of the n-th block of PDUs with the same size in the HS-DSCH DATA FRAME TYPE 2. Multiple logical channels may be carried on the same Transport Network flow.

Value range: {0-15}, where 0-14 identifies logical channels 1-15, 15 identifies BCCH or PCCH logical channel mapped on HS-DSCH.

Field length: 4 bits.

#### 6.2.5.25	Total Number of PDU blocks

Description: The field indicates the number of blocks of block of PDUs with the same size in this HS-DSCH DATA FRAME.

Value range: {0-31}, 0 – not used.

Field length: 5 bits.

#### 6.2.5.26	MAC-d PDU length in block n

Description: The value of this field indicates the length of every MAC-d PDU in the n-th block of PDUs with the same size in number of octets.

Value range: {0-1505}, 0 – not used.

Field length: 11 bits.

#### 6.2.5.27	Number of MAC-d PDUs in block n (#PDUs in block n)

Description: Indicates the number of MAC-d PDUs in the n-th block of PDUs with the same size.

Value range: {0-15}, 0 – not used.

Field length: 4 bits.

#### 6.2.5.28	DRT Indicator

Description: Indicates whether a DRT is present.

Value range: {0 = DRT not present, 1= DRT present}.

Field length: 1 bit.

#### 6.2.5.29	FACH Indicator (FI) [FDD and 1.28Mcps TDD]

Description: Indicates whether D-RNTI and UE-ID Type Indicator are present (i.e. whether UE in CELL_FACH).

Value range: {0 = D-RNTI and UE-ID Type Indicator not present, 1= D-RNTI and UE-ID Type Indicator present}.

Field length: 1 bit.

NOTE:	In case FI is set to 1, PI shall be set to 0

#### 6.2.5.30	S-RNTI [FDD and 1.28Mcps TDD]

Void.

#### 6.2.5.31	UE-ID Type Indicator (UE-ID Type)

Void.

#### 6.2.5.32	FSN/DRT Reset

Description: When the Node B receives a HS-DSCH DATA FRAME where the 1-bit FSN/DRT Reset IE is set to 1, the Node B should reset any state of congestion estimation based on previously received FSN and DRT values. Node B may instead decide to start a new estimation of congestion detection initiated with the FSN and DRT values included in this HS-DSCH data frame. FSN/DRT Reset IE set to 1 may indicate a discontinuity in the sequence of the transmitted DRT and FSN values in the transmitted HS-DSCH data frames belonging to the associated MAC-d flow.

If the 1-bit FSN/DRT Reset IE is set to 0, Node B may use the included DRT and FSN values for congestion detection.

Value range:

0	Node B may use the included DRT and FSN values for congestion detection.

1	Node B should not use previously received FSN and DRT values for congestion detection.

Field length: 1 bit.

#### 6.2.5.33	MAC-d Type 2 PDU

Description: A MAC-d Type 2 PDU contains the MAC-ehs SDU [FDD and 1.28Mcps TDD – or MAC-c SDU] as defined in TS 25.321 [9].

Field length: For length of MAC-d Type 2 PDU of block n, see the value of the MAC-d PDU length in block n IE.

#### 6.2.5.34	PCH Indicator (PI) [FDD]

Description: Indicates whether the D-RNTI and DRX Cycle Length Coefficient are present(i.e. whether UE in CELL/URA_PCH).

Value range: {0 = D-RNTI and DRX Cycle Length Coefficient not present, 1= D-RNTI and DRX Cycle Length Coefficient present}.

Field length: 1 bit.

NOTE:	In case PI is set to 1, FI shall be set to 0.

#### 6.2.5.35	DRX Cycle Length Coefficient [FDD]

Description: The DRX Cycle Length Coefficient is used as input for the formula to establish the paging occasions to be used in DRX.

DRX Cycle Length Coefficient is given in the unit DRX_CYCLE_LENGTH_COEFFICIENT where:

DRX_CYCLE_LENGTH_COEFFICIENT = 0		Reserved

DRX_CYCLE_LENGTH_COEFFICIENT = 1		DRX Cycle Length Coefficient = 3

DRX_CYCLE_LENGTH_COEFFICIENT = 2		DRX Cycle Length Coefficient = 4

...

DRX_CYCLE_LENGTH_COEFFICIENT = 6		DRX Cycle Length Coefficient = 8

DRX_CYCLE_LENGTH_COEFFICIENT = 7		DRX Cycle Length Coefficient = 9

Value range: {3…9}.

Field length: 3 bits.

#### 6.2.5.36	Number of Subframes [FDD and 1.28Mcps TDD]

Description: The Number of Subframes field indicates how many subframes that follows in the frame.

NOTE:	A subframe has both a header portion and a payload portion in the frame.

Value range: {1-16}

The binary coding is derived from the value minus 1. E.g. value 1 is coded as binary “0000” and value 16 is coded as binary “1111”.

Values {11, 12, 13, 14, 15, 16}: Reserved in this user plane revision. Shall be ignored by the receiver.

Field length: 4 bits.

#### 6.2.5.37	Number of MAC-is SDU in frame [FDD and 1.28Mcps TDD]

Description: Total number of MAC-is SDUs in all MAC-is PDUs in the E-DCH DATA FRAME.

Value range: {0-4095}.

Field length: 12 bits.

#### 6.2.5.38	Number of MAC-is PDUs [FDD and 1.28Mcps TDD]

Description: Indicates the number of MAC-is PDUs in the user data frame in the payload part for the corresponding subframe number.

Value range: {0-15}

Field length: 4 bits.

#### 6.2.5.39	Subframe Number [FDD and 1.28Mcps TDD]

Description: Indicates the subframe number in which the payload was received.

Value range: {0-4}

Field length: 3 bits.

#### 6.2.5.40	E-DCH User Buffer Size [FDD and 1.28Mcps TDD]

Description: Indicates the total size of the E-DCH DATA FRAME in octets

Value range: {0-262 140}

Field length: 18 bits.

#### 6.2.5.41	MAC-is PDU descriptor [FDD and 1.28Mcps TDD]

Description: The MAC-is PDU descriptor contains the Length (L), Logical channel identifier (LCH-ID) and Flag (F) fields mapped directly from the "MAC-i Header n" field (n>0) received over the Uu (TS 25.321 [9]).

Field length: variable; length of MAC-is PDU descriptor in octets = 2  * number of MAC-is SDU contained in the corresponding MAC-is PDU as described in TS 25.321 [9].

#### 6.2.5.42	eDRX Indication [FDD]

Description: Indicates whether the UE in Cell_FACH state supports Enhanced UE DRX or not.

Value range: {0 = Enhanced UE DRX not supported, 1= Enhanced UE DRX supported}.

Field length: 1 bit.

#### 6.2.5.43	Cell Portion LCR ID [1.28Mcps TDD]

Description: Cell Portion LCR ID indicates the cell portion with best received quality during RACH access. Cell Portion LCR ID is configured by O&M.

Value range: {0-255}.

Field Length: 8 bits.

#### 6.2.5.44	TS0 Indication [1.28Mcps TDD]

Description: Indicates whether the UE supports TS0 enhancement or not.

Value range: {0 = Enhanced TS0 not supported, 1= Enhanced TS0 supported}.

Field length: 1 bit.

#### 6.2.5.45	MU-MIMO Indication(MI) [1.28Mcps TDD]

Description: Indicates whether the UE in Cell_FACH state supports MU-MIMO or not.

Value range: {0 = MU-MIMO not supported, 1= MU-MIMO supported}.

Field length: 1 bit.

#### 6.2.5.46	CPC Recovery (CPC Recov) [FDD]

Description: Describes if UE is CPC non-uniform related to HS-SCCH order and CPC recovery is needed in the serving DRNS. It indicates that the dynamic DTX/DRX (de)activation using HS-SCCH order is needed.

Value range: {0=no Recovery, 1=Recovery}.

Field Length: 1 bit.

Note: This is not applicable when HS-SCCH order is not used.

#### 6.2.5.47	FeDRX Indication [FDD]

Description: Indicates whether the UE in Cell_FACH state supports Further Enhanced UE DRX or not.

Value range: {0 = Further Enhanced UE DRX not supported, 1= Further Enhanced UE DRX supported}.

Field length: 1 bit.

#### 6.2.5.48	NodeB Triggered HS-DPCCH Transmission(NTHT) [FDD]

Description: Indicates whether the UE in Cell_FACH and CELL_PCH state supports NodeB Triggered HS-DPCCH Transmission or not.

Value range: {0 = NodeB Triggered HS-DPCCH Transmission not supported, 1= NodeB Triggered HS-DPCCH Transmission supported}.

Field length: 1 bit.

#### 6.2.5.49	TTI length [FDD]

Description: Indicates which type of TTI length UE in Cell_FACH and Idle state is in use for 2ms/10ms concurrent deployment.

Value range: {0 = 2ms, 1= 10ms}.

Field length: 1 bit.

#### 6.2.5.50	SN, Frame Number

Description: SN is a sequence number assigned to each HS-DSCH data frame by RNC and can be used by Node B to identify the set of MAC-d PDU’s sent in the frame. This is also used by RNC to indicate the HS-DSCH data frame that the Node B can discard.

Value range: {0…32767}.

Field length: 15 bits.

#### 6.2.5.51	S/D, Store/Discard indicator

Description: The S/D indicator indicates if Node B can store or discard data frame associated with SN.

Value range: {0 = Discard data associated with SN, 1 = Store and associate MAC-d PDUs in the frame with SN}.

Field length: 1 bit

#### 6.2.5.52	Connection Frame Number (CFN)

Description: The CFN corresponding to the SFN of the frame in which the payload was received on Uplink. If the payload was received in several frames, the CFN corresponding to the last Uu frame in which the information was received shall be indicated.

Value range: (other): {0..255}.

Field length: (other): 8 bits.

#### 6.2.5.53	CR Ind (Cell Reselection Indication)

Description: Indicates that the Node B may release the common E-DCH resource due to a cell reselection indication.

Value range: {0 = Release common E-DCH resource not permitted due to a cell reselection indication, 1 = Release common E-DCH resource permitted due to a cell reselection indication}.

Field length: 1 bit.

#### 6.2.5.54	CRR Rec (Cell Reselection Request Received)

Description: Indicates that the Node B has received Cell Reselection Request from UE or not.

Value range: {0 = Cell Reselection Request from UE not received, 1 = Cell Reselection Request from UE received}.

Field length: 1 bit.

#### 6.2.5.55	Extended S-RNTI

Description: Extended S-RNTI is defined in TS 25.401 [5]. Extended S-RNTI is used in UL control frames [FDD and E-DCH data frames] to identify the UE context in the SRNC.

Value range: {0-4194303}.

Field length: 22 bits.

#### 6.2.5.56	Operation Preference of Common E-DCH resource Indication (OPCE Ind) [FDD]

Description: The OPCE Ind indicates there is a preference for the DRNC’s Node B to trigger acquisition of the common E-DCH resource or to release the allocated common E-DCH resource for the particular UE. In case of NTHT is not set or set to zero, the OPCE Ind bit is not used.

Value range: {0 = Preference to release the allocated common E-DCH resource, 1 = Preference to trigger acquisition of the common E-DCH resource}.

Field length: 1 bit.

## 6.3	Control Frame structure

### 6.3.1	Introduction

Control Frames are used to transport control information between SRNC and DRNC.

Figure 13 defines the Control Frame structure for common transport channels.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Control Frame CRC |  |  |  |  |  |  | FT |  | Header |
| Control Frame Type |  |  |  |  |  |  |  |  |  |
| Control Information |  |  |  |  |  |  |  |  | Payload |
| Control Information (cont) |  |  |  |  |  |  |  |  |  |

Figure 13: Iur Common Transport Channel Control Frame Format

The Control Frame Type IE defines the type of the control frames.

The length of the payload is variable accordingly to the control frame type.

The structure of the header and the payload of the control frames is defined in the following subclauses.

### 6.3.2	Header structure of the control frames

#### 6.3.2.1	Control frame CRC

Description: Cyclic Redundancy Checksum calculated on a control frame with polynomial X^7+X^6+X^2+1. The CRC calculation shall cover all bits in the control frame, starting from bit 0 in the first byte (FT field) up to the end of the control frame.

Value range: {0-127}.

Field length: 7 bits.

#### 6.3.2.2	Frame type (FT)

Refer to subclause 6.2.5.2.

#### 6.3.2.3	Control Frame Type

Description: Indicates the type of the control information (information elements and length) contained in the payload (=type of control frame).

Value: values of the Control Frame Type IE parameter are defined in the table 1.

Table 1: Control Frame Type

| Type of control frame | Value |
| --- | --- |
| FACH Flow Control | 0000 0010 |
| FACH Capacity Request | 0000 0011 |
| DSCH Capacity Request | 0000 0100 |
| DSCH Capacity Allocation | 0000 0101 |
| DL NODE SYNCHRONISATION | 0000 0110 |
| UL NODE SYNCHRONISATION | 0000 0111 |
| HS-DSCH Capacity Request | 0000 1010 |
| HS-DSCH Capacity Allocation TYPE 1 | 0000 1011 |
| HS-DSCH Capacity Allocation TYPE 2 | 0000 1100 |
| HS-DSCH UL Synchronization Establishment Failure (1.28Mcps TDD only) | 0000 1101 |
| HS-DSCH PDU Drop Indication | 0000 1110 |

### 6.3.3	Payload structure and information elements

#### 6.3.3.1	FACH FLOW CONTROL

Figure 14 shows the payload structure when the control frame is used for the above mentioned purpose. This control information is sent in the UL only.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SRNTI |  |  |  |  |  |  |  |
| SRNTI (cont) |  |  |  |  |  |  |  |
| SRNTI (cont) |  |  |  | CmCH-PI |  |  |  |
| Credits |  |  |  |  |  |  |  |
| New IE fl 7(E); New IE fl 6; New IE fl 5; New IE fl 4; New IE fl 3; New IE fl 2; New IE fl 1; New IE fl 0. |
| Extended S-RNTI |  |  |  |  |  |  |  |
| Extended S-RNTI (cont) |  |  |  |  |  |  |  |
| Extended S-RNTI (cont) |  |  |  |  |  | Spare bits 1-0 |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 14: FACH FLOW CONTROL payload structure

Bit 0 of New IE Flags in FACH FLOW CONTROL FRAME indicates if the Extended S-RNTI is present (1) or not (0). If the Extended S-RNTI is included, the S-RNTI shall be igored by the receiver.

Bits 1 through 6 of New IE Flags in FACH FLOW CONTROL FRAME shall be set to 0.

[FDD - Field length of Spare Extension IE in FACH FLOW CONTROL FRAME is 0-28 octets.]

##### 6.3.3.1.1	S-RNTI

Refer to subclause 6.2.5.4.

##### 6.3.3.1.2	Common Transport Channel Priority Indicator (CmCH-PI)

Refer to subclause 6.2.5.7.

##### 6.3.3.1.3	Credits

Description: The Credits IE indicates the number of MAC-c/sh SDUs that a user may transmit.

Value range: {0-255, where 0=stop transmission, 255=unlimited}.

Field length: 8 bits.

##### 6.3.3.1.4	Spare Extension

Description: Indicates the location where new IEs can in the future be added in a backward compatible way.

Field length: 0-32 octets.

##### 6.3.3.1.5	Extended S-RNTI

Refer to subclause 6.2.5.55.

#### 6.3.3.2	DSCH CAPACITY REQUEST [TDD]

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spare bits 7-4 |  |  |  | Cm CH-PI |  |  |  |
| User Buffer Size |  |  |  |  |  |  |  |
| User Buffer Size (cont) |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 15: CAPACITY REQUEST payload structure

DSCH Capacity Request is sent for each priority group to indicate the user buffer size. The control frame is sent by the DSCH CAPACITY REQUEST is sent for each priority group to indicate the user buffer size. The control frame is sent by the SRNC when the SRNC considers the user buffer status needs an increased buffer reporting frequency. This may be sent to signal an event, such as, data arrival or user-buffer discard. This control frame is used to improve user-buffer reporting above the level produced by the user-buffer reporting associated with the DSCH DATA FRAMEs.

##### 6.3.3.2.1	Common Transport Channel Priority Indicator (CmCH-PI)

Refer to subclause 6.2.5.7.

##### 6.3.3.2.2	User Buffer Size

Refer to subclause 6.2.5.12.

##### 6.3.3.2.3	Spare Extension

Refer to subclause 6.3.3.1.4.

#### 6.3.3.3	DSCH CAPACITY ALLOCATION [TDD]

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spare bits 7-4 |  |  |  | Cm CH-PI |  |  |  |
| Max MAC –c/sh SDU Length (cont) |  |  |  |  |  |  |  |
| Max MAC –c/sh SDU Length |  |  |  |  | Spare bits 2-0 |  |  |
| Credits |  |  |  |  |  |  |  |
| Interval |  |  |  |  |  |  |  |
| Repetition Period |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 16: CAPACITY ALLOCATION payload structure

The CAPACITY ALLOCATION Control Frame describes an allocation that the SRNC may use. When the Credits IE has a value of 0 it signifies that there is no resources allocated for transmission and to thus stop transmission. When the Credits IE has a value of 255, it signifies unlimited capacity for transmission of SDUs. When the Repetition Period IE has a value of 0, it signifies that the allocation (Maximum MAC-c/sh SDU Length, Credits and Interval IEs) can be repeated without limit.

##### 6.3.3.3.1	Common Transport Channel Priority Indicator (CmCH-PI)

Refer to subclause 6.2.5.7.

##### 6.3.3.3.2	Maximum MAC-c/sh SDU Length

Description: The values indicated the maximum allowable SDU size. MAC-c/sh SDU contains the C/T field of the MAC header followed by one RLC PDU.

Field length: See the value of the MAC-c/sh SDU Length IE.

##### 6.3.3.3.3	Credits

Description: The Credits IE indicates the number of MAC-c/sh SDUs that a SRNC may transmit during one Interval granted in the DSCH CAPACITY ALLOCATION Control Frame.

Value range: Refer to subclause 6.3.3.1.3.

Field Length: Refer to subclause 6.3.3.1.3.

##### 6.3.3.3.4	Interval

Description: The value of this field indicates the time interval during which the Credits granted in the DSCH CAPACITY ALLOCATION Control Frame may be used. The first interval starts immediately after reception of the DSCH CAPACITY ALLOCATION control frame, subsequent intervals start immediately after the previous interval has elapsed. This value is only applied to the DSCH transport channel.

Value range: {0-2550 ms}. Value 0 shall be interpreted that none of the credits shall be used.

Granularity: 10ms.

Field Length: 8 bits.

##### 6.3.3.3.5	Repetition Period

Description: The value of this field indicates the number of subsequent intervals that the Credits IE granted in the DSCH CAPACITY ALLOCATION Control Frame may be used. These values represent an integer number of Intervals (see subclause 6.3.3.3.4). This field is only applied to the DSCH transport channel.

Value range: {0-255, where 0= unlimited repetition period}.

Field Length: 8 bits.

##### 6.3.3.3.6	Spare Extension

Refer to subclause 6.3.3.1.4.

#### 6.3.3.4	FACH CAPACITY REQUEST

Figure 17 shows the payload structure when the control frame is used for the above mentioned purpose. This control information is sent in the DL only.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DRNTI |  |  |  |  |  |  |  |
| DRNTI (cont) |  |  |  |  |  |  |  |
| DRNTI (cont) |  |  |  | CmCH -PI |  |  |  |
| User Buffer Size |  |  |  |  |  |  |  |
| User Buffer Size (cont) |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 17: FACH CAPACITY REQUEST payload structure

##### 6.3.3.4.1	D-RNTI

Refer to subclause 6.2.5.3.

##### 6.3.3.4.2	Common Transport Channel Priority Indicator (CmCH-PI)

Refer to subclause 6.2.5.7.

##### 6.3.3.4.3	User Buffer Size

Refer to subclause 6.2.5.12.

##### 6.3.3.4.4	Spare extension

Refer to subclause 6.3.3.1.4.

#### 6.3.3.5	HS-DSCH CAPACITY REQUEST

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spare bits 7-4 |  |  |  | Cm CH-PI |  |  |  |
| User Buffer Size |  |  |  |  |  |  |  |
| User Buffer Size (cont) |  |  |  |  |  |  |  |
| New IE fl 7(E) | New IE fl 6 | New IE fl 5 | New IE fl 4 | New IE fl 3 | New IE fl 2 | New IE fl 1 | New IE fl 0 |
| DRNTI |  |  |  |  |  |  |  |
| DRNTI (cont) |  |  |  |  |  |  |  |
| DRNTI (cont) |  |  |  | Spare bits 3-0 |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 18: CAPACITY REQUEST payload structure

The HS-DSCH CAPACITY REQUEST control frame is sent for each priority group to indicate the user buffer size. The control frame is sent by the SRNC when the SRNC considers the user buffer status needs an increased buffer reporting frequency. This may be sent to signal an event, such as, data arrival or user-buffer discard. This control frame is used to improve user-buffer reporting above the level produced by the user-buffer reporting associated with the HS-DSCH DATA FRAMEs (TYPE 1 or TYPE 2).

[FDD and 1.28Mcps TDD - Bit 0 of New IE Flags in HS-DSCH CAPACITY REQUEST indicates if a DRNTI is present (1) or not (0) in the 3 octets following the New IE Flags IE. Bits 1 through 6 of New IE Flags in HS-DSCH CAPACITY REQUEST shall be set to 0. Field length of Spare Extension IE in HS-DSCH CAPACITY REQUEST is 0-28 octets.]

##### 6.3.3.5.1	Common Transport Channel Priority Indicator (CmCH-PI)

Description: CmCH-PI, configured via the Scheduling Priority Indicator in RNSAP (TS 25.423 [8]), is the relative priority of the data frame and the SDUs included.

Value range: {0-15, where 0=lowest priority, 15=highest priority}.

Field length: 4 bits.

##### 6.3.3.5.2	User Buffer Size

Refer to subclause 6.2.5.12.

##### 6.3.3.5.3	Spare Extension

Refer to subclause 6.3.3.1.4.

##### 6.3.3.5.4	DRNTI [FDD]

Refer to subclause 6.2.5.3.

#### 6.3.3.6	HS-DSCH CAPACITY ALLOCATION

Two types of HS-DSCH CAPACITY ALLOCATION Control Frames exist for the HS-DSCH capacity allocation, i.e. HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame and HS-DSCH CAPACITY ALLOCATION TYPE 2 Control Frame.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spare bits 7-6 |  | Congestion Status |  | Cm CH-PI |  |  |  |
| Max MAC –d PDU Length |  |  |  |  |  |  |  |
| Max MAC –d PDU Length(cont) |  |  |  |  | HS-DSCH Credits |  |  |
| HS-DSCH Credits (cont) |  |  |  |  |  |  |  |
| HS-DSCH Interval |  |  |  |  |  |  |  |
| HS-DSCH Repetition Period |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 19: CAPACITY ALLOCATION TYPE 1 payload structure

The CAPACITY ALLOCATION TYPE 1 Control Frame describes an allocation that the SRNC may use. When the HS-DSCH Credits IE has a value of 0 it signifies that there is no resources allocated for transmission and to thus stop transmission. When the HS-DSCH Credits IE has a value of 2047, it signifies unlimited capacity for transmission of PDUs. When the HS-DSCH Repetition Period IE has a value of 0, it signifies that the allocation (Maximum MAC-d PDU Length, HS-DSCH Credits and HS-DSCH Interval IEs) can be repeated without limit. In addition to this the CAPACITY ALLOCATION TYPE 1 Control Frame may inform the SRNC about the detection of congestion in the DL transport network layer with the Congestion Status Bits.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spare bits 7-6 |  | Congestion Status |  | Cm CH-PI |  |  |  |
| Spare bits 7-4 |  |  |  | SRNTI Indicator | Max MAC –d PDU |  |  |
| Max MAC –d PDU Length Type 2(cont) |  |  |  |  |  |  |  |
| HS-DSCH Credits |  |  |  |  |  |  |  |
| HS-DSCH Credits (cont) |  |  |  |  |  |  |  |
| HS-DSCH Interval |  |  |  |  |  |  |  |
| HS-DSCH Repetition Period |  |  |  |  |  |  |  |
| S-RNTI |  |  |  |  |  |  |  |
| S-RNTI (cont) |  |  |  |  |  |  |  |
| S-RNTI (cont) |  |  |  | Spare bits 3-0 |  |  |  |
| New IE fl 7(E); New IE fl 6; New IE fl 5; New IE fl 4; New IE fl 3; New IE fl 2; New IE fl 1; New IE fl 0. |
| Extended S-RNTI |  |  |  |  |  |  |  |
| Extended S-RNTI(cont) |  |  |  |  |  |  |  |
| Extended S-RNTI(cont) |  |  |  |  |  | Spare bits 1-0 |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 20: CAPACITY ALLOCATION TYPE 2 payload structure

The CAPACITY ALLOCATION TYPE 2 Control Frame describes an allocation that the SRNC may use. When the HS-DSCH Credits IE has a value of 0 it signifies that there is no resources allocated for transmission and to thus stop transmission. When the HS-DSCH Credits IE has a value of 65535, it signifies unlimited capacity for transmission of PDUs. When the HS-DSCH Repetition Period IE has a value of 0, it signifies that the allocation (Maximum MAC-d PDU Length Type 2, HS-DSCH Credits and HS-DSCH Interval IEs) can be repeated without limit. In addition to this the CAPACITY ALLOCATION TYPE 2 Control Frame may inform the SRNC about the detection of congestion in the DL transport network layer with the Congestion Status Bits.

Bit 0 of New IE Flags in CAPACITY ALLOCATION TYPE 2 FRAME indicates if the Extended S-RNTI IE is present (1) or not (0). If the Extended S-RNTI IE is included, the S-RNTI IE shall be igored by the receiver.

Bits 1 through 6 of New IE Flags in CAPACITY ALLOCATION TYPE 2 FRAME shall be set to 0.

[FDD - Field length of Spare Extension IE in CAPACITY ALLOCATION TYPE 2 FRAME is 0-28 octets.]

##### 6.3.3.6.1	Common Transport Channel Priority Indicator (CmCH-PI)

Refer to subclause 6.2.5.7.

##### 6.3.3.6.2	Maximum MAC-d PDU Length

Description: The Maximum MAC-d PDU Length IE is used in HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame. The value indicates the maximum allowable PDU size among the MAC-d PDU sizes configured via RNSAP (TS 25.423 [8]).

Value range: Refer to subclause 6.2.5.16.

Field length: Refer to subclause 6.2.5.16.

##### 6.3.3.6.3	HS-DSCH Credits

Description: The HS-DSCH Credits IE is used in HS-DSCH CAPACITY ALLOCATION (TYPE 1 and TYPE 2) Control Frame. It is used to indicate the granted amount of MAC-d PDU data that a SRNC may transmit during one HS-DSCH Interval.

In case of HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame, it indicates the number of MAC-d PDU that a SRNC may transmit during one HS-DSCH Interval granted in the HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame.

In case of HS-DSCH CAPACITY ALLOCATION TYPE 2 Control Frame, the granted amount of MAC-d PDU data in octets is obtained by multiplying the maximum MAC-d PDU length (indicated by the Maximum MAC-d PDU Length Type 2 IE) with the number of MAC-d PDUs (indicated by the HS-DSCH Credits IE).

Value range: {0-2047, where 0=stop transmission, 2047=unlimited} in case of HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame, {0-65535, where 0=stop transmission, 65535=unlimited} in case of HS-DSCH CAPACITY ALLOCATION TYPE 2 Control Frame.

Field length: 11 bits in case of HS-DSCH CAPACITY ALLOCATION TYPE 1 Control Frame, 16 bits in case of HS-DSCH CAPACITY ALLOCATION TYPE 2 Control Frame.

##### 6.3.3.6.4	HS-DSCH Interval

Description: The value of this field indicates the time interval during which the HS-DSCH Credits IE granted in the HS-DSCH CAPACITY ALLOCATION (TYPE 1 or TYPE 2) Control Frame may be used. The first interval starts immediately after reception of the HS-DSCH CAPACITY ALLOCATION (TYPE 1 or TYPE 2) control frame, subsequent intervals start immediately after the previous interval has elapsed. This value is only applied to the HS-DSCH transport channel.

Value range: {0-2550 ms}. Value 0 shall be interpreted that none of the credits shall be used.

Granularity: 10ms.

Field Length: 8 bits.

##### 6.3.3.6.5	HS-DSCH Repetition Period

Description: The value of this field indicates the number of subsequent intervals that the HS-DSCH Credits IE granted in the HS-DSCH CAPACITY ALLOCATION (TYPE 1 or TYPE 2) Control Frame may be used. These values represent an integer number of Intervals (see subclause 6.3.3.6.4). This field is only applied to the HS-DSCH transport channel.

Value range: {0-255, where 0= unlimited repetition period}.

Field Length: 8 bits.

##### 6.3.3.6.8	Spare Extension

Refer to subclause 6.3.3.1.4.

##### 6.3.3.6.9	Congestion Status

Description: The Congestion Status Bits are used by the DRNC to indicate a congestion status received. The DRNC provides the congestion status in HS-DSCH CAPACITY ALLOCATION (TYPE 1 or TYPE 2) Control Frame when it has been received towards the SRNC.

Value range:

0	No TNL Congestion

1	Reserved for future use

2	TNL Congestion – detected by delay build-up

3	TNL Congestion – detected by frame loss

Field Length: 2 bits.

##### 6.3.3.6.10	SRNTI Indicator [FDD and 1.28Mcps TDD]

Description: Indicates whether S-RNTI is present.

Value range: {0 = S-RNTI not present, 1= S-RNTI present or Extended S-RNTI present}.

Field length: 1 bit.

##### 6.3.3.6.11	S-RNTI [FDD and 1.28Mcps TDD]

Refer to subclause 6.2.5.4.

##### 6.3.3.6.12	Maximum MAC-d PDU Length Type 2

Description: The Maximum MAC-d PDU Length Type 2  IE is used in HS-DSCH CAPACITY ALLOCATION TYPE 2 Control Frame. The value is a factor in the granted amount of  MAC-d PDU data that a SRNC may transmit during one HS-DSCH Interval. The amount of MAC-d PDU data in octets is obtained by multiplying the MACd PDU length (indicated by the Maximum MAC-d PDU Length Type 2 IE) with the number of MAC-d PDUs (indcated by the HS-DSCH Credits IE).

Value range: {0-1505}, 0 – not used.

Field length: 11 bits.

##### 6.3.3.6.13	Extended S-RNTI [FDD]

Refer to subclause 6.2.5.55.

#### 6.3.3.7	DL NODE SYNCHRONISATION

##### 6.3.3.7.1	Payload structure

Figure 21 shows the structure of the payload for the DL NODE SYNCHRONISATION control frame.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |
| T1 (cont) |  |  |  |  |  |  |  |
| T1 (cont) |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 21: Structure of the payload for the DL NODE SYNCHRONISATION control frame

##### 6.3.3.7.2	T1

Description: RNC specific frame number (RFN) that indicates the time when RNC sends the frame through the SAP to the transport layer.

Value range: {0-40959.875 ms}.

Field length: 24 bits.

##### 6.3.3.7.3	Spare Extension

The Spare Extension IE is described in subclause 6.3.3.1.4.

#### 6.3.3.8	UL NODE SYNCHRONISATION

##### 6.3.3.8.1	Payload structure

The payload of the UL NODE SYNCHRONISATION control frames is shown in figure 22.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 |  |  |  |  |  |  |  |
| T1 (cont) |  |  |  |  |  |  |  |
| T1 (cont) |  |  |  |  |  |  |  |
| T2 |  |  |  |  |  |  |  |
| T2 (cont) |  |  |  |  |  |  |  |
| T2 (cont) |  |  |  |  |  |  |  |
| T3 |  |  |  |  |  |  |  |
| T3 (cont) |  |  |  |  |  |  |  |
| T3 (cont) |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 22: Structure of the payload for UL NODE SYNCHRONISATION control frame

##### 6.3.3.8.2	T1

Description: T1 timer is extracted from the correspondent DL NODE SYNCHRONISATION control frame.

Value range: {0-40959.875 ms}.

Granularity: 0.125 ms.

Field length: 24 bits.

##### 6.3.3.8.3	T2

Description: Node B specific frame number (BFN) that indicates the time when Node B received the correspondent DL NODE SYNCHRONISATION control frame through the SAP from the transport layer.

Value range: {0-40959.875 ms}.

Granularity: 0.125 ms.

Field length: 24 bits.

##### 6.3.3.8.4	T3

Description: Node B specific frame number (BFN) that indicates the time when Node B sends the frame through the SAP to the transport layer.

Value range: {0-40959.875 ms}.

Granularity: 0.125 ms.

Field length: 24 bits.

##### 6.3.3.8.5	Spare Extension

The Spare Extension IE is described in subclause 6.3.3.1.4.

#### 6.3.3.9	HS-DSCH UL SYNCHRONIZATION ESTABLISHMENT FAILURE [1.28Mcps TDD]

##### 6.3.3.9.1	Payload structure

The payload of the HS-DSCH UL Synchronization Establishment Failure control frames is shown in figure 23.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S-RNTI |  |  |  |  |  |  |  |
| S-RNTI (cont) |  |  |  |  |  |  |  |
| S-RNTI (cont) |  |  |  | Spare bits 3-0 |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 23: HS-DSCH UL SYNCHRONIZATION ESTABLISHMENT FAILURE payload structure

##### 6.3.3.9.2	S-RNTI

Refer to subclause 6.2.5.4.

#### 6.3.3.10	HS-DSCH PDU DROP INDICATION

##### 6.3.3.10.1	Payload structure

The payload of the HS-DSCH PDU Drop Indication control frame is shown in figure 24.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Spare bits 7-3 |  |  |  |  | Drop Reason |  |  |
| Number of PDUs Dropped |  |  |  |  |  |  |  |
| SN 1 |  |  |  |  |  |  |  |
| SN 1 (cont) |  |  |  |  |  |  | Spare |
| PDU 1 index |  |  |  |  |  |  |  |
| SN N |  |  |  |  |  |  |  |
| SN N(cont) |  |  |  |  |  |  | Spare |
| PDU N Index |  |  |  |  |  |  |  |
| Spare Extension |  |  |  |  |  |  |  |

Figure 24: HS-DSCH PDU DROP INDICATION payload structure

##### 6.3.3.10.2	Drop Reason

Description: The Drop Reason IE is used in HS-DSCH PDU DROP INDICATION Control Frame. It indicates the reason of drop of HS-DSCH PDUs in DRNS.

Value range:

0	too long delay in MAC-hs/ehs buffer

1	unsuccessful HARQ due to T1 timer expiry

2	TNL Congestion – detected by frame loss

3 to 7 are reserved

Field length: 3 bits

##### 6.3.3.10.3	PDU Index

Description: The PDU Index IE identifies the PDU Index in the HS-DSCH DATA FRAME.

Value range: {0-255}

Field length: 8 bits

# 7	Handling of Unknown, Unforeseen and Erroneous Protocol Data

## 7.1	General

A Frame Protocol frame with an unknown IE or an illegal IE value shall be ignored.

Annex A (informative):
Change History

| TSG # | TSG Doc. | CR | Rev | Subject/Comment | New |
| --- | --- | --- | --- | --- | --- |
| 09/2008 | - | - | - | Creation of Rel-8 version based on v 7.8.0 | 8.0.0 |
| 41 | RP-080587 | 0129 | 1 | Introduction of frame formats supporting E-DCH for CELL_FACH and Idle over Iur | 8.0.0 |
| 41 | RP-080588 | 0131 |  | Introduction of Enhanced UE DRX | 8.0.0 |
| 42 | RP-080850 | 0133 | 1 | Introduction of the Enhanced CELL_FACH, CELL_PCH, URA_PCH state for 1.28Mcps TDD | 8.1.0 |
| 42 | RP-080837 | 0136 |  | Correction of CR127, adding codepoints for the new control frames | 8.1.0 |
| 42 | RP-080858 | 0137 |  | Corrections to IE definitions for E-DCH DATA FRAME and "Definitions" chapter | 8.1.0 |
| 43 | RP-090073 | 0139 | 1 | Correction of handling of Capacity Allocation and HS-DSCH Credits | 8.2.0 |
| 44 | RP-090630 | 0141 |  | Correction of the value range of MAC PDU Size Extended IE in CCH FP Iur | 8.3.0 |
| 46 | RP-091181 | 0145 |  | Correction on value range for Number of MAC-is SDU in frame | 8.4.0 |
| 46 | RP-091180 | 0146 | 1 | Correction to the NodeB procedure when receiving uplink synchronization failure from physical layer for 1.28Mcp TDD | 8.4.0 |
| 12/2009 | - | - | - | Creation of Rel-9 version based on v. 8.4.0 | 9.0.0 |
| 46 | RP-091188 | 0143 |  | Introduction of Cell Portion for 1.28 Mcps TDD | 9.0.0 |
| 47 | RP-100229 | 0147 | - | Corrections to the TS0 enhancement for the UE in CELL_FACH state for 1.28Mcps TDD | 9.1.0 |
| 47 | RP-100217 | 0149 | - | Clarification of eDRX Indication for 1.28Mcp TDD | 9.1.0 |
| 12/2010 |  |  |  | Creation of Rel-10 version based on v. 9.1.0 | 10.0.0 |
| 50 | RP-101277 | 0150 |  | Introduction of MU-MIMO to CTCH of Iur user plane | 10.0.0 |
| SP-49 | SP-100629 |  |  | Clarification on the use of References (TS 21.801 CR#0030) | 10.0.1 |
| 52 | RP-110684 | 0155 |  | Correction of references | 10.1.0 |
| 52 | RP-110681 | 0157 |  | Correction of Logical Channel ID range used in Enhanced Cell-FACH | 10.1.0 |
| 52 | RP-110688 | 0158 |  | Correction to mapping of MAC-is SDU to MAC-d PDU | 10.1.0 |
| 54 | RP-111645 | 0162 |  | Indicator for DTXDRX HS-SCCH order | 10.2.0 |
| 09/2012 |  |  |  | Update to Rel-11 version (MCC) | 11.0.0 |
| 58 | RP-121725 | 0165 |  | Introduction of Further Enhancements to CELL_FACH feature | 11.1.0 |
| 58 | RP-121727 | 0166 | 2 | Supporting Multiflow Operation | 11.1.0 |
| 58 | RP-121725 | 0167 | 1 | 2nd DRX cycle and fallback to PRACH R99 | 11.1.0 |
| 59 | RP-130206 | 0169 | 1 | Clean-up for Multiflow | 11.2.0 |
| 60 | RP-130643 | 0171 |  | Correction of Spare bit allocation | 11.3.0 |
| 06/2014 |  |  |  | Update to Rel-12 version (MCC) | 12.0.0 |
| 64 | RP-140875 | 0180 | 3 | Introduction of Cell Reselection indications | 12.0.0 |
| 65 | RP-141608 | 0181 | 2 | Rapporteur Review – change figures to table style | 12.1.0 |
| 66 | RP-142083 | 0186 | 1 | Correction: add Extended S-RNTI in the Iur user plane | 12.2.0 |
| 70 | RP-152090 | 0187 | 2 | Introduction of IDLE to CONNECTED mode transition optimization | 13.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-03 | SA#75 |  |  |  |  | Promotion to Release 14 without technical change | 14.0.0 |
| 2018-07 | SA#80 | - | - | - | - | Promotion to Release 15 without technical change | 15.0.0 |
| 2020-07 | - | - | - | - | - | Update to Rel-16 version (MCC) | 16.0.0 |
| 2022-03 | SA#95-e |  |  |  |  | Promotion to Release 17 without technical change | 17.0.0 |
| 2024-03 | SA#103- | - | - | - | - | Update to Rel-18 version (MCC) | 18.0.0 |
| 2025-09 | - | - | - | - | - | Update to Rel-19 version (MCC) | 19.0.0 |