3GPP TS 36.455 V19.0.0 (2025-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

LTE Positioning Protocol A (LPPa)

(Release 19)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

LTE, radio, positioning

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	General	9

4.1	Procedure specification principles	9

4.2	Forwards and backwards compatibility	9

4.3	Specification notations	9

5	LPPa services	10

5.1	LPPa procedure modules	10

5.2	Parallel transactions	10

6	Services expected from lower layer	10

7	Functions of LPPa	10

8	LPPa procedures	11

8.1	Elementary procedures	11

8.2	Location Information Transfer Procedures	12

8.2.1	E-CID Measurement Initiation	12

8.2.1.1	General	12

8.2.1.2	Successful Operation	12

8.2.1.3	Unsuccessful Operation	13

8.2.1.4	Abnormal Conditions	13

8.2.2	E-CID Measurement Failure Indication	13

8.2.2.1	General	13

8.2.2.2	Successful Operation	13

8.2.2.3	Unsuccessful Operation	13

8.2.2.4	Abnormal Conditions	13

8.2.3	E-CID Measurement Report	14

8.2.3.1	General	14

8.2.3.2	Successful Operation	14

8.2.3.3	Unsuccessful Operation	14

8.2.3.4	Abnormal Conditions	14

8.2.4	E-CID Measurement Termination	14

8.2.4.1	General	14

8.2.4.2	Successful Operation	15

8.2.4.3	Unsuccessful Operation	15

8.2.4.4	Abnormal Conditions	15

8.2.5	OTDOA Information Exchange	15

8.2.5.1	General	15

8.2.5.2	Successful Operation	15

8.2.5.3	Unsuccessful Operation	16

8.2.5.4	Abnormal Conditions	16

8.2.6	UTDOA Information Exchange	16

8.2.6.1	General	16

8.2.6.2	Successful Operation	16

8.2.6.3	Unsuccessful Operation	17

8.2.6.4	Abnormal Conditions	17

8.2.7	UTDOA Information Update	17

8.2.7.1	General	17

8.2.7.2	Successful Operation	17

8.2.7.3	Unsuccessful Operation	17

8.2.7.4	Abnormal Conditions	18

8.3	Management Procedures	18

8.3.1	Error Indication	18

8.3.1.1	General	18

8.3.1.2	Successful Operation	18

8.3.1.3	Abnormal Conditions	18

8.4	Assistance Information Transfer Procedures	18

8.4.1	Assistance Information Control	18

8.4.1.1	General	18

8.4.1.2	Successful Operation	19

8.4.1.3	Abnormal Conditions	19

8.4.2	Assistance Information Feedback	19

8.4.2.1	General	19

8.4.2.2	Successful Operation	19

8.4.2.3	Abnormal Conditions	20

9	Elements for LPPa Communication	20

9.0	General	20

9.1	Message Functional Definition and Content	20

9.1.1	Messages for Location Information Transfer Procedures	20

9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST	20

9.1.1.2	E-CID MEASUREMENT INITIATION RESPONSE	21

9.1.1.3	E-CID MEASUREMENT INITIATION FAILURE	21

9.1.1.4	E-CID MEASUREMENT FAILURE INDICATION	22

9.1.1.5	E-CID MEASUREMENT REPORT	22

9.1.1.6	E-CID MEASUREMENT TERMINATION COMMAND	22

9.1.1.7	OTDOA INFORMATION REQUEST	22

9.1.1.8	OTDOA INFORMATION RESPONSE	23

9.1.1.9	OTDOA INFORMATION FAILURE	24

9.1.1.10	UTDOA INFORMATION REQUEST	24

9.1.1.11	UTDOA INFORMATION RESPONSE	24

9.1.1.12	UTDOA INFORMATION FAILURE	25

9.1.1.13	UTDOA INFORMATION UPDATE	25

9.1.2	Messages for Management Procedures	25

9.1.2.1	ERROR INDICATION	25

9.1.3	Messages for Assistance Information Transfer Procedures	25

9.1.3.1	ASSISTANCE INFORMATION CONTROL	25

9.1.3.2	ASSISTANCE INFORMATION FEEDBACK	26

9.2	Information Element definitions	26

9.2.0	General	26

9.2.1	Cause	26

9.2.2	Criticality Diagnostics	27

9.2.3	Message Type	28

9.2.4	LPPa Transaction ID	28

9.2.5	E-CID Measurement Result	29

9.2.6	ECGI	29

9.2.7	OTDOA Cell Information	30

9.2.8	E-UTRAN Access Point Position	32

9.2.9	PRS Muting Configuration	32

9.2.10	Requested SRS Transmission Characteristics	33

9.2.11	UL Configuration	33

9.2.12	Cell Portion ID	35

9.2.13	Inter-RAT Measurement Result	35

9.2.14	MBSFN subframe Configuration	36

9.2.15	WLAN Measurement Result	37

9.2.16	NPRS configuration	37

9.2.17	NPRS Muting Configuration	39

9.2.18	Offset of NB-IoT Channel Number to EARFCN	39

9.2.19	PRS Frequency Hopping Configuration	39

9.2.20	Assistance Information	40

9.2.21	PosSIB Segments	40

9.2.22	Assistance Information Meta Data	40

9.2.23	Positioning SIB Type	41

9.2.24	Assistance Information Failure List	41

9.2.25	TDD Configuration	42

9.2.26	NR CGI	42

9.3	Message and Information Element Abstract Syntax (with ASN.1)	43

9.3.1	General	43

9.3.2	Usage of Private Message Mechanism for Non-standard Use	43

9.3.3	Elementary Procedure Definitions	43

9.3.4	PDU Definitions	48

9.3.5	Information Element definitions	56

9.3.6	Common definitions	74

9.3.7	Constant definitions	75

9.3.8	Container definitions	77

9.4	Message transfer syntax	82

9.5	Timers	82

10	Handling of unknown, unforeseen and erroneous protocol data	82

Annex A (informative):	Change History	83

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the control plane radio network layer signalling procedures between eNB and E-SMLC. LPPa supports the concerned functions by signalling procedures defined in this document. LPPa is developed in accordance with the general principles stated in TS 36.401 [2].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.401:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Architecture Description".

[3]	3GPP TS 36.413:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[4]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[5]	3GPP TS 36.104:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Base Station (BS) radio transmission and reception".

[6]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[7]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[8] 	3GPP TS 36.133:"Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[9]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[10]	3GPP TS 36.331:"Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[11]	IEEE Std 802.11™-2012, IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area network.

[12]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[13]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Elementary Procedure: LPPa protocol consists of Elementary Procedures (EPs). An LPPa Elementary Procedure is a unit of interaction between the eNB and the E-SMLC. An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success or failure),

-	Class 2: Elementary Procedures without response.

Cell Portion: A geographical part of a cell. A cell portion is semi-static, and identical for both the UL and the DL. Within a cell, a cell portion is uniquely identified by its Cell Portion ID.

Transmission Point (TP): A set of geographically co-located transmit antennas for one cell, part of one cell or one PRS-only TP. Transmission Points can include base station (eNB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a PRS-only TP, etc. One cell can be formed by one or multiple transmission points. For a homogeneous deployment, each transmission point may correspond to one cell.

PRS-only TP: A TP which only transmits PRS signals for PRS-based TBS positioning and is not associated with a cell.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BSSID	Basic Service Set IDentifier

CID	Cell-ID (positioning method)

DL	Downlink

E-CID	Enhanced Cell-ID (positioning method)

eNB	E-UTRAN NodeB

EP	Elementary Procedure

EPC	Evolved Packet Core

E-SMLC	Evolved Serving Mobile Location Centre

E-UTRAN	Evolved UTRAN

GNSS	Global Navigation Satellite System

HESSID	Homogeneous Extended Service Set IDentifier

IE	Information Element

LCS	LoCation Services

LPP	LTE Positioning Protocol

LPPa	LTE Positioning Protocol Annex

MME	Mobility Management Entity

NW	Network

OTDOA	Observed Time Difference of Arrival

RSSI	Received Signal Strength Indicator

S1AP	S1 Application Protocol

SBAS	Satellite-based Augmentation System

SRS	Sounding Reference Signal

SSID	Service Set IDentifier

TP	Transmission Point

UE	User Equipment

UL	Uplink

UTDOA	Uplink Time Difference of Arrival

WLAN	Wireless Local Area Network

# 4	General

## 4.1	Procedure specification principles

The principle for specifying the procedure logic is to specify the functional behaviour of the terminating eNB exactly and completely. Any rule that specifies the behaviour of the originating eNB shall be possible to be verified with information that is visible within the system.

The following specification principles have been applied for the procedure text in clause 8:

-	The procedure text discriminates between:

1)	Functionality which "shall" be executed

The procedure text indicates that the receiving node "shall" perform a certain function Y under a certain condition. If the receiving node supports procedure X but cannot perform functionality Y requested in the initiating message of a Class 1 EP, the receiving node shall respond with the message used to report unsuccessful outcome for this procedure, containing an appropriate cause value.

2)	Functionality which "shall, if supported" be executed

The procedure text indicates that the receiving node "shall, if supported," perform a certain function Y under a certain condition. If the receiving node supports procedure X, but does not support functionality Y, the receiving node shall proceed with the execution of the EP, possibly informing the requesting node about the not supported functionality.

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included. For requirements on including Criticality Diagnostics IE, see section 10.

## 4.2	Forwards and backwards compatibility

The forwards and backwards compatibility of the protocol is assured by a mechanism where all current and future messages, and IEs or groups of related IEs, include ID and criticality fields that are coded in a standard format that will not be changed in the future. These parts can always be decoded regardless of the standard version.

## 4.3	Specification notations

For the purposes of the present document, the following notations apply:

Procedure	When referring to an elementary procedure in the specification the Procedure Name is written with the first letters in each word in upper case characters followed by the word "procedure", e.g. Handover Preparation procedure.

Message	When referring to a message in the specification the MESSAGE NAME is written with all letters in upper case characters followed by the word "message", e.g. ERROR INDICATION message.

IE	When referring to an information element (IE) in the specification the Information Element Name is written with the first letters in each word in upper case characters and all letters in Italic font followed by the abbreviation "IE", e.g. Cause IE.

Value of an IE	When referring to the value of an information element (IE) in the specification the "Value" is written as it is specified in sub clause 9.2 enclosed by quotation marks, e.g. "Value".

# 5	LPPa services

The present clause describes the services an eNB offers to the E-SMLC.

## 5.1	LPPa procedure modules

The procedures are divided into two modules as follows:

1.	LPPa Location Information Transfer Procedures;

2.	LPPa Management Procedures;

The LPPa Location Information Transfer Procedures module contains procedures used to handle the transfer of positioning related information between eNB and E-SMLC.

The Management Procedures module contains procedures that are not related specifically to positioning, i.e. error handling.

## 5.2	Parallel transactions

Unless explicitly indicated in the procedure specification, at any instance in time one protocol peer may have more than one ongoing LPPa procedure.

# 6	Services expected from lower layer

Within E-UTRAN, LPPa protocol uses the services provided by the S1AP protocol. An LPPa message is carried inside an S1AP message.

S1AP signalling is described in TS 36.413 [3].

# 7	Functions of LPPa

The LPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the eNB to exchange location information with the E-SMLC for the purpose of E-CID positioning.

-	OTDOA Information Transfer. This function allows the eNB to exchange information with the E-SMLC for the purpose of OTDOA positioning.

-	UTDOA Information Transfer. This function allows the eNB to exchange information with the E-SMLC for the purpose of supporting UTDOA.

-	Assistance Information Transfer. This function allows the E-SMLC to exchange information with the eNB for the purpose of assistance information broadcasting.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

The mapping between the above functions and LPPa EPs is shown in the table below.

Table 7-1: Mapping between LPPa functions and LPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| UTDOA Information Transfer | a) UTDOA Information Exchange b) UTDOA Information Update |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |

# 8	LPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |  |
| UTDOA Information Exchange | UTDOA INFORMATION REQUEST | UTDOA INFORMATION RESPONSE | UTDOA INFORMATION FAILURE |  |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| UTDOA Information Update | UTDOA INFORMATION UPDATE |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |

## 8.2	Location Information Transfer Procedures

### 8.2.1	E-CID Measurement Initiation

#### 8.2.1.1	General

The purpose of E-CID Measurement Initiation procedure is to allow the E-SMLC to request the eNB to report E-CID measurements used by E-SMLC to compute the location of the UE.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: E-CID Measurement Initiation procedure, successful operation

The E-SMLC initiates the procedure by sending an E-CID MEASUREMENT INITIATION REQUEST message. If the eNB is able to initiate the requested E-CID measurements, it shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT INITIATION RESPONSE message when measurement results other than the "Cell-ID" have been requested.

If the Report Characteristics IE is set to "OnDemand", the eNB shall return the result of the measurement in the E-CID MEASUREMENT INITIATION RESPONSE message including, if available, the E-UTRAN Access Point Position IE in the E-CID Measurement Result IE, and the E-SMLC shall consider that the E-CID measurements for the UE has been terminated by the eNB. If available, the eNB shall include the Cell Portion ID IE in the E-CID MEASUREMENT INITIATION RESPONSE message. Upon reception of the Cell Portion ID IE, the E-SMLC may use the value as the cell portion for the measurement. If the Report Characteristics IE is set to "OnDemand" and the Inter-RAT Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the eNB shall, if supported, provide the corresponding measurements, if available in the eNB, in the Inter-RAT Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message. If the Report Characteristics IE is set to "OnDemand" and the WLAN Measurement Quantities IE is included in the E-CID MEASUREMENT INITIATION REQUEST message, the eNB shall, if supported, provide the corresponding measurements, if available in the eNB, in the WLAN Measurement Result IE in E-CID MEASUREMENT INITIATION RESPONSE message.

If the Report Characteristics IE is set to "Periodic", the eNB shall initiate the requested measurements and shall reply with the E-CID MEASUREMENT INITIATION RESPONSE message without including either the E-CID Measurement Result IE or the Cell Portion ID IE in this message. The eNB shall then periodically initiate the E-CID Measurement Report procedure for the measurements, with the requested reporting periodicity.

#### 8.2.1.3	Unsuccessful Operation

Figure 8.2.1.3-1: E-CID Measurement Initiation procedure, unsuccessful operation

If the eNB is not able to initiate at least one of the requested E-CID measurements, the eNB shall respond with an E-CID MEASUREMENT INITIATION FAILURE message.

#### 8.2.1.4	Abnormal Conditions

Void

### 8.2.2	E-CID Measurement Failure Indication

#### 8.2.2.1	General

The purpose of the E-CID Measurement Failure Indication procedure is for the eNB to notify the E-SMLC that the E-CID measurements previously requested with the E-CID Measurement Initiation procedure can no longer be reported.

#### 8.2.2.2	Successful Operation

Figure 8.2.2.2-1: E-CID Measurement Failure Indication, successful operation

Upon reception of the E-CID MEASUREMENT FAILURE INDICATION message, the E-SMLC shall consider that the E-CID measurements for the UE have been terminated by the eNB.

#### 8.2.2.3	Unsuccessful Operation

Not applicable.

#### 8.2.2.4	Abnormal Conditions

Void.

### 8.2.3	E-CID Measurement Report

#### 8.2.3.1	General

The purpose of E-CID Measurement Report procedure is for the eNB to provide the E-CID measurements for the UE to the E-SMLC.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: E-CID Measurement Report procedure, successful operation

The eNB initiates the procedure by sending an E-CID MEASUREMENT REPORT message. The E-CID MEASUREMENT REPORT message contains the E-CID measurement results according to the measurement configuration in the respective E-CID MEASUREMENT INITIATION REQUEST message.

The Measured Results IE shall be included in the E-CID Measurement Result IE of the E-CID MEASUREMENT REPORT message when measurement results other than the "Cell-ID" have been requested.

If available, the eNB shall include the E-UTRAN Access Point Position IE which is the configured estimated serving antenna position in the E-CID Measurement Result IE within the E-CID MEASUREMENT REPORT message. Upon reception of this E-UTRAN Access Point Position IE, the E-SMLC may use the value as the geographical position of the E-UTRAN access point.

If available, the eNB shall include the Cell Portion ID IE in the E-CID MEASUREMENT REPORT message. Upon reception of the Cell Portion ID IE, the E-SMLC may use the value as the cell portion for the measurement.

#### 8.2.3.3	Unsuccessful Operation

Not applicable.

#### 8.2.3.4	Abnormal Conditions

Void.

### 8.2.4	E-CID Measurement Termination

#### 8.2.4.1	General

The purpose of E-CID Measurement Termination procedure is to terminate periodical E-CID measurements for the UE performed by the eNB.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: E-CID Measurement Termination procedure, successful operation

The E-SMLC initiates the procedure by generating an E-CID MEASUREMENT TERMINATION COMMAND message.

#### 8.2.4.3	Unsuccessful Operation

Not applicable.

#### 8.2.4.4	Abnormal Conditions

Void.

### 8.2.5	OTDOA Information Exchange

#### 8.2.5.1	General

The purpose of the OTDOA Information Exchange procedure is to allow the E-SMLC to request the eNB to transfer OTDOA information to the E-SMLC.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: OTDOA Information Exchange procedure, successful operation

The E-SMLC initiates the procedure by sending an OTDOA INFORMATION REQUEST message. The eNB responds with OTDOA INFORMATION RESPONSE message that contains the available OTDOA information applicable to the relevant cells/TPs.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: OTDOA Information Exchange procedure, unsuccessful operation

If the eNB does not have any OTDOA information to report, the eNB shall respond with an OTDOA INFORMATION FAILURE message.

#### 8.2.5.4	Abnormal Conditions

Void.

### 8.2.6	UTDOA Information Exchange

#### 8.2.6.1	General

The UTDOA Information Exchange procedure is initiated by the E-SMLC to indicate to the eNB the need to configure the UE to transmit periodic SRS signals and to retrieve the SRS configuration from the eNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: UTDOA Information Exchange procedure, successful operation

The E-SMLC initiates the procedure by sending a UTDOA INFORMATION REQUEST message to the eNB. This message may contain the bandwidth and number of SRS transmissions desired. If the E-SMLC requests a number of SRS transmissions, the eNB may take this information into account when configuring SRS transmissions for the UE. The eNB shall reply with the UTDOA INFORMATION RESPONSE message.

The UTDOA INFORMATION RESPONSE message contains the SRS configuration for the UE. The eNB shall include the deltaSS IE in the UTDOA INFORMATION RESPONSE message whenever SRS sequence hopping is enabled for the requested measurement. If the deltaSS IE is received by the E-SMLC in the UTDOA INFORMATION RESPONSE message, the E-SMLC shall consider that SRS sequence hopping is enabled for that particular measurement.

#### 8.2.6.3	Unsuccessful Operation

Figure 8.2.6.3-1: UTDOA Information Exchange procedure, unsuccessful operation

If the eNB is unable to configure any SRS transmissions for the UE, the eNB shall respond with a UTDOA INFORMATION FAILURE message. If a handover of the target UE has been triggered, the eNB shall send a UTDOA INFORMATION FAILURE message with an appropriate cause value.

#### 8.2.6.4	Abnormal Conditions

Void.

### 8.2.7	UTDOA Information Update

#### 8.2.7.1	General

The UTDOA Information Update procedure is sent by the eNB to indicate to the E-SMLC that a change has occurred in the SRS configuration, either due to a change in SRS configuration parameters in one or more cells, or because a cell change has been triggered.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: UTDOA Information Update procedure, successful operation

The eNB initiates the procedure by sending a UTDOA INFORMATION UPDATE message to the E-SMLC. This message contains, in the case of a change in SRS configuration parameters, the SRS configuration information for all cells with SRS configured. The eNB shall include the deltaSS IE in the UTDOA INFORMATION UPDATE message whenever SRS sequence hopping is enabled for the requested measurement. If the deltaSS IE is received by the E-SMLC in the UTDOA INFORMATION UPDATE message, the E-SMLC shall consider that SRS sequence hopping is enabled for that particular measurement.

#### 8.2.7.3	Unsuccessful Operation

Not Applicable.

#### 8.2.7.4	Abnormal Conditions

Void.

## 8.3	Management Procedures

### 8.3.1	Error Indication

#### 8.3.1.1	General

The Error Indication procedure is initiated by a node to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Error Indication procedure, E-SMLC originated, successful operation

Figure 8.3.1.2-2: Error Indication procedure, eNB originated, successful operation

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE.

#### 8.3.1.3	Abnormal Conditions

Not applicable.

## 8.4	Assistance Information Transfer Procedures

### 8.4.1	Assistance Information Control

#### 8.4.1.1	General

The purpose of the Assistance Information Control procedure is to allow the E-SMLC to signal positioning assistance information to the eNB for assistance information broadcasting.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2-1: Assistance Information Control procedure

The E-SMLC initiates the procedure by sending an ASSISTANCE INFORMATION CONTROL message.

If the Assistance Information IE is included in the ASSISTANCE INFORMATION CONTROL message, the eNB shall replace any previously stored assistance information and use the received information to configure assistance information broadcasting.

If the Broadcast Priority IE is included in the Assistance Information IE, the eNB may take it into account when configuring broadcasting for the relevant information. Assistance information having the same Broadcast Priority value should receive the same treatment (i.e. broadcast by the eNB or not broadcast).

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", the eNB may start broadcasting the assistance information. If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "stop", the eNB may stop broadcasting the assistance information.

#### 8.4.1.3	Abnormal Conditions

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", and no assistance information is available, the eNB shall consider the procedure as failed.

If neither the Assistance Information IE nor the Broadcast IE are included in the ASSISTANCE INFORMATION CONTROL message, the eNB shall consider the procedure as failed.

### 8.4.2	Assistance Information Feedback

#### 8.4.2.1	General

The purpose of the Assistance Information Feedback procedure is to allow the eNB to give feedback to the E-SMLC on assistance information broadcasting.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Assistance Information Feedback procedure

If the Assistance Information Failure List IE is included in the ASSISTANCE INFORMATION FEEDBACK message, the E-SMLC shall consider that assistance information broadcasting could not be configured for the relevant information.

#### 8.4.2.3	Abnormal Conditions

Void.

# 9	Elements for LPPa Communication

## 9.0	General

Sub clauses 9.1 and 9.2 describe the structure of the messages and information elements required for the LPPa protocol in tabular format. Sub clause 9.3 provides the corresponding ASN.1 definition.

The following attributes are used for the tabular description of the messages and information elements: Presence, Range Criticality and Assigned Criticality. Their definition and use can be found in TS 36.413 [3].

NOTE:	The messages have been defined in accordance to the guidelines specified in TR 25.921 [9].

## 9.1	Message Functional Definition and Content

### 9.1.1	Messages for Location Information Transfer Procedures

#### 9.1.1.1	E-CID MEASUREMENT INITIATION REQUEST

This message is sent by E-SMLC to initiate E-CID measurements.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Report Characteristics | M |  | ENUMERATED(OnDemand, Periodic,…) |  | YES | reject |
| Measurement Periodicity | C-ifReportCharacteristicsPeriodic |  | ENUMERATED(120ms, 240ms, 480ms, 640ms, 1024ms, 2048ms, 5120ms, 10240ms, 1min, 6min, 12min, 30min, 60min,…) |  | YES | reject |
| Measurement Quantities |  | 1 .. <maxnoMeas> |  |  | YES | reject |
| >Measurement Quantities Item | M |  | ENUMERATED (Cell-ID, Angle of Arrival, Timing Advance Type 1, Timing Advance Type 2, RSRP, RSRQ,…) |  | EACH | reject |
| Inter-RAT Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | YES | ignore |
| >Inter-RAT Measurement Quantities Item | M |  | ENUMERATED(GERAN, UTRAN,… , NR) |  | EACH | reject |
| WLAN Measurement Quantities |  | 0 .. <maxnoMeas> |  |  | YES | ignore |
| >WLAN Measurement Quantities Item | M |  | ENUMERATED (WLAN, ...) |  | EACH | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value “Periodic”. |

#### 9.1.1.2	E-CID MEASUREMENT INITIATION RESPONSE

This message is sent by eNB to indicate that the requested E-CID measurement is successfully initiated.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| eNB UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| E-CID Measurement Result | O |  | 9.2.5 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |
| Inter-RAT Measurement Result | O |  | 9.2.13 |  | YES | ignore |
| WLAN Measurement Result | O |  | 9.2.15 |  | YES | ignore |

#### 9.1.1.3	E-CID MEASUREMENT INITIATION FAILURE

This message is sent by eNB to indicate that the requested E-CID measurement cannot be initiated.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.4	E-CID MEASUREMENT FAILURE INDICATION

This message is sent by eNB to indicate that the previously requested E-CID measurement can no longer be reported.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| eNB UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| Cause | M |  | 9.2.1 |  | YES | ignore |

#### 9.1.1.5	E-CID MEASUREMENT REPORT

This message is sent by eNB to report the results of the requested E-CID measurement.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| eNB UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| E-CID Measurement Result | M |  | 9.2.5 |  | YES | ignore |
| Cell Portion ID | O |  | 9.2.12 |  | YES | ignore |

#### 9.1.1.6	E-CID MEASUREMENT TERMINATION COMMAND

This message is sent by the E-SMLC to terminate the requested E-CID measurement.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| E-SMLC UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |
| eNB UE Measurement ID | M |  | INTEGER(1..15,…) |  | YES | reject |

#### 9.1.1.7	OTDOA INFORMATION REQUEST

This message is sent by E-SMLC to request OTDOA information.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| OTDOA Information Type |  | 1 .. <maxnoOTDOAtypes> |  |  | YES | reject |
| >OTDOA Information Item | M |  | ENUMERATED (pci, cellid, tac, earfcn, prsBandwidth, prsConfigIndex, cpLength, noDlFrames, noAntennaPorts, sFNInitTime, …, e-UTRANAccessPointPosition, prsmutingconfiguration, prsid, tpid, tpType, crsCPlength, MBSFNsubframeConfiguration, nPRSConfiguration, offsetNBChanneltoEARFCN, operationModeInfo, NPRS-ID, dlBandwidth, multipleprsConfigurationsperCell, prsOccasionGroup, prsFrequencyHoppingConfiguration, repetitionNumberofSIB1-NB, nPRSsequenceInfo, NPRS Type 2, tddConfig) |  | EACH | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoOTDOAtypes | Maximum no. of OTDOA information types that can be requested and reported with one message. Value is 63. |

#### 9.1.1.8	OTDOA INFORMATION RESPONSE

This message is sent by eNB to provide OTDOA information.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| OTDOA Cells |  | 1 .. <maxCellineNB> |  | Served cells/TPs that broadcast PRS. May be used to signal multiple PRS configurations per cell/TPs (up to 3 are supported in this release). | GLOBAL | ignore |
| >OTDOA Cell Information | M |  | 9.2.7 |  | - | - |
| Additional OTDOA Cells |  | 0 .. <maxCellineNB-ext> |  | Served cells/TPs that broadcast PRS. May be used to signal multiple PRS configurations per cell/TPs (up to 3 are supported in this release). | GLOBAL | ignore |
| >OTDOA Cell Information | M |  | 9.2.7 |  | - | - |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxCellineNB-ext | Maximum no. of additional cells/TPs that can be served by an eNB. Value is 3840. |

#### 9.1.1.9	OTDOA INFORMATION FAILURE

This message is sent by eNB to indicate that the OTDOA information cannot be provided.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.10	UTDOA INFORMATION REQUEST

This message is sent by the E-SMLC to indicate to the eNB the need to configure the UE to transmit periodic SRS signals for UTDOA positioning.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.10 |  | YES | ignore |

#### 9.1.1.11	UTDOA INFORMATION RESPONSE

This message is sent by the eNB to provide the configured SRS information to the E-SMLC.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| UL Configuration | M |  | 9.2.11 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.12	UTDOA INFORMATION FAILURE

This message is sent by the eNB to indicate that no SRS transmissions could be configured for the UE for UTDOA positioning.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.13	UTDOA INFORMATION UPDATE

This message is sent by the eNB to indicate that the SRS configuration for the UE, for one or more cells, has changed.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| UL Configuration | O |  | 9.2.11 |  | YES | ignore |

### 9.1.2	Messages for Management Procedures

#### 9.1.2.1	ERROR INDICATION

This message is used to indicate that some error has been detected in the eNB or in the E-SMLC.

Direction: E-SMLC  eNB and eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| LPPa Transaction ID | M |  | 9.2.4 |  | – |  |
| Cause | O |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

### 9.1.3	Messages for Assistance Information Transfer Procedures

#### 9.1.3.1	ASSISTANCE INFORMATION CONTROL

This message is sent by the E-SMLC to transfer assistance information.

Direction: E-SMLC  eNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information | O |  | 9.2.20 |  | YES | reject |
| Broadcast | O |  | ENUMERATED (start, stop, ...) |  | YES | reject |

#### 9.1.3.2	ASSISTANCE INFORMATION FEEDBACK

This message is sent by the eNB to give feedback on assistance information broadcasting.

Direction: eNB  E-SMLC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| LPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information Failure List | O |  | 9.2.24 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

## 9.2	Information Element definitions

### 9.2.0	General

When specifying information elements which are to be represented by bit strings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bit strings from other specifications, the first bit of the bit string contains the first bit of the concerned information.

### 9.2.1	Cause

The purpose of the cause information element is to indicate the reason for a particular event for the whole protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, Requested Item not Supported, Requested Item Temporarily not Available, ... ) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Unspecified, Abstract Syntax Error (Falsely Constructed Message),...) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. In general, "not supported" cause values indicate that the concerned capability is missing. On the other hand, "not available" cause values indicate that the concerned capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network Layer related |
| Requested Item not Supported | The eNB does not support the requested measurement object, or cannot provide the requested information item. |
| Requested Item Temporarily not Available | The eNB can temporarily not provide the requested measurement object or information item. |

| Protocol cause | Meaning |
| --- | --- |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerned criticality indicated "reject" (see sub clause 10.3) |
| Abstract Syntax Error (Ignore and Notify) | The received message included an abstract syntax error and the concerned criticality indicated "ignore and notify" (see sub clause 10.3) |
| Abstract syntax error (falsely constructed message) | The received message contained IEs or IE groups in wrong order or with too many occurrences (see sub clause 10.3) |
| Message not Compatible with Receiver State | The received message was not compatible with the receiver state (see sub clause 10.4) |
| Semantic Error | The received message included a semantic error (see sub clause 10.4) |
| Transfer Syntax Error | The received message included a transfer syntax error (see sub clause 10.2) |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related |

| Miscellaneous cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies and the cause is not related to any of the categories Radio Network Layer, Transport Network Layer or Protocol. |

### 9.2.2	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the eNB or E-SMLC when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing. The conditions for inclusion of the LPPa Transaction ID IE are described in clause 10.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error. |
| Triggering Message | O |  | ENUMERATED (initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| Procedure Criticality | O |  | ENUMERATED (reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| LPPa Transaction ID | O |  | 9.2.4 |  |
| Information Element Criticality Diagnostics |  | 0 .. <maxNrOfErrors> |  |  |
| >IE Criticality | M |  | ENUMERATED (reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value "ignore'" shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE ID of the not understood or missing IE. |
| >Type Of Error | M |  | ENUMERATED (not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxNrOfErrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxNroOfErrors is 256. |

### 9.2.3	Message Type

The Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Procedure Code | M |  | INTEGER (0..255) |  |
| Type of Message | M |  | CHOICE (Initiating Message, Successful Outcome, Unsuccessful Outcome, …) |  |

### 9.2.4	LPPa Transaction ID

The LPPa Transaction ID IE is used to associate all the messages belonging to the same procedure. Messages belonging to the same procedure shall use the same LPPa Transaction ID.

The LPPa Transaction ID is determined by the initiating peer of a procedure.

The LPPa Transaction ID shall uniquely identify a procedure among all ongoing parallel procedures using the same procedure code, and initiated by the same protocol peer.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| LPPa Transaction ID | M |  | INTEGER (0..32767) |  |

### 9.2.5	E-CID Measurement Result

The purpose of the E-CID Measurement Result information element is to provide the E-CID measurement result.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Serving Cell ID | M |  | ECGI 9.2.6 | E-UTRAN Cell Identifier of the serving cell |
| Serving Cell TAC | M |  | OCTET STRING(2) | Tracking Area Code of the serving cell |
| E-UTRAN Access Point Position | O |  | 9.2.8 | The configured estimated geographical position of the antenna of the cell. |
| Measured Results |  | 0 .. <maxnoMeas> |  |  |
| >CHOICE Measured Results Value | M |  |  |  |
| >>Value Angle of Arrival |  |  | INTEGER (0..719) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 1 |  |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Value Timing Advance Type 2 |  |  | INTEGER (0..7690) | According to mapping in TS 36.133 [8] |
| >>Result RSRP |  | 1 .. <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRP | M |  | INTEGER(0..97, …) |  |
| >>Result RSRQ |  | 1 . <maxCellReport> |  |  |
| >>>PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the reported cell |
| >>>EARFCN | M |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5] |
| >>>ECGI | O |  | ECGI 9.2.6 | E-UTRAN Cell Global Identifier of the reported cell |
| >>>Value RSRQ | M |  | INTEGER(0..34, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxCellReport | Maximum no. of cells that can be reported with one message. Value is 9. |

### 9.2.6	ECGI

The E-UTRAN Cell Global Identifier (ECGI) is used to globally identify a cell.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PLMN identity | M |  | OCTET STRING (SIZE (3)) | PLMN identity - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n -The Selected PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |
| E-UTRAN Cell Identifier | M |  | BIT STRING (28) |  |

### 9.2.7	OTDOA Cell Information

This IE contains OTDOA information of a cell/TP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| OTDOA Cell Information |  | 1 .. <maxnoOTDOAtypes> |  |  |
| >CHOICE OTDOA Cell Information Item | M |  |  |  |
| >>PCI |  |  | INTEGER (0..503, …) | Physical Cell ID |
| >>Cell ID |  |  | ECGI 9.2.6 |  |
| >>TAC |  |  | OCTET STRING(2) | Tracking Area Code |
| >>EARFCN |  |  | INTEGER (0..65535, ..., 65536..262143) | Corresponds to NDL for FDD and NDL/UL for TDD in ref. TS 36.104 [5]. For an inband mode NB-IoT Cell, this IE indicates the E-UTRAN EARFCN. |
| >>PRS Bandwidth |  |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100, ...) | Transmission bandwidth of PRS |
| >>PRS Configuration Index |  |  | INTEGER (0..4095, ...) | PRS Configuration Index, ref TS 36.211 [6] |
| >>CP Length |  |  | ENUMERATED (Normal, Extended,...) | Cyclic prefix length of the PRS |
| >>Number of DL Frames |  |  | ENUMERATED (sf1, sf2, sf4, sf6,…) | Number of consecutive downlink subframes NPRS with PRS, ref TS 36.211 [6] |
| >>Number of Antenna Ports |  |  | ENUMERATED(n1-or-n2, n4,…) | Number of used antenna ports, where n1-or-n2 corresponds to 1 or 2 ports, n4 corresponds to 4 ports |
| >>SFN Initialisation Time |  |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. |
| >>E-UTRAN Access Point Position |  |  | 9.2.8 | The configured estimated geographical position of the antenna of the cell/TP. |
| >>PRS Muting Configuration |  |  | 9.2.9 | The configuration of positioning reference signals muting pattern. |
| >>PRS-ID |  |  | INTEGER (0..4095,…) | PRS ID, ref TS 36.211 [6]. |
| >>TP-ID |  |  | INTEGER (0..4095,…) | Identity of the transmission point. This IE together with the PCI and/or PRS-ID may be used to identify the transmission point in case the same physical cell ID is shared by multiple transmission points. |
| >>TP Type |  |  | ENUMERATED (prs-only-tp, …) | A TP which transmits PRS only. |
| >>Number of DL Frames-Extended |  |  | INTEGER (1..160,…) | Number of consecutive downlink subframes NPRS with PRS, ref TS 36.211 [6]. |
| >>CRS CP Length |  |  | ENUMERATED (Normal, Extended,...) | Cyclic prefix length of the CRS. |
| >>MBSFN subframe Configuration |  |  | 9.2.14 | The MBSFN subframe configuration. |
| >>NPRS configuration |  |  | 9.2.16 | The NPRS configuration with the mapping to resource elements as specified for the Type 1 NPRS in TS 36.211 sub-clause 10.2.6A.2 [6]. Only applicable for inband mode NB-IoT operation. |
| >>Offset of NB-IoT Channel Number to DL EARFCN |  |  | Offset of NB-IoT Channel Number to EARFCN 9.2.18 | Corresponds to MDL in TS 36.104 [5] |
| >>operationModeInfo |  |  | ENUMERATED (inband, guardband, standalone,...) |  |
| >>NPRS-ID |  |  | INTEGER (0..4095,…) | NPRS ID, ref TS 36.211 [6]. |
| >>DL Bandwidth |  |  | ENUMERATED (bw6, bw15, bw25, bw50, bw75, bw100, ...) | DL transmission bandwidth expressed in units of resource blocks NRB, ref TS 36.104 [5]. |
| >>PRS Occasion Group |  |  | ENUMERATED (og2, og4, og8, og16, og32, og64, og128, ...) | PRS occasion group in a PRS period, ref TS 36.211 [6]. |
| >>PRS Frequency Hopping Configuration |  |  | 9.2.19 | PRS frequency hopping configuration. |
| >>Repetition Number of SIB1-NB |  |  | ENUMERATED (r4, r8, r16, ...) | Repetition Number of SIB1-NB, refer to TS36.213 [12]. Value r4 corresponds to 4 repetitions, r8 to 8 repetitions, and r16 to 16 repetitions. |
| >>NPRSsequenceInfo |  |  | INTEGER (0..174,…) | The index of the PRB containing the NPRS as defined in the table nprsSequenceInfo to E-UTRA PRB index relation, refer to TS 36.355 [13]. Only included in case of inband mode NB-IoT operation. |
| >>NPRS Type 2 |  |  | 9.2.16 | The NPRS configuration with the mapping to resource elements as specified for the Type 2 NPRS in TS 36.211 sub-clause 10.2.6A.2 [6]. |
| >>TDD Configuration |  |  | 9.2.25 | TDD specific physical channel configuration. |

| Range bound | Explanation |
| --- | --- |
| maxnoOTDOAtypes | Maximum no. of OTDOA information types that can be requested and reported with one message. Value is 63. |

### 9.2.8	E-UTRAN Access Point Position

E-UTRAN Access Point Position IE is used to identify the geographical position of an E-UTRAN Access Point. It is expressed as ellipsoid point with altitude and uncertainty ellipsoid according to TS 23.032 [7].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER (0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degrees (0°.. 90°). |
| Degrees Of Longitude | M |  | INTEGER (-223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degrees (-180°..+180°). |
| Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| Altitude | M |  | INTEGER (0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a < N+1, except for N=215-1 for which the range is extended to include all greater values of (a). |
| Uncertainty semi-major | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Uncertainty semi-minor | M |  | INTEGER (0..127) | The uncertainty "r" is derived from the "uncertainty code" k by r = 10x(1.1k-1). |
| Orientation of major axis | M |  | INTEGER (0..179) |  |
| Uncertainty Altitude | M |  | INTEGER (0..127) | The uncertainty altitude “h” expressed in metres is derived from the “uncertainty code” k, by: h=45x(1.025k-1). |
| Confidence | M |  | INTEGER(0..100) | In percentage |

### 9.2.9	PRS Muting Configuration

The PRS Muting Configuration IE is used to describe the configuration of PRS muting patterns for the concerned cell/TP, according to TS 36.211 [6] and TS 36.133 [8].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE PRS Muting Configuration | M |  |  |  |
| >Two |  |  | BIT STRING (2) | If a bit is set to "0", it indicates that the PRS is muted in the corresponding PRS positioning occasion (numbering from any sub frame for which SFN=0) in a periodic cycle of length equal to the length of the bit string |
| >Four |  |  | BIT STRING (4) | Same as above |
| >Eight |  |  | BIT STRING (8) | Same as above |
| >Sixteen |  |  | BIT STRING (16) | Same as above |
| >thirty-two |  |  | BIT STRING (32) | Same as above |
| >sixty-four |  |  | BIT STRING (64) | Same as above |
| >one-hundred-and-twenty-eight |  |  | BIT STRING (128) | Same as above |
| >two-hundred-and-fifty-six |  |  | BIT STRING (256) | Same as above |
| >five-hundred-and-twelve |  |  | BIT STRING (512) | Same as above |
| >one-thousand-and-twenty-four |  |  | BIT STRING (1024) | Same as above |

### 9.2.10	Requested SRS Transmission Characteristics

The purpose of the Requested SRS Transmissions Characteristics information element is to inform the eNB of the number and bandwidth of periodic SRS transmissions requested for the UE for the purpose of UTDOA positioning.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number Of Transmissions | M |  | INTEGER (0..500,…) | The number of periodic SRS transmissions requested. The value of ‘0’ represents an infinite number of SRS transmissions. |
| Bandwidth | M |  | INTEGER (1..100,…) | The requested bandwidth of the SRS transmissions, the value of which corresponds to the number of resource blocks requested to be allocated. |

### 9.2.11	UL Configuration

The purpose of the Uplink Configuration information element is to inform the E-SMLC of the uplink configuration parameters.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PCI | M |  | INTEGER (0..503, …) | Physical Cell Identifier of the PCell |
| UL EARFCN | M |  | INTEGER (0..262143, …) | The uplink E-UTRA carrier frequency of the PCell |
| TA Type1 | O |  | INTEGER (0..7690) | Timing advance measurement, the mapping of the reported quantity is defined in TS 36.133 [8] |
| TA Type2 | O |  | INTEGER (0..7690) | Timing advance measurement, the mapping of the reported quantity is defined in TS 36.133 [8] |
| Number of Transmissions | M |  | INTEGER (0..500,…) | The number of periodic SRS transmissions. The value of ‘0’ represents an infinite number of SRS transmissions. |
| SRS Configuration | M | 1 ..<maxServCell> |  | Configuration of SRS for corresponding serving cells. |
| >PCI | M |  | INTEGER (0..503, …) | Physical Cell ID. |
| >UL EARFCN | M |  | INTEGER (0..262143, ...) | The uplink E-UTRA carrier frequency of the corresponding serving cell. Corresponds to NUL in TS 36.104 [5]. |
| >UL-bandwidth | M |  | ENUMERATED (n6, n15, n25, n50, n75, n100) | Cell transmission bandwidth configuration in uplink corresponding to an E-UTRA channel bandwidth TS 36.104 [5], Table 5.6-1. Value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. |
| >UL-CyclicPrefixLength | M |  | ENUMERATED (Normal, Extended) | Uplink cyclic prefix. |
| >srs-BandwidthConfig | M |  | ENUMERATED (bw0, bw1, bw2, bw3, bw4, bw5, bw6, bw7) | Cell-specific SRS bandwidth configuration TS 36.211 [6]. bw0 corresponds to value 0, bw1 to value 1 and so on |
| >srs-Bandwidth | M |  | ENUMERATED (bw0, bw1, bw2, bw3) | UE-specific SRS bandwidth configuration TS 36.211 [6] |
| >srs-AntennaPort | M |  | ENUMERATED (an1, an2, an4, ...) | .Number of antenna ports for SRS transmission. TS 36.211 [6] |
| >srs-HoppingBandwidth | M |  | ENUMERATED (hbw0, hbw1, hbw2, hbw3) | SRS frequency hopping bandwidth configuration TS 36.211 [6] |
| >srs-cyclicShift | M |  | ENUMERATED (cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7) | SRS-Cyclic shift TS 36.211 [6] |
| >srs-ConfigIndex | M |  | INTEGER (0..1023) | SRS configuration index TS 36.211 [6] |
| >MaxUpPt | C- IfTDD |  | ENUMERATED (true) | MaxUpPt TS 36.211[6] |
| >transmissionComb | M |  | INTEGER (0..1) | Transmission comb TS 36.211 [6] |
| >freqDomainPosition | M |  | INTEGER (0..23) | Frequency domain position TS 36.211 [6] |
| >groupHoppingEnabled | M |  | BOOLEAN | Group-hopping-enabled TS 36.211 [6] |
| >deltaSS | O |  | INTEGER (0..29) | deltaSS TS 36.211 [6] |
| >SFN Initialisation Time | M |  | BIT STRING (64) | Time in seconds relative to 00:00:00 on 1 January 1900 (calculated as continuous time without leap seconds and traceable to a common time reference) where binary encoding of the integer part is in the first 32 bits and binary encoding of the fraction part in the last 32 bits. The fraction part is expressed with a granularity of 1 /2**32 second. |

| Condition | Explanation |
| --- | --- |
| IfTDD | This IE shall be present if the UL-EARFCN IE refers to TDD operation. |

| Range bound | Explanation |
| --- | --- |
| maxServCell | Maximum number of serving cells with SRS configuration. Value is 5. |

### 9.2.12	Cell Portion ID

This parameter gives the current Cell Portion associated with the target UE. The Cell Portion ID is the unique identifier for a cell portion within a cell.

Table 9.2.12-1: Cell Portion

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Portion ID | M |  | INTEGER (0..255, …, 256..4095) |  |

### 9.2.13	Inter-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-RAT Measured Results |  | 1.. <maxnoMeas> |  |  | - |  |
| >CHOICE Inter-RAT Measured Results Value | M |  |  |  | - |  |
| >>Result GERAN |  | 1..<maxGERANMeas> |  |  | - |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  | - |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  | - |  |
| >>>RSSI | M |  | INTEGER(0..63, ...) |  | - |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  | - |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  | - |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  | - |  |
| >>>>Physical CellId UTRA FDD |  |  | INTEGER (0..511, ...) |  | - |  |
| >>>>Physical CellId UTRA TDD |  |  | INTEGER (0..127, ...) |  | - |  |
| >>>UTRA RSCP | O |  | INTEGER(-5..91, ...) |  | - |  |
| >>>UTRA EcNo | O |  | INTEGER(0..49, ...) | This IE applies to FDD only. | - |  |
| >>Result NR |  | 1..<maxNRMeas> |  |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0.. 3279165) |  | - |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR SS-RSRP | O |  | INTEGER (0..127) | Cell level measurement result of NR SS-RSRP | - |  |
| >>>NR SS-RSRQ | O |  | INTEGER (0..127) | Cell level measurement result of NR SS-RSRQ | - |  |
| >>>ResultsPerSSB Index List |  | 0..1 |  |  | YES | ignore |
| >>>>ResultsPerSSB Index Item |  | 1..<maxResultsPerSSBIndex> |  |  |  |  |
| >>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>NR SS-RSRP beam value | O |  | INTEGER (0..127) | Beam level measurement result of NR SS-RSRP | - |  |
| >>>>>NR SS-RSRQ beam value | O |  | INTEGER (0..127) | Beam level measurement result of NR SS-RSRQ | - |  |
| >>>NR CGI | O |  | NR CGI 9.2.26 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 32. |
| maxResultsPerSSBIndex | Maximum no. of NR SSB indices that can be reported with one message. Value is 64. |

### 9.2.14	MBSFN subframe Configuration

The MBSFN subframe Configuration IE describes the MBSFN subframe configuration for the concerned cell/TP.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| MBSFN subframe Configuration Value |  | 1 .. < maxMBSFN-Allocations > |  |  |
| >Radio Frame Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32) | Corresponds to information provided in radioFrameAllocationPeriod contained in the MBSFN-SubframeConfig IE as defined in TS 36.331 [10] |
| >Radio Frame Allocation Offset | M |  | INTEGER (0..7) | Corresponds to information provided in radioFrameAllocationOffset contained in the MBSFN-SubframeConfig IE as defined in TS 36.331 [10] |
| > CHOICE Subframe Allocation | M |  |  | Corresponds to information provided in subframeAllocation contained in the MBSFN-SubframeConfig IE as defined in TS 36.331 [10] |
| >>oneFrame |  |  | BIT STRING (SIZE(6)) |  |
| >>fourFrames |  |  | BIT STRING (SIZE(24)) |  |

| Range bound | Explanation |
| --- | --- |
| maxMBSFN-Allocations | Maximum number of MBSFN frame allocations with different offset as defined in TS 36.331 [10]. Value is 8. |

### 9.2.15	WLAN Measurement Result

The WLAN Measurement Result information element provides the WLAN measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| WLAN Measured Results |  | 1.. <maxnoMeas> |  |  |
| >WLAN RSSI | M |  | INTEGER (0..141, ...) |  |
| >SSID | O |  | OCTET STRING (SIZE(1..32)) | Includes the SSID field as defined in subclause 8.4.2.2 of IEEE 802.11™ [11]. |
| >BSSID | O |  | OCTET STRING (SIZE(6)) | Includes the BSSID field as defined in subclause 8.2.4.3.4 of IEEE 802.11™ [11]. |
| >HESSID | O |  | OCTET STRING (SIZE(6)) | Includes the HESSID field as defined in subclause 8.4.2.94 of IEEE 802.11™ [11]. |
| >Operating Class | O |  | INTEGER (0..255) | Indicates the WLAN Operating Class as defined in IEEE 802.11™ [11]. |
| >Country Code |  |  | ENUMERATED (unitedStates, europe, japan, global, …) | Indicates the WLAN country code as defined in IEEE 802.11™ [11]. |
| >WLAN Channel List |  | 0..1 |  |  |
| >>WLAN Channel List Item |  | 1..<maxWLANchannels> |  |  |
| >>>WLAN Channel |  |  | INTEGER (0..255) | Indicates the WLAN channel number as defined in IEEE 802.11™ [11]. |
| >WLAN Band | O |  | ENUMERATED (band2dot4, band5, …) | Indicates the WLAN band as defined in IEEE 802.11™ [11]. |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxWLANchannels | Maximum no. of WLAN channels that can be reported within one list. Value is 16. |

### 9.2.16	NPRS configuration

The NPRS Configuration IE is used to describe the configuration of NPRS for the concerned NB-IOT carrier.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NPRS subframe configuration Part A | O |  |  | For NPRS subframe configuration Part A and NPRS subframe configuration Part B, at least one of them must be present in NPRS configuration. |
| >CHOICE Bitmaps for NPRS subframes | M |  |  |  |
| >>Ten |  |  | BIT STRING(SIZE(10)) | Subframes not containing NPRS are indicated with ‘0’. Subframes containing NPRS are indicated with ‘1’ |
| >> Forty |  |  | BIT STRING(SIZE(40)) | Same as above |
| >>Ten-TDD |  |  | BIT STRING(SIZE(8)) | Subframes not containing NPRS are indicated with ‘0’. Subframes containing NPRS are indicated with ‘1’. The subframe #1 and #2 are not included in this Bit String. This IE applies to NB-IoT TDD only. |
| >> Forty-TDD |  |  | BIT STRING(SIZE(32)) | Same as above. |
| >NPRS Muting Configuration | O |  | 9.2.17 |  |
| NPRS subframe configuration Part B | O |  |  | For NPRS subframe configuration Part A and NPRS subframe configuration Part B, at least one of them must be present in NPRS configuration. |
| >Number of NPRS subframes in one occasion | M |  | ENUMERATED (sf10, sf20, sf40, sf80, sf160, sf320, sf640, sf1280, …, sf2560) | Number of consecutive subframes containing NPRS in one NPRS occasion. The values sf10 and sf20 are only applicable to FDD mode. The value sf2560 is only applicable to TDD mode. |
| >Periodicity of NPRS | M |  | ENUMERATED (sf160, sf320, sf640, sf1280, …, sf2560) | Periodicity of NPRS occasion TNPRS |
| >starting subframe offset of NPRS occasion | M |  | ENUMERATED (zero, one-Eighth, two-Eighths, three-Eighths, four-Eighths, five-Eighths, six-Eighths, seven-Eighths, …) | For a given periodicity of NPRS occasion TNPRS, the starting subframe offset of NPRS occasion = a* TNPRS. |
| >NPRS Muting Configuration | O |  | 9.2.17 |  |
| >SIB1-NB-Subframe-TDD | O |  | ENUMERATED (sf0, sf4, sf0and5, ...) | The subframe(s) in which the SIB1-NB is transmitted. Values sf0 and sf4 correspond with subframe #0 and #4 respectively. Value sf0and5 corresponds with subframes #0 and #5. This IE applies to NB-IoT TDD only. |

### 9.2.17	NPRS Muting Configuration

The NPRS Muting Configuration IE is used to describe the configuration of NPRS muting patterns for the concerned NB-IOT carrier.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE NPRS Muting Configuration | M |  |  |  |
| >Two |  |  | BIT STRING (SIZE(2)) | Each bit in a muting pattern corresponds to: for Part A, consecutive 10 subframes, for Part B, one NPRS occasion. The first bit of the NPRS muting sequence corresponds to the first NPRS positioning occasion (for Part B) or the first NPRS subframes (for PartA) that starts from any subframe for which SFN=0. The sequence is valid for all subframes after the target device has received the nprs-MutingInfo. |
| >Four |  |  | BIT STRING (SIZE(4)) | Same as above |
| >Eight |  |  | BIT STRING (SIZE(8)) | Same as above |
| >Sixteen |  |  | BIT STRING (SIZE(16)) | Same as above |

### 9.2.18	Offset of NB-IoT Channel Number to EARFCN

This IE is used to indicate the offset of the NB-IoT Channel Number to the EARFCN (TS 36.104 [5]).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Offset of NB-IoT Channel Number to DL EARFCN | M |  | ENUMERATED (-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9,...) |  |

### 9.2.19	PRS Frequency Hopping Configuration

The PRS Frequency Hopping Configuration IE is used to describe the configuration of PRS frequency hopping for the concerned cell/TP, according to TS 36.211 [6].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Number of Frequency Hopping Bands | M |  | ENUMERATED (twobands, fourbands, ...) | Number of bands for frequency hopping. |
| Band Positions |  | 1.. <maxnoFreqHoppingBandsMinusOne,...> |  |  |
| >NarrowBand Index | M |  | INTEGER (0..15, ...) | Narrowband Index |

| Range bound | Explanation |
| --- | --- |
| maxnoFreqHoppingBandsMinusOne | Maximum no. of frequency hopping bands minus one. Value is 7. |

### 9.2.20	Assistance Information

This IE contains the assistance information.

Table 9.2.20-1: Assistance Information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information | M |  |  |  |
| >System Information |  | 1..<maxNrOfPosSImessage> |  | Corresponds to the number of SI messages with posSIBs to be scheduled |
| >>Broadcast Periodicity | M |  | ENUMERATED (ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ...) | Corresponds to information provided in posSI-Periodicity contained in the SystemInformationBlockType1 IE as defined in TS 36.331 [10] |
| >>Pos SIBs |  | 1..<maxNrOfPosSIBs> |  | Number of posSIBs in the System Information. |
| >>>PosSIB-Type | M |  | Positioning SIB-Type 9.2.23 |  |
| >>>PosSIB Segments | M |  | 9.2.21 |  |
| >>>Assistance Information Meta Data | O |  | 9.2.22 |  |
| >>>Broadcast Priority | O |  | INTEGER (1..16, ...) | The priority of the assistance Information where 1 represents the highest priority and 16 the lowest priority |

| Range bound | Explanation |
| --- | --- |
| maxNrOfPosSImessage | Maximum number of positioning system information messages. Value is 32. |
| maxNrOfPosSIBs | Maximum number of positioning system information blocks included in the message. Value is 32. |

### 9.2.21	PosSIB Segments

This IE provides one posSIB or two or more posSIB segments which must be scheduled in series in consecutive transmissions of the same SI message.

Table 9.2.21-1: PosSIB Segments

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PosSIB Segments |  | 1..<maxNrOfSegments> |  |  |
| >Assistance Data SIB Element | M |  | OCTET STRING | Includes the assistanceDataSIBElement IE as defined in TS 36.355 [13] |

| Range bound | Explanation |
| --- | --- |
| maxNrOfSegments | Maximum number of positioning SIB segments (in case of Assistance Information Element contains segmented data according to TS 36.355 [13]). Value is 64. |

### 9.2.22	Assistance Information Meta Data

This parameter contains meta data for an assistance information element.

Table 9.2.22-1: Assistance Information Meta Data

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Encrypted | O |  | ENUMERATED (true, …) | Corresponds to information provided in encrypted contained in the SystemInformationBlockType1 IE as defined in TS 36.331 [10] |
| GNSS ID | O |  | ENUMERATED (gps, sbas, qzss, galileo, glonass, bds, ..., navic) | Corresponds to information provided in gnss-id contained in the SystemInformationBlockType1IE as defined in TS 36.331 [10] |
| SBAS ID | O |  | ENUMERATED (waas, egnos, msas, gagan, ...) | Corresponds to to information provided in sbas-id contained in the SystemInformationBlockType1IE as defined in TS 36.331 [10] |

### 9.2.23	Positioning SIB Type

This parameter defines a specific positioning SIB, as defined in TS 36.331 [10].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning SIB Type | M |  | ENUMERATED ( posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType3-1, ..., posSibType4-1, posSibType5-1, posSibType2-24, posSibType2-25) |  |

### 9.2.24	Assistance Information Failure List

This parameter identifies the assistance information for which the eNB failed to configure broadcasting.

Table 9.2.24-1: Assistance Information Failure List

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information Failure List |  | 1..<maxnoAssistInfoFailureListItems> |  |  |
| >PosSIB-Type | M |  | Positioning SIB-Type 9.2.23 |  |
| >Outcome | M |  | ENUMERATED (failed, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoAssistInfoFailureListItems | Maximum no. of assistance information failure list items that can be signaled with one message. Value is 32. |

### 9.2.25	TDD Configuration

This IE is used to indicate the TDD specific physical channel configuration.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Subframe Assignment | M |  | ENUMERATED ( sa0, sa1, sa2, sa3, sa4, sa5, sa6, … ) | sa0 points to Configuration 0, sa1 to Configuration 1 etc. as specified in TS 36.211 [6, table 4.2-2]. Configurations 0 and 6 are not applicable for NB-IoT. |

### 9.2.26	NR CGI

This IE is used to globally identify an NR cell (see TS 38.300 [45]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | OCTET STRING (SIZE (3)) | PLMN identity - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1 - bits 8 to 5 of octet n encoding digit 2n -The Selected PLMN identity consists of 3 digits from MCC followed by either -a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or -3 digits from MNC (in case of a 3 digit MNC). |
| NR Cell Identity | M |  | BIT STRING (SIZE(36)) |  |

## 9.3	Message and Information Element Abstract Syntax (with ASN.1)

### 9.3.1	General

Sub clause 9.3 presents the Abstract Syntax of the LPPa protocol with ASN.1. In case there is contradiction between the ASN.1 definition in this sub clause and the tabular format in sub clause 9.1 and 9.2, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional elements, in which the tabular format shall take precedence.

The ASN.1 definition specifies the structure and content of LPPa messages. LPPa messages can contain any IEs specified in the object set definitions for that message without the order or number of occurrence being restricted by ASN.1. However, for this version of the standard, a sending entity shall construct an LPPa message according to the PDU definitions module and with the following additional rules (Note that in the following, "IE" means an IE in the object set with an explicit id. If one IE needs to appear more than once in one object set, then the different occurrences have different IE ids):

-	IEs shall be ordered (in an IE container) in the order they appear in object set definitions.

-	Object set definitions specify how many times IEs may appear. An IE shall appear exactly once if the presence field in an object has value "mandatory". An IE may appear at most once if the presence field in an object has value "optional" or "conditional". If in a tabular format there is multiplicity specified for an IE (i.e. an IE list) then in the corresponding ASN.1 definition the list definition is separated into two parts. The first part defines an IE container list in which the list elements reside. The second part defines list elements. The IE container list appears as an IE of its own. For this version of the standard an IE container list may contain only one kind of list elements.

If an LPPa message that is not constructed as defined above is received, this shall be considered as Abstract Syntax Error, and the message shall be handled as defined for Abstract Syntax Error in clause 10.

### 9.3.2	Usage of Private Message Mechanism for Non-standard Use

The private message mechanism for non-standard use may be used:

-	for special operator (and/or vendor) specific features considered not to be part of the basic functionality, i.e. the functionality required for a complete and high-quality specification in order to guarantee multivendor inter-operability.

-	by vendors for research purposes, e.g. to implement and evaluate new algorithms/features before such features are proposed for standardisation.

The private message mechanism shall not be used for basic functionality. Such functionality shall be standardised.

### 9.3.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

LPPA-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode,

LPPATransactionID

FROM LPPA-CommonDataTypes

ErrorIndication,

PrivateMessage,

E-CIDMeasurementInitiationRequest,

E-CIDMeasurementInitiationResponse,

E-CIDMeasurementInitiationFailure,

E-CIDMeasurementFailureIndication,

E-CIDMeasurementReport,

E-CIDMeasurementTerminationCommand,

OTDOAInformationRequest,

OTDOAInformationResponse,

OTDOAInformationFailure,

UTDOAInformationRequest,

UTDOAInformationResponse,

UTDOAInformationFailure,

UTDOAInformationUpdate,

AssistanceInformationControl,

AssistanceInformationFeedback

FROM LPPA-PDU-Contents

id-errorIndication,

id-privateMessage,

id-e-CIDMeasurementInitiation,

id-e-CIDMeasurementFailureIndication,

id-e-CIDMeasurementReport,

id-e-CIDMeasurementTermination,

id-oTDOAInformationExchange,

id-uTDOAInformationExchange,

id-uTDOAInformationUpdate,

id-assistanceInformationControl,

id-assistanceInformationFeedback

FROM LPPA-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

LPPA-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&procedureCode			ProcedureCode	UNIQUE,

&criticality			Criticality		DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME	&UnsuccessfulOutcome]

PROCEDURE CODE			&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

LPPA-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode		LPPA-ELEMENTARY-PROCEDURE.&procedureCode		({LPPA-ELEMENTARY-PROCEDURES}),

criticality			LPPA-ELEMENTARY-PROCEDURE.&criticality			({LPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

lppatransactionID	LPPATransactionID,

value				LPPA-ELEMENTARY-PROCEDURE.&InitiatingMessage	({LPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode		LPPA-ELEMENTARY-PROCEDURE.&procedureCode		({LPPA-ELEMENTARY-PROCEDURES}),

criticality			LPPA-ELEMENTARY-PROCEDURE.&criticality			({LPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

lppatransactionID	LPPATransactionID,

value				LPPA-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({LPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode		LPPA-ELEMENTARY-PROCEDURE.&procedureCode		({LPPA-ELEMENTARY-PROCEDURES}),

criticality			LPPA-ELEMENTARY-PROCEDURE.&criticality			({LPPA-ELEMENTARY-PROCEDURES}{@procedureCode}),

lppatransactionID	LPPATransactionID,

value				LPPA-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({LPPA-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

-- Interface Elementary Procedure List

--

-- **************************************************************

LPPA-ELEMENTARY-PROCEDURES LPPA-ELEMENTARY-PROCEDURE ::= {

LPPA-ELEMENTARY-PROCEDURES-CLASS-1			|

LPPA-ELEMENTARY-PROCEDURES-CLASS-2			,

...

}

LPPA-ELEMENTARY-PROCEDURES-CLASS-1 LPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementInitiation		|

oTDOAInformationExchange		,

...,

uTDOAInformationExchange

}

LPPA-ELEMENTARY-PROCEDURES-CLASS-2 LPPA-ELEMENTARY-PROCEDURE ::= {

e-CIDMeasurementFailureIndication		|

e-CIDMeasurementReport					|

e-CIDMeasurementTermination				|

errorIndication							|

privateMessage							,

...,

uTDOAInformationUpdate					|

assistanceInformationControl			|

assistanceInformationFeedback

}

-- **************************************************************

--

-- Interface Elementary Procedures

--

-- **************************************************************

e-CIDMeasurementInitiation LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementInitiationRequest

SUCCESSFUL OUTCOME		E-CIDMeasurementInitiationResponse

UNSUCCESSFUL OUTCOME	E-CIDMeasurementInitiationFailure

PROCEDURE CODE			id-e-CIDMeasurementInitiation

CRITICALITY				reject

}

e-CIDMeasurementFailureIndication LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementFailureIndication

PROCEDURE CODE			id-e-CIDMeasurementFailureIndication

CRITICALITY				ignore

}

e-CIDMeasurementReport LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementReport

PROCEDURE CODE			id-e-CIDMeasurementReport

CRITICALITY				ignore

}

e-CIDMeasurementTermination LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		E-CIDMeasurementTerminationCommand

PROCEDURE CODE			id-e-CIDMeasurementTermination

CRITICALITY				reject

}

oTDOAInformationExchange LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		OTDOAInformationRequest

SUCCESSFUL OUTCOME		OTDOAInformationResponse

UNSUCCESSFUL OUTCOME	OTDOAInformationFailure

PROCEDURE CODE			id-oTDOAInformationExchange

CRITICALITY				reject

}

uTDOAInformationExchange LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UTDOAInformationRequest

SUCCESSFUL OUTCOME		UTDOAInformationResponse

UNSUCCESSFUL OUTCOME	UTDOAInformationFailure

PROCEDURE CODE			id-uTDOAInformationExchange

CRITICALITY				reject

}

uTDOAInformationUpdate LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UTDOAInformationUpdate

PROCEDURE CODE			id-uTDOAInformationUpdate

CRITICALITY				ignore

}

assistanceInformationControl LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationControl

PROCEDURE CODE			id-assistanceInformationControl

CRITICALITY				reject

}

assistanceInformationFeedback LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AssistanceInformationFeedback

PROCEDURE CODE			id-assistanceInformationFeedback

CRITICALITY				reject

}

errorIndication LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		ErrorIndication

PROCEDURE CODE			id-errorIndication

CRITICALITY				ignore

}

privateMessage			LPPA-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		PrivateMessage

PROCEDURE CODE			id-privateMessage

CRITICALITY				ignore

}

END

-- ASN1STOP

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for LPPa.

--

-- **************************************************************

LPPA-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

E-CID-MeasurementResult,

OTDOACells,

OTDOA-Information-Item,

Measurement-ID,

MeasurementPeriodicity,

MeasurementQuantities,

ReportCharacteristics,

RequestedSRSTransmissionCharacteristics,

ULConfiguration,

Cell-Portion-ID,

InterRATMeasurementQuantities,

InterRATMeasurementResult,

Add-OTDOACells,

WLANMeasurementQuantities,

WLANMeasurementResult,

Assistance-Information,

Broadcast,

AssistanceInformationFailureList

FROM LPPA-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Single-Container{},

LPPA-PRIVATE-IES,

LPPA-PROTOCOL-EXTENSION,

LPPA-PROTOCOL-IES,

LPPA-PROTOCOL-IES-PAIR

FROM LPPA-Containers

maxnoOTDOAtypes,

id-Cause,

id-CriticalityDiagnostics,

id-E-SMLC-UE-Measurement-ID,

id-OTDOACells,

id-OTDOA-Information-Type-Group,

id-OTDOA-Information-Type-Item,

id-ReportCharacteristics,

id-MeasurementPeriodicity,

id-MeasurementQuantities,

id-eNB-UE-Measurement-ID,

id-E-CID-MeasurementResult,

id-RequestedSRSTransmissionCharacteristics,

id-ULConfiguration,

id-Cell-Portion-ID,

id-InterRATMeasurementQuantities,

id-InterRATMeasurementResult,

id-AddOTDOACells,

id-WLANMeasurementQuantities,

id-WLANMeasurementResult,

id-Assistance-Information,

id-Broadcast,

id-AssistanceInformationFailureList

FROM LPPA-Constants;

-- **************************************************************

--

-- E-CID MEASUREMENT INITIATION REQUEST

--

-- **************************************************************

E-CIDMeasurementInitiationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationRequest-IEs}},

...

}

E-CIDMeasurementInitiationRequest-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-E-SMLC-UE-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID						PRESENCE mandatory}|

{ ID id-ReportCharacteristics			CRITICALITY reject	TYPE ReportCharacteristics				PRESENCE mandatory}|

{ ID id-MeasurementPeriodicity			CRITICALITY reject	TYPE MeasurementPeriodicity				PRESENCE conditional}|

-- The IE shall be present if the Report Characteritics IE is set to the value “periodic” --

{ ID id-MeasurementQuantities			CRITICALITY reject	TYPE MeasurementQuantities				PRESENCE mandatory}|

{ ID id-InterRATMeasurementQuantities	CRITICALITY ignore	TYPE InterRATMeasurementQuantities	PRESENCE optional}|

{ ID id-WLANMeasurementQuantities		CRITICALITY ignore	TYPE WLANMeasurementQuantities			PRESENCE optional},

...

}

-- **************************************************************

--

-- E-CID MEASUREMENT INITIATION RESPONSE

--

-- **************************************************************

E-CIDMeasurementInitiationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{E-CIDMeasurementInitiationResponse-IEs}},

...

}

E-CIDMeasurementInitiationResponse-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-E-SMLC-UE-Measurement-ID	CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-eNB-UE-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID					PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult		CRITICALITY ignore	TYPE E-CID-MeasurementResult		PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional}|

{ ID id-Cell-Portion-ID				CRITICALITY ignore	TYPE Cell-Portion-ID				PRESENCE optional}|

{ ID id-InterRATMeasurementResult	CRITICALITY ignore	TYPE InterRATMeasurementResult		PRESENCE optional}|

{ ID id-WLANMeasurementResult		CRITICALITY ignore	TYPE WLANMeasurementResult			PRESENCE optional},

...

}

-- **************************************************************

--

-- E-CID MEASUREMENT INITIATION FAILURE

--

-- **************************************************************

E-CIDMeasurementInitiationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementInitiationFailure-IEs}},

...

}

E-CIDMeasurementInitiationFailure-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-E-SMLC-UE-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

-- E-CID MEASUREMENT FAILURE INDICATION

--

-- **************************************************************

E-CIDMeasurementFailureIndication ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementFailureIndication-IEs}},

...

}

E-CIDMeasurementFailureIndication-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-E-SMLC-UE-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory},

...

}

-- **************************************************************

--

-- E-CID MEASUREMENT REPORT

--

-- **************************************************************

E-CIDMeasurementReport ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementReport-IEs}},

...

}

E-CIDMeasurementReport-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-E-SMLC-UE-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-E-CID-MeasurementResult			CRITICALITY ignore	TYPE E-CID-MeasurementResult	PRESENCE mandatory}|

{ ID id-Cell-Portion-ID					CRITICALITY ignore	TYPE Cell-Portion-ID			PRESENCE optional},

...

}

-- **************************************************************

--

-- E-CID MEASUREMENT TERMINATION COMMAND

--

-- **************************************************************

E-CIDMeasurementTerminationCommand ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{E-CIDMeasurementTerminationCommand-IEs}},

...

}

E-CIDMeasurementTerminationCommand-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-E-SMLC-UE-Measurement-ID		CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory}|

{ ID id-eNB-UE-Measurement-ID			CRITICALITY reject	TYPE Measurement-ID				PRESENCE mandatory},

...

}

-- **************************************************************

--

-- OTDOA INFORMATION REQUEST

--

-- **************************************************************

OTDOAInformationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{OTDOAInformationRequest-IEs}},

...

}

OTDOAInformationRequest-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-OTDOA-Information-Type-Group		CRITICALITY reject	TYPE OTDOA-Information-Type			PRESENCE mandatory},

...

}

OTDOA-Information-Type ::= SEQUENCE (SIZE(1..maxnoOTDOAtypes)) OF ProtocolIE-Single-Container { { OTDOA-Information-TypeIEs} }

OTDOA-Information-TypeIEs	LPPA-PROTOCOL-IES ::= {

{ ID id-OTDOA-Information-Type-Item		CRITICALITY reject	TYPE OTDOA-Information-Type-Item	PRESENCE mandatory},

...

}

OTDOA-Information-Type-Item ::= SEQUENCE {

oTDOA-Information-Type-Item		OTDOA-Information-Item,

iE-Extensions					ProtocolExtensionContainer { { OTDOA-Information-Type-ItemExtIEs} } OPTIONAL,

...

}

OTDOA-Information-Type-ItemExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

-- OTDOA INFORMATION RESPONSE

--

-- **************************************************************

OTDOAInformationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{OTDOAInformationResponse-IEs}},

...

}

OTDOAInformationResponse-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-OTDOACells						CRITICALITY ignore	TYPE OTDOACells					PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-AddOTDOACells					CRITICALITY ignore	TYPE Add-OTDOACells				PRESENCE optional},

...

}

-- **************************************************************

--

-- OTDOA INFORMATION FAILURE

--

-- **************************************************************

OTDOAInformationFailure ::= SEQUENCE {

protocolIEs						ProtocolIE-Container		{{OTDOAInformationFailure-IEs}},

...

}

OTDOAInformationFailure-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

-- UTDOA INFORMATION REQUEST

--

-- **************************************************************

UTDOAInformationRequest ::= SEQUENCE {

protocolIEs				ProtocolIE-Container		{{UTDOAInformationRequest-IEs}},

...

}

UTDOAInformationRequest-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-RequestedSRSTransmissionCharacteristics		CRITICALITY ignore	TYPE RequestedSRSTransmissionCharacteristics	PRESENCE optional},

...

}

-- **************************************************************

--

-- UTDOA INFORMATION RESPONSE

--

-- **************************************************************

UTDOAInformationResponse ::= SEQUENCE {

protocolIEs				ProtocolIE-Container		{{UTDOAInformationResponse-IEs}},

...

}

UTDOAInformationResponse-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-ULConfiguration					CRITICALITY reject	TYPE ULConfiguration			PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

-- UTDOA INFORMATION FAILURE

--

-- **************************************************************

UTDOAInformationFailure ::= SEQUENCE {

protocolIEs				ProtocolIE-Container		{{UTDOAInformationFailure-IEs}},

...

}

UTDOAInformationFailure-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-Cause							CRITICALITY ignore	TYPE Cause						PRESENCE mandatory}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

-- UTDOA INFORMATION UPDATE

--

-- **************************************************************

UTDOAInformationUpdate ::= SEQUENCE {

protocolIEs					ProtocolIE-Container		{{UTDOAInformationUpdate-IEs}},

...

}

UTDOAInformationUpdate-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-ULConfiguration		CRITICALITY ignore	TYPE ULConfiguration	PRESENCE optional},

...

}

-- **************************************************************

--

-- ASSISTANCE INFORMATION CONTROL

--

-- **************************************************************

AssistanceInformationControl ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{AssistanceInformationControl-IEs}},

...

}

AssistanceInformationControl-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-Assistance-Information	CRITICALITY reject	TYPE Assistance-Information		PRESENCE optional}|

{ ID id-Broadcast				CRITICALITY reject	TYPE Broadcast					PRESENCE optional},

...

}

-- **************************************************************

--

-- ASSISTANCE INFORMATION FEEDBACK

--

-- **************************************************************

AssistanceInformationFeedback ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{AssistanceInformationFeedback-IEs}},

...

}

AssistanceInformationFeedback-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-AssistanceInformationFailureList	CRITICALITY reject	TYPE AssistanceInformationFailureList	PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional},

...

}

-- **************************************************************

--

-- ERROR INDICATION

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs		ProtocolIE-Container	{{ErrorIndication-IEs}},

...

}

ErrorIndication-IEs LPPA-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional},

...

}

-- **************************************************************

--

-- PRIVATE MESSAGE

--

-- **************************************************************

PrivateMessage ::= SEQUENCE {

privateIEs		PrivateIE-Container	{{PrivateMessage-IEs}},

...

}

PrivateMessage-IEs LPPA-PRIVATE-IES ::= {

...

}

END

-- ASN1STOP

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

LPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-ResultsPerSSB-Index-List,

id-ResultsPerSSB-Index-Item,

id-NR-CGI,

maxCellineNB,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-InterRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxNRmeas,

maxResultsPerSSBIndex,

maxCellineNB-ext,

maxWLANchannels,

maxMBSFN-Allocations,

maxnoFreqHoppingBandsMinusOne,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs

FROM LPPA-Constants

Criticality,

LPPATransactionID,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM LPPA-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

LPPA-PROTOCOL-EXTENSION,

LPPA-PROTOCOL-IES

FROM LPPA-Containers;

-- A

Add-OTDOACells ::= SEQUENCE (SIZE (1.. maxCellineNB-ext)) OF SEQUENCE {

add-OTDOACellInfo				Add-OTDOACell-Information,

iE-Extensions					ProtocolExtensionContainer { {Add-OTDOACells-ExtIEs} } OPTIONAL,

...

}

Add-OTDOACells-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

Add-OTDOACell-Information ::= SEQUENCE (SIZE (1..maxnoOTDOAtypes)) OF OTDOACell-Information-Item

Assistance-Information ::= SEQUENCE {

systemInformation				SystemInformation,

iE-Extensions					ProtocolExtensionContainer { { Assistance-Information-ExtIEs} } OPTIONAL,

...

}

Assistance-Information-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

AssistanceInformationFailureList ::= SEQUENCE (SIZE (1..maxnoAssistInfoFailureListItems)) OF SEQUENCE {

posSIB-Type						PosSIB-Type,

outcome							Outcome,

iE-Extensions					ProtocolExtensionContainer { {AssistanceInformationFailureList-ExtIEs} }	OPTIONAL,

...

}

AssistanceInformationFailureList-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

AssistanceInformationMetaData ::= SEQUENCE {

encrypted			ENUMERATED {true, ...}	OPTIONAL,

gNSSID				ENUMERATED {gps, sbas, gzss, galileo, glonass, bds, ..., navic}	OPTIONAL,

sBASID				ENUMERATED {waas, egnos, msas, gagan, ...}				OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { AssistanceInformationMetaData-ExtIEs} }	OPTIONAL,

...

}

AssistanceInformationMetaData-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

-- B

BCCH ::= INTEGER (0..1023, ...)

BitmapsforNPRS	::= CHOICE {

ten			BIT STRING(SIZE (10)),

forty		BIT STRING(SIZE (40)),

...,

ten-tdd		BIT STRING(SIZE (8)),

forty-tdd	BIT STRING(SIZE (32))}

Broadcast ::= ENUMERATED {

start,

stop,

...

}

BroadcastPeriodicity ::= ENUMERATED {

ms80,

ms160,

ms320,

ms640,

ms1280,

ms2560,

ms5120,

...

}

BSSID ::= OCTET STRING (SIZE(6))

-- C

Cause ::= CHOICE {

radioNetwork		CauseRadioNetwork,

protocol			CauseProtocol,

misc				CauseMisc,

...

}

CauseMisc ::= ENUMERATED {

unspecified,

...

}

CauseProtocol ::= ENUMERATED {

transfer-syntax-error,

abstract-syntax-error-reject,

abstract-syntax-error-ignore-and-notify,

message-not-compatible-with-receiver-state,

semantic-error,

unspecified,

abstract-syntax-error-falsely-constructed-message,

...

}

CauseRadioNetwork ::= ENUMERATED {

unspecified,

requested-item-not-supported,

requested-item-temporarily-not-available,

...

}

Cell-Portion-ID ::= INTEGER (0..255,... , 256..4095)

CPLength ::= ENUMERATED {

normal,

extended,

...

}

CriticalityDiagnostics ::= SEQUENCE {

procedureCode					ProcedureCode					OPTIONAL,

triggeringMessage				TriggeringMessage				OPTIONAL,

procedureCriticality			Criticality						OPTIONAL,

lppatransactionID				LPPATransactionID				OPTIONAL,

iEsCriticalityDiagnostics		CriticalityDiagnostics-IE-List	OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {CriticalityDiagnostics-ExtIEs} }	OPTIONAL,

...

}

CriticalityDiagnostics-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

CriticalityDiagnostics-IE-List ::= SEQUENCE (SIZE (1..maxNrOfErrors)) OF

SEQUENCE {

iECriticality			Criticality,

iE-ID					ProtocolIE-ID,

typeOfError				TypeOfError,

iE-Extensions			ProtocolExtensionContainer { {CriticalityDiagnostics-IE-List-ExtIEs} } OPTIONAL,

...

}

CriticalityDiagnostics-IE-List-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

-- D

DL-Bandwidth ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

-- E

E-CID-MeasurementResult ::= SEQUENCE {

servingCell-ID					ECGI,

servingCellTAC					TAC,

e-UTRANAccessPointPosition		E-UTRANAccessPointPosition	OPTIONAL,

measuredResults					MeasuredResults				OPTIONAL,

...

}

ECGI ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

eUTRANcellIdentifier		EUTRANCellIdentifier,

iE-Extensions				ProtocolExtensionContainer { {ECGI-ExtIEs} } OPTIONAL,

...

}

ECGI-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

EUTRANCellIdentifier ::= BIT STRING (SIZE (28))

EARFCN ::= INTEGER (0..65535, ..., 65536..262143)

E-UTRANAccessPointPosition ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

latitude					INTEGER (0..8388607),

longitude					INTEGER (-8388608..8388607),

directionOfAltitude			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),

uncertaintySemi-major		INTEGER (0..127),

uncertaintySemi-minor		INTEGER (0..127),

orientationOfMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100),

...

}

-- F

-- G

-- H

HESSID ::= OCTET STRING (SIZE(6))

-- I

InterRATMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {InterRATMeasurementQuantities-ItemIEs} }

InterRATMeasurementQuantities-ItemIEs LPPA-PROTOCOL-IES ::= {

{ ID id-InterRATMeasurementQuantities-Item	CRITICALITY reject	TYPE InterRATMeasurementQuantities-Item PRESENCE mandatory}}

InterRATMeasurementQuantities-Item ::= SEQUENCE {

interRATMeasurementQuantitiesValue			InterRATMeasurementQuantitiesValue,

iE-Extensions								ProtocolExtensionContainer { { InterRATMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

InterRATMeasurementQuantitiesValue-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

InterRATMeasurementQuantitiesValue ::= ENUMERATED {

geran,

utran,

...,

nr

}

InterRATMeasurementResult ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF InterRATMeasuredResultsValue

InterRATMeasuredResultsValue ::= CHOICE {

resultGERAN			ResultGERAN,

resultUTRAN			ResultUTRAN,

...,

resultNR			ResultNR

}

-- J

-- K

-- L

-- M

Measurement-ID ::= INTEGER (1..15, ...)

MeasurementPeriodicity ::= ENUMERATED {

ms120,

ms240,

ms480,

ms640,

ms1024,

ms2048,

ms5120,

ms10240,

min1,

min6,

min12,

min30,

min60,

...

}

MeasurementQuantities ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF ProtocolIE-Single-Container { {MeasurementQuantities-ItemIEs} }

MeasurementQuantities-ItemIEs LPPA-PROTOCOL-IES ::= {

{ ID id-MeasurementQuantities-Item	CRITICALITY reject	TYPE MeasurementQuantities-Item		PRESENCE mandatory}

}

MeasurementQuantities-Item ::= SEQUENCE {

measurementQuantitiesValue							MeasurementQuantitiesValue,

iE-Extensions										ProtocolExtensionContainer { { MeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

MeasurementQuantitiesValue-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

MeasurementQuantitiesValue ::= ENUMERATED {

cell-ID,

angleOfArrival,

timingAdvanceType1,

timingAdvanceType2,

rSRP,

rSRQ,

...

}

MeasuredResults ::= SEQUENCE (SIZE (1.. maxNoMeas)) OF MeasuredResultsValue

MeasuredResultsValue ::= CHOICE {

valueAngleOfArrival				INTEGER (0..719),

valueTimingAdvanceType1			INTEGER (0..7690),

valueTimingAdvanceType2			INTEGER (0..7690),

resultRSRP						ResultRSRP,

resultRSRQ						ResultRSRQ,

...

}

MBSFNsubframeConfiguration ::= SEQUENCE (SIZE (1.. maxMBSFN-Allocations)) OF MBSFNsubframeConfigurationValue

MBSFNsubframeConfigurationValue ::=			SEQUENCE {

radioframeAllocationPeriod			ENUMERATED {n1, n2, n4, n8, n16, n32},

radioframeAllocationOffset			INTEGER (0..7),

subframeAllocation					Subframeallocation

}

#### -- N

NarrowBandIndex ::= INTEGER (0..15,...)

NRCellIdentity ::= BIT STRING (SIZE(36))

NR-CGI ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

nRCellIdentity		NRCellIdentity,

iE-Extensions		ProtocolExtensionContainer { {NR-CGI-ExtIEs} } OPTIONAL,

...

}

NR-CGI-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

NPRSConfiguration ::=			SEQUENCE {

nPRSSubframePartA			NPRSSubframePartA 		OPTIONAL,

nPRSSubframePartB			NPRSSubframePartB 		OPTIONAL,

...

}

NPRSMutingConfiguration ::= CHOICE {

two 				BIT STRING (SIZE (2)),

four 				BIT STRING (SIZE (4)),

eight 				BIT STRING (SIZE (8)),

sixteen 			BIT STRING (SIZE (16)),

...

}

NPRSSubframePartA ::=			SEQUENCE {

bitmapsforNPRS				BitmapsforNPRS,

nPRSMutingConfiguration		NPRSMutingConfiguration 		OPTIONAL,

...

}

NPRSSubframePartB ::=			SEQUENCE {

numberofNPRSOneOccasion		ENUMERATED {sf10, sf20, sf40, sf80, sf160, sf320, sf640, sf1280, ..., sf2560},

periodicityofNPRS			ENUMERATED {sf160, sf320, sf640, sf1280, ..., sf2560},

startingsubframeoffset		ENUMERATED {zero, one-Eighth, two-Eighths, three-Eighths, four-Eighths, five-Eighths, six-Eighths, seven-Eighths, ...},

nPRSMutingConfiguration		NPRSMutingConfiguration 		OPTIONAL,

...,

sIB1-NB-Subframe-TDD			ENUMERATED {	sf0, sf4, sf0and5, ...} 		OPTIONAL

}

NumberOfAntennaPorts ::= ENUMERATED {

n1-or-n2,

n4,

...

}

NumberOfDlFrames ::= ENUMERATED {

sf1,

sf2,

sf4,

sf6,

...

}

NumberOfDlFrames-Extended ::= INTEGER (1..160,...)

NumberOfFrequencyHoppingBands ::= ENUMERATED {

twobands,

fourbands,

...

}

NPRSSequenceInfo ::= INTEGER (0..174,...)

NRARFCN ::= INTEGER (0.. 3279165)

NRPCI ::= INTEGER (0..1007)

#### -- O

OffsetNBChanneltoEARFCN ::= ENUMERATED {

minusTen,

minusNine,

minusEight,

minusSeven,

minusSix,

minusFive,

minusFour,

minusThree,

minusTwo,

minusOne,

minusZeroDotFive,

zero,

one,

two,

three,

four,

five,

six,

seven,

eight,

nine,

...

}

OperationModeInfo ::= ENUMERATED {

inband,

guardband,

standalone,

...

}

OTDOACells ::= SEQUENCE (SIZE (1.. maxCellineNB)) OF SEQUENCE {

oTDOACellInfo					OTDOACell-Information,

iE-Extensions					ProtocolExtensionContainer { {OTDOACells-ExtIEs} } OPTIONAL,

...

}

OTDOACells-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

OTDOACell-Information ::= SEQUENCE (SIZE (1..maxnoOTDOAtypes)) OF OTDOACell-Information-Item

OTDOACell-Information-Item ::= CHOICE {

pCI							PCI,

cellId						ECGI,

tAC							TAC,

eARFCN						EARFCN,

pRS-Bandwidth				PRS-Bandwidth,

pRS-ConfigurationIndex		PRS-Configuration-Index,

cPLength					CPLength,

numberOfDlFrames			NumberOfDlFrames,

numberOfAntennaPorts		NumberOfAntennaPorts,

sFNInitialisationTime		SFNInitialisationTime,

e-UTRANAccessPointPosition	E-UTRANAccessPointPosition,

...,

pRSMutingConfiguration		PRSMutingConfiguration,

prsid						PRS-ID,

tpid						TP-ID,

tpType						TP-Type,

numberOfDlFrames-Extended	NumberOfDlFrames-Extended,

crsCPlength					CPLength,

mBSFNsubframeConfiguration	MBSFNsubframeConfiguration,

nPRSConfiguration			NPRSConfiguration,

offsetNBChanneltoEARFCN		OffsetNBChanneltoEARFCN,

operationModeInfo			OperationModeInfo,

nPRS-ID						INTEGER (0..4095, ...),

dL-Bandwidth				DL-Bandwidth,

pRSOccasionGroup			PRSOccasionGroup,

pRSFreqHoppingConfig		PRSFrequencyHoppingConfiguration,

repetitionNumberofSIB1-NB	RepetitionNumberofSIB1-NB,

nPRSSequenceInfo			NPRSSequenceInfo,

nPRSType2					NPRSConfiguration,

tddConfiguration			TDDConfiguration

}

OTDOA-Information-Item ::= ENUMERATED {

pci,

cellid,

tac,

earfcn,

prsBandwidth,

prsConfigIndex,

cpLength,

noDlFrames,

noAntennaPorts,

sFNInitTime,

...,

e-UTRANAccessPointPosition,

prsmutingconfiguration,

prsid,

tpid,

tpType,

crsCPlength,

mBSFNsubframeConfiguration,

nPRSConfiguration,

offsetNBChannelNumbertoEARFCN,

operationModeInfo,

nPRS-ID,

dlBandwidth,

multipleprsConfigurationsperCell,

prsOccasionGroup,

prsFrequencyHoppingConfiguration,

repetitionNumberofSIB1-NB,

nPRSSequenceInfo,

nPRSType2,

tddConfig

}

Outcome ::= ENUMERATED {

failed,

...

}

#### -- P

PCI ::= INTEGER (0..503, ...)

PhysCellIDGERAN ::= INTEGER (0..63, ...)

PhysCellIDUTRA-FDD ::= INTEGER (0..511, ...)

PhysCellIDUTRA-TDD ::= INTEGER (0..127, ...)

PLMN-Identity ::= OCTET STRING (SIZE(3))

PosSIBs ::= SEQUENCE (SIZE (1.. maxNrOfPosSIBs)) OF SEQUENCE {

posSIB-Type						PosSIB-Type,

posSIB-Segments					PosSIB-Segments,

assistanceInformationMetaData	AssistanceInformationMetaData		OPTIONAL,

broadcastPriority				INTEGER (1..16,...)				OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { PosSIBs-ExtIEs} }	OPTIONAL,

...

}

PosSIBs-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSIB-Segments ::= SEQUENCE (SIZE (1.. maxNrOfSegments)) OF SEQUENCE {

assistanceDataSIBelement			OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { { PosSIB-Segments-ExtIEs} }	OPTIONAL,

...

}

PosSIB-Segments-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

PosSIB-Type ::= ENUMERATED {

posSibType1-1,

posSibType1-2,

posSibType1-3,

posSibType1-4,

posSibType1-5,

posSibType1-6,

posSibType1-7,

posSibType2-1,

posSibType2-2,

posSibType2-3,

posSibType2-4,

posSibType2-5,

posSibType2-6,

posSibType2-7,

posSibType2-8,

posSibType2-9,

posSibType2-10,

posSibType2-11,

posSibType2-12,

posSibType2-13,

posSibType2-14,

posSibType2-15,

posSibType2-16,

posSibType2-17,

posSibType2-18,

posSibType2-19,

posSibType3-1,

...,

posSibType4-1,

posSibType5-1,

posSibType2-24,

posSibType2-25

}

PRS-Bandwidth ::= ENUMERATED {

bw6,

bw15,

bw25,

bw50,

bw75,

bw100,

...

}

PRS-Configuration-Index ::= INTEGER (0..4095, ...)

PRS-ID	::= INTEGER (0..4095, ...)

PRSMutingConfiguration ::= CHOICE {

two 			BIT STRING (SIZE (2)),

four 			BIT STRING (SIZE (4)),

eight 			BIT STRING (SIZE (8)),

sixteen 		BIT STRING (SIZE (16)),

...,

thirty-two						BIT STRING (SIZE (32)),

sixty-four						BIT STRING (SIZE (64)),

one-hundred-and-twenty-eight	BIT STRING (SIZE (128)),

two-hundred-and-fifty-six		BIT STRING (SIZE (256)),

five-hundred-and-twelve			BIT STRING (SIZE (512)),

one-thousand-and-twenty-four	BIT STRING (SIZE (1024))

}

PRSOccasionGroup ::= ENUMERATED {

og2,

og4,

og8,

og16,

og32,

og64,

og128,

...

}

PRSFrequencyHoppingConfiguration ::= SEQUENCE {

noOfFreqHoppingBands		NumberOfFrequencyHoppingBands,

bandPositions				SEQUENCE(SIZE (1..maxnoFreqHoppingBandsMinusOne)) OF NarrowBandIndex,

iE-Extensions				ProtocolExtensionContainer { { PRSFrequencyHoppingConfiguration-Item-IEs} } OPTIONAL,

...

}

PRSFrequencyHoppingConfiguration-Item-IEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

-- Q

-- R

RepetitionNumberofSIB1-NB ::= ENUMERATED {

r4,

r8,

r16,

...

}

ReportCharacteristics ::= ENUMERATED {

onDemand,

periodic,

...

}

RequestedSRSTransmissionCharacteristics ::= SEQUENCE {

numberOfTransmissions	INTEGER (0..500, ...),

bandwidth				INTEGER (1..100, ...),

...

}

ResultRSRP ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRP-Item

ResultRSRP-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRP		ValueRSRP,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRP-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRP-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultRSRQ ::= SEQUENCE (SIZE (1.. maxCellReport)) OF ResultRSRQ-Item

ResultRSRQ-Item ::= SEQUENCE {

pCI				PCI,

eARFCN			EARFCN,

eCGI			ECGI OPTIONAL,

valueRSRQ		ValueRSRQ,

iE-Extensions	ProtocolExtensionContainer { { ResultRSRQ-Item-ExtIEs} } OPTIONAL,

...

}

ResultRSRQ-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultGERAN ::= SEQUENCE (SIZE (1.. maxGERANMeas)) OF ResultGERAN-Item

ResultGERAN-Item ::= SEQUENCE {

bCCH				BCCH,

physCellIDGERAN		PhysCellIDGERAN,

rSSI				RSSI,

iE-Extensions		ProtocolExtensionContainer { { ResultGERAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultGERAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultUTRAN ::= SEQUENCE (SIZE (1.. maxUTRANMeas)) OF ResultUTRAN-Item

ResultUTRAN-Item ::= SEQUENCE {

uARFCN				UARFCN,

physCellIDUTRAN		CHOICE {

physCellIDUTRA-FDD		PhysCellIDUTRA-FDD,

physCellIDUTRA-TDD		PhysCellIDUTRA-TDD

},

uTRA-RSCP			UTRA-RSCP OPTIONAL,

uTRA-EcN0			UTRA-EcN0 OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultUTRAN-Item-ExtIEs} } OPTIONAL,

...

}

ResultUTRAN-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

ResultNR ::= SEQUENCE (SIZE (1.. maxNRmeas)) OF ResultNR-Item

ResultNR-Item ::= SEQUENCE {

nRARFCN				NRARFCN,

nRPCI				NRPCI,

sS-NRRSRP			SS-NRRSRP	OPTIONAL,

sS-NRRSRQ			SS-NRRSRQ	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultNR-Item-ExtIEs} } OPTIONAL,

...

}

ResultNR-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ResultsPerSSB-Index-List							CRITICALITY ignore	EXTENSION ResultsPerSSB-Index-List					PRESENCE optional}|

{ ID id-NR-CGI												CRITICALITY ignore	EXTENSION NR-CGI											PRESENCE optional},

...

}

ResultsPerSSB-Index-List ::= SEQUENCE (SIZE(1..maxResultsPerSSBIndex)) OF ResultsPerSSB-Index-Item

ResultsPerSSB-Index-Item ::=SEQUENCE {

sSB-Index					SSB-Index,

sS-NRRSRPBeamValue			SS-NRRSRP			OPTIONAL,

sS-NRRSRQBeamValue			SS-NRRSRQ 				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ResultsPerSSB-Index-Item-ExtIEs} }	OPTIONAL,

...

}

ResultsPerSSB-Index-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

-- S

SFNInitialisationTime ::= BIT STRING (SIZE (64))

SRSConfigurationForAllCells ::= SEQUENCE (SIZE (1.. maxServCell)) OF SRSConfigurationForOneCell

SRSConfigurationForOneCell ::= SEQUENCE {

pci							PCI,

ul-earfcn					EARFCN,

ul-bandwidth				ENUMERATED {n6, n15, n25, n50, n75, n100},

ul-cyclicPrefixLength		CPLength,

srs-BandwidthConfig			ENUMERATED {bw0, bw1, bw2, bw3, bw4, bw5, bw6, bw7},

srs-Bandwidth				ENUMERATED {bw0, bw1, bw2, bw3},

srs-AntennaPort				ENUMERATED {an1, an2, an4, ...},

srs-HoppingBandwidth		ENUMERATED {hbw0, hbw1, hbw2, hbw3},

srs-cyclicShift				ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7},

srs-ConfigIndex				INTEGER (0..1023),

maxUpPts					ENUMERATED {true}			OPTIONAL,	-- This IE shall be present if the UL-EARFCN IE refers to TDD operation. --

transmissionComb			INTEGER (0..1),

freqDomainPosition			INTEGER (0..23),

groupHoppingEnabled			BOOLEAN,

deltaSS						INTEGER (0..29)				OPTIONAL,

sfnInitialisationTime		SFNInitialisationTime,

...

}

Subframeallocation ::= CHOICE {

oneFrame						BIT STRING (SIZE(6)),

fourFrames						BIT STRING (SIZE(24))

}

SS-NRRSRP ::= INTEGER (0..127)

SS-NRRSRQ ::= INTEGER (0..127)

SSB-Index ::= INTEGER (0..63)

SSID ::= OCTET STRING (SIZE(1..32))

SystemInformation ::= SEQUENCE (SIZE (1.. maxNrOfPosSImessage)) OF SEQUENCE {

broadcastPeriodicity				BroadcastPeriodicity,

posSIBs								PosSIBs,

iE-Extensions						ProtocolExtensionContainer { { SystemInformation-ExtIEs} }	OPTIONAL,

...

}

SystemInformation-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

-- T

TAC ::= OCTET STRING (SIZE(2))

TDDConfiguration ::= SEQUENCE {

subframeAssignment				ENUMERATED { sa0, sa1, sa2, sa3, sa4, sa5, sa6, ... },

iE-Extensions					ProtocolExtensionContainer { { TDDConfiguration-ExtIEs} }	OPTIONAL,

...

}

TDDConfiguration-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

TP-ID ::= INTEGER (0..4095, ...)

TP-Type ::= ENUMERATED { prs-only-tp, ... }

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

#### -- U

ULConfiguration ::= SEQUENCE {

pci							PCI,

ul-earfcn					EARFCN,

timingAdvanceType1			INTEGER (0..7690) 			OPTIONAL,

timingAdvanceType2			INTEGER (0..7690)			OPTIONAL,

numberOfTransmissions		INTEGER (0..500,...),

srsConfiguration			SRSConfigurationForAllCells,

...

}

UARFCN ::= INTEGER (0..16383, ...)

UTRA-EcN0 ::= INTEGER (0..49, ...)

UTRA-RSCP ::= INTEGER (-5..91, ...)

-- V

ValueRSRP ::= INTEGER (0..97, ...)

ValueRSRQ ::= INTEGER (0..34, ...)

-- W

WLANMeasurementQuantities ::= SEQUENCE (SIZE (0.. maxNoMeas)) OF ProtocolIE-Single-Container { {WLANMeasurementQuantities-ItemIEs} }

WLANMeasurementQuantities-ItemIEs LPPA-PROTOCOL-IES ::= {

{ ID id-WLANMeasurementQuantities-Item	CRITICALITY reject	TYPE WLANMeasurementQuantities-Item PRESENCE mandatory}}

WLANMeasurementQuantities-Item ::= SEQUENCE {

wLANMeasurementQuantitiesValue			WLANMeasurementQuantitiesValue,

iE-Extensions							ProtocolExtensionContainer { { WLANMeasurementQuantitiesValue-ExtIEs} } OPTIONAL,

...

}

WLANMeasurementQuantitiesValue-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

WLANMeasurementQuantitiesValue ::= ENUMERATED {

wlan,

...

}

WLANMeasurementResult ::= SEQUENCE (SIZE (1..maxNoMeas)) OF WLANMeasurementResult-Item

WLANMeasurementResult-Item ::= SEQUENCE {

wLAN-RSSI			WLAN-RSSI,

sSID				SSID					OPTIONAL,

bSSID				BSSID					OPTIONAL,

hESSID				HESSID					OPTIONAL,

operatingClass		WLANOperatingClass		OPTIONAL,

countryCode			WLANCountryCode			OPTIONAL,

wLANChannelList		WLANChannelList			OPTIONAL,

wLANBand			WLANBand				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { WLANMeasurementResult-Item-ExtIEs } }	OPTIONAL,

...

}

WLANMeasurementResult-Item-ExtIEs	LPPA-PROTOCOL-EXTENSION ::= {

...

}

WLAN-RSSI ::= INTEGER (0..141, ...)

WLANBand ::= ENUMERATED {band2dot4, band5, ...}

WLANChannelList ::= SEQUENCE (SIZE (1..maxWLANchannels)) OF WLANChannel

WLANChannel ::= INTEGER (0..255)

WLANCountryCode ::= ENUMERATED {

unitedStates,

europe,

japan,

global,

...

}

WLANOperatingClass ::= INTEGER (0..255)

-- X

-- Y

-- Z

END

-- ASN1STOP

### 9.3.6	Common definitions

-- ASN1START

-- **************************************************************

--

-- Common definitions

--

-- **************************************************************

LPPA-CommonDataTypes {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-CommonDataTypes (3)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- Extension constants

--

-- **************************************************************

maxPrivateIEs 									INTEGER ::= 65535

maxProtocolExtensions 							INTEGER ::= 65535

maxProtocolIEs									INTEGER ::= 65535

-- **************************************************************

--

-- Common Data Types

--

-- **************************************************************

Criticality		::= ENUMERATED { reject, ignore, notify }

LPPATransactionID		::= INTEGER (0..32767)

Presence		::= ENUMERATED { optional, conditional, mandatory }

PrivateIE-ID	::= CHOICE {

local				INTEGER (0.. maxPrivateIEs),

global				OBJECT IDENTIFIER

}

ProcedureCode		::= INTEGER (0..255)

ProtocolIE-ID		::= INTEGER (0..maxProtocolIEs)

TriggeringMessage	::= ENUMERATED { initiating-message, successful-outcome, unsuccessful-outcome}

END

-- ASN1STOP

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

LPPA-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM LPPA-CommonDataTypes;

-- **************************************************************

--

-- Elementary Procedures

--

-- **************************************************************

id-errorIndication												ProcedureCode ::= 0

id-privateMessage												ProcedureCode ::= 1

id-e-CIDMeasurementInitiation									ProcedureCode ::= 2

id-e-CIDMeasurementFailureIndication							ProcedureCode ::= 3

id-e-CIDMeasurementReport										ProcedureCode ::= 4

id-e-CIDMeasurementTermination									ProcedureCode ::= 5

id-oTDOAInformationExchange										ProcedureCode ::= 6

id-uTDOAInformationExchange										ProcedureCode ::= 7

id-uTDOAInformationUpdate										ProcedureCode ::= 8

id-assistanceInformationControl									ProcedureCode ::= 9

id-assistanceInformationFeedback								ProcedureCode ::= 10

-- **************************************************************

--

-- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellineNB								INTEGER ::= 256

maxNoMeas									INTEGER ::= 63

maxCellReport								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxGERANMeas								INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxCellineNB-ext							INTEGER ::= 3840

maxMBSFN-Allocations						INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNRmeas									INTEGER ::= 32

maxResultsPerSSBIndex						INTEGER	::=	64

-- **************************************************************

--

-- IEs

--

-- **************************************************************

id-Cause																	ProtocolIE-ID ::= 0

id-CriticalityDiagnostics													ProtocolIE-ID ::= 1

id-E-SMLC-UE-Measurement-ID													ProtocolIE-ID ::= 2

id-ReportCharacteristics													ProtocolIE-ID ::= 3

id-MeasurementPeriodicity													ProtocolIE-ID ::= 4

id-MeasurementQuantities													ProtocolIE-ID ::= 5

id-eNB-UE-Measurement-ID													ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult													ProtocolIE-ID ::= 7

id-OTDOACells																ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group												ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item												ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item												ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics									ProtocolIE-ID ::= 12

id-ULConfiguration															ProtocolIE-ID ::= 13

id-Cell-Portion-ID															ProtocolIE-ID ::= 14

id-InterRATMeasurementQuantities											ProtocolIE-ID ::= 15

id-InterRATMeasurementQuantities-Item										ProtocolIE-ID ::= 16

id-InterRATMeasurementResult												ProtocolIE-ID ::= 17

id-AddOTDOACells															ProtocolIE-ID ::= 18

id-WLANMeasurementQuantities												ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item											ProtocolIE-ID ::= 20

id-WLANMeasurementResult													ProtocolIE-ID ::= 21

id-Assistance-Information													ProtocolIE-ID ::= 22

id-Broadcast																ProtocolIE-ID ::= 23

id-AssistanceInformationFailureList											ProtocolIE-ID ::= 24

id-ResultsPerSSB-Index-List													ProtocolIE-ID ::= 25

id-ResultsPerSSB-Index-Item													ProtocolIE-ID ::= 26

id-NR-CGI																	ProtocolIE-ID ::= 27

END

-- ASN1STOP

### 9.3.8	Container definitions

-- ASN1START

-- **************************************************************

--

-- Container definitions

--

-- **************************************************************

LPPA-Containers {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-Containers (5)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

maxPrivateIEs,

maxProtocolExtensions,

maxProtocolIEs,

Criticality,

Presence,

PrivateIE-ID,

ProtocolIE-ID

FROM LPPA-CommonDataTypes;

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

LPPA-PROTOCOL-IES ::= CLASS {

&id				ProtocolIE-ID			UNIQUE,

&criticality	Criticality,

&Value,

&presence		Presence

}

WITH SYNTAX {

ID				&id

CRITICALITY		&criticality

TYPE			&Value

PRESENCE		&presence

}

-- **************************************************************

--

-- Class Definition for Protocol IEs

--

-- **************************************************************

LPPA-PROTOCOL-IES-PAIR ::= CLASS {

&id						ProtocolIE-ID		UNIQUE,

&firstCriticality		Criticality,

&FirstValue,

&secondCriticality		Criticality,

&SecondValue,

&presence				Presence

}

WITH SYNTAX {

ID						&id

FIRST CRITICALITY 		&firstCriticality

FIRST TYPE				&FirstValue

SECOND CRITICALITY 		&secondCriticality

SECOND TYPE				&SecondValue

PRESENCE				&presence

}

-- **************************************************************

--

-- Class Definition for Protocol Extensions

--

-- **************************************************************

LPPA-PROTOCOL-EXTENSION ::= CLASS {

&id					ProtocolIE-ID		UNIQUE,

&criticality		Criticality,

&Extension,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

EXTENSION			&Extension

PRESENCE			&presence

}

-- **************************************************************

--

-- Class Definition for Private IEs

--

-- **************************************************************

LPPA-PRIVATE-IES ::= CLASS {

&id					PrivateIE-ID,

&criticality		Criticality,

&Value,

&presence			Presence

}

WITH SYNTAX {

ID					&id

CRITICALITY			&criticality

TYPE				&Value

PRESENCE			&presence

}

-- **************************************************************

--

-- Container for Protocol IEs

--

-- **************************************************************

ProtocolIE-Container { LPPA-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Single-Container { LPPA-PROTOCOL-IES : IEsSetParam} ::=

ProtocolIE-Field {{IEsSetParam}}

ProtocolIE-Field { LPPA-PROTOCOL-IES : IEsSetParam} ::= SEQUENCE {

id				LPPA-PROTOCOL-IES.&id					({IEsSetParam}),

criticality		LPPA-PROTOCOL-IES.&criticality			({IEsSetParam}{@id}),

value			LPPA-PROTOCOL-IES.&Value				({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container for Protocol IE Pairs

--

-- **************************************************************

ProtocolIE-ContainerPair { LPPA-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (0..maxProtocolIEs)) OF

ProtocolIE-FieldPair {{IEsSetParam}}

ProtocolIE-FieldPair { LPPA-PROTOCOL-IES-PAIR : IEsSetParam} ::= SEQUENCE {

id					LPPA-PROTOCOL-IES-PAIR.&id					({IEsSetParam}),

firstCriticality	LPPA-PROTOCOL-IES-PAIR.&firstCriticality	({IEsSetParam}{@id}),

firstValue			LPPA-PROTOCOL-IES-PAIR.&FirstValue			({IEsSetParam}{@id}),

secondCriticality	LPPA-PROTOCOL-IES-PAIR.&secondCriticality	({IEsSetParam}{@id}),

secondValue			LPPA-PROTOCOL-IES-PAIR.&SecondValue			({IEsSetParam}{@id})

}

-- **************************************************************

--

-- Container Lists for Protocol IE Containers

--

-- **************************************************************

ProtocolIE-ContainerList {INTEGER : lowerBound, INTEGER : upperBound, LPPA-PROTOCOL-IES : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-Container {{IEsSetParam}}

ProtocolIE-ContainerPairList {INTEGER : lowerBound, INTEGER : upperBound, LPPA-PROTOCOL-IES-PAIR : IEsSetParam} ::=

SEQUENCE (SIZE (lowerBound..upperBound)) OF

ProtocolIE-ContainerPair {{IEsSetParam}}

-- **************************************************************

--

-- Container for Protocol Extensions

--

-- **************************************************************

ProtocolExtensionContainer { LPPA-PROTOCOL-EXTENSION : ExtensionSetParam} ::=

SEQUENCE (SIZE (1..maxProtocolExtensions)) OF

ProtocolExtensionField {{ExtensionSetParam}}

ProtocolExtensionField { LPPA-PROTOCOL-EXTENSION : ExtensionSetParam} ::= SEQUENCE {

id					LPPA-PROTOCOL-EXTENSION.&id				({ExtensionSetParam}),

criticality			LPPA-PROTOCOL-EXTENSION.&criticality	({ExtensionSetParam}{@id}),

extensionValue		LPPA-PROTOCOL-EXTENSION.&Extension		({ExtensionSetParam}{@id})

}

-- **************************************************************

--

-- Container for Private IEs

--

-- **************************************************************

PrivateIE-Container { LPPA-PRIVATE-IES : IEsSetParam} ::=

SEQUENCE (SIZE (1..maxPrivateIEs)) OF

PrivateIE-Field {{IEsSetParam}}

PrivateIE-Field { LPPA-PRIVATE-IES : IEsSetParam} ::= SEQUENCE {

id				LPPA-PRIVATE-IES.&id			({IEsSetParam}),

criticality		LPPA-PRIVATE-IES.&criticality	({IEsSetParam}{@id}),

value			LPPA-PRIVATE-IES.&Value			({IEsSetParam}{@id})

}

END

-- ASN1STOP

## 9.4	Message transfer syntax

LPPa shall use the ASN.1 Basic Packed Encoding Rules (BASIC-PER) Aligned Variant as transfer syntax, as specified in ref. ITU-T Rec. X.691 [4].

## 9.5	Timers

Void.

# 10	Handling of unknown, unforeseen and erroneous protocol data

Section 10 of TS 36.413 [3] is applicable for the purposes of the present document, with the following additions:

-	In case of Abstract Syntax Error, when reporting the Criticality Diagnostics IE for not comprehended IE/IEgroups or missing IE/IE groups, the LPPa Transaction ID IE shall also be included;

-	In case of Logical Error, when reporting the Criticality Diagnostics IE, the LPPa Transaction ID IE shall also be included.

Annex A (informative):
Change History

| TSG # | TSG Doc. | CR | Rev | Subject/Comment | New |
| --- | --- | --- | --- | --- | --- |
| 11/2009 |  |  |  | First version is created | 0.0.0 |
| 12/2009 |  |  |  | Increasing the version to 2.0.0 for approval at RAN#46 | 2.0.0 |
| 46 | RP-091213 |  |  | Approved at RAN#46 | 9.0.0 |
| 47 | RP-100225 | 0001 | 3 | Inclusion of Geographical Area and E-UTRAN Access Point Position information | 9.1.0 |
| 47 | RP-100225 | 0003 | 1 | Introduction of new cause values in LPPa | 9.1.0 |
| 47 | RP-100225 | 0004 |  | Introduction of EARFCN information in E-CID measurement results over LPPa | 9.1.0 |
| 47 | RP-100225 | 0007 |  | Rapporteur’s update of LPPa protocol | 9.1.0 |
| 48 | RP-100600 | 0010 | 1 | Clarification on E-CID MEASUREMENT INITIATION procedure | 9.2.0 |
| 48 | RP-100600 | 0011 |  | Correction of signalling of E-UTRAN Access Point Position | 9.2.0 |
| 48 | RP-100600 | 0013 | 2 | Addition of PRS Muting Configuration information to LPPa | 9.2.0 |
| 48 | RP-100600 | 0015 | 2 | Access Point reporting for OTDOA | 9.2.0 |
| 49 | RP-100906 | 0016 |  | Rapporteur’s update | 9.3.0 |
| 50 | RP-101270 | 0017 |  | Object ID for LPPa modules | 9.4.0 |
| 12/2010 |  |  |  | Created Rel-10 version based v. 9.4.0 | 10.0.0 |
| SP-49 | SP-100629 |  |  | Clarification on the use of References (TS 21.801 CR#0030) | 10.0.1 |
| 52 | RP-110689 | 0018 | 1 | Correction of Measured Result IE | 10.1.0 |
| 52 | RP-110686 | 0019 | 1 | Rapporteur’s proposal following review of TS 36.455 | 10.1.0 |
| 52 | RP-110685 | 0020 |  | Reference review outcome in TS 36.455 | 10.1.0 |
| 53 | RP-111196 | 0021 |  | Encoding of SFN Initialisation Time | 10.2.0 |
| 56 | RP-120744 | 0026 |  | Correction of SFN Initialization Time | 10.3.0 |
| 56 | RP-120744 | 0027 |  | Correction of E-UTRAN Acess Point Position | 10.3.0 |
| 57 | RP-121131 | 0030 | 2 | Correction on E-CID Measurements | 10.4.0 |
| 09/2012 |  |  |  | Update to Rel-11 version (MCC) | 11.0.0 |
| 58 | RP-121736 | 0036 |  | Correction on Uncertainty Altitude | 11.1.0 |
| 59 | RP-130237 | 0042 |  | Extending maxEARFCN | 11.2.0 |
| 60 | RP-130840 | 0045 | 3 | Network Based Positioning Support in LTE | 11.3.0 |
| 64 | RP-140905 | 0046 | 4 | Adding Cell Portion to E-CID Measurement Reporting | 12.0.0 |
| 64 | RP-140904 | 0047 | 4 | Modifications of LPPa to Include inter-RAT Measurements | 12.0.0 |
| 66 | RP-142093 | 0048 | 1 | LPPa Rapporteur Update | 12.1.0 |
| 66 | RP-142094 | 0049 | 1 | Corrections to Inter-RAT Measurements in TS 36.455 | 12.1.0 |
| 67 | RP-150356 | 0050 | 1 | ASN.1 Corrections for LPPa | 12.2.0 |
| 12/2015 |  |  |  | Update to Rel-13 version (MCC) | 13.0.0 |
| 71 | RP-160449 | 0054 |  | LPPa Rapporteur Update | 13.1.0 |
| 74 | RP-162334 | 0055 | 7 | Reusing Available WLAN Measurements to Enhance E-CID | 14.0.0 |
| 74 | RP-162334 | 0069 | 1 | Introduction of Transmission Points for OTDOA in Shared Cell-ID Scenario and PRS based Terrestrial Beacon Systems | 14.0.0 |
| 74 | RP-162334 | 0071 |  | Cell Portion ID Extension | 14.0.0 |

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 03/2017 | RP-75 | RP-170539 | 0072 |  | B | Introduction of OTDOA enhancements for NB-IOT | 14.1.0 |
| 03/2017 | RP-75 | RP-170691 | 0073 | 1 | B | OTDOA Enhancements for FeMTC | 14.1.0 |
| 06/2017 | RP-76 | RP-171323 | 0073 | 1 | F | Correction on NB-IoT OTDOA | 14.2.0 |
| 06/2017 | RP-76 | RP-171324 | 0075 | 1 | F | Rapporteur’s Review of LPPa Editorials | 14.2.0 |
| 09/2017 | RP-77 | RP-171974 | 0077 | - | F | Correction on MTC positioning | 14.3.0 |
| 12/2017 | RP-78 | RP-172673 | 0086 | 1 | F | Corrections on OTDOA information transmission in NB-IoT | 14.4.0 |
| 06/2018 | RP-80 | RP-181341 | 0082 | 5 | B | Assistance Information Broadcasting | 15.0.0 |
| 06/2018 | RP-80 | RP-181241 | 0090 | - | D | LPPa Rapporteur Update | 15.0.0 |
| 06/2018 | RP-80 | RP-181313 | 0093 | 2 | B | Support of OTDOA in NB-IoT enhancement | 15.0.0 |
| 09/2018 | RP-81 | RP-181926 | 0095 | 1 | A | Introduction of NPRS enhancement | 15.1.0 |
| 12/2018 | RP-82 | RP-182451 | 0100 | 1 | F | NB-IoT TDD correction | 15.2.0 |
| 12/2018 | RP-82 | RP-182449 | 0102 | 3 | F | Addition of TDD UL/DL configuration to OTDOA assistance data | 15.2.0 |
| 01/2019 | RP-82 |  |  |  |  | Editorial Corrections: adding "-- ASN1START" and "-- ASN1STOP" TAGs to the ASN.1 | 15.2.1 |
| 2020-03 | RP-87-e | RP-200425 | 0104 | 3 | C | Addition of broadcast of barometric pressure and TBS assistance data | 16.0.0 |
| 2020-03 | RP-87-e | RP-200420 | 0106 | 5 | B | CR of TS 36.455 for introducing NavIC in LTE – core part | 16.0.0 |
| 2020-03 | RP-87-e | RP-200425 | 0107 | 3 | B | Inter-RAT Measurement of NR Cells for E-CID | 16.0.0 |
| 2020-09 | RP-89-e | RP-201952 | 0111 | 1 | F | Introducing NR beam level measurement in inter-RAT measurement in E-CID measurement over LPPa. | 16.1.0 |
| 2020-09 | RP-89-e | RP-201952 | 0113 | - | F | Addition of NR CGI in Inter-RAT measurement result of E-CID measurement over LPPa | 16.1.0 |
| 2022-03 | SA#95-e |  |  |  |  | Promotion to Release 17 without technical change | 17.0.0 |
| 2023-03 | RAN#99 | RP-230592 | 0118 | 2 | F | LPPA corrections of references to RRC | 17.1.0 |
| 2024-03 | RAN#103 | RP-240617 | 0119 | 1 | D | LPPa Rapporteur Update | 18.0.0 |
| 2024-06 | RAN#104 | RP-241113 | 0122 | 1 | D | Correcting ASN.1 comments for conditional present IE | 18.1.0 |
| 2025-09 | - | - | - | - | - | Update to Rel-19 version (MCC) | 19.0.0 |