| 3GPP TS 37.470 V19.0.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; W1 interface; General aspects and principles (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	General aspects	7

4.0	General	7

4.1	W1 interface general principles	7

4.2	W1 interface specification objectives	8

4.3	W1 interface capabilities	8

4.4	W1 interface characteristics	8

5	Functions of the W1 interface	8

5.1	General	8

5.2	W1-C functions	8

5.2.1	Interface management function	8

5.2.2	System Information management function	9

5.2.3	Paging function	9

5.2.4	UE context management function	9

5.2.5	RRC message transfer function	10

5.2.6	Warning messages information transfer function	10

5.3	W1-U functions	10

5.3.1	Transfer of user data	10

5.3.2	Flow control function	10

6	Procedures of the W1 interface	10

6.1	Control plane procedures	10

6.1.1	Interface Management procedures	10

6.1.2	Context Management procedures	10

6.1.3	RRC Message Transfer procedures	11

6.1.3A	Void	11

6.1.4	Warning Message Transmission procedures	11

6.1.5	Paging procedures	11

6.2	User plane procedures	11

6.2.1	User Data Transfer	11

6.2.2	Flow Control	11

7	W1 interface protocol structure	12

7.1	W1 Control Plane Protocol (W1-C)	12

7.2	W1 User Plane Protocol (W1-U)	12

8	Other W1 interface specifications	12

8.0	General	12

8.1	E-UTRAN and NG-RAN; W1 interface: layer 1 (3GPP TS 37.471)	13

8.2	E-UTRAN and NG-RAN; W1 signalling transport (3GPP TS 37.472)	13

8.3	E-UTRAN and NG-RAN; W1 application protocol (W1AP) (3GPP TS 37.473)	13

8.4	NG-RAN; NR user plane protocol (3GPP TS 38.425)	13

Annex A (informative):	Change history	14

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is an introduction to the 3GPP TS 37.4xx series of technical specifications that define the W1 interface. The W1 interface provides means for interconnecting an ng-eNB-CU and an ng-eNB-DU of an ng-eNB within an NG-RAN.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.425: "NR user plane protocol".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2"

[4]	3GPP TS 37.471: "E-UTRAN and NG-RAN; W1 interface: layer 1".

[5]	3GPP TS 37.472: "E-UTRAN and NG-RAN; W1 signalling transport".

[6]	3GPP TS 37.473: "E-UTRAN and NG-RAN; W1 application protocol (W1AP)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

W1: interface between an ng-eNB-CU and an ng- eNB-DU, providing an interconnection point between the ng-eNB-CU and the ng-eNB-DU.

W1-C: Reference point for the control plane protocol between ng-eNB-CU and ng-eNB-DU.

ng-eNB-CU: a logical node hosting RRC, SDAP and PDCP protocols of the ng-eNB that controls the operation of one or more ng-eNB-DUs. The ng-eNB-CU terminates the W1 interface connected with the ng-eNB-DU.

ng-eNB-DU: a logical node hosting RLC, MAC and PHY layers of the ng-eNB, and its operation is partly controlled by ng-eNB-CU. One ng-eNB-DU supports one or multiple cells. One cell is supported by only one ng-eNB-DU. The ng-eNB-DU terminates the W1 interface connected with the ng-eNB-CU.

ng-eNB: as defined in TS 38.300 [3].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

DRB	Data Radio Bearers

W1-U	W1 User plane interface

W1-C	W1 Control plane interface

W1AP	W1 Application Protocol

GTP-U	GPRS Tunnelling Protocol

IP	Internet Protocol

NR-MIB	NR-Master Information Block

O&M	Operation and Maintenance

PA	Paging Area

PF	Paging Frame

PO	Paging Occasion

QoS	Quality of Service

RRC	Radio Resource Control

SCTP	Stream Control Transmission Protocol

SRB	Signalling Radio Bearers

SIB1	System Information Block 1

TNL	Transport Network Layer

# 4	General aspects

## 4.0	General

This clause captures the W1 interface principles and characteristics.

## 4.1	W1 interface general principles

The general principles for the specification of the W1 interface are as follows:

-	An ng-eNB may consist of an ng-eNB-CU and ng-eNB-DUs. An ng-eNB-CU and an ng-eNB-DU is connected via W1 logical interface.

-	One ng-eNB-CU controls one or more ng-eNB-DUs.

-	One ng-eNB-DU supports one or multiple cells. One cell is supported by only one ng-eNB-DU.

-	ng-eNB-DU ID is used to identify ng-eNB-DU only over W1AP procedures, ng-eNB-DU ID is not connected to cell identifier.

-	The ng-eNB-CU terminates W1 interface connected with the ng-eNB-DU.

-	The ng-eNB-DU terminates W1 interface connected with the ng-eNB-CU.

-	The W1 interface shall separate Radio Network Layer and Transport Network Layer.

-	The W1 interface shall enable exchange of UE associated information and non-UE associated information.

-	The W1 interface is open;

-	From a logical standpoint, the W1 is a point-to-point interface between an ng-eNB-CU and an ng-eNB-DU.

NOTE:	A point-to-point logical interface should be feasible even in the absence of a physical direct connection between the endpoints.

-	The W1 interface supports control plane and user plane separation;

-	The W1 interface enables exchange of UE associated information and non-UE associated information;

-	The standard should not prevent to separated CP and UP.

## 4.2	W1 interface specification objectives

The W1 interface specifications facilitate the following:

-	inter-connection of an ng-eNB-CU and an ng-eNB-DU supplied by different manufacturers.

## 4.3	W1 interface capabilities

The W1 interface supports:

-	procedures to establish, maintain and release radio bearers for the NG-RAN part of PDU sessions and for E-UTRAN Radio Access Bearers;

-	the separation of each UE on the protocol level for user specific signalling management;

-	the transfer of RRC signalling messages between the UE and the ng-eNB-CU.

## 4.4	W1 interface characteristics

# 5	Functions of the W1 interface

## 5.1	General

The following clauses describe the functions supported over W1-C and W1-U.

## 5.2	W1-C functions

### 5.2.1	Interface management function

The W1 setup function allows to exchange application level data needed for the ng-eNB-DU and ng-eNB-CU to interoperate correctly on the W1 interface. The W1 setup is initiated by the ng-eNB-DU.

The ng-eNB-CU Configuration Update and ng-eNB-DU Configuration Update functions allow to update application level configuration data needed between ng-eNB-CU and ng-eNB-DU to interoperate correctly over the W1 interface, and may activate or deactivate cells.

For NG-RAN, the W1 setup and ng-eNB-DU Configuration Update functions allow to inform the S-NSSAI(s) supported by the ng-eNB-DU.

The error indication function is used by the ng-eNB-DU or ng-eNB-CU to indicate to the ng-eNB-CU or ng-eNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the ng-eNB-DU and the ng-eNB-CU.

The W1 resource coordination function is used to transfer information about frequency resource sharing between ng-eNB-CU and ng-eNB-DU.

The ng-eNB-DU status indication function allows the ng-eNB-DU to indicate overload status to ng-eNB-CU.

### 5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the ng-eNB-DU. The ng-eNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The ng-eNB-DU is responsible for the encoding of ng-eNB-MIB. In case broadcast of SIB1 and other SI messages is needed, the ng-eNB-DU is responsible for assembling SIB1, SIB2, SIB3, SIB8 and SIB16 and the ng-eNB-CU is responsible for assembling other SIBs.

### 5.2.3	Paging function

The ng-eNB-DU is responsible for transmitting the paging information.

The ng-eNB-CU provides paging information to enable the ng-eNB-DU to calculate the exact PO and PF. The ng-eNB-CU determines the PA. The ng-eNB-DU consolidates all the paging records for a particular PO, PF and PA, and encodes the final RRC message and broadcasts the paging message on the respective PO, PF in the PA.

### 5.2.4	UE context management function

The W1 UE context management function supports the establishment and modification of the necessary overall UE context.

The establishment of the W1 UE context is initiated by the ng-eNB-CU and accepted or rejected by the ng-eNB-DU based on admission control criteria (e.g., resource not available).

The modification of the W1 UE context can be initiated by either ng-eNB-CU or ng-eNB-DU. The receiving node can accept or reject the modification. The W1 UE context management function also supports the release of the context previously established in the ng-eNB-DU. The release of the context is triggered by the ng-eNB-CU either directly or following a request received from the ng-eNB-DU. The ng-eNB-CU requests the ng-eNB-DU to remove the UE Context when the UE enters RRC_IDLE or RRC_INACTIVE.

This function can be also used to manage DRBs and SRBs, i.e., establishing, modifying and releasing DRB and SRB resources. The establishment and modification of DRB resources are triggered by the ng-eNB-CU and accepted/rejected by the ng-eNB-DU based on resource reservation information and QoS information to be provided to the ng-eNB-DU. For each DRB to be setup or modified, the S-NSSAI may be provided by ng-eNB-CU to the ng-eNB-DU in the UE Context Setup procedure and the UE Context Modification procedure.

For NG-RAN, the mapping between QoS flows and radio bearers is performed by ng-eNB-CU and the granularity of bearer related management over W1 is radio bearer level, and the ng-eNB-CU decides an aggregated DRB QoS profile for each radio bearer based on received QoS flow profile, and provides both aggregated DRB QoS profile and QoS flow profile to the ng-eNB-DU, and the ng-eNB-DU either accepts the request or rejects it with appropriate cause value.

With this function, ng-eNB-DU could also notify ng-eNB-CU whether the QoS for already established DRBs is not fulfilled any longer or it is fulfilled again.

The UE Inactivity Notification function is initiated by the ng-eNB-DU to indicate the UE activity event.

The Notify function is to enable the ng-eNB-DU to inform the ng-eNB-CU that the QoS of an already established GBR DRB cannot by fulfilled any longer or that it can be fulfilled again.

With this function, the ng-eNB-CU indicates the UL UE AMBR limit to the ng-eNB-DU, and the ng-eNB-DU enforces the indicated limit.

With this function, the ng-eNB-CU indicates the UL UE AMBR limit to the ng-eNB-DU, and the ng-eNB-DU enforces the indicated limit.

With this function, ng-eNB-CU requests the ng-eNB-DU to setup or change of the SpCell for the UE, and the ng-eNB-DU either accepts or rejects the request with appropriate cause value.

With this function, the ng-eNB-CU requests the setup of the SCell(s) at the ng-eNB-DU side, and the ng-eNB-DU accepts all, some or none of the SCell(s) and replies to the ng-eNB-CU. The ng-eNB-CU requests the removal of the SCell(s) for the UE.

With this function, the ng-eNB-DU indicates that a bearer, or a UE is inactive or active. The ng-eNB-CU consolidates all the serving ng-eNB-DUs for the UE and takes further action.

### 5.2.5	RRC message transfer function

This function allows to transfer RRC messages between ng-eNB-CU and ng-eNB-DU. RRC messages are transferred over W1-C. The ng-eNB-CU is responsible for the encoding of the dedicated RRC message with assistance information provided by ng-eNB-DU. This function also allows ng-eNB-DU to report to ng-eNB-CU if the downlink RRC message has been successfully delivered to UE or not.

### 5.2.6	Warning messages information transfer function

This function allows to cooperate with the warning message transmission procedures over NG interface. The ng-eNB-CU is responsible for encoding the warning related SI message and sending it together with other warning related information for the ng-eNB-DU to broadcast over the radio interface.

## 5.3	W1-U functions

### 5.3.1	Transfer of user data

This function allows to transfer of user data between ng-eNB-CU and ng-eNB-DU.

### 5.3.2	Flow control function

This function allows to control the downlink user data flow to the ng-eNB-DU. The detailed protocol is specified in TS 38.425 [2]

# 6	Procedures of the W1 interface

## 6.1	Control plane procedures

### 6.1.1	Interface Management procedures

The W1 Interface management procedures are listed below:

-	Reset procedure

-	Error Indication procedure

-	W1 Setup procedure

-	ng-eNB-DU Configuration Update procedure

-	ng-eNB-CU Configuration Update procedure

-	ng-eNB-DU Resource Coordination procedure

-	ng-eNB-DU Status Indication procedure

### 6.1.2	Context Management procedures

The W1 Context management procedures are listed below:

-	UE Context Setup procedure

-	UE context Release Request (ng-eNB-DU initiated) procedure

-	UE context Release (ng-eNB-CU initiated) procedure

-	UE Context Modification (ng-eNB-CU initiated) procedure

-	UE Context Modification Required (ng-eNB-DU initiated) procedure

-	UE Inactivity Notification procedure

-	Notify procedure

### 6.1.3	RRC Message Transfer procedures

The W1 RRC message transfer procedures are listed below:

-	Initial UL RRC Message Transfer procedure

-	UL RRC Message Transfer procedure

-	DL RRC Message Transfer procedure

-	RRC Delivery Report procedure

### 6.1.3A	Void

### 6.1.4	Warning Message Transmission procedures

The W1 Warning message transmission procedures are listed below:

-	Write-Replace Warning procedure

-	PWS Cancel procedure

-	PWS Restart Indication procedure

-	PWS Failure Indication procedure

### 6.1.5	Paging procedures

The W1 Paging procedures are listed below:

-	Paging

## 6.2	User plane procedures

### 6.2.1	User Data Transfer

Void

### 6.2.2	Flow Control

Void

# 7	W1 interface protocol structure

## 7.1	W1 Control Plane Protocol (W1-C)

Figure 7.1-1 shows the protocol structure for W1-C. The TNL is based on IP transport, comprising the SCTP on top of IP. The application layer signalling protocol is referred to as W1AP (W1 Application Protocol).

Figure 7.1-1: Interface protocol structure for W1-C

## 7.2	W1 User Plane Protocol (W1-U)

Figure 7.2-1 shows the protocol structure for W1-U. The TNL is based on IP transport, comprising the UDP and GTP-U on top of IP.

Figure 7.2-1: Interface protocol structure for W1-U

# 8	Other W1 interface specifications

## 8.0	General

This clause contains the description of the other related 3GPP specifications.

## 8.1	E-UTRAN and NG-RAN; W1 interface: layer 1 (3GPP TS 37.471)

3GPP TS 37.471 [4] specifies the physical layer technologies that may be used to support the W1 interface.

## 8.2	E-UTRAN and NG-RAN; W1 signalling transport (3GPP TS 37.472)

3GPP TS 37.472 [5] specifies the signalling bearers for the W1AP for the W1-C interface.

## 8.3	E-UTRAN and NG-RAN; W1 application protocol (W1AP) (3GPP TS 37.473)

3GPP TS 37.473 [6] specifies the F1AP protocol for radio network control plane signalling over the W1 interface.

## 8.4	NG-RAN; NR user plane protocol (3GPP TS 38.425)

3GPP TS 38.425 [2] specifies the user plane protocol being used over the W1-U interface.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2018-05 | RAN3#100 | R3-183552 |  |  |  | BL TS submission for approval. |  |
| 2018-06 | RAN3#100 | R3-183602 |  |  |  | Merging the agreements in R3-183571 and R3-183552. |  |
| 2018-11 | RAN3#102 | R3-187270 |  |  |  | Add definition of functions and procedure for the W1AP. |  |
| 2019-02 | RAN3#103 | R3-191163 |  |  |  | Add definition of functions and procedure for the W1AP. |  |
| 2019-04 | RAN3#103bis | R3-191970 |  |  |  | Add procedure definition of Interface Management procedures and UE Context Management procedures. |  |
| 2019-05 | RAN3#104 | R3-193174 |  |  |  | Add Some descriptions and procedures | 0.4.0 |
| 2019-08 | RAN3#105 | R3-194676 |  |  |  | Add the description of procedure for the W1AP | 0.4.0 |
| 2019-08 | RAN3#105 | R3-194562 |  |  |  | Add the description related to SpCell and SCell management | 0.4.0 |
| 2019-10 | RAN3#105-Bis | R3-196138 |  |  |  | Added the description of References, Definitions and abbreviations ,General aspects, - QoS notification, inactivity notification | 0.4.0 |
| 2019-12 | RP-86 | RP-192951 |  |  |  | TS submission to TSG RAN plenary for approval | 1.2.0 |
| 2019-12 | RP-86 |  |  |  |  | TS approved by TSG RAN plenary | 16.0.0 |
| 2020-03 | RP-87-e | RP-200429 | 0001 |  | F | Miscellaneous corrections to 37.470 | 16.1.0 |
| 2020-07 | RP-88-e | RP-201084 | 0002 | 1 | F | Miscellaneous corrections to 37.470 | 16.2.0 |
| 2022-03 | SA#95-e |  |  |  |  | Promotion to Release 17 without technical change | 17.0.0 |
| 2024-03 | SA#103- | - | - | - | - | Update to Rel-18 version (MCC) | 18.0.0 |
| 2025-09 | - | - | - | - | - | Update to Rel-19 version (MCC) | 19.0.0 |