3GPP TSG-RAN WG3 #100							R3-181719

Busan, Korea 21 - 25 May 2018

Agenda Item:	10.2.1.1

Source:	InterDigital Communications

Title:	Registration with AMF Reallocation based on Slice Suitability

# Introduction

SA2 has agreed the concept of AMF relocation to select the appropriate slice or AMF to serve a UE. RAN3 so far has not addressed AMF reallocation except to blindly copy the NAS Redirect message when grabbing relevant procedures from 36.413. The discussion in SA2 was around two methods that involve RAN3, the first method (option A) transfers the NAS registration message from the initial AMF to the target AMF directly, the second method (option B) involves returning the NAS registration message to RAN with enough information so that the RAN can send to the correct target AMF. The figure below is the message sequence chart in TS 23.502 section 4.2.2.2.3:

At last meeting two fundamental approaches were proposed for method A:

- A new procedure is defined to notify the NG-RAN node that an AMF redirection has occurred.

- Modify from 4G when the core network can send the DOWNLINK NAS TRANSPORT/INITIAL CONTEXT SETUP REQUEST so that it can be an implicit indication that AMF redirection has occurred.

Along with that there are open issues with either method:

- Do we need to explicitly signal either the new AMF name, the old AMF name or both? If yes then those parameters would need to the added to the AMF REDIRECTION REQUEST if the new procedure is used, or if we are using existing messages it would be added in the DOWNLINK NAS TRANSPORT and INITIAL CONTEXT SETUP REQUEST

- Is there a need for the NG-RAN node to modify the RAN UE NGAP ID when the AMF is changed due to redirection? The new procedure allows for that to happen, using the existing message would need a new procedure to modify the ID.

- Just for information if NSSAI parameters are needed they would be added to the same messages as above for AMF name.

# pCR for New Procedure

# 8	NGAP Procedures

## 8.1 List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 8.1-1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| AMF Configuration Update | AMF CONFIGURATION UPDATE | AMF CONFIGURATION UPDATE ACKNOWLEDGE | AMF CONFIGURATION UPDATE FAILURE |
| RAN Configuration Update | RAN CONFIGURATION UPDATE | RAN CONFIGURATION UPDATE ACKNOWLEDGE | RAN CONFIGURATION UPDATE FAILURE |
| Handover Cancellation | HANDOVER CANCEL | HANDOVER CANCEL ACKNOWLEDGE |  |
| Handover Preparation | HANDOVER REQUIRED | HANDOVER COMMAND | HANDOVER PREPARATION FAILURE |
| Handover Resource Allocation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER FAILURE |
| Initial Context Setup | INITIAL CONTEXT SETUP REQUEST | INITIAL CONTEXT SETUP RESPONSE | INITIAL CONTEXT SETUP FAILURE |
| NG Reset | NG RESET | NG RESET ACKNOWLEDGE |  |
| NG Setup | NG SETUP REQUEST | NG SETUP RESPONSE | NG SETUP FAILURE |
| Path Switch Request | PATH SWITCH REQUEST | PATH SWITCH REQUEST ACKNOWLEDGE | PATH SWITCH REQUEST FAILURE |
| PDU Session Resource Modify | PDU SESSION RESOURCE MODIFY REQUEST | PDU SESSION RESOURCE MODIFY RESPONSE |  |
| PDU Session Resource Modify Indication | PDU SESSION RESOURCE MODIFY INDICATION | PDU SESSION RESOURCE MODIFY CONFIRM |  |
| PDU Session Resource Release | PDU SESSION RESOURCE RELEASE COMMAND | PDU SESSION RESOURCE RELEASE RESPONSE |  |
| PDU Session Resource Setup | PDU SESSION RESOURCE SETUP REQUEST | PDU SESSION RESOURCE SETUP RESPONSE |  |
| UE Context Modification | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Release | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| AMF Redirection | AMF REDIRECTION REQUEST | AMF REDIRECTION RESPONSE | AMF REDIRECTION FAILURE |

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |

### 8.6.x	AMF Redirection

#### 8.6.x.1	General

The purpose of the AMF Redirection procedure is to notify the NG-RAN node that the INITIAL UE MESSAGE message was forwarded to a new AMF to handle the UE context.

Editor’s Note:	Further details are FFS.

#### 8.6.x.2	Successful Operation

Figure 8.6.x.2-1: AMF Redirection: successful operation

The AMF initiates the procedure by sending an AMF REDIRECTION REQUEST message including the appropriate data to the NG-RAN node. The NG-RAN node responds with an AMF REDIRECTION RESPONSE message including the appropriate data.

If the New RAN UE NGAP ID IE is included in the AMF REDIRECTION RESPONSE message, the AMF shall, use it as the new RAN reference to this UE.

Upon successful completion of the procedure, the NG-RAN node establishes the UE-associated logical NG-connection.

Editor’s Note:	Further details are FFS.

#### 8.6.x.3	Unsuccessful Operation

Figure 8.7.1.3-1: AMF REDIECTION: unsuccessful operation

If the NG-RAN node cannot accept the redirection, it should respond with an AMF REDIRECTION FAILURE message and appropriate cause value.

Editor’s Note:	Further details are FFS.

#### 8.6.x.4	Abnormal Conditions

Editor’s Note:	Further details are FFS.

#### 9.2.5.x	AMF REDIRECTION REQUEST

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the AMF and is used for notifying the NG-RAN node that an AMF Redirection has occurred.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |

#### 9.2.5.y	AMF REDIRECTION RESPONSE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the NG-RAN node and is used to acknowledge an AMF Redirection.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| New RAN UE NGAP ID | O |  | RAN UE NGAP ID 9.3.3.2 |  | YES | reject |

#### 9.2.5.z	AMF REDIRECTION FAILURE

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the NG-RAN node to indicate that the redirection AMF was unsuccessful.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | ignore |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

Editor’s Note:	Further details FFS.

# pCR for Existing Procedure

### 8.6.2	Downlink NAS Transport

#### 8.6.2.1	General

Editor’s Note:	Further details are FFS.

#### 8.6.2.2	Successful Operation

Figure 8.6.2.2-1: Downlink NAS transport

If the AMF only needs to send a NAS message transparently via the NG-RAN node to the UE, the AMF shall send a DOWNLINK NAS TRANSPORT message to the NG-RAN node including the NAS message as a NAS-PDU IE. If the UE-associated logical NG-connection is not established, the AMF shall allocate a unique AMF UE NGAP ID to be used for the UE and include that in the DOWNLINK NAS TRANSPORT message; by receiving the AMF UE NGAP ID IE in the DOWNLINK NAS TRANSPORT message, the NG-RAN node establishes the UE-associated logical NG-connection.

If the RAN Paging Priority IE is included in the DOWNLINK NAS TRANSPORT message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

The NAS-PDU IE contains an AMF – UE message that is transferred without interpretation in the NG-RAN node.

If the Handover Restriction List IE is contained in the DOWNLINK NAS TRANSPORT message, the NG-RAN node shall store this information in the UE context. The NG-RAN node shall use the information in the Handover Restriction List IE if present in the DOWNLINK NAS TRANSPORT message to:

-	determine a target for subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the DOWNLINK NAS TRANSPORT message and there is no previously stored handover restriction information, the NG-RAN node shall consider that no roaming and no access restriction apply to the UE.

If the Index to RAT/Frequency Selection Priority IE is included in the DOWNLINK NAS TRANSPORT message, the NG-RAN node shall, if supported, use it as defined in TS 23.501 [9].

If the UE Aggregate Maximum Bit Rate IE is included in the DOWNLINK NAS TRANSPORT message, the NG-RAN node shall store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for all non-GBR QoS flows for the concerned UE as specified in TS 23.501 [9].

Editor’s Note:	Further details are FFS.

#### 8.6.2.3	Abnormal Conditions

Editor’s Note:	Further details are FFS.

# Conclusion

There have been stage 2/3 agreements in SA2 and CT4 on the need for a AMF-AMF transfer method, and it has been shown that at least one method to allow for AMF Redirection through direct AMF-AMF communications is doable in RAN3.

Proposal 1: Agree to answers for the following open issues:

- Do we need to explicitly signal either the new AMF name, the old AMF name or both?

- Is there a need for the NG-RAN node to modify the RAN UE NGAP ID when the AMF is changed due to redirection?

Proposal 2: Agree to use either the existing procedures or a new procedure