3GPP TSG RAN WG3 Meeting #100	Tdoc R3-182757

Busan, Korea, 21st May – 25th May 2018

Agenda Item:	10.10.2

Source:	Ericsson

Title:	(TP for SA BL CR for TS 38.473): Other SI delivery command

Document for:	TP for SA BL CR for TS 38.473

# Introduction

During previous RAN3 meetings, the encoding of the system information (SI) in the disaggregated (CU-DU) architecture was discussed. The following agreements were captured:

- MIB is encoded in DU;

- RMSI is encoded in DU;

- Other SIBs are encoded in CU.

RAN3 also agreed on the introduction of the “System Information Delivery Command” procedure to enable the on-demand broadcast of the Other SI. Based on these agreements, in this contribution we address some remaining issues related to the delivery of the SI and solve the related FFS accordingly.

# Discussion

The NR SI are divided into Minimum SI and Other SI. The Minimum SI comprise the NR-MIB and the NR-SIB1 that are periodically broadcasted. The Other SI comprise the NR-SIBs that encompass everything not broadcasted in the Minimum SI and may either be broadcasted, or provisioned in a dedicated manner, either triggered by the network or upon request from the UE. If the Other SI are requested by the UE, the network may broadcast them for a predefined time interval.

An open issue is whether the time interval for broadcasting the SI should be configured by the gNB-DU or by the gNB-CU. The gNB-DU and its cells are configured via O&M and the gNB-DU is responsible for scheduling the SI. Therefore, it seems logical that the O&M also configures a default time interval for broadcasting the Other SI in the gNB-DU (e.g., as part of the cell configuration). It is also worth noting that in case that the UE requests the Other SI using MSG1, the gNB-DU must have this information to know for how long to broadcast the Other SI. The possibility for the gNB-CU to request a different broadcast time interval for a given request, which is possible only in case that the UE requests the Other SI using MSG3, is an optimization that may be discussed at a later time. Also, we would assume that the broadcast time interval is rather static. Based on that we believe that the broadcast time interval should be configured in the gNB-DU by O&M.

Proposal 1	The O&M configures a default broadcast time interval in the gNB-DU.

Another issue has to do with the gNB-CU system information. We propose to remove the FFS and keep the reference to TS 38.331. More information will be added when RAN2 will progress further. At the moment, there isn’t anything else that can be added since it all depends on RAN2 progress.

Proposal 2	Remove the FFS in the gNB-CU system information while waiting for further input from RAN2.

Based on the above we propose in the Annex 1 changes to the BL CR for 38.473

Proposal 3	RAN3 is kindly asked to agree with the text proposal in Annex I.

# Conclusion

In this contribution, we discussed issues related to the delivery of SI and made the following proposals

Proposal 1	The O&M configures a default broadcast time interval in the gNB-DU.

Proposal 2	Remove the FFS in the gNB-CU system information while waiting for further input from RAN2.

Proposal 3	RAN3 is kindly asked to agree with the text proposal in Annex I.

# Annex 1: TP for the BL CR to TS 38.473

Start of Text Proposal 1 for TS 38.473

### 8.x2.1	System Information Delivery

#### 8.x2.1.1	General

The purpose of the System Information Delivery procedure is to command the gNB-DU to broadcast the requested Other SI. The procedure uses non-UE associated signalling.

8.x2.1.2	Successful Operation

Figure 8.x2.1.2-1: System Information Delivery procedure. Successful operation.

The gNB-CU initiates the procedure by sending a SYSTEM INFORMATION DELIVERY COMMAND message to the gNB-DU.

Start of Text Proposal 2 for TS 38.473

### 9.2.x2	System Information messages

#### 9.2.x2.1	SYSTEM INFORMATION DELIVERY COMMAND

This message is sent by the gNB-CU and is used to deliver the Other SI to enable the gNB-DU to broadcast for a defined time interval.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| NR CGI | M |  | 9.3.1.12 | NR cell identifier | - |  |
| SI type list |  | 1 |  |  | YES | reject |
| >SI type item Ies |  | 1 .. <maxnoofSITypes> |  |  | EACH | reject |
| >>Other SI Type | M |  | ENUMERATED (FFS, …) | Other SI Type |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSITypes | Maximum no. of SI Types. Value is FFS |

Start of Text Proposal 3 for TS 38.473

#### 9.3.1.x6	gNB-CU System Information

This IE contains the system information encoded by the gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SI message | M |  | OCTET STRING | SI message containing all SIBs other than SIB1, as defined in TS 38.331 [8]. |

End of Text Proposal for TS 38.473