3GPP TSG-RAN WG3#101	R3-184448

Gothenburg, Sweden, 20 - 24 August 2018

Agenda item:	31.3.4.12

Source:	Nokia, Nokia Shanghai Bell

Title:	(TP for NR BL CR for TS 38.413): Correction of dedicated PDU Session transport resources

Document for:	Discussion and Decision

# 1	Description and proposals

At the last SA2#127bis meeting, SA2 agreed the CR in [3] introducing the signalling for the network instance. The network instance had been introduced to detect a packet from a certain IP domain or to forward the packet to the peer user plane node belonging to a certain IP domain.

More generally, according to the SA2 CR, the network instance enables the user plane node to select a particular transport resource associated with a PDU session whenever it sends a user plane packet. Examples given in [3] are to implement transport network isolation, separation of IP domains between NG-RAN nodes, or selection of a specific L2 technology, VPN etc:

NOTE 1: 	a Network Instance can be defined e.g. to separate IP domains, e.g. when a UPF is connected to 5G-ANs in different IP domains, overlapping UE IP addresses assigned by multiple Data Networks, transport network isolation in the same PLMN, etc.

NOTE 2:		As an example, the UPF can use the Network Instance included in the FAR, together with other information such as Outer header creation (IP address part) and Destination interface in the FAR, to determine the interface in UPF (e.g. VPN or Layer 2 technology) for forwarding of the traffic.

In this SA2 CR, which network instance the user plane shall use is determined by the control plane node (the SMF) based on control plane information and then sent to the user plane node (the UPF). Example of control plane information used by the SMF is given in [3]:

The SMF may determine the Network Instance for N3, N9 and N6 interfaces, based on the e.g. UE location, registered PLMN ID of UE, S-NSSAI of the PDU Session, DNN, etc.

As seen above the SMF determines the network instance for N3 interface. It sends it to the UPF for UPF to use the proper transport resource, L2 resource or IP domain for N3 endpoint but it is necessary that NG-RAN node uses consistent transport resource and IP domain for the peer N3 endpoint.

Therefore, the SMF must provide the network instance it has decided to use also to the NG-RAN node.

Proposal 1: SMF to send the Network Instance IE to NG-RAN node for consistent selection of transport resource across N3 interface.

One special case is if NG-RAN node is in split architecture. In this case the CU-CP will receive the network instance but the configuration of the mapping between Network Instance and Transport resource is to be in the CU UP (similar mapping between Network Instance and Transport resource in the UPF).

Proposal 2: propagate the Network Instance IE over E1AP to the NG-RAN user plane node CU UP.

The TS 38.413 is presented below. And TP for TS 38.463 is presented in [4].

# 2	References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- S2-186133, Use of Network Instance

- R3-184449, Correction of dedicated PDU Session Transport Resource

# Text Proposal for TS 38.413

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: PDU session resource setup: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node.

-	The PDU SESSION RESOURCE SETUP REQUEST message shall contain the information required by the NG-RAN node to setup PDU session related NG-RAN configuration consisting of at least one PDU session resource and for each PDU session resource to setup include a PDU Session Resource Setup Request Item IEs IE.

Upon reception of the PDU SESSION RESOURCE SETUP REQUEST message, and if resources are available for the requested configuration, the NG-RAN node shall execute the requested NG-RAN configuration and allocate associated resources over NG and over Uu for each PDU session listed in the PDU Session Resource Setup Request Item IEs IE.

If the RAN Paging Priority IE is included in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

For each requested PDU session, if resources are available for the requested configuration, the NG-RAN node shall establish at least one or several Data Radio Bearers and associate each accepted QoS flow of the PDU session to a Data Radio Bearer established.

For each PDU session successfully established the NG-RAN node shall pass to the UE the PDU Session NAS-PDU IE, if available, and the value contained in the PDU Session ID IE received for the PDU session. The NG-RAN node shall not send to the UE the PDU Session NAS PDUs associated to the failed PDU sessions.

For each PDU session the NG-RAN node shall store the UP transport layer information included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and use it as the uplink termination point for the user plane data for this PDU session.

For each PDU session, if the Additional Transport Layer Information IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node may forward the UP transport layer information to the S-NG-RAN node as the uplink termination point for the user plane data for this PDU session split in different tunnel.

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, use it when selecting transport network resource as specified in [9].

For each PDU session for which the PDU Session Type IE is included in the PDU SESSION RESOURCE SETUP REQUEST message and is set to "ethernet" or "unstructured", the NG-RAN node shall not perform header compression for the concerned PDU session.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the NG-RAN node shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU session. If the NG-RAN node cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU session resources with an appropriate cause value.

If the NG-RAN node is an ng-eNB, it shall reject all PDU sessions for which the Integrity Protection Indication IE is set to "required".

For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the NG-RAN node should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU SESSION RESOURCE SETUP RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource Setup Request Transfer IE of the PDU SESSION RESOURCE SETUP REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "not needed", then the NG-RAN node shall not perform user plane integrity protection nor perform ciphering for the concerned PDU session.

For each PDU session in the PDU SESSION RESOURCE SETUP REQUEST message the NG-RAN node shall enforce the traffic corresponding to the received PDU Session Resource Aggregate Maximum Bit Rate IE. The NG-RAN node shall use the received Aggregate Maximum Bit Rate for the concerned PDU session and concerned UE as specified in TS 23.501 [9].

For each PDU session in the PDU SESSION RESOURCE SETUP REQUEST message, if the Additional QoS Flow Information IE is included, the NG-RAN node may consider it for the DRB allocation process. It is up to NG-RAN node implementation to decide whether and how to use it.

For each QoS flow requested to be setup the NG-RAN node shall take into account the received QoS Flow Level QoS Parameters IE. For each QoS flow the NG-RAN node shall establish or modify the resources according to the values of the Allocation and Retention Priority IE (priority level and pre-emption indicators) and the resource situation as follows:

-	The NG-RAN node shall consider the priority level of the requested QoS flow, when deciding on the resource allocation.

-	The priority levels and the pre-emption indicators may (individually or in combination) be used to determine whether the QoS flow setup has to be performed unconditionally and immediately. If the requested QoS flow is marked as "may trigger pre-emption" and the resource situation requires so, the NG-RAN node may trigger the pre-emption procedure which may then cause the forced release of a lower priority QoS flow which is marked as "pre-emptable". Whilst the process and the extent of the pre-emption procedure are operator-dependent, the pre-emption indicators shall be treated as follows:

1.	The values of the last received Pre-emption Vulnerability IE and Priority Level IE shall prevail.

2.	If the Pre-emption Capability IE is set to "may trigger pre-emption", then this allocation request may trigger the pre-emption procedure.

3.	If the Pre-emption Capability IE is set to "shall not trigger pre-emption", then this allocation request shall not trigger the pre-emption procedure.

4.	If the Pre-emption Vulnerability IE is set to "pre-emptable", then this QoS flow shall be included in the pre-emption process.

5.	If the Pre-emption Vulnerability IE is set to "not pre-emptable", then this QoS flow shall not be included in the pre-emption process.

6.	If the Priority Level IE is set to "no priority" the given values for the Pre-emption Capability IE and Pre-emption Vulnerability IE shall not be considered. Instead the values "shall not trigger pre-emption" and "not pre-emptable" shall prevail.

-	The NG-RAN node pre-emption process shall keep the following rules:

1.	The NG-RAN node shall only pre-empt QoS flows with lower priority, in ascending order of priority.

2.	The pre-emption may be done for QoS flows belonging to the same UE or to other UEs.

NOTE:	The text above on NG-RAN node pre-emption process may need to be refined.

For each QoS flow which has been successfully established, the NG-RAN node stores the mapped E-RAB ID if included in the PDU Session Resource Setup Request Transfer IE contained in the PDU SESSION RESOURCE SETUP REQUEST message and uses it as specified in TS 38.300 [8].

The NG-RAN node shall report to the AMF in the PDU SESSION RESOURCE SETUP RESPONSE message the result for each individual PDU session resource requested to be setup. In particular, for each PDU session resource successfully setup, it shall include the PDU Session Resource Setup Response Transfer IE containing the following information:

-	The list of QoS flows which have been successfully established in the QoS Flow Setup Response List IE.

-	The list of QoS flows which have failed to be established, if any, in the QoS Flow Failed to Setup List IE.

-	The UP transport layer information to be used for the PDU session.

For each PDU session resource successfully setup at the NG-RAN, the NG-RAN node may allocate resources for an additional NG-U PDU session resource GTP-U tunnel, indicated in the Additional PDU Session Resource Setup Response IE.

The list of PDU session resources which failed to be setup, if any, shall be reported in the PDU SESSION RESOURCE SETUP RESPONSE message with an appropriate cause value within the PDU Session Resource Failed to Setup List IE.

Upon reception of the PDU SESSION RESOURCE SETUP RESPONSE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Setup Response Transfer IE to each SMF associated with the concerned PDU session. In case the splitting PDU session is not used by the NG-RAN node, the SMF should remove the Additional Transport Layer Information, if any.

When the NG-RAN node reports unsuccessful establishment of a QoS flow, the cause value should be precise enough to enable the SMF to know the reason for an unsuccessful establishment.

Upon reception of the PDU SESSION RESOURCE SETUP REQUEST message to setup a QoS flow for IMS voice, if successful IMS voice over NG-RAN is not able to be supported, the NG-RAN node shall initiate EPS fallback or RAT fallback for IMS voice procedure as specified in TS 23.501 [9] and report unsuccessful establishment of the QoS flow in the PDU Session Resource Setup Response Transfer IE with cause value "IMS voice EPS fallback or RAT fallback triggered".

Next change

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

The AMF initiates the procedure by sending a PDU SESSION RESOURCE MODIFY REQUEST message to the NG-RAN node.

-	The PDU SESSION RESOURCE MODIFY REQUEST message shall contain the information required by the NG-RAN node, which may trigger the NG-RAN configuration modification for the existing PDU sessions listed in the PDU Session Resource Modify Request Item IEs IE.

Upon reception of the PDU SESSION RESOURCE MODIFY REQUEST message, if the NG-RAN configuration is triggered to be modified and if resources are available for the modified NG-RAN configuration, the NG-RAN node shall execute the configuration modification for the requested PDU session.

If the RAN Paging Priority IE is included in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Modify Request Transfer IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, use it as specified in [9].

For each PDU session included in the PDU Session Resource Modify Request Item IEs IE:

-	For each QoS flow included in the QoS Flow Add or Modify Request List IE, based on the QoS Flow Level QoS Parameters IE, the NG-RAN node may establish, modify or release the Data Radio Bearer configuration and may change allocation of resources on NG or Uu accordingly. The NG-RAN node shall associate each QoS flow accepted to setup or modify with a Data Radio Bearer of the PDU session. The associated Data Radio Bearer for the QoS flow accepted to modify may not change.

-	For each QoS flow included in the QoS Flow to Release List IE, the NG-RAN node shall de-associate the QoS flow with the previously associated Data Radio Bearer.

-	The NG-RAN node shall pass the NAS-PDU IE received for the PDU session to the UE when modifying the PDU session configuration. The NG-RAN node does not send the NAS PDUs associated to the failed PDU sessions to the UE.

-	The NG-RAN node may change allocation of resources on NG according to the requested target configuration.

-	If the PDU Session Aggregate Maximum Bit Rate IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall use the received Aggregate Maximum Bit Rate for the concerned PDU session and concerned UE as specified in TS 23.501 [9].

-	If the UL NG-U UP TNL Information IE is included in the PDU Session Resource Modify Request Transfer IE, the NG-RAN node shall update the transport layer information for the uplink data accordingly for the concerned PDU session.

The NG-RAN node shall report to the AMF, in the PDU SESSION RESOURCE MODIFY RESPONSE message, the result for each PDU session requested to be modified listed in the PDU SESSION RESOURCE MODIFY REQUEST message:

-	For each PDU session which is successfully modified, the PDU Session Resource Modify Response Transfer IE shall be included to report:

1.	The list of QoS flows which have been successfully setup or modified, if any, shall be included in the QoS Flow Add or Modify Response List IE in case the PDU Session Resource Modify procedure is triggered by QoS flow setup or modification.

2.	The list of QoS flows which have failed to be setup or modified, if any, shall be included in the QoS Flow Failed to Add or Modify List IE in case the PDU Session Resource Modify procedure is triggered by QoS flow setup or modification.

-	For each PDU session which failed to be modified, the failure cause shall be included in the Cause IE for each PDU session within the PDU Session Resource Failed to Modify List IE.

The NG-RAN node shall, if supported, report in the PDU SESSION RESOURCE MODIFY RESPONSE message location information of the UE in the User Location Information IE.

For a PDU session or a QoS flow which failed to be modified, the NG-RAN node shall fall back to the configuration of the PDU session or the QoS flow as it was configured prior to the reception of the PDU SESSION RESOURCE MODIFY REQUEST message.

Upon reception of the PDU SESSION RESOURCE MODIFY REQUEST message to setup a QoS flow for IMS voice, if successful IMS voice over NG-RAN is not able to be supported, the NG-RAN node shall initiate EPS fallback or RAT fallback for IMS voice procedure as specified in TS 23.501 [9] and report unsuccessful establishment of the QoS flow in the PDU Session Resource Setup Response Transfer IE with cause value "IMS voice EPS fallback or RAT fallback triggered".

If the User Location Information IE is included in the PDU SESSION RESOURCE MODIFY RESPONSE message, the AMF shall handle this information as specified in TS 23.501 [9].

Next Change

#### 9.3.4.1	PDU Session Resource Setup Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.4 |  |
| UL NG-U UP TNL Information | M |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. |
| Additional UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the additional NG-U transport bearer, for delivery of UL PDUs. |
| Data Forwarding Not Possible | O |  | 9.3.1.63 |  |
| PDU Session Type | M |  | 9.3.1.52 |  |
| Security Indication | O |  | 9.3.1.27 |  |
| Network Instance | O |  | 9.3.1.x |  |
| QoS Flow Setup Request List |  | 1 |  |  |
| >QoS Flow Setup Request Item IEs |  | 1..<maxnoofQoSFlows> |  |  |
| >>QoS Flow Indicator | M |  | 9.3.1.51 |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.3.1.12 |  |
| >>E-RAB ID | O |  | 9.3.2.3 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

#### 9.3.4.3	PDU Session Resource Modify Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.4 |  |
| UL NG-U UP TNL Information | O |  | UP Transport Layer Information 9.3.2.2 | UPF endpoint of the NG-U transport bearer, for delivery of UL PDUs. |
| Network Instance | O |  | 9.3.1.x |  |
| QoS Flow Add or Modify Request List |  | 0..1 |  |  |
| >QoS Flow Add or Modify Request Item IEs |  | 1..<maxnoofQoSFlows> |  |  |
| >>QoS Flow Indicator | M |  | 9.3.1.51 |  |
| >>QoS Flow Level QoS Parameters | O |  | 9.3.1.12 | The presence of this IE may need to be refined |
| >>E-RAB ID | O |  | 9.3.2.3 |  |
| QoS Flow to Release List | O |  | QoS Flow List 9.3.1.13 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |

Next change

#### 9.3.1.x	Network Instance

This IE provides the network instance to be used by the NG-RAN node when selecting a particular transport network resource as described in [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Network Instance | M |  | INTEGER (1..256, …) |  |

Next Change

#### -- N

N3IWF-ID ::= CHOICE {

n3IWF-ID				BIT STRING (SIZE(16)),

...

}

NAS-PDU ::= OCTET STRING

NetworkInstance ::= INTEGER (1..256, ...)

NewSecurityContextInd ::= ENUMERATED {

true,

...

}

NextHopChainingCount ::= INTEGER (0..7)

NextPagingAreaScope ::= ENUMERATED {

same,

changed,

...

}

PDUSessionResourceModifyRequestTransfer ::= SEQUENCE {

pDUSessionAggregateMaximumBitRate		BitRate															OPTIONAL,

uL-NGU-UP-TNLInformation				UPTransportLayerInformation										OPTIONAL,

networkInstance							NetworkInstance												OPTIONAL,

qosFlowAddOrModifyRequestList			QosFlowAddOrModifyRequestList									OPTIONAL,

qosFlowToReleaseList					QosFlowList														OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceModifyRequestTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceModifyRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

Next change

PDUSessionResourceSetupRequestTransfer ::= SEQUENCE {

pDUSessionAggregateMaximumBitRate		BitRate,

uL-NGU-UP-TNLInformation				UPTransportLayerInformation,

additionalUL-NGU-UP-TNLInformation		UPTransportLayerInformation										OPTIONAL,

dataForwardingNotPossible				DataForwardingNotPossible										OPTIONAL,

pDUSessionType							PDUSessionType,

securityIndication						SecurityIndication												OPTIONAL,

networkInstance							NetworkInstance												OPTIONAL,

qosFlowSetupRequestList					QosFlowSetupRequestList,

iE-Extensions		ProtocolExtensionContainer { {PDUSessionResourceSetupRequestTransfer-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupRequestTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}