3GPP TSG RAN WG3 Meeting #101	R3-184524

Gothenburg/Sweden, 20-24th August 2018

Title:	NTN Overview and scenarios

Source: 	Thales

Type:	pCR

Document for:	Agreement

Agenda Item:	20.1 Scenarios

Study item:	FS_NR_NTN_solutions: Study on solutions for NR to support non-terrestrial networks (NTN)

# Abstract

This technical contribution aims at proposing a text for the clause “NTN Overview and scenarios” of the technical report related to 3GPP TR 38.821 “Study on solutions for NR to support non terrestrial networks (NTN)”.

# Proposed Text

We suggest the following text for the chapter 3. “Definitions, symbols and abbreviations” and 4. “NTN Overview and scenarios”.

* * * Start of changes * * * * (NEW TEXT)

# 3. Definitions, symbols and abbreviations

## 3.1 Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Availability: % of time during which the RAN is available for the targeted communication. Unavailable communication for shorter period than [Y] ms shall not be counted. The RAN may contain several access network components among which an NTN to achieve multi-connectivity or link aggregation.

Beam throughput: data rate provided in a beam

Bentpipe payload: payload that changes the frequency carrier of the uplink RF signal, filters and amplifies it before transmitting it on the downlink

Gateway: an earth station interfacing with the satellite via a feeder link with antenna system, RF front end, set of modems and related radio resource control functions and providing network interfaces

Geostationary Earth orbit: Circular orbit at 35,786 kilometres above the Earth's equator and following the direction of the Earth's rotation. An object in such an orbit has an orbital period equal to the Earth's rotational period and thus appears motionless, at a fixed position in the sky, to ground observers.

Low Earth Orbit: Orbit around the Earth with an altitude between 300 kilometres, and 1500 kilometres.

Medium Earth Orbit: region of space around the Earth above low Earth orbit and below geostationary Earth Orbit.

Minimum Elevation angle: minimum angle under which the satellite or UAS platform can be seen by a terminal.

Mobile Services: a radio-communication service between mobile and land stations, or between mobile stations

Mobile Satellite Services: A radio-communication service between mobile earth stations and one or more space stations, or between space stations used by this service; or between mobile earth stations by means of one or more space stations

Non Geostationary Satellites: Satellites (LEO and MEO) orbiting around the Earth with a period that varies approximately between 1.5 hour and 10 hours. It is necessary to have a constellation of several Non Geostationary satellites associated with handover mechanisms to ensure a service continuity.

Non-terrestrial networks: Networks, or segments of networks, using an airborne or space-borne vehicle to embark a transmission equipment relay node or base station.

On Board processing: digital processing carried out on uplink RF signals aboard a satellite or an aerial.

One way latency: time required to propagate through a telecommunication system from a terminal to the public data network or from the public data network to the terminal. This is especially used for voice and video conference applications.

Regenerative payload: payload that transforms and amplifies an uplink RF signal before transmitting it on the downlink. The transformation of the signal refers to digital processing that may include demodulation, decoding, re-encoding, re-modulation and/or filtering.

Round Trip Delay: time required for a network communication to travel from a terminal to the gateway or from the gateway to the terminal and back. This is especially used for web based applications.

Satellite: a space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO), Medium-Earth Orbit (MEO), or Geostationary Earth Orbit (GEO).

Satellite cell: a cell corresponding to the earth foot print of a satellite beam generated from a satellite platform

Unmanned Aircraft Systems: Systems encompassing tethered UAS (TUA), Lighter than Air UAS (LTA), Heavier than Air UAS (HTA), all operating in altitudes typically between 8 and 50 km including High Altitude Platforms (HAPs)

User Connectivity: capability to establish and maintain data / voice / video transfer between networks and Terminals

User Throughput: data rate provided to a terminal

## 3.2 Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

EIRP 	Equivalent Isotropic Radiated Power

FSS	Fixed Satellite Services

GEO	Geostationary Earth Orbiting

gNB	next Generation Node B

GW	Gateway

HAPS	High Altitude Platform Station

HEO	Highly Elliptical Orbiting

ISL	Inter-Satellite Links

LEO	Low Earth Orbiting

Mbps	Mega bit per second

MEO	Medium Earth Orbiting

MS	Mobile Services

MSS	Mobile Satellite Services

NGEO	Non Geostationary Earth Orbiting

NTN 	Non Terrestrial Network

RAN	Radio Access Network

RTD	Round Trip Delay

SNR 	Signal-to-Noise Ratio

Rx		Receiver

UAS	Unmanned Aircraft System

UE:	User Equipment

# 4. NTN Overview and scenarios

## 4.1 NTN overview

A non-terrestrial network refers to a network, or segment of networks using RF resources on board a satellite (or UAS platform).

The typical architecture of a non-terrestrial network providing access to user equipment is depicted below:

Figure 1: Non-terrestrial network typical physical architecture based on transparent payload

Figure 2: Non-terrestrial network typical physical architecture based on regenerative payload

Non-Terrestrial Network typically features the following elements:

-	One or several gateways that connect the Non-Terrestrial Network to a public data network

-	a GEO satellite is fed by one or several Gateways which are deployed across the satellite targeted coverage (e.g. regional or even continental coverage). We assume that UE in a cell are served by only one gateway

-	A Non-GEO satellite served successively by one gateway at a time. The system ensures service and feeder link continuity between the successive serving gateways with sufficient time duration to proceed with mobility anchoring and hand-over

-	A Feeder link or radio link between a Gateway and the satellite (or UAS platform)

-	A service link or radio link between the user equipment and the satellite (or UAS platform).

-	A satellite (or UAS platform) which may implement either a transparent or a regenerative (with on board processing) payload. The satellite (or UAS platform) generate beams typically generate several beams over a given service area bounded by its field of view. The foot prints of the beams are typically of elliptic shape. The field of view of a satellite (or UAS platform) depends on the on board antenna diagram and min elevation angle.

-	A transparent payload: Radio Frequency filtering, Frequency conversion and amplification. Hence the waveform signal repeated by the payload is un-changed;

-	A regenerative payload: Radio Frequency filtering, Frequency conversion and amplification as well as demodulation/decoding, switch and/or routing, coding/modulation. This is effectively equivalent to having all or part of base station functions (e.g. gNB) on board the satellite (or UAS platform).

-	Inter satellite links (ISL) optionally in case of a constellation of satellites. This will require regenerative payloads on board the satellites. ISL may operate in RF frequency or optical bands.

-	User Equipment are served by the satellite (or UAS platform) within the targeted service area.

There may be different types of satellite (or UAS platform) listed here under:

Table 1: Types of NTN platforms

| Platforms | Altitude range | Orbit | Typical beam foot print size |
| --- | --- | --- | --- |
| Low-Earth Orbit (LEO) satellite | 300 – 1500 km | Circular around the earth | 100 – 500 km |
| Medium-Earth Orbit (MEO) satellite | 7000 – 25000 km |  | 100 – 500 km |
| Geostationary Earth Orbit (GEO) satellite | 35 786 km | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | 200 – 1000 km |
| UAS platform (including HAPS) | 8 – 50 km (20 km for HAPS) |  | 5 - 200 km |
| High Elliptical Orbit (HEO) satellite | 400 – 50000 km | Elliptical around the earth | 200 – 1000 km |

Typically

- GEO satellite and UAS are used to provide continental, regional or local service.

- a constellation of LEO and MEO is used to provide services in both Northern and Southern hemispheres. In some case, the constellation can even provide global coverage including polar regions. For the later, this requires appropriate orbit inclination, sufficient beams generated and inter satellite links.

HEO satellite systems are not considered in this document.

## 4.2 NTN reference scenarios

We shall consider in this document non terrestrial networks providing access to user equipment in two reference scenarios including

- Circular orbiting and notional station keeping platforms.

- Highest RTD constraint

- Highest Doppler constraint

- A transparent and a regenerative payload

- One ISL case and one without ISL. Regenerative payload is mandatory in the case of inter satellite links.

| Scenarios | GEO based non-terrestrial access network (Scenario A) | LEO based non-terrestrial access network (Scenario B & C) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35 786 km | 600 km |
| Spectrum (service link) | <6 GHz and > 6 GHz e.g. 2 GHz and 30 GHz |  |
| Max channel bandwidth (service link) | 30 MHz for band <6 GHz 400 MHz for band > 6 GHz |  |
| Inter Satellite link | No | Scenario B: No if transparent payload Scenario C: Yes if regenerative payload |
| Payload | Transparent (including radio frequency function only) | Regenerative (including all or part of RAN functions) or transparent |
| Max beam foot print diameter at nadir | 500 km | 200 km |
| Min Elevation angle for both gateway and user equipment | 10° | 10° |
| Max distance between satellite and user equipment at min elevation angle | 40 586 km | 1932 km |
| Max Round Trip Delay (propagation delay only) | 541 ms (service and feeder links) | Scenario B: 25.76 ms (transparent payload: service and feeder links) Scenario C: 12.88 ms (regenerative payload: service link only) |
| Max delay variation within a beam (earth fixed user equipment) | 16ms | 4.4ms |
| Max differential delay within a beam | 1.6 ms | 0.65 ms |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27ppm/s |
| User equipment motion on the earth | 1000 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1000 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBic Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 4 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP defined New Radio | 3GPP or non 3GPP defined Radio interface |

In terms of mobility, one should distinguish between

- Inter-beam hand-over with satellite and gateway unchanged

- Satellite hand-over when the serving gateway is unchanged. See below

- Gateway hand-over when satellite is unchanged. This typically may occur when changing satellite beam which are managed by different gateways.

All of these hand-over may apply to both GEO and LEO satellites.

Figure 3: Satellite hand-over

Figure 4: Gateway hand-over

* * * End of changes * * * *