3GPP TSG-RAN WG3 #101	   R3-184688

Gothenburg, Sweden, 20th August – 24th August 2018

Source: Qualcomm Incorporated

Title: (TP for MR-DC with 5GC BL CR for TS 38.423): RRC Container correction for MR-DC with 5GC

Document for: Approval

Agenda Item: 10.8.4.4

# Introduction

In this contribution we discuss on what information is needed in the RRC Container carried over Xn for performing the various MR-DC with 5GC procedures (like SN addition/modification/release/change or inter-MN handover/SN change).

# Discussion

Xn messages defined in TS 38.423 to perform the MR-DC with 5GC procedures are common for both NGEN-DC and NE-DC. To support the delta configuration during handover over Xn, i.e. NGEN-DC/NE-DC Inter-MN HO or Inter-RAT/Intra-system HO, the Handover Request/Acknowledge messages exchange the RRC Context between the nodes. As per TS 38.423, the RRC context refers to the HandoverPreparationInformation message defined in the corresponding target RAT RRC specification. That is, if the HO is from MngeNB->MngeNB, gNB->ngeNB the RRC context refers to the definition in LTE RRC specification TS36.331 for eLTE. However, if the HO is from ngeNB->gNB, gNB-> gNB the RRC context refers to the definition in NR RRC specification TS38.331.

Observation 1: Single set of Xn messages are used for MR-DC with 5GC, i.e. for both NGEN-DC and NE-DC, and the source adapts the encoding format of the target RAT(either eLTE or NR) for the RRC context transferred during HO.

We think that it is efficient to follow similar logic as MR-DC handover for deciding the encoding format of the MR-DC (SN addition/modification/release/change) procedures based on the SN RAT. That is, eLTE Inter-Node RRC messages used when the SN is ng-eNB for NE-DC and NR Inter-Node RRC messages used when the SN is gNB for NGEN-DC. Following this approach, we do not need to change the NR RRC specification for eLTE specific changes and the respective specifications will have the updates for the corresponding RAT.

Proposal 1: MN adapts the SN RAT encoding format for the Transparent RRC container exchanged over Xn during the MR-DC (SN addition/modification/release/change) procedures, i.e. NR Inter-Node RRC messages used for NGEN-DC and eLTE Inter-Node RRC messages used for NE-DC.

# Conclusion

Based on above discussion, we have following observations and proposals.

Observation 1: Single set of Xn messages are used for MR-DC with 5GC, i.e. for both NGEN-DC and NE-DC, and the source adapts the encoding format of the target RAT(either eLTE or NR) for the RRC context transferred during HO.

Proposal 1: MN adapts the SN RAT encoding format for the Transparent RRC container exchanged over Xn during the MR-DC (SN addition/modification/release/change) procedures, i.e. NR Inter-Node RRC messages used for NGEN-DC and eLTE Inter-Node RRC messages used for NE-DC.

Text proposal to 38.423 is provided in appendix.

# Appendix: Text Proposal

-----------------------Start of TP ------------------------

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 | This IE may need to be refined. | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 |  | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 | This IE may need to be refined. | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>Bearer Configurations To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Info – SN terminated | M |  | 9.2.1.5 |  | – |  |
| >>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Info – MN terminated | M |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Either includes the SCG-ConfigInfo message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB, or the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10], if the S-NG-RAN node is a gNB. | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | OCTET STRING | This IE may need to be revised | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |

---------------------- Next change -------------------------------

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| PDU Session Resources Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>Bearer Configurations Admitted To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | ignore |
| >>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Response Info – SN terminated | M |  | 9.2.1.6 |  | – |  |
| >>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Response Info – MN terminated | M |  | 9.2.1.8 |  | – |  |
| PDU Session Resources Not Admitted List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | O |  | PDU Session Resources Not Admitted List 9.2.1.3 | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Either includes the SCG-Config message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB, or the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10], if the S-NG-RAN node is a gNB. | YES | reject |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

---------------------- Next change -------------------------------

#### 9.1.2.4	S-NODE RECONFIGURATION COMPLETE

This message is sent by the M-NG-RAN node to the S-NG-RAN node to indicate whether the configuration requested by the S-NG-RAN node was applied by the UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Response Information | M |  |  |  | YES | ignore |
| >CHOICE Response Type | M |  |  |  | – |  |
| >>Configuration successfully applied |  |  |  |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Either includes the RRCConnectionReconfigurationComplete message as defined in subclause 6.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB, or the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10], if the S-NG-RAN node is a gNB. | – |  |
| >>Configuration rejected by the M-NG-RAN node |  |  |  |  | – |  |
| >>>Cause | M |  | 9.2.3.2 |  | – |  |
| >>>M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Either includes the SCG-ConfigInfo message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB, or the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331, if the S-NG-RAN node is a gNB. | – |  |

---------------------- Next change -------------------------------

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation to modify S-NG-RAN node resources for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| SCG Change Indication | O |  | ENUMERATED (true, ...) | This IE may need to be refined. | YES | ignore |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 | This IE may need to be refined. | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.3.49 | This IE may need to be refined. | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.3.51 |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>>Bearer Configurations To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Info – SN terminated | M |  | 9.2.1.5 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Setup Info – MN terminated |  |  | 9.2.1.7 |  | – |  |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>Bearer Configurations To Be Modified |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Info – SN terminated |  |  | 9.2.1.9 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Info – MN terminated |  |  | 9.2.1.11 |  | – |  |
| > PDU Session Resources To Be Released List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Released List – SN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| >>PDU Session Resources to Be Released List – MN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Either includes the SCG-ConfigInfo message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB or the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10] , if the S-NG-RAN node is a gNB. | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. | YES | ignore |

---------------------- Next change -------------------------------

#### 9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session Resources Admitted List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources Admitted To Be Added List |  | 1 |  |  | – |  |
| >>PDU Session Resources Admitted To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>Bearer Configurations Admitted To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | ignore |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>> PDU Session Resource Setup Response Info – SN terminated | M |  | 9.2.1.6 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Setup Response Info – MN terminated | M |  | 9.2.1.8 |  | – |  |
| >PDU Session Resources Admitted To Be Modified List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources Admitted To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>Bearer Configurations Admitted To Be Modified |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | ignore |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Response Info – SN terminated | M |  | 9.2.1.10 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Response Info – MN terminated | M |  | 9.2.1.12 |  | – |  |
| >PDU Session Resources Admitted To Be Released List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources admitted to be released List – SN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| >>PDU Session Resources admitted to be released List – MN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| PDU Session Resources Not Admitted to be Added List | O |  |  |  | YES | ignore |
| >PDU Session Resources Not Admitted List – SN terminated | M |  | PDU Session Resources Not Admitted List 9.2.1.3 | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. | – |  |
| >PDU Session Resources Not Admitted List – MN terminated | M |  | PDU Session Resources Not Admitted List 9.2.1.3 | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. | – |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Either includes the SCG-Config message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB, or the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10] , if the S-NG-RAN node is a gNB. | YES | ignore |
| Admitted Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | ignore |
| Admitted Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs release | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

---------------------- Next change -------------------------------

#### 9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| SCG Change Indication | O |  | ENUMERATED (true, ...) | This IE may need to be refined. | YES | ignore |
| PDU Session Resources To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>NG-U DL UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>Cause | M |  | 9.2.3.2 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Either includes the SCG-Config message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB, or the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10] , if the S-NG-RAN node is a gNB. | YES | ignore |

---------------------- Next change -------------------------------

#### 9.1.2.9	S-NODE MODIFICATION CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node about the successful modification.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Either includes the SCG-ConfigInfo message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB or the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10], if the S-NG-RAN node is a gNB. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

---------------------- Next change -------------------------------

#### 9.1.2.10	S-NODE MODIFICATION REFUSE

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the S-NG-RAN node initiated S-NG-RAN node Modification has failed.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Either includes the SCG-ConfigInfo message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB or the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10], if the S-NG-RAN node is a gNB. | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

---------------------- Next change -------------------------------

#### 9.1.2.14	S-NODE RELEASE REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the release of resources.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  | – |  |
| >>>SCG Bearer |  |  |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>>UL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | Identifies the Xn transport bearer used for forwarding of UL PDUs | – |  |
| >>>>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | Identifies the Xn transport bearer. used for forwarding of DL PDUs | – |  |
| >>>Split Bearer |  |  |  |  | – |  |
| >>>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>>DL Forwarding UP TNL Information | O |  | UP Transport Layer Information 9.2.3.30 | Identifies the Xn transport bearer. used for forwarding of DL PDUs | – |  |
| UE Context Kept Indicator | O |  | ENUMERATED (true, ...) | This IE may need to be refined. | YES | ignore |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Either includes the SCG-ConfigInfo message as defined in subclause 10.2.2 of TS 36.331 [14], if the S-NG-RAN node is an ng-eNB or the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10], if the S-NG-RAN node is a gNB. | YES | ignore |

--------------------------- End of TP ----------------------------