3GPP TSG-RAN WG3#101                                                                        R3-184762

Goteborg, Sweden, 20-24 August 2018

Agenda Item:	10.8.2.1

Source:	CATT

Title:	Introduction of PDCP data recovery request/PDCP change indication

during SN modification procedure

Document for:	Discussion, Decision

1. Introduction

In this contribution, we discussed the removal of SCG change indication and the need for an indication from the MN to SN to enable request of PDCP data recovery in SN.At the same time,we also propose to change the SCG change indication into PDCP change indication from SN to MN,which is similar with EN-DC scenario.

2. Discussion

2.1 PDCP change indication from SN to MN

In EN-DC, for SN initiated SN modification procedure, PDCP change indication IE which is defined as follows is introduced instead of SCG change indication IE.

PDCP Change Indication

The PDCP Change Indication IE is used to require the MeNB to either initiate the security key update or to perform PDCP data recovery towards the UE (see TS 37.340 [15]).

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDCP Change Indication | M |  | ENUMERATED (S-KgNB update required, PDCP data recovery required, …) | The value of S-KgNB update required indicates that the security key in en-gNB needs to be updated. The value of PDCP data recovery required indicates that MeNB needs to perform PDCP data recovery. |

For option 7 and option 4, it is similar with EN-DC.SN may request the update of S-KgNB or request PDCP data recovery in MN node during SN initiated SN change procedure. So, we propose to modify the SCG Change indication IE as PDCP change indication IE in S-Node Modification Required message.

Proposal 1: In S-NODE Modification Required message, it is proposed to modify the SCG Change indication IE into PDCP change indication IE and use the same definition for PDCP change indication IE as in EN-DC.

2.2 PDCP data recovery request from MN to SN

For MN initiated SN change procedure, in EN-DC scenario, it is agreed to remove the SCG change indication IE in SgNB Modification Request message. The reason is that SN could know security key needs to be updated during SN modification procedure via the presence of S-NG-RAN node Security Key IE. While there is no scenario that PDCP data recovery is needed for all bearers whose PDCP/SDAP are located in SN node.

For option 4 and NR-NR DC, a new scenario i.e. intra-MN handover without security key change needs to be supported. In this scenario, since RLC would be re-established in MN node, PDCP data recovery needs to be performed for all SN terminated bearers. To support this scenario, a new IE PDCP data recovery request needs to be introduced.

Proposal 2: In S-NODE Modification Request message, it is proposed to remove SCG Change Indication IE and introduce a new IE PDCP data recovery request.

3. Conclusion

In this contribution, we discussed the removal of SCG change indication from MN to SN and also the need for PDCP data recovery indication from the MN to the SN for the case of synchronous reconfiguration at MN without security key change. The following observation and proposal were made:

Proposal 1: In S-NODE Modification Required message, it is proposed to modify the SCG Change indication IE into PDCP change indication IE and use the same definition for PDCP change indication IE as in EN-DC.

Proposal 2: In S-NODE Modification Request message, it is proposed to remove SCG Change Indication IE and introduce a new IE PDCP data recovery request.

4. TP for 38.423

<<<<<<<<<<<<<<<<<<<<Start of the first change >>>>>>>>>>>>>>>>>>>>

9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDCP data recovery request | O |  | 9.2.2.xx |  |  |  |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 | This IE may need to be refined. | YES | ignore |
| SCG Configuration Query | O |  | 9.2.3.27 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | 9.2.3.49 | This IE may need to be refined. | – |  |
| >S-NG-RAN node Security Key | O |  | 9.2.3.51 |  | – |  |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate 9.2.3.17 |  | – |  |
| >Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | – |  |
| >Lower Layer presence status change | O |  | 9.2.3.60 |  | – |  |
| >PDU Session Resources To Be Added List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>>Bearer Configurations To Be Added |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>PDU Session Resource Setup Info – SN terminated | M |  | 9.2.1.5 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Setup Info – MN terminated | M |  | 9.2.1.7 |  | – |  |
| >PDU Session Resources To Be Modified List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>>Bearer Configurations To Be Modified |  | 1 .. <maxnoofBearerConfigs> |  |  | EACH | reject |
| >>>>CHOICE Bearer Configuration | M |  |  |  | – |  |
| >>>>>SN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Info – SN terminated | M |  | 9.2.1.9 |  | – |  |
| >>>>>MN terminated Bearer |  |  |  |  | – |  |
| >>>>>>PDU Session Resource Modification Info – MN terminated | M |  | 9.2.1.11 |  | – |  |
| > PDU Session Resources To Be Released List |  | 0..1 |  |  | – |  |
| >>PDU Session Resources To Be Released List – SN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| >>PDU Session Resources to Be Released List – MN terminated | O |  | OCTET STRING | A value for PDU Session ID shall only be present once in PDU Session Resources Admitted List IE and in PDU Session Resources Not Admitted List IE. This IE may need to be refined. | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2. of TS 38.331 [10] | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | ignore |
| Requested Split SRBs release | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested to be released. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofBearerConfigs | Maximum no. of BearerConfigurations Value is 2. This value may need to be refined. So far, only MN- and SN-terminated bearer configurations are considered. |

<<<<<<<<<<<<<<<<<<<<Start of the second change >>>>>>>>>>>>>>>>>>>>

9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDCP Change Indication | O |  | 9.2.2.YY |  | YES | ignore |
| PDU Session Resources To Be Modified List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Modified Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>NG-U DL UP TNL Information at NG-RAN | O |  | UP Transport Layer Information 9.2.3.30 | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – |  |
| PDU Session Resources To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU Session Resources To Be Released Item |  | 1 .. <maxnoofPDUSessions> |  |  | EACH | ignore |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>Cause | M |  | 9.2.3.2 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

<<<<<<<<<<<<<<<<<<<<Start of the third change >>>>>>>>>>>>>>>>>>>>

9.2.2.xx	PDCP data recovery request

The PDCP data recovery request IE is used to request the S-NG-RAN node to perform PDCP data recovery .

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDCP data recovery request | M |  | ENUMERATED (True, …) |  |

9.2.YY	PDCP Change Indication

The PDCP Change Indication IE is used to require the MN to either initiate the security key update or to perform PDCP data recovery towards the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| PDCP Change Indication | M |  | ENUMERATED (S-NG-RAN node key update required, PDCP data recovery required, …) | S-NG-RAN node key update required indicates that the security key in SN needs to be updated. The value of PDCP data recovery required indicates that MN needs to perform PDCP data recovery. |

<<<<<<<<<<<<<<<<<<<<End of the change >>>>>>>>>>>>>>>>>>>>