3GPP TSG-RAN WG3 Meeting #101	     R3-184930

Gothenburg, Sweden, 20th - 24th August 2018

Agenda Item:	24.1.2.2

Source:	Ericsson

Title:	TP to TR 38.874 on QoS Handling for the Adaptation layer above RLC layer

Document for:	pCR to TR 38.874

# Introduction

In RAN-2 discussion about scheduler and QoS impacts on IAB-architectures [1], it was agreed that the IAB network should schedule the wireless resource to meet UE bearer’s requirement regardless of the number of hops a given UE is away from the Donor DU. Furthermore, it was decided during the RAN-3 discussion on IAB QoS handling [2] that the N2 admission control procedure for ensuring the QoS profiles can be extended to IAB network.

In this contribution, we further explain our views on the QoS/fairness handling for IAB network in the context of Adaptation layer above the RLC layer and clarify some points in TPs [1,2] that are approved and included in TR 38.874.

# QoS Support for Adaptation Above RLC Layer

As explained in [3] and shown in the Annex, a scalable, yet simple solution for providing QoS differentiation and ensuring fairness, is to share smartly the backhaul bearers among QoS classes of IAB nodes that are different hops away from the donor DU. For example, traffic belonging to a distant IAB node with high priority can have a dedicated bearer at all the links/hops. Meanwhile, low priority traffic belonging to the same IAB node can share backhaul bearer with high priority traffic for nearby (to donor DU) IAB node. Furthermore, the existing mechanism for QoS in NR, i.e. SDAP layer, maps several QoS flows with different 5QI values into a single DRB. In the context of gNB CU-DU split, the gNB CU decides flow-to-DRB mapping and performs the F1 UE Context Setup procedure to setup one or more bearers in the gNB DU. Consequently, the gNB DU holds information at the UE bearer level only, which means that QoS differentiation per QoS flow level at the backhaul links is not supported even for the one-to-one mapping between UE bearers and RLC channels in the CU-DU split case without IAB support.

Existing mechanism for QoS in NR maps several QoS flows with different 5QI values into a single DRB at the SDAP layer.

It is not possible to enforce QoS differentiation per single QoS flow level at the backhaul links irrespective of the choice of mapping scheme between the UE bearers and RLC channels.

When it comes to the implementation of a smart sharing scheme, the CU can configure the backhaul bearers for end-user traffic and configure the mapping rules in the Adaptation layer. The mapping rules for aggregating/multiplexing of bearers can also be decided by the CU. The donor DU could then enforce the mapping rules as configured by the CU to forward the packets over the right backhaul bearer (how the Adaptation layer is set up and configured is discussed in [4]). Furthermore, the CU can employ both the IP address of an IAB node and the DSCP field of the IP header to convey QoS related information to the DU.

When an IAB node attaches to the network, the CU/DU can determine and store the number of hops between the donor node and the IAB node. The CU can then maintain a mapping/table of IAB node IP addresses and corresponding hop counts. This way, the CU can map traffic for a distant IAB node to a higher priority backhaul bearer, compared to the traffic destined to a nearby IAB node. Furthermore, the CU can keep track on how many UEs and access bearers are connected to each IAB node. This information can be provided to intermediate nodes (e.g. on demand, periodically, when a new UE gets connected, when a UE changes its path, etc.) enabling them to enforce fairness between IAB nodes based on how many UEs are connected to that IAB node. Thus, it is possible to support both options for implementing QoS/fairness defined in [1] (i.e. based on the number of UE bearers carried over the backhaul links and/or the number of descendant IAB nodes supported over the backhaul links)

Adaptation layer above RLC can support both options for enforcing fairness across backhaul links (i.e. based on the number of UE bearers and/or the number of descendant IAB nodes supported over backhaul links).

# QoS Requirements for Network Services

For RAN to support the QoS profiles imposed by the core network, an N2 procedure is defined in TS 23.502, allowing RAN to reject the QoS profiles requested by the core network when RAN cannot support these profiles. To implement this procedure in an IAB network, the Donor CU has to check with all the intermediate IAB node DUs along the path to a UE (including access DU) before approving a QoS profile. In our view, this step is not always needed. It may depend on QoS requirements associated with the new profile as well as the currently configured backhaul bearers. It may be required for QoS profiles associated with guaranteed services, while best-effort profiles may be mapped to existing backhaul bearers without checking for admission control with all intermediate nodes. Exact CU behaviour could be up to implementation and configuration.

The CU does not always need to involve the intermediate IAB nodes in the N2 admission control procedure for ensuring the requirements of QoS profiles.

Furthermore, in an IAB network, data packets may have to traverse several hops, where the Adaptation layer of each hop will process and forward the packets to the next IAB node. This can incur additional latency of delivery of packets to the intended UE. For applications with strict delay budget (e.g. voice), some mechanism is needed to ensure that the packet delay budget is met across the IAB network. One possible approach is to add timestamps for the delay budget to the data packets at the Adaptation layer of the Donor DU. Adding timestamp could also be beneficial for active queue management in the sense that packets with already expired timestamps can be dropped to avoid congestion, as it is pointless to keep packets that will be discarded anyway by the receiving entity. More details on the type of value for timestamp and required length for timestamp subfield is FFS.

For services with strict delay tolerance, some mechanism is required (e.g. timestamps) for ensuring that the packet delay budget is met in an IAB network.

Adding timestamp could also be beneficial for active queue management as packets with already expired timestamps can be dropped to avoid congestion.

# Conclusion

In this contribution, we have observed that:

Observation 1	Existing mechanism for QoS in NR maps several QoS flows with different 5QI values into a single DRB at the SDAP layer.

Observation 2	It is not possible to enforce QoS differentiation per single QoS flow level at the backhaul links irrespective of the choice of mapping scheme between the UE bearers and RLC channels.

Observation 3	Adaptation layer above RLC can support both options for enforcing fairness across backhaul links (i.e. based on the number of UE bearers and/or the number of descendant IAB nodes supported over backhaul links).

Observation 4	The CU does not always need to involve the intermediate IAB nodes in the N2 admission control procedure for ensuring the requirements of QoS profiles.

Observation 5	For services with strict delay tolerance, some mechanism is required (e.g. timestamps) for ensuring that the packet delay budget is met in an IAB network.

Observation 6	Adding timestamp could also be beneficial for active queue management as packets with already expired timestamps can be dropped to avoid congestion.

Based on these observations, we have provided a TP in Section 6 to be captured in TR 38.874.

# References

[1] R2-1810972; TP for TR 38.874 Scheduler and QoS impacts #AdHoc1807, Montreal, Canada, July 2018.

[2] R3-184369; IAB QoS Handling, RAN3#AdHoc1807, Montreal, Canada, July 2018.

[3] R2-1810567; QoS Handling in the Adaptation Layer of IAB System, Ericsson, RAN2 AH 1807, Montreal, Canada, July 2018.

[4] R3-184931; TP to TR 38.874 on setup procedure for the Adaptation layer of an IAB network, Ericsson, RAN3#101, Gothenburg, Sweden, August 2018.

# Text Proposal to TR 38.874

------------------------------------------Change 1-------------------------------------------

### 8.2.4	Scheduler and QoS impacts

#### 8.2.4.1		Bearer mapping

An IAB node needs to multiplex the UE DRBs to the BH RLC-Channel. The following two options can be considered on bearer mapping in IAB node.

Figure 8.2.4.1-1 example of one-to-one mapping between UE DRB and BH RLC-Channel

Option 1. One-to-one mapping

In this option, each UE DRB is mapped onto a separate BH RLC-Channel. Further, the each BH RLC-Channel is mapped onto a separate BH RLC-Channel on the next hop. The number of established BH RLC-Channels is equal to the number of established UE DRBs.

Since the IAB node just relays a data block between UE DRBs and BH RLC-Channels, there is no need to multiplex UE DRBs, and no need to identify the data block.

Figure 8.2.4.1-2 example of aggregated mapping between UE DRB and BH RLC-Channel

Option 2. Aggregated mapping

For the aggregated mapping, the IAB node establishes BH RLC-Channels based on information such as the QoS profile of the traffic and target IAB node (or hop count). The exact information to be used for the aggregation is FFS. The IAB node can multiplex DRBs or QoS flows with similar QoS characteristics and/or hop count into a single BH RLC-Channel even in case they belong to different UEs. Further, the each BH RLC-channel may be mapped onto the different BH RLC-Channels according to QoS profiles on the next hop.

Since the BH RLC-Channel is  shared by multiple bearers, UEs or IAB nodes, each data block transmitted in the BH RLC-Channel may contain an identifier of the UE, IAB node, and DRB it belongs to (the identifiers may or may not be at the Adaptation layer header depending on the architecture option).

The mapping as well as the QoS scheduling in the intermediate nodes can be updated due to dynamic aspects such as changing radio conditions, load conditions (e.g. number of connected users), time already spent by packets on flight, etc. The intermediate nodes could get the information required via the Adaptation layer header or direct CU signalling, the details are FFS.

Table 8.2.4.1-1: Observations for bearer mapping

|  | One-to-One | Aggregated |
| --- | --- | --- |
| # of BH RLC-Channels | # of DRBs | Depends on configuration |
| RB multiplexing | No | Yes |
| In-band identification | None | IAB node ID + other IDs (UE, DRB) depending on the architecture option |
| QoS guarantee | Yes | Yes |
| QoS granularity | Per Access UE bearer | Per BH RLC-Channel |

------------------------------------------Change 2-------------------------------------------

#### 8.2.4.2	Enforcement of Fairness Schemes

An IAB network should attempt to schedule the wireless resources to meet each UE bearer’s requirement regardless of the number of hops a given UE is away from the Donor DU.

The scheduler on the wireless backhaul link can distinguish the QoS profiles associated with different RLC channels. It may also apply information regarding the number of hops a packet needs to traverse, in addition to the QoS profile of the bearers, in order to provide hop-agnostic performance.

When one-on-one mapping is used between UE bearer and RLC-channel on the backhaul, the IAB node has explicit information on each UE bearer and can therefore apply appropriate QoS differentiation among QoS profiles, as well as fairness among UE bearers with same QoS profile.

While QoS differentiation is still possible when UE bearers are aggregated to backhaul RLC-channels, enforcement of fairness across UE bearers become less granular.

Figure 8.2.4.2-1 IAB network with 3 hops and 12 UEs

Figure 8.2.4.2-1 shows an example scenario of an IAB network with 3 hops and 12 UEs attached. The UEs are assumed to have one bearer each with same QoS profile (e.g. default bearer). The UE-bearers are assumed to share the same RLC channel on BH links. Consequently, each backhaul link carries different number of UE-bearers (Table 8.2.4-y).

Below are the two options for applying fairness schemes across backhaul and access links (other options are not precluded):

- Option 1: The DU scheduler obtains information about the number of UE bearers carried on each backhaul link. This enables the scheduler to apply fairness schemes. For this, the scheduler has to be updated whenever the number of UE bearers change on one of its backhaul RLC-channels. Alternatively, the scheduler derives the number of UE bearers carried on the backhaul RLC-channel from packet inspection.

- Option 2: The DU scheduler obtains information about the number of descendant IAB-nodes supported by each backhaul link. This allows enforcing fairness schemes as long as the total traffic is balanced across IAB-nodes.

------------------------------------------Change 3-------------------------------------------

## 9.5	Satisfying the QoS requirements

IAB mode of operation may impose additional requirements on the RAN design, in order for the RAN to support the QoS profiles imposed by the Core network. These additional requirements may be due to e.g. the latency associated with multiple hops, congestion and failure of wireless backhaul links. However, in both IAB and non-IAB mode of operation, RAN may not always be able to meet the QoS profiles requested by the core network. To handle this scenario, the TS 23.502 [3] in Section 4.3.2. defines an N2 procedure which allows the RAN to reject the QoS profiles requested by the core network, in case the RAN cannot meet these QoS profiles. This N2 procedure is applicable to both IAB and non-IAB mode of operation.

With regards to the aforementioned N2 procedure, after receiving a flow QoS request from the core network, the IAB-donor CU may inform, via F1-AP, the corresponding access-IAB-node-DU and some or all intermediate IAB-node DUs about this flow and its QoS requirement. The inquired DUs shall accept/reject the request. In order to guarantee latency bounds, the CU should include in the QoS request to the DUs some assistance information including at least e.g. some hop-count-related information pertaining to the route to the access-IAB-node-DU. Further details of this information to be provided are FFS.

Since the IAB-specific constraints on QoS depend on the particular IAB architecture option, the study will capture the tradeoff among the various IAB architecture options with respect to their impact on QoS.

------------------------------------------End of changes-------------------------------------------

# Annex – Example of QoS Support for IAB Network

Figure 1 Example of QoS unaware mapping

Figure 2 Example of separate backhaul bearer for each QoS class per subsequent IAB node

Figure 3 Example of sharing backhaul bearers among different QoS classes