3GPP TSG-RAN WG3 Meeting #103	R3-190161

Athens, Greece, EU, 25 Feb – 1 Mar 2019

Agenda item:	9.3.18

Source:	Nokia, Nokia Shanghai Bell

Title:	Confusing definition of the desired buffer size for the NR UP

# 1	Introduction

Currently, the Downlink Data Delivery Status PDU contains, among others, following information:

b)	the desired buffer size in bytes for the concerned data radio bearer;

Handling of this information is described as follows (introduced in [1]):

-	regards the desired buffer size under b) […] above as the amount of data to be sent from the hosting node:

-	If the value of the desired buffer size is 0, the hosting node shall stop sending any data per bearer.

-	If the value of the desired buffer size in b) above is greater than 0, the hosting node may send up to this amount of data per bearer beyond the "Highest Delivered NR PDCP SN" for RLC AM, or the hosting node may send up to this amount of data per bearer beyond the "Highest Transmitted NR PDCP SN" for RLM UM.

The first point, concerning value ‘0’ is defined clearly: no more data is expected from the hosting node. However, the other point, may trigger confusion regarding the amount of data expected. In this paper we address this point.

# 2	Problem and solution

The corresponding node hosts normally a transmission buffer, where data received from the hosting node is kept until it is transmitted towards the UE. Only once it is transmitted or delivered, the PDU is acknowledged to the hosting node. Therefore, the highest PDU SN sent from the hosting node to the corresponding node is typically much higher than the last SN reported as transmitted delivered.

The definition cited above refers to the amount of data expected from the hosting node as counted “beyond the Highest Delivered NR PDCP SN” (or the “Highest Transmitted NR PDCP SN”). This hints that the reported desired buffer size is the total buffer and that the hosting node shall perform necessary calculations to derive the actual amount of data that may still be sent (signalled desired buffer size minus data sent towards the corresponding node, but not yet delivered/transmitted to the UE).

If this is kept, there are two issues:

- The motivation to repeat the same value in each and every DDDS message is questionable (the desired buffer size is mandatory field in the message), as it likely changes rarely.

- Handling of ‘zero’ and non-zero values is different (in fact, handling of values of desired buffer size that are lower than the amount of not yet delivered/transmitted data is problematic).

It would be much simpler to handle the value of the desired buffer size as the amount of data that the corresponding node expects to be delivered from the hosting node. This spares the hosting node from unnecessary calculations and enables handling the value in a simple way.

We therefore propose to remove the statement starting from “beyond” (from both RLC modes), so that the hosting node interprets the desired buffer size as the amount of data it is expected to send on top of what the corresponding node has already received.

# 3	Conclusions

In this paper, we discuss interpretation of the desired buffer size, when it is reported as higher than zero. We come to the conclusion that the way it is described causes unnecessary complication for the hosting node. Hence, we propose:

To remove the statement starting from “beyond” (from both RLC modes), so that the hosting node interprets the desired buffer size as the amount of data it is expected to send on top of what the corresponding node has already received.

The needed CR is proposed in [2].

# 4	References

- R3-182423, RAN3 #99-bis

- R3-190162, RAN3 #103