3GPP TSG-RAN WG3 Meeting #103bis	R3-191695

# Xian, P.R. China 8th – 12th April 2019

Title:	[Draft] Reply LS on Automatic Restoration of GFBR QOS

Reply to:	 LS on Automatic Restoration of GFBR QOS (S2-1902906)

Release:	Rel-16

Work Item:	FS_5G_URLLC

Source:	Ericsson

To:	3GPP SA WG2

Cc:	3GPP RAN WG2, 3GPP RAN WG1

Contact Person:

#### Name:	Nianshan Shi

E-mail Address:	Nianshan.shi@ericsson.com

1. Overall Description:

RAN3 has discussed the LS on Automatic Restoration of GFBR QOS, in which SA 2 invites RAN 2 and RAN 3 to provide other feedback that they consider relevant with regards to Annex B and the multiple solutions in TR 23.725 v16.1.0 for Key Issue #7.

RAN3 would like to provide the below feedback regarding the components in Annex B.

## Components A/B/C

- RAN3 agrees to enhance the Rel-15 Notification Control function to also be applied at admission control. NG-RAN shall be allowed to temporarily fail the admission control at resource setup / handover, the QoS flow is accepted to be setup in NG-RAN, treated as a GBR QoS flow, but it is in a “Notification Control” state. It is RAN internal implementation matter how to decide when a QoS flow can be put out of the “Notification state.

- RAN3 agrees that there is no need to indicate to UE that the QoS flow is setup with “reasonable admission”, or if the QoS flow is fulfilled/not fulfilled.

## Component D

- RAN3 confirm that tt is up to implementation for RAN mechanisms attempt to restore the QoS.

## Component E

- RAN3 agrees to that it is the source RAN to decide the handover target and it is already possible that the source RAN will choose a new target in the failure of handover to the first target. Target may feedback that it cannot Guarantee the GFBR flow.

2. Actions:

To SA2

ACTION: RAN3 kindly ask SA2 to take above into consideration.

3. Date of Next TSG-RAN3 Meetings:

TSG RAN3 Meeting #104		13th -  17th May 2019, 	Reno, U.S.A