3GPP TSG RAN WG3 Meeting #103bis	R3-191785

Xi’an, P.R. China, April 8th – 12th 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0085 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Making DBS Useful for Dual Connectivity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-03-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Enabling proper operation of the Desired Buffer Size concept in Dual Connectivity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the Buffer Dwell Target Time IE to the DL USER DATA PDU This is a backwards-compatible CR with isolated impact. The corresponding discussion is presented in R3-191784. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Malfunctioning of the Desired Buffer Size concept in Dual Connectivity. Arrival of a large amount of out-of-order packets to the UE, leading to UE’s reordering timer expiry and out-of-order data delivery to higher layer, leading to throughput degradations or loss of higher-layer control data. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.4.1.1, 5.5.2.1, 5.5.3.5, 5.5.3.x (new), 5.5.3.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Change 1-------------------------------------------

## 5.2	NR user plane protocol layer services

The following functions are provided by the NR user plane protocol:

-	Provision of NR user plane specific sequence number information for user data transferred from the node hosting NR PDCP to the corresponding node for a specific data radio bearer.

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for user data associated with a specific data radio bearer.

-	Information of NR PDCP PDUs that were not delivered to the UE or not transmitted to the lower layers.

-	Information of NR PDCP PDUs transmitted to the lower layers for user data associated with a specific data radio bearer.

-	Information of downlink NR PDCP PDUs to be discarded for user data associated with a specific data radio bearer;

-	Information of the currently desired buffer size at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer.

-	Provision of the buffer dwell target time from to the corresponding node, used by the corresponding node for setting the desired buffer size for transmitting to the UE user data associated with a specific data radio bearer.

-	Information of the currently desired data rate in bytes at the corresponding node for transmitting to the UE user data associated with a specific data radio bearer;

-	Information of successful in sequence delivery of NR PDCP PDUs to the UE from the corresponding node for retransmission user data associate with a specific data radio bearer;

-	Information of NR PDCP PDUs transmitted to the lower layers for retransmission user data associated with a specific data radio bearer.

-	Information of the specific events at the corresponding node.

-	Information on Radio Link Quality from the corresponding node for user data associated with a specific data radio bearer.

-------------------------------------------Change 2-------------------------------------------

#### 5.4.1.1	Successful operation

The purpose of the Transfer of Downlink User Data procedure is to provide NR-U specific sequence number information at the transfer of user data carrying a DL NR PDCP PDU from the node hosting the NR PDCP entity to the corresponding node.

An NR user plane protocol instance making use of the Transfer of Downlink User Data procedure is associated to a single data radio bearer only.

The node hosting the NR PDCP entity shall assign consecutive NR-U sequence numbers to each transferred NR-U packet. A retransmitted NR PDCP PDU shall be assigned a new NR-U sequence number.

The node hosting the NR PDCP entity indicates to the corresponding node whether this NR-U packet is a retransmission of NR PDCP PDU.

The node hosting the NR PDCP entity can indicate to the corresponding node to either discard all NR PDCP PDUs up to and including a defined DL discard NR PDCP PDU SN or discard one or a number of blocks of downlink NR PDCP PDUs.

If the Assistance Information Report Polling Flag is equal to 1, the corresponding node shall, if supported, send the ASSISTANCE INFORMATION DATA to the node hosting the NR PDCP entity.

If the Buffer Dwell Target Time is present in the DL USER DATA, the corresponding node shall, if supported, use this value for setting the desired buffer size.

>>>>>>>>>>>>>>Skipped text<<<<<<<<<<<<<<

-------------------------------------------Change 3-------------------------------------------

#### 5.5.2.1	DL USER DATA (PDU Type 0)

This frame format is defined to allow the corresponding node to detect lost NR-U packets and is associated with the transfer of a Downlink NR PDCP PDU.

The following shows the respective DL USER DATA frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  | Buffer Target Time Ind | DL Discard Blocks | DL Flush | Report polling | 1 |
| Spare |  |  |  | Report Delivered | User data existence flag | Assistance Info. Report Polling Flag | Retransmission flag | 1 |
| NR-U Sequence Number |  |  |  |  |  |  |  | 3 |
| DL discard NR PDCP PDU SN |  |  |  |  |  |  |  | 0 or 3 |
| DL discard Number of blocks |  |  |  |  |  |  |  | 0 or 1 |
| DL discard NR PDCP PDU SN start (first block) |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (first block) |  |  |  |  |  |  |  | 0 or 1 |
| … |  |  |  |  |  |  |  |  |
| DL discard NR PDCP PDU SN start (last block) |  |  |  |  |  |  |  | 0 or 3 |
| Discarded Block size (last block) |  |  |  |  |  |  |  | 0 or 1 |
| DL report NR PDCP PDU SN |  |  |  |  |  |  |  | 0 or 3 |
| Buffer dwell target time |  |  |  |  |  |  |  | 0 or 2 |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.1-1: DL USER DATA (PDU Type 0) Format

-------------------------------------------Change 4-------------------------------------------

#### 5.5.3.5	Desired buffer size for the data radio bearer

Description: This parameter indicates the desired buffer size in bytes for the concerned data radio bearer as specified in clause 5.4.2.1. The desired buffer size shall be estimated while taking into account the buffer dwell time target.

Value range: {0..232-1}.

Field length: 4 octets.

-------------------------------------------Change 5-------------------------------------------

#### 5.5.3.x	Buffer Target Time Indication

Description: This parameter indicates the presence of Buffer Dwell Target Time.

Value range: {0= Buffer Dwell Target Time not present, 1= Buffer Dwell Target Time present}.

Field length: 1 bit.

#### 5.5.3.y	Buffer Dwell Target Time

Description: This parameter defines the buffer dwell target time in milliseconds to be used in calculating the desired buffer size as defined in section 5.5.3.5.

Value range: {0..216-1}.

Field length: 2 octets.

-------------------------------------------End of changes ------------------------------------------