3GPP TSG-RAN WG3 Meeting #103bis	                               R3-191827

Xi'an, China, 8th - 12th April 2019

Agenda item:	25.2.3.4

Source:	CMCC

Title:	Load Balancing Optimization

Document for:	Discussion and decision

Introduction

One of the key use cases in the SI “Study on RAN-centric data collection and utilization for LTE and NR” was “load sharing/balancing related optimization”. In this document, we describe the load balancing use case, investigate architecture aspects for load balancing use case and introduce the main alternatives.

Discussion

Load balancing has the task to handle uneven distribution of the traffic load over multiple cells [1]. The automating of such optimisation can provide high quality user experience, simultaneous improving the system capacity and also to minimize human intervention in the network management and optimization tasks.

According to the majority opinion from the email discussion [2], LTE MLB mechanism could be used as baseline and study of MLB for NR could consider the new features of NR, like slicing, SUL, MR-DC, CU/DU function split, etc. Specifically, the definition of load information to be exchanged or reported, as well as the distributed and centralized load balance approaches should be studied.

Load information

In LTE system, the mobility load balancing mechanism is based on cell reselection and handover parameters self-tuning in order to redistribute traffic from highly loaded cells to underutilized cells.

In general, mobility load balancing consists of the following functions [1]:

-	Load measurement

-	Cell load information exchange over interfaces

-	Load balancing action based on handovers

-	Handover and/or cell reselection parameter reconfiguration and adapting, e.g. in the gNB or OAM entity

One key issue is what kind of load information should be reported. The setting of load information determines the trigger conditions, the general behaviors and performance of the load balancing algorithm. In LTE system, the cell specific load information adopted for the purpose of load balancing optimization consists of [1]:

-	Radio resource usage

-	HW load indicator

-	TNL load indicator

-	Cell Capacity Class value

-	Capacity value

Such parameters designed in LTE could be reused in NR. Moreover, new features of NR, like slicing, SUL, MR-DC, CU/DU function split, etc. may introduce new parameters for the purpose of load balancing optimization.

Proposal 1: To use LTE design as a baseline and consider new features of NR for studying the load information for load balancing algorithm in NR.

Architecture aspects

Besides the update of load information caused by the 5G new features, it is also important to look into the architecture to perform SON algorithms. Generally speaking, different architecture will result in different approaches to achieve the same functions indicated by a specific use case. To make the architectural aspects easier to be understood, we start with the MLB case. Specifically, for MLB, different architectures will have an impact on the interfaces that support load information exchanges and the location to decide the handovers as well as the parameter adapting.  And possible architectural alternatives for MLB case can be classified into two groups: centralized architecture and distributed architecture (potentially plus the hybrid architecture), which follows similar classification criteria as in TR28.861 [3] and TS32.500 [4], where the self-organizing network is further divided into C-SON, D-SON and H-SON based on different types of SON algorithm deployment on both (either) 3GPP management system and (or) Network Functions (NF, which includes CN and RAN). However, the architectural analysis on load balancing optimization is not captured in both [3] and [4], which could be discussed within the scope of RAN3.

Observation 1: In TR28.861 and TS32.500, the concept of SON has been classified as C-SON, D-SON and H-SON.

Observation 2: The architectural analysis on load balancing optimization is not captured in TR28.861 and TS32.500, which could be discussed within the scope of RAN3.

Generally speaking, the load balancing optimization can be further divided into intra-gNB (when CU-DU split is introduced) and inter-gNB load balancing aspects. In our opinion, intra-gNB and inter-gNB cases should be discussed separately in terms of architecture because of the assurance of the unified usage of the term ‘centralized’ within each scope.

For intra-gNB case, considering CU/DU split is introduced in NR, the case of intra-CU inter-DU load sharing and balancing should be studied. However, in this contribution, following the conventional method as in LTE, the inter-gNB load balancing case is firstly investigated, with which we believe specific architecture option could also be applied to intra-gNB case.

Proposal 2: The architecture for inter-gNB load balancing case should be studied in the SI.

Different types of architectures for inter-gNB load balancing case are discussed below.

Centralized architecture

In this alternative, the load report analysis, the handover decision and the parameter adapting decision are centralized in one separate entity/logical node, which could be in the domain of OAM or RAN. Specifically, the centralized SON algorithm could be located in OAM or a logical node in charge of multiple gNBs. The detailed procedures are illustrated in figure 1:

Figure 1 Centralized architecture

- The gNBs monitor the cell load and report their current load information to a centralized SON entity/node. The gNB reports its load periodically, or triggered by some pre-defined criteria (e.g. reports only when the load exceeds a given load threshold value).

- The centralized SON entity/node could also ask the gNB to report its load as needed.

- The centralized SON entity/node makes a decision on whether to modify cell reselection/handover parameters taking the received cell load information into account.

- If yes, the cell reselection/handover parameters between the congested cell and its neighbouring cells are modified, and the modification is updated in the centralized SON entity/node and informed to the gNBs. Meanwhile, the updated cell reselection parameters are broadcast to the UEs in System Information.

4a. The centralized SON entity/node performs load balancing action by sending handover command.

- According to the cell reselection/handover mechanisms, some UEs reselect/handover to the less congested cell.

It should be noted that Step 4a is not supported when the centralized SON algorithm is located in OAM. Moreover, based on the characteristic of OAM, it is not appropriate to generate HO command (i.e. perform RRM operation) from OAM. Therefore, a logical RAN node in charge of multiple gNBs could be introduced as the centralized SON entity/node to perform handover actions.

As shown in figure 1, the cell load coordination goes through the centralized SON entity/node. The advantage of one centralized entity is to avoid non-unified load balance decisions among multi-vendor gNBs. In addition, centralized architecture can be more robust against network instability caused by simultaneous operation of multiple SON features. Moreover, the centralized architecture could introduce potentially new functions for multiple use cases which could bring extra benefits to the current network. For example, the centralized SON node could perform online prediction on the cell load based on the historical traffic and measurements data. However, the centralized entity requires all involved gNBs to communicate with it when the load balancing algorithm is triggered, which would require extra capability for the centralized SON node to perform fast convergence of LB decisions.

Distributed architecture

Another alternative is to adapt parameters and perform handover actions within gNB. When the load balancing algorithm is triggered, the following procedures are performed as shown in figure 2:

Figure 2 Distributed architecture

- The gNBs monitor the cell load and exchange their current load information with neighboring gNBs on Xn interface. The gNB reports its load periodically, or the gNB requests cell load information from neighboring gNBs based on the measured cell load with some predefined criteria (eg. a pre-defined load threshold).

- The gNB makes a decision whether to modify cell reselection/handover parameters taking into account of its own and the received cell load information.

- If yes, the cell reselection/handover parameters are modified and informed to neighboring gNBs. Meanwhile, the updated cell reselection parameters are broadcast to the UEs in System Information.

- The gNB can also initiate handover procedures with target gNB(s) to perform handover actions.

- According to the cell reselection/handover mechanisms, some UEs reselect/handover to the less congested cell.

This distributed architecture would allow gNBs to directly communicate with each other and make quicker decisions before the network reaches a loaded condition. However, distributed algorithms are not performed well in case of multi-vendor because of the non-unified load balance algorithms by the gNBs.

Hybrid architecture

A third approach, as shown in figure 3, is a mixture of the above mentioned two architectures in terms of the location to achieve the SON algorithm; namely, the SON algorithm is partially located in the centralized SON entity/node and partially located in gNB, and the centralized SON entity/node and gNB work together in a coordinated manner to build up the complete functionality of the use case.

Figure 3 Hybrid architecture

Specifically, the complete MLB functionality could be realized through two functions which are achieved by different architectures: 1. the gNBs negotiate the load information with each other in order to decide whether to execute a handover in a distributed manner; and, 2. the centralized SON entity/node collects the load report from gNBs, makes decisions on whether to modify cell reselection/handover parameters and performs parameter adapting in a centralized manner.

This hybrid architecture would minimize the coordination procedure for parameter adapting while allowing gNBs to directly communicate with each other. However, hybrid solutions also face the challenges of maintaining different interfaces and extra signaling load.

Comparison

The following table aims at comparing the alternative solutions described in the above sections.

Table 1 Comparison table among alternatives

|  | Centralized architecture (the centralized entity = the OAM entity) | Centralized architecture (the centralized entity = a logical RAN node) | Distributed architecture | Hybrid architecture |
| --- | --- | --- | --- | --- |
| Response time | Long | Short | Short | FFS (depends on the location of the centralized entity, and the function split between the centralized node and gNBs) |
| Load on Xn interface | Low : No exchange | Low : No exchange | High |  |
| Load on OAM interface | High: Distinct messages to indicate cell A and cell B loads | Nearly no traffic | Nearly no traffic |  |
| New interface between gNB and the centralized RRM entity | No | Yes | No |  |
| gNB complexity | Low | Low | High: Support of Load balance algorithms Storage of neighbour cells load if periodic load exchange |  |
| OAM complexity | Support of RRM functionalities (unrealistic) | Low | Low |  |
| Scalability | The centralized SON entity concentrates all the decision and the load information exchanges | The centralized SON entity concentrates all the decision and the load information exchanges | The decision and the load information exchanges are distributed among nodes |  |
| Load information exchange between gNB | Not supported | Not supported | Supported if periodic load exchange |  |
| Multi-vendor support | Good | Good | Best performance in a vendors own network |  |

Proposal 3: To study and evaluate the proposed architecture alternatives for load balancing optimization in the SI.

Proposal 4: To capture the text proposal of the architecture aspects into the TR.

Proposal

The paper discussed the load information and the architecture aspects of load balancing use case, and came to the following proposals:

Proposal 1: To use LTE design as a baseline and consider new features of NR for studying the load information for load balancing algorithm in NR.

Observation 1: In TR28.861 and TS32.500, the concept of SON has been classified as C-SON, D-SON and H-SON.

Observation 2: The architectural analysis on load balancing optimization is not captured in TR28.861 and TS32.500, which could be discussed within the scope of RAN3.

Proposal 2: The architecture for inter-gNB load balancing case should be studied in the SI.

Proposal 3: To study and evaluate the proposed architecture alternatives for load balancing optimization in the SI.

Proposal 4: To capture the text proposal of the architecture aspects into the TR.

References

- TR36.300, Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2(Release 15)

- R3-190565, Report of offline coordination of uses cases for RAN-centric DCU, CMCC (Rapporteur)

- 3GPP TR28.861, Study on the Self-Organizing Networks (SON) for 5G networks (Release 16)

- 3GPP TS32.500, Self-Organizing Networks (SON); Concepts and requirements (Release 15)