3GPP TSG RAN WG3#103bis 	                                                                           R3-192031

Xian, China, 8th  April – 12th April 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0067 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NGAP Further Clarification of S-NSSAI Update for EPS to 5GS HO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-04-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As agreed in RAN3#103, the S-NSSAI per PDU session may be updated for the EPS to 5GS handover for home-routed roaming scenario, hence the updated S-NSSAI value is required to be included in the PDU SESSION RESOURCE MODIFY REQUEST message to support the inter-system handover for home-routed roaming scenario. However, it is not clear whether in other scenarios, e.g. intra-5GS HO, 5GC can also use PDU SESSION RESOURCE MODIFY REQUEST message to update the S-NSSAI value per PDU session. As concluded in SA2 and RAN3 previously, slice remapping is not supported in Rel-15, hence we propose to reflect above agreeement explicitly, so as to avoid abusing of such remapping feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To refer to TS23.502, highlighting the limitation that only in home-routed roaming scenario, 5GC can use PDU SESSION RESOURCE MODIFY REQUEST message to update the S-NSSAI value per PDU session in Rel-15. Impact analysis: Impact assessment towards the previous version of the specification (same release): - This CR has an impact under functional point of view. - The impact can be considered isolated because the change only affects the S-NSSAI update during inter-system handover in home-routed roaming scenario |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether in other scenarios, 5GC can also use PDU SESSION RESOURCE MODIFY REQUEST message to update the S-NSSAI value per PDU session |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Change Begins

8.2.3	PDU Session Resource Modify

8.2.3.1	General

The purpose of the PDU Session Resource Modify procedure is to enable configuration modifications of already established PDU session(s) for a given UE. It is also to enable the setup, modification and release of the QoS flow for already established PDU session(s). The procedure uses UE-associated signalling.

8.2.3.2	Successful Operation

Figure 8.2.3.2-1: PDU session resource modify: successful operation

<Unchanged Text Omitted>

If the RAN Paging Priority IE is included in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node may use it to determine a priority for paging the UE in RRC_INACTIVE state.

For each PDU session, if the S-NSSAI IE is included in the PDU Session Resource Modify Request Item IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall replace the previously provided S-NSSAI by the received S-NSSAI for the concerned PDU session and use it as specified in TS 23.502 [10].

For each PDU session, if the Network Instance IE is included in the PDU Session Resource Modify Request Transfer IE contained in the PDU SESSION RESOURCE MODIFY REQUEST message, the NG-RAN node shall, if supported, use it as specified in TS 23.501 [9].

Change Ends