3GPP TSG-RAN WG3 Meeting #103bis                                                           R3-192065

Xi’an, China, April 8th – 12th

Agenda item:	13.2.2

Source: 		Qualcomm Incorporated

Title: 	CB #36 LS to RAN2

Document for:	Discussion

# 1	Introduction

This document captures CB discussion:

CB: # 36_LStoRAN2 - capture agreement & WA (“full picture”), ask for confirmation that the # of bearers supported with this agreement+WA is sufficient (QC) LS to RAN2 in R3-192065 For 1:1 mapping, the use of GTP tunnel ID to identify a DRB between donor CU and donor DU is confirmed WA: adopt IPv6 flow labels for 1:1 mapping; FFS whether to also use DSCP - ask RAN2 to confirm above assumption; if RAN2 confirms, it seems we can turn WA into agreement

The following includes a draft LS to RAN2.

# 2	Draft LS to RAN2

3GPP TSG-RAN WG3 Meeting #103	bis                                                            R3-19xxxx

Xi’an, China, April 8th – 12th

Title:	[Draft] LS to RAN2 on bearer limit with IPv6 flow label

Response to:	-

Release:	Rel-16

Work Item:	NR_IAB

Source:	Qualcomm (will be RAN3)

To:	RAN2

CC:

Contact Person:

#### Name:	Georg Hampel

E-mail Address:	ghampel [at] qti [dot] qualcomm [dot] com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description

RAN3 discussed intra-donor transport, where the IAB-donor gNB is split into IAB-donor DU and IAB-donor CU, which are interconnected by a wireline network.

In such a scenario, the IAB-donor DU needs to forward F1-U and F1-C packets arriving from the CU on the wireline network to the IAB-node on the wireless backhaul, and vice versa.

In downstream direction, the IAB-donor-DU has to identify the RLC-channel on the wireless BH link, where the F1 packet is forwarded based on information carried on the header(s) of the packet arriving on the wireline network.

RAN3 acknowledges that there is benefit if the bearer can be identified on the packet header between donor CU and donor DU.

RAN3 achieved the following agreements:

- For 1:1 mapping, the use of GTP tunnel ID to identify a DRB between donor CU and donor DU is confirmed - Working assumption: Adopt IPv6 Flow Labels for 1:1 mapping; FFS whether to also use DSCP

2. Action

RAN3 kindly asks RAN2 for confirmation that the number of bearers supported with this agreement and working assumption is sufficient.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG3 Meeting #104				 13 - 17 May 2019  	Reno, USA

TSG-RAN WG3 Meeting #105	26 - 30 Aug 2019 	Ljubljana , SI