3GPP TSG RAN WG#3 Meeting #103bis	R3-192103

Xi’an, China, 8th – 12th April 2019

Agenda item:	20.1

Source:	Nokia, Nokia Shanghai Bell, Thales

Title:	 TP on small corrections on scenarios

Document for:	Discussion and Decision

# 1 Introduction

The minimal elevation angle for feeder link is 5° in TR 38.811 and Table 8.1-1 in TR 38.821 , but is 10° in Table 4.2-2 in TR 38.821 [1][2]. To align with the assumptions in TR 38.811 and TR 38.821, this contribution provides corresponding text proposal for TR 38.821. This contribution also provides more accurate value for some parameters.

# 2 Text Proposal for TR 38.821

Start of changes

## 4.2	Non-Terrestrial Networks reference scenarios

We shall consider in this document non-terrestrial networks providing access to user equipment in six reference scenarios including

- Circular orbiting and notional station keeping platforms.

- Highest RTD constraint

- Highest Doppler constraint

- A transparent and a regenerative payload

- One ISL case and one without ISL. Regenerative payload is mandatory in the case of inter-satellite links.

- Fixed or steerable beams resulting respectively in moving or fixed beam foot print on the ground

Six scenarios are considered as depicted in Table 4.2-1 and are detailed in Table 4.2-2.

Table 4.2-1: Reference scenarios

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A and B) | LEO based non-terrestrial access network (Scenario C & D) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth (service link) | 30 MHz for band < 6 GHz 400 MHz for band > 6 GHz |  |
| Payload | Scenario A: Transparent (including radio frequency function only) Scenario B: regenerative (including all or part of RAN functions) | Scenario C: Transparent (including radio frequency function only) Scenario D: Regenerative (including all or part of RAN functions) |
| Inter-Satellite link | No | Scenario C: No Scenario D: Yes |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) Scenario D1: Yes (steerable beams), see note 1 Scenario D2: No (the beams move with the satellite) |
| Max beam foot print diameter at nadir | 500 km | 200 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,131 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) Scenario B: 270.73 ms (service link only) | Scenario C: (transparent payload: service and feeder links) - 25.77 ms (600 km) - 41.77 ms (1200 km) Scenario D: (regenerative payload: service link only) - 12.89 ms (600 km) - 20.89 ms (1200 km) |
| Max delay variation within a beam (earth fixed user equipment) | 16 ms | 4.44 ms (600 km) 6.44 ms (1200 km) |
| Max differential delay within a beam | 1.6 ms | 0.65 ms (600km and 1200km) |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600 km) 21 ppm(1200 km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27 ppm/s (600 km) 0.13 ppm/s(1200 km) |
| User equipment motion on the earth | 1000 km/h (e.g. aircraft) | 500 km/h (e.g. high speed train) Possibly 1000 km/h (e.g. aircraft) |
| User equipment antenna types | Omnidirectional antenna (linear polarisation), assuming 0 dBi Directive antenna (up to 60 cm equivalent aperture diameter in circular polarisation) |  |
| User equipment Tx power | Omnidirectional antenna: UE power class 3 with up to 200 mW Directive antenna: up to 4 W |  |
| User equipment Noise figure | Omnidirectional antenna: 7 dB Directive antenna: 1.2 dB |  |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP or non-3GPP defined Radio interface | 3GPP or non-3GPP defined Radio interface |

NOTE 1:	Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite.

NOTE 2:	Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment.

NOTE 3:	Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir.

NOTE 4: Speed of light used for delay calculation is 299792458 m/s.

End of changes

Start of changes

## 7.1	Requirements and key issues

## 7.1.1 	Delay

In order to reduce the standardization work, the table here below identifies the worst case NTN scenarios to be considered for the delay constraint.

Table 8.1-1: NTN scenarios versus delay constraints, Source [2]

| NTN scenarios | A | B | C1 | C2 | D1 | D2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | GEO transparent payload | GEO regenerative payload | LEO transparent payload |  | LEO regenerative payload |  |
| Satellite altitude | 35 786 km |  | 600 km |  |  |  |
| Relative speed of Satellite wrt earth | negligible |  | 7.56 km per second |  |  |  |
| Min elevation for both feeder and service links | 10° for service link and 10° for feeder link |  |  |  |  |  |
| Typical Min / Max NTN beam foot print diameter (note 1) | 100 km / 1000 km |  | 50 km / 500 km |  |  |  |
| Maximum Round Trip Delay on the radio interface between the gNB and the UE | 541.46 ms (Worst case) | 270.73 ms | 25.77 ms |  | 12.89 ms |  |
| Minimum Round Trip Delay on the radio interface between the gNB and the UE | 477.48 ms | 238.74 ms | 8 ms |  | 4 ms |  |
| Maximum Delay variation as seen by the UE (note 2) | Negligible |  | Up to +/- 40 µs/sec (Worst case) |  | Up to +/- 20 µs/sec |  |
| Maximum delay difference within a NTN beam as seen by the UE (note 3) | 16 ms (Worst case) |  | 4.44 ms |  |  |  |
| Max rate of hand-over (FFS) |  |  |  |  |  |  |
| NOTE 1: The beam foot print diameter are indicative. The diameter depends on the orbit, earth latitude, antenna design and radio resource management strategy in a given system. NOTE 2: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario NOTE 3: The delay difference compares the delay (function of UE-satellite-NTN gateway distance) experienced by two different UEs served by the same beam at a given time NOTE 4: Speed of light used for delay calculation is 299792458 m/s. |  |  |  |  |  |  |

End of changes

# Reference

- TR 38.811, “Study on New Radio (NR) to support non terrestrial networks”, 3GPP.

- TR 38.821, “Solutions on NR to support non-terrestrial networks”, 3GPP.