3GPP TSG RAN WG3#104 	                                                                               R3-192269

Reno, USA, 13th – 17th  May 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.821 | CR | pCR | rev |  | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for TR38.821 Clarification of Supporting Xn Interface over NTN Transport Links |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2019-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current baseline TR38.821, for gNB on board case in sub-section 5.2.1.1, it has been described that: “The gNB on board different satellites may be connected to the same 5GCN on the ground.” and “If the satellite hosts more than one gNB, the same SRI will transport all the corresponding NG interface instances.”. Similarly, for gNB-DU on board case in sub-section 5.3.2.1, it has been also described that:” DU on board different satellites may be connected to the same CU on ground. and “If the satellite hosts more than one DU, the same SRI will transport all the corresponding F1 interface instances.” However, the counterpart transport support with ISL is still missing, i.e. whether the same ISL will transport all the corresponding Xn interface instances as well? It is not fully clear whether the gNB on board can setup/maintain Xn interface with gNB on ground indirectly via NTN-GWs, if the SRI transport link is as good as ISL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal 1: To agree and capture that “If the satellite hosts more than one gNB, the same ISL will transport all the corresponding Xn interface instances.” Proposal 2: To agree and capture that “For gNB on board, SRL can also transport Xn interface instance with gNB on ground, if configured”. Proposal 2bis: “If the satellite hosts more than one gNB, the same SRI will transport all the corresponding Xn interface instances.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion or incompleteness exists |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First change, ommited text not changed ***

5.2.1.1	Overview

The NG-RAN logical architecture as described in TS 38.401 is used as baseline for NTN scenarios.

The satellite payload implements regeneration of the signals received from Earth.

- NR-Uu radio interface on the service link between the UE and the satellite

- Satellite Radio Interface (SRI) on the feeder link between the NTN gateway and the satellite.

SRI (Satellite Radio Interface) is a transport link between NTN GW and satellite.

Figure 5.3-1: Regenerative satellite without ISL, gNB processed payload

Note: The satellite may embark additional traffic routing functions that are out of RAN scope. (FFS)

The satellite payload also provides Inter-Satellite Links (ISL) between satellites

ISL (Inter-Satellite Links) is a transport link between satellites. ISL may be a radio interface or an optical interface that may be 3GPP or non 3GPP defined but this is out of the study item scope.

The NTN GW is a Transport Network Layer node, and supports all necessary transport protocols.

Figure 5.3-2: Regenerative satellite with ISL, gNB processed payload (FFS)

The figure above illustrates that UE served by a gNB on board a satellite could access the 5GCN via ISL.

The gNB on board different satellites may be connected to the same 5GCN on the ground.

If the satellite hosts more than one gNB, the same SRI will transport all the corresponding NG interface instances.

If the satellite hosts more than one gNB, the same ISL will transport all the corresponding Xn interface instances.

*** Next change, ommited text not changed ***

8.4.3.4	Transporting Xn over SRI

Transporting Xn over an Earth-satellite link between on board NTN gNB and terrestrial gNB directly or indirectly via NTN GWs (IP Routers) has challenges, but it is not prohibited from configuring so, if SRI is good enough.

For example, in a LEO scenario, when a satellite moves below the horizon, all its Xn interfaces to terrestrial gNBs will become unavailable, and this may trigger subsequent actions at application protocol and/or SCTP level in the relevant terrestrial gNBs. The opposite will happen when the satellite appears at the horizon: Xn setups may be triggered to some terrestrial gNBs. This creates a technical issue, as it will lead to CP signalling surges corresponding to changes in visibility of the LEO satellites.

Furthermore depending on the outage performance of the SRI, Xn may become unavailable for some periods of time. This may trigger interface re-establishment toward all corresponding terrestrial gNBs, generating CP signalling surges at every outage event. This would happen for all Xn interface terminated in the on board NTN -gNB impacted by the outage event.

Therefore the benefit of this configuration is FFS.

*** End change ***