3GPP TSG-RAN WG3 Meeting #104	R3-192474

# Reno, Nevada, USA, May 13 – 17, 2019									           Revision of R3-191374

Agenda item:	19.4

Source: 	Qualcomm Incorporated

Title: 	Broadcast of Location Assistance Data by NG-RAN

Document for:	Discussion and Decision

# 1. 	Introduction

A new work item on "NR Positioning Support" has been agreed at RAN#83 [1]. The objectives include:

Define signaling for broadcast assistance data delivery [RAN2/RAN3/SA2/SA3]

Support for broadcast of location assistance data was added in Release 15 for E-UTRA connected to EPC as described in TS 36.305 [2] and TS 23.271 [3] for stage 2 and in TS 36.331 [4], TS 36.455 [5], TS 36.355 [6], TS 29.171 [7], TS 24.301 [8] and TS 23.008 [9] for stage 3. Location assistance data that can be broadcast for E-UTRA connected to EPC in Release 15 comprises GNSS, RTK, and OTDOA assistance data elements defined in TS 36.355 [6]. The same assistance data elements are already supported for NR in Release 15 and additional assistance data elements (e.g., for new RAT dependent position methods for NR and for GNSS PPP-RTK [1]) will be added in Release 16.

At RAN2#105bis, it was agreed that broadcast of assistance data for NR is supported for at least A-GNSS, RTK and OTDOA assistance information [10]. The OTDOA assistance data may also be useful for other positioning methods requiring DL-PRS and/or TRP information.

The benefits of broadcasting location assistance data can include reduced network signaling and reduced load on network elements (e.g. LMF, AMF, gNB) for a high density of recipient UEs in each cell, reduced latency in obtaining assistance data at a UE, and reduced UE signaling and power usage. As an example, IoT UEs would be able to determine and track their locations while in idle state which could reduce both UE power and load on the network. The assistance data can also be ciphered which can provide a revenue stream for operators.

The support of broadcast of location assistance data for NR can also be used to help support broadcast of location assistance data for E-UTRA connected to 5GC, since the same signalling and procedures could be used on the network side and SIB support on the radio interface can reuse the existing RRC support in TS 36.331 [4].

# 2. 	Procedures for Broadcast of Location Assistance Data

For E-UTRA connected to EPC the procedures for broadcast of assistance data are specified in TS 36.305 clause 7.6 [2]. The broadcast procedures for NG-RAN connected to 5GC can be based these procedures with suitable changes for NG-RAN/5GC as shown in Figure 1 below.

Figure 1: Procedures to support broadcast of assistance data by NG-RAN.

1.	The LMF sends an NRPPa Assistance Data Information Control message to the NG-RAN node with an indication to start broadcasting assistance information. The message includes one or more System Information groups, where each group contains the broadcast periodicity and one or more pre-coded posSIB types together with meta data. Each posSIB type may be ciphered and/or segmented at the LMF. The meta data may include an indication whether the posSIB type in the System Information group is ciphered or not, as well as an indication of an applicable GNSS type.

2.	The NG-RAN node includes the received System Information groups in RRC System Information Messages and corresponding scheduling information in SIBx (e.g. SIB1). The UE applies system information acquisition procedures for acquiring the assistance data information that is broadcast.

NOTE:	The SIBx for scheduling information is FFS.

3.	If the posSIB types were ciphered by the LMF, the LMF provides the used ciphering keys, together with a validity time and validity area for each key, to the AMF. The AMF returns a Ciphering Key Data Result message back to the LMF indicating whether the AMF was able to successfully store the ciphering data sets. The AMF may then distribute successfully stored ciphering keys and their validity times and validity areas to suitably subscribed UEs using a mobility management message. The AMF repeats this procedure whenever a ciphering key changes.

4.	At any time after Step 1, the NG-RAN node may send an NRPPa Assistance Information Feedback message to the LMF providing feedback on assistance information broadcasting. The message may include an assistance information failure list indicating that certain posSIB types could not be configured for broadcasting by the NG-RAN node.

5.	If the assistance information in a System Information group changes, the LMF provides updated information in an NRPPa Assistance Information Control message.

6.	The NG-RAN node replaces the previously stored System Information groups with the new information received at Step 5 and includes the new System Information groups in RRC System Information Messages.

7.	If the LMF wants to abort the broadcast of a System Information Group, it sends an NRPPa Assistance Information Control message to the NG-RAN node including an indication to stop broadcasting the assistance information.

Observation 1:	The procedures for broadcast of location assistance data for NG-RAN can be based on the procedure for broadcast of location assistance data for E-UTRA connected to EPC in TS 36.305 [2].

# 3. 	Definition of Assistance Data Elements

For E-UTRA connected to EPC, the assistance data elements which can be broadcast are defined in TS 36.355 [6]. The definition includes the following.

-	Mapping of posSIB types to assistance data elements (clause 7.2 of TS 36.355 [6]).

-	UE procedures related to receiving broadcast information elements (clause 7.3 of TS 36.355 [6]).

-	Common content of broadcast information elements including support of ciphering, segmentation and update of information content (clause 7.4 of TS 36.355 [6]).

-	Content of a broadcast information element for OTDOA (clause 7.4.2 of TS 36.355 [6]).

-	Content of broadcast information elements for A-GNSS and RTK (clause 6.5.2.2 of TS 36.355 [6]).

-	Ciphering of broadcast information elements (clause 7.5 of TS 36.355 [6]).

The broadcast information elements for A-GNSS and RTK defined in clause 6.5.2.2 of TS 36.355 [6] and the common content of broadcast information elements to support ciphering, segmentation and update of information content defined in clause 7.4 of TS 36.355 [6] can be reused for NR (and E-UTRA connected to 5GC). Additional GNSS assistance data elements to support PPP-RTK (SSR) will be added in Release 16 [1]. The same ciphering support described in clause 7.5 of TS 36.355 [6] can be reused for NR together with corresponding handling of posSIB segments and deciphering (clause 7.3 of TS 36.305).

The OTDOA posSIB Type 3.1 contains the LTE OTDOA Assistance Data elements (Reference Cell and Neighbour Cell Information). NR-OTDOA may be added to the existing LPP OTDOA positioning method, which currently supports OTDOA for LTE and NB-IoT. This would allow "hybrid" LTE and NR OTDOA (similar to the current possibility of using "hybrid" LTE and NB-IoT OTDOA).  A new posSIB Type (e.g. Type 3.2) can be added which includes the new NR specific OTDOA assistance data. Both posSIB Types may then be broadcast by a gNB and ng-eNB. Therefore, the existing OTDOA posSIB Type 3.1 can be reused for NG-RAN and a new NR specific OTDOA posSIB Type (e.g. 3.2) can be added.

An additional OTDOA posSIB Type (e.g. Type 3.3) would be needed for the TRP related assistance data to support UE-based OTDOA. In UMTS [11], there are separate SIB definitions for UE-assisted and UE-based OTDOA. As discussed in e.g. [12] there is a good reason for the use of different SIBs for UMTS broadcast of UE-assisted versus UE-based OTDOA assistance data, because it allows UE-based assistance data to be optionally ciphered while allowing UE-assisted assistance data to be broadcast unciphered. This would not easily be possible if the assistance data were included in the same SIB, and would be almost impossible if the assistance data were included in one SIB as part of the same ASN.1 structure. The reason is that complete ASN.1 decoding would not be possible without prior deciphering and prior deciphering would not be possible without some ASN.1 decoding to determine which parts of the SIB to decipher. Placing different types of OTDOA assistance data in different posSIBs (i.e. posSIB Type 3.1, 3.2, 3.3 as discussed above) allow UEs to more easily access assistance data that the UE prefers or supports (e.g. NR or LTE only, UE-based or UE-assisted only, etc.).

Therefore, the LPP impacts are restricted to the definition of additional posSIB Types for OTDOA together with corresponding ASN.1 definition and additional posSIB Types for GNSS PPP-RTK (SSR) [13],[14]:

|  | posSibType | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
|  | posSibType2-20 | GNSS-SSR-URA |
|  | posSibType2-21 | GNSS-SSR-PhaseBias |
|  | posSibType2-22 | GNSS-SSR-STEC-Correction |
|  | posSibType2-23 | GNSS-SSR-GriddedCorrectrion |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted (LTE) |
|  | posSibType3-2 | OTDOA-UE-Assisted (NR) |
|  | posSibType3-3 | OTDOA-UE-Based |

posSIB Types which change relatively infrequent (e.g., posSIBTypes3-x) could be packed into a single posSI message, and therefore, would require a single SI window only. This would require segmentation, but latency in receiving long-term valid posSIBs should generally not be of a concern.

The posSIBType3-3 for UE-Based OTDOA support may also be split into two separate posSIBs, one containing the static TRP coordinates and one containing the more dynamic time synchronization differences (RTDs [15]). This may allow different broadcast periodicities for the static and more dynamic data or may allow a deployment to broadcast the dynamic data (e.g., RTDs) only and providing the static data via LPP (e.g., using 5GC MO-LR [16]).

Observation 2:	The specification for broadcast of assistance data as defined in clause 7 of TS 36.355 can be re-used for NR. New posSIB Types are needed to support NR-OTDOA and GNSS PPP-RTK (SSR) assistance data.

# 4. 	Transfer of Assistance Data from an LMF to NG RAN Node

For E-UTRA connected to EPC, LPPa in TS 36.455 [5] is used to transfer assistance data elements from an E-SMLC to an eNB using the following messages and procedures:

-	LPPa Assistance Information Control (TS 36.455 clause 8.4.1, 9.1.3.1 [5]).

-	LPPa Assistance Information Feedback (TS 36.455 clause 8.4.2, 9.1.3.2 [5]).

The LPPa Assistance Information Control message is used to transfer assistance information from an E-SMLC to an eNB by including the following information for each positioning SI message.

-	Broadcast periodicity.

-	For each positioning SIB (posSIB) in the positioning SI message:

-	posSIB type,

-	one or more posSIB segments,

-	optional indications of a GNSS, SBAS and use of encryption,

-	broadcast priority.

The LPPa Assistance Information Feedback enables an eNB to provide feedback to an E-SMLC identifying posSIBs which the eNB is not able to broadcast.

The procedures and messages supported in LPPa can be used as the basis for identical procedures and messages in NRPPa to support transfer of assistance data from an LMF to an NG-RAN node and feedback from an NG-RAN node to an LMF. Annex A of this contribution shows the NRPPa changes required to support broadcast of assistance data in NRPPa according to Section 2 above.  This is based on the corresponding LPPa messages with changes for NG-RAN. Additional information elements/changes which may be required in NRPPa (compared to LPPa) are highlighted in green based on the discussion in [19].

NOTE:	A small update to LPPa would be needed to support the additional GNSS SSR assistance data elements (discussed in section 3 above) for E-UTRA connected to EPC. I.e., the additional posSIB Types 2-20 to 2-23 would need to be added to the IE Positioning SIB Type in LPPa [5], similar to what is shown in Annex A for NRPPa.

Observation 3:	The procedures and messages supported in LPPa for transfer of assistance data from an E-SMLC to an eNB can be used as the basis for identical procedures and messages in NRPPa [17] to support transfer of assistance data from an LMF to an NG-RAN node.

# 5. 	Broadcast of Assistance Data by an NG RAN Node

# 5.1	Broadcast by an ng-eNB

# An ng-eNB can use the existing RRC support for posSIBs and posSIB scheduling defined for E-UTRA connected to EPC in TS 36.331 [4]. This can be used to support existing broadcast assistance data elements for A-GNSS, RTK and LTE-OTDOA. The posSIB Types to support NR-OTDOA and UE-based mode as well as PPP-RTK (SSR) as discussed in section 3 above need to be added to the pos SI message and SIB scheduling information in TS 36.331 as discussed in [19].

Observation 4:	Broadcast of assistance data by an ng-eNB can reuse existing RRC support for posSIBs and posSIB scheduling in TS 36.331 [4] . Additional posSIB Types for NR-OTDOA and UE-based mode as well as for PPP-RTK (SSR) need to be added to the pos SI message and posSIB scheduling information.

# 5.2	Broadcast by a gNB

New positioning SIBs (posSIBs) can be added to RRC in TS 38.331 [18] in a similar manner to inclusion of posSIBs for E-UTRA connected to EPC in TS 36.331 [4], as described in further detail in [19].

Observation 5:	Required RRC changes for support of broadcast of assistance data by an gNB are relative minor since new positioning SIBs (posSIBs) can be added to RRC in TS 38.331 [18] in a similar manner to inclusion of posSIBs for E-UTRA connected to EPC in TS 36.331 [4].

# 6. 	Summary

In this contribution we discussed the broadcast of location assistance data by NG-RAN. The following observations have been made.

Observation 1:	The procedures for broadcast of location assistance data for NG-RAN can be based on the procedure for broadcast of location assistance data for E-UTRA connected to EPC in TS 36.305 [2].

Observation 2:	The specification for broadcast of assistance data as defined in clause 7 of TS 36.355 can be re-used for NR. New posSIB Types are needed to support NR-OTDOA and GNSS PPP-RTK (SSR) assistance data.

Observation 3:	The procedures and messages supported in LPPa for transfer of assistance data from an E-SMLC to an eNB can be used as the basis for identical procedures and messages in NRPPa [17] to support transfer of assistance data from an LMF to an NG-RAN node.

Observation 4:	Broadcast of assistance data by an ng-eNB can reuse existing RRC support for posSIBs and posSIB scheduling in TS 36.331 [4] . Additional posSIB Types for NR-OTDOA and UE-based mode as well as for PPP-RTK (SSR) need to be added to the pos SI message and posSIB scheduling information.

Observation 5:	Required RRC changes for support of broadcast of assistance data by an gNB are relative minor since new positioning SIBs (posSIBs) can be added to RRC in TS 38.331 [18] in a similar manner to inclusion of posSIBs for E-UTRA connected to EPC in TS 36.331 [4].

Based on these observations, the following proposals are made to support broadcast of location assistance data by NG-RAN.

Proposal 1:	Support broadcast of location assistance data based on the broadcast procedure for E-UTRA connected to EPC in TS 36.305 [2], whereby the LMF in 5GCN performs segmentation and ciphering and provides pre-coded positioning SIBs for broadcast in SI messages. Any ciphering keys are provided by an LMF to an AMF for distribution to suitably subscribed UEs using mobility management messages.

Proposal 2:	Support transfer of assistance data from an LMF to an NG-RAN node using procedures and messages in NRPPa which are based on the procedures and messages supported in LPPa for transfer of assistance data from an E-SMLC to an eNB, as shown in Annex A.

# References

[1]	RP-190752, "New WID: NR Positioning Support".

[2]		3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[3]		3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[5]	3GPP TS 36.455: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[6]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[7]	3GPP TS 29.171: "Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[8]		3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[9]		3GPP TS 23.008: "Organization of subscriber data".

[10]	R2-1905186, "Report from session on Rel-15 LTE Positioning, Rel-15 and 16 NR Positioning", RAN2#105bis.

[11]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol specification".

[12]	R2-1808105, "Introduction of UE-Based OTDOA Positioning", Qualcomm Incorporated.

[13]	R2-1906781, "GNSS SSR Assistance Data for NG-RAN", Qualcomm Incorporated.

[14]	R2-1906782, "Running LPP CR for PPP-RTK support (SSR)", Qualcomm Incorporated.

[15]	R2-1906784, "System Level Aspects of UE-Based Positioning", Qualcomm Incorporated.

[16]		3GPP TS 23.273: "5G System Location Services (LCS)".

[17]	3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

[18]		3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[19]	R2-1906783, "Broadcast of Location Assistance Data by NG-RAN", Qualcomm Incorporated.

# Annex A

# NRPPa changes for Assistance Information Broadcasting

START OF CHANGES

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.413:"NG-RAN; NG Application Protocol (NGAP)".

[3]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[4]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error handling".

[6]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) ".

[7]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Base Station (BS) radio transmission and reception".

[8]	3GPP TS 23.032:"Technical Specification Group Services and System Aspects; Universal Geographical Area Description (GAD)".

[9]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[10]	3GPP TS 36.211:"Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Physical Channels and Modulation".

[11]	IEEE Std 802.11™-2012, IEEE Standard for Information technology - Telecommunications and information exchange between systems - Local and metropolitan area network.

[12]	3GPP TS 36.455: " Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol A (LPPa)".

[xx]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[yy]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[zz]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

NEXT CHANGE

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CID	Cell-ID (positioning method)

E-CID	Enhanced Cell-ID (positioning method)

GNSS	Global Navigation Satellite System

LMF	Location Management Function

OTDOA	Observed Time Difference of Arrival

SBAS	Satellite-based Augmentation System

NEXT CHANGE

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |

NEXT CHANGE

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| E-CID Measurement Failure Indication | E-CID MEASUREMENT FAILURE INDICATION |
| E-CID Measurement Report | E-CID MEASUREMENT REPORT |
| E-CID Measurement Termination | E-CID MEASUREMENT TERMINATION COMMAND |
| Error Indication | ERROR INDICATION |
| Assistance Information Control | ASSISTANCE INFORMATION CONTROL |
| Assistance Information Feedback | ASSISTANCE INFORMATION FEEDBACK |

NEXT CHANGE

## 8.x	Assistance Information Transfer Procedures

### 8.x.1	Assistance Information Control

#### 8.x.1.1	General

The purpose of the Assistance Information Control procedure is to allow the LMF to signal positioning assistance information to the NG-RAN node for assistance information broadcasting.

#### 8.x.1.2	Successful Operation

Figure 8.x.1.2-1: Assistance Information Control procedure

The LMF initiates the procedure by sending an ASSISTANCE INFORMATION CONTROL message.

If the Assistance Information IE is included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN node shall replace any previously stored assistance information and use the received information to configure assistance information broadcasting.

If the Broadcast Priority IE is included in the Assistance Information IE, the NG-RAN node may take it into account when configuring broadcasting for the relevant information. Assistance information having the same Broadcast Priority value should receive the same treatment (i.e. broadcast by the NG-RAN node or not broadcast).

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", the NG-RAN node may start broadcasting the assistance information. If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "stop", the NG-RAN node may stop broadcasting the assistance information.

#### 8.x.1.3	Abnormal Conditions

If the Broadcast IE is included in the ASSISTANCE INFORMATION CONTROL message and set to "start", and no assistance information is available, the NG-RAN node shall consider the procedure as failed.

If neither the Assistance Information IE nor the Broadcast IE are included in the ASSISTANCE INFORMATION CONTROL message, the NG-RAN node shall consider the procedure as failed.

### 8.x.2	Assistance Information Feedback

#### 8.x.2.1	General

The purpose of the Assistance Information Feedback procedure is to allow the NG-RAN node to give feedback to the LMF on assistance information broadcasting.

#### 8.x.2.2	Successful Operation

Figure 8.x.2.2-1: Assistance Information Feedback procedure

If the Assistance Information Failure List IE is included in the ASSISTANCE INFORMATION FEEDBACK message, the LMF shall consider that assistance information broadcasting could not be configured for the relevant information.

#### 8.x.2.3	Abnormal Conditions

Void.

NEXT CHANGE

### 9.1.x	Messages for Assistance Information Transfer Procedures

#### 9.1.x.1	ASSISTANCE INFORMATION CONTROL

This message is sent by the LMF to transfer assistance information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information | O |  | 9.2.x |  | YES | reject |
| Broadcast | O |  | ENUMERATED (start, stop, ...) |  | YES | reject |

#### 9.1.x.2	ASSISTANCE INFORMATION FEEDBACK

This message is sent by the NG-RAN node to give feedback on assistance information broadcasting.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Assistance Information Failure List | O |  | 9.2.z |  | YES | reject |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

NEXT CHANGE

### 9.2.x	Assistance Information

This IE contains the assistance information.

Table 9.2.x-1: Assistance Information

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information | M |  |  |  |
| >Area ID | O |  | BIT STRING (24) | System information area see TS 38.331 [xx]. |
| >System Information |  | 1..<maxNrOfPosSImessage> |  | Corresponds to the number of SI messages with posSIBs to be scheduled |
| >>Broadcast Periodicity | M |  | ENUMERATED (ms80, ms160, ms320, ms640, ms1280, ms2560, ms5120, ...) | Broadcast Peiriodicity for the Pos SIBs, see TS 36.331 [yy] and TS 38.331 [xx] |
| >>Pos SIBs |  | 1..<maxNrOfPosSIBs> |  | Number of posSIBs in the System Information. |
| >>>PosSIB-Type | M |  | 9.2.y |  |
| >>>PosSIB Segments | M |  | 9.2.a |  |
| >>>Assistance Information Meta Data | O |  | 9.2.b |  |
| >>>Broadcast Priority | O |  | INTEGER (1..16, ...) | The priority of the assistance Information where 1 represents the highest priority and 16 the lowest priority |

| Range bound | Explanation |
| --- | --- |
| maxNrOfPosSImessage | Maximum number of positioning system information messages. Value is 32. |
| maxNrOfPosSIBs | Maximum number of positioning system information blocks included in the message. Value is 32. |

NEXT CHANGE

### 9.2.a	PosSIB Segments

This IE provides one posSIB or two or more posSIB segments which must be scheduled in series in consecutive transmissions of the same SI message.

Table 9.2.a-1: PosSIB Segments

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PosSIB Segments |  | 1..<maxNrOfSegments> |  |  |
| >Assistance Data SIB Element | M |  | OCTET STRING | TS 36.355 [zz] |

| Range bound | Explanation |
| --- | --- |
| maxNrOfSegments | Maximum number of positioning SIB segments (in case of Assistance Information Element contains segmented data according to TS 36.355 [zz]). Value is 64. |

NEXT CHANGE

### 9.2.b	Assistance Information Meta Data

This parameter contains meta data for an assistance information element.

Table 9.2.a-1: Assistance Information Meta Data

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Encrypted | O |  | ENUMERATED (true, …) | TS 36.331 [yy] TS 38.331 [xx] |
| GNSS ID | O |  | ENUMERATED (gps, sbas, qzss, galileo, glonass, bds, ...) | TS 36.331 [yy] TS 38.331 [xx] |
| SBAS ID | O |  | ENUMERATED (waas, egnos, msas, gagan, ...) | TS 36.331 [yy] TS 38.331 [xx] |
| Area Scope | O |  | ENUMERATED (true, …) | TS 38.331 [xx] |

NEXT CHANGE

### 9.2.y	Positioning SIB Type

This parameter defines a specific positioning SIB, as defined in TS 36.355 [zz].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Positioning SIB Type | M |  | ENUMERATED ( posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType3-1, posSibType3-2, posSibType3-3, ... ) |  |

NEXT CHANGE

### 9.2.z	Assistance Information Failure List

This parameter identifies the assistance information for which the NG-RAN failed to configure broadcasting.

Table 9.2.z-1: Assistance Information Failure List

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Assistance Information Failure List |  | 1..<maxnoAssistInfoFailureListItems> |  |  |
| >PosSIB-Type | M |  | 9.2.y |  |
| >Outcome | M |  | ENUMERATED (failed, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoAssistInfoFailureListItems | Maximum no. of assistance information failure list items that can be signaled with one message. Value is 32. |