3GPP TSG-RAN WG3 Meeting #104	R3-192780

# Reno, USA, 13 – 17 May, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0137 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network sharing with multiple Cell ID broadcast |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Need to support network sharing with multiple Cell ID broadcast in the first 5G release. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend served cell information and neighbour information for NR and E-UTRA cells to include CGI/TAC/RANAC for each PLMN Identity as broadcast in SIB1. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol and functional point of view. This CR is backwards compatible at ASN.1 level. The impact can be considered isolated to Xn Setup and NG-RAN Node Configuration Update procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Mandatory feature not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.11, 9.2.2.12, 9.2.2.13, 9.2.2.14, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | Y |  | Other core specifications |  |  |  | TS 38.300 CR#... TS 38.401 CR#... TS 38.473 CR#... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – | – |
| NR CGI | M |  | 9.2.2.7 |  | – | – |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – | – |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – | – |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. This IE is ignored if the Broadcast PLMNs Enhanced IE is present. | – | – |
| >PLMN Identity | M |  | 9.2.2.4 |  | – | – |
| CHOICE NR-Mode-Info | M |  |  |  | – | – |
| >FDD |  |  |  |  | – | – |
| >>FDD Info |  | 1 |  |  | – | – |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – | – |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – | – |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – | – |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – | – |
| >TDD |  |  |  |  | – | – |
| >>TDD Info |  | 1 |  |  | – | – |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – | – |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – | – |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. | – | – |
| Connectivity Support | M |  | 9.2.2.28 |  | – | – |
| Broadcast PLMNs Enhanced |  | 0..<maxnoofBPLMNs> |  | Broadcast PLMNs. The PLMN Identities and associated information contained in this IE shall be provided in the same order as broadcast in SIB1 within the PLMN-IdentityInfoList IE defined in TS 38.331 [8]. | YES | ignore |
| >PLMN Identity | M |  | 9.2.2.4 |  | – | – |
| >NR CGI | O |  | 9.2.2.7 |  | – | – |
| >TAC | O |  | 9.2.2.5 |  | – | – |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

#### 9.2.2.12	Served Cell Information E-UTRA

This IE contains cell configuration information of an E-UTRA cell that a neighbour NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA PCI | M |  | INTEGER (0..503, …) | E-UTRA Physical Cell ID | – | – |
| ECGI | M |  | E-UTRA CGI 9.2.2.8 |  | – | – |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – | – |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – | – |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs. The PLMN Identities and associated information contained in this IE shall be provided in the same order as broadcast in SIB1 within the cellAccessRelatedInfo-5GC-r15 IE defined in the SystemInformationBlockType1-v1530-IEs IE in TS 36.331 [14]. NOTE: In this version of the specification, it is possible to broadcast only up to 6 PLMN IDs. | – | – |
| >PLMN Identity | M |  | 9.2.2.4 |  | – | – |
| >ECGI | O |  | E-UTRA CGI 9.2.2.8 |  | YES | ignore |
| >TAC | O |  | 9.2.2.5 |  | YES | ignore |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  | YES | ignore |
| CHOICE E-UTRA-Mode-Info | M |  |  |  | – | – |
| >FDD |  |  |  |  | – | – |
| >>FDD Info |  | 1 |  |  | – | – |
| >>>UL EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NUL in TS 36.104 [25] for E-UTRA operating bands for which it is defined; ignored for E-UTRA operating bands for which NUL is not defined | – | – |
| >>>DL EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NDL in TS 36.104 [25] | – | – |
| >>>UL E-UTRA Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.2.2.22 | Same as DL Transmission Bandwidth in this release; ignored in case UL EARFCN value is ignored | – | – |
| >>>DL E-UTRA Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.2.2.22 |  | – | – |
| >TDD |  |  |  |  | – | – |
| >>TDD Info |  | 1 |  |  | – | – |
| >>>EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NDL/NUL in TS 36.104 [25] | – | – |
| >>>E-UTRA Transmission Bandwidth | M |  | 9.2.2.22 |  | – | – |
| >>>Subframe Assignment | M |  | ENUMERATED (sa0, sa1, sa2, sa3, sa4, sa5, sa6, ...) | Uplink-downlink subframe configuration information defined in TS 36.211 [26] | – | – |
| >>>Special Subframe Info |  | 1 |  | Special subframe configuration information defined in TS 36.211 [26] | – | – |
| >>>>Special Subframe Patterns | M |  | ENUMERATED (ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, ssp9, ssp10, ...) |  | – | – |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED (Normal, Extended,…) |  | – | – |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED (Normal, Extended, ...) |  | – | – |
| Number of Antenna Ports E-UTRA | O |  | 9.2.2.23 |  | – | – |
| PRACH Configuration | O |  | E-UTRA PRACH Configuration 9.2.2.25 |  | – | – |
| MBSFN Subframe Info |  | 0..<maxnoofMBSFN> |  | MBSFN subframe defined in TS 36.331 [14] | – | – |
| >Radioframe Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32, …) |  | – | – |
| >Radioframe Allocation Offset | M |  | INTEGER (0..7, ...) |  | – | – |
| >MBSFN Subframe Allocation E-UTRA | M |  | 9.2.2.26 |  | – | – |
| E-UTRA Multiband Info List | O |  | 9.2.2.24 |  | – | – |
| FreqBandIndicatorPriority | O |  | ENUMERATED (not-broadcast, broadcast, ...) | This IE indicates that the eNodeB supports FreqBandIndicationPriority, and whether FreqBandIndicatorPriority is broadcast in SIB 1 (see TS 36.331 [14]) | – | – |
| BandwidthReducedSI | O |  | ENUMERATED (scheduled, ...) | This IE indicates that the SystemInformationBlockType1-BR is scheduled in the cell (see TS 36.331 [14]) | – | – |
| Protected E-UTRA Resource Indication | O |  | 9.2.2.29 | This IE indicates which E-UTRA control/reference signal resources are protected and are not subject to E-UTRA - NR Cell Resource Coordination. | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. The value is 12. |
| maxnoofMBSFN | Maximum no. of MBSFN frame allocation with different offset. Value is 8. |

#### 9.2.2.13	Neighbour Information NR

This IE contains cell configuration information of NR cells that a neighbour NG-RAN node may need to properly operate its own served cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Neighbour Information NR |  | 1 .. <maxnoofNeighbours> |  |  | – | – |
| >NRPCI | M |  | INTEGER (0..1007) | NR Physical Cell ID | – | – |
| >NR-CGI | M |  | 9.2.2.7 |  | – | – |
| >TAC | M |  | 9.2.2.5 | Tracking Area Code | – | – |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  | – | – |
| >CHOICE NR-Mode-Info | M |  |  |  | – | – |
| >>FDD |  |  |  |  | – | – |
| >>>FDD Info |  | 1 |  |  | – | – |
| >>>>UL NR FreqInfo | M |  | NR Frequency Info 9.2.2.19 |  | – | – |
| >>>>DL NR FreqInfo | M |  | NR Frequency Info 9.2.2.19 |  | – | – |
| >>TDD |  |  |  |  | – | – |
| >>>TDD Info |  | 1 |  |  | – | – |
| >>>>NR FreqInfo | M |  | NR ARFCN Frequency Info 9.2.2.19 |  | – | – |
| >Connectivity Support | M |  | 9.2.2.28 |  | – | – |
| >Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the neighbour cell, as defined in TS 38.331 [10]. | – | – |
| > Broadcast PLMNs Enhanced |  | 0..<maxnoofBPLMNs> |  | Broadcast PLMNs. The PLMN Identities and associated information contained in this IE shall be provided in the same order as broadcast in SIB1 within the PLMN-IdentityInfoList IE defined in TS 38.331 [8]. | YES | ignore |
| >>PLMN Identity | M |  | 9.2.2.4 |  | – | – |
| >>NR CGI | O |  | 9.2.2.7 |  | – | – |
| >>TAC | O |  | 9.2.2.5 |  | – | – |
| >>RANAC | O |  | RAN Area Code 9.2.2.6 |  | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

#### 9.2.2.14	Neighbour Information E-UTRA

This IE contains cell configuration information of E-UTRA cells that a neighbour NG-RAN node may need to properly operate its own served cells.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Neighbour Information E-UTRA |  | 1 .. <maxnoofNeighbours> |  |  | – | – |
| >E-UTRA PCI | M |  | INTEGER (0..503, …) | E-UTRA Physical Cell Identifier of the neighbour cell | – | – |
| >ECGI | M |  | E-UTRA CGI 9.2.2.8 |  | – | – |
| >EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | DL EARFCN for FDD or EARFCN for TDD | – | – |
| >TAC | M |  | 9.2.2.5 | Tracking Area Code | – | – |
| >RANAC | O |  | RAN Area Code 9.2.2.6 |  | – | – |
| >Broadcast PLMNs EUTRA |  | 0..<maxnoofBPLMNs> |  | Broadcast PLMNs. The PLMN Identities and associated information contained in this IE shall be provided in the same order as broadcast in SIB1 within the cellAccessRelatedInfo-5GC-r15 IE defined in the SystemInformationBlockType1-v1530-IEs IE in TS 36.331 [14]. | YES | ignore |
| >>PLMN Identity | M |  | 9.2.2.4 |  | – | – |
| >>ECGI | O |  | E-UTRA CGI 9.2.2.8 |  | – | – |
| >>TAC | O |  | 9.2.2.5 |  | – | – |
| >>RANAC | O |  | RAN Area Code 9.2.2.6 |  | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given served cell. Value is 1024. |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-Additional-UL-NG-U-TNLatUPF-List,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-MaxIPrate-DL,

id-broadcastPLMNs-Enhanced,

id-broadcastPLMNs-EUTRA,

id-E-UTRA-CGI,

id-TAC,

id-RANAC,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFSets,

<<< skip unchanged ASN.1 >>>

#### -- B

BitRate	::= INTEGER (0..4000000000000,...)

BroadcastPLMNs ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF PLMN-Identity

BroadcastPLMNs-Enhanced ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BroadcastPLMNs-Enhanced-Item

BroadcastPLMNs-Enhanced-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

cellID				NR-CGI		OPTIONAL,

tac					TAC			OPTIONAL,

ranac				RANAC		OPTIONAL,

iE-Extension		ProtocolExtensionContainer { { BroadcastPLMNs-Enhanced-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNs-Enhanced-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastPLMNs-EUTRA ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF BroadcastPLMNs-EUTRA-Item

BroadcastPLMNs-EUTRA-Item ::= SEQUENCE {

pLMN-Identity		PLMN-Identity,

ECGI				E-UTRA-CGI	OPTIONAL,

tac					TAC			OPTIONAL,

ranac				RANAC		OPTIONAL,

iE-Extension		ProtocolExtensionContainer { { BroadcastPLMNs-EUTRA-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNs-EUTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

BroadcastPLMNinTAISupport-Item ::= SEQUENCE {

plmn-id							PLMN-Identity,

tAISliceSupport-List			SliceSupport-List,

iE-Extension					ProtocolExtensionContainer { {BroadcastPLMNinTAISupport-Item-ExtIEs} } OPTIONAL,

...

}

BroadcastPLMNinTAISupport-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

<<< skip unchanged ASN.1 >>>

#### -- N

NeighbourInformation-E-UTRA ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-E-UTRA-Item

NeighbourInformation-E-UTRA-Item ::= SEQUENCE {

e-utra-PCI			E-UTRAPCI,

e-utra-cgi			E-UTRA-CGI,

earfcn				E-UTRAARFCN,

tac					TAC,

ranac				RANAC																		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NeighbourInformation-E-UTRA-Item-ExtIEs} } 	OPTIONAL,

...

}

NeighbourInformation-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-broadcastPLMNs-EUTRA	CRITICALITY ignore	EXTENSION BroadcastPLMNs-EUTRA		PRESENCE optional },

...

}

NeighbourInformation-NR ::= SEQUENCE (SIZE(1..maxnoofNeighbours)) OF NeighbourInformation-NR-Item

NeighbourInformation-NR-Item ::= SEQUENCE {

nr-PCI								NRPCI,

nr-cgi								NR-CGI,

tac									TAC,

ranac								RANAC														OPTIONAL,

nr-mode-info						NeighbourInformation-NR-ModeInfo,

connectivitySupport					Connectivity-Support,

measurementTimingConfiguration		OCTET STRING,

iE-Extensions			ProtocolExtensionContainer { {NeighbourInformation-NR-Item-ExtIEs} } 	OPTIONAL,

...

}

NeighbourInformation-NR-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::={

{ ID id-broadcastPLMNs-Enhanced	CRITICALITY ignore	EXTENSION BroadcastPLMNs-Enhanced		PRESENCE optional },

...

}

<<< skip unchanged ASN.1 >>>

##### -- Served Cells E-UTRA IEs

ServedCellInformation-E-UTRA ::= SEQUENCE {

e-utra-pci								E-UTRAPCI,

e-utra-cgi								E-UTRA-CGI,

tac										TAC,

ranac									RANAC																					OPTIONAL,

broadcastPLMNs							SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedCellInformation-E-UTRA-perBPLMN,

e-utra-mode-info						ServedCellInformation-E-UTRA-ModeInfo,

numberofAntennaPorts					NumberOfAntennaPorts-E-UTRA																OPTIONAL,

prach-configuration						E-UTRAPRACHConfiguration																OPTIONAL,

mBSFNsubframeInfo						MBSFNSubframeInfo-E-UTRA																OPTIONAL,

multibandInfo							E-UTRAMultibandInfoList																	OPTIONAL,

freqBandIndicatorPriority				ENUMERATED {not-broadcast, broadcast, ...} 												OPTIONAL,

bandwidthReducedSI						ENUMERATED {scheduled, ...}																OPTIONAL,

protectedE-UTRAResourceIndication		ProtectedE-UTRAResourceIndication														OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-ExtIEs} }					OPTIONAL,

...

}

ServedCellInformation-E-UTRA-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

ServedCellInformation-E-UTRA-perBPLMN ::= SEQUENCE {

plmn-id					PLMN-Identity,

iE-Extensions			ProtocolExtensionContainer { {ServedCellInformation-E-UTRA-perBPLMN-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-E-UTRA-perBPLMN-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-E-UTRA-CGI 			CRITICALITY ignore  EXTENSION E-UTRA-CGI			PRESENCE optional    }|

{ ID id-TAC					CRITICALITY ignore  EXTENSION TAC       			PRESENCE optional    }|

{ ID id-RANAC       			CRITICALITY ignore  EXTENSION RANAC               	PRESENCE optional    },

...

}

<<< skip unchanged ASN.1 >>>

##### -- Served Cells NR IEs

ServedCellInformation-NR ::= SEQUENCE {

nrPCI								NRPCI,

cellID								NR-CGI,

tac									TAC,

ranac								RANAC						OPTIONAL,

broadcastPLMN						BroadcastPLMNs,

nrModeInfo							NRModeInfo,

measurementTimingConfiguration		OCTET STRING,

connectivitySupport					Connectivity-Support,

iE-Extensions						ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-broadcastPLMNs-Enhanced	CRITICALITY ignore	EXTENSION BroadcastPLMNs-Enhanced		PRESENCE optional },

...

}

### 9.3.7	Constant definitions

<<< skip unchanged ASN.1 >>>

id-PDUSession-List-withDataForwardingRequest														ProtocolIE-ID ::= 106

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 111

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 112

id-MaxIPrate-DL																						ProtocolIE-ID ::= 113

id-SecurityResult																					ProtocolIE-ID ::= 114

id-S-NSSAI																							ProtocolIE-ID ::= 115

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 116

id-broadcastPLMNs-Enhanced																			ProtocolIE-ID ::= xxx

id-broadcastPLMNs-EUTRA																				ProtocolIE-ID ::= xxx

id-E-UTRA-CGI																						ProtocolIE-ID ::= xxx

id-TAC																								ProtocolIE-ID ::= xxx

id-RANAC																							ProtocolIE-ID ::= xxx

END

-- ASN1STOP