3GPP TSG-RAN3 Meeting #104	R3-192942

Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0087 | rev |  | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Higher layer duplication for solution 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_IIOT |  |  |  |  |  | Date: |  |  | 2019-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS Rel-16 23. 501, to support the redundant URLLC communication, the support of redundant transmission on N3/N9 interfaces is introduced. Hence there is a need to capture the redundant transmission on N3/N9 interfaces into RAN specifications. Specifically, • Redundant indication for duplicated QoS flows; • Two NG UP tunnels are setup for PDU sessions with duplicated QoS flows • Redundant Network instance for the Redundant NG UP tunnel • Release indication of the redundant NG UP tunnel |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update bearer context setup and modification procedure to support redundant transmission between UPF and CP-UP by supporting two GTP-U tunnels for a single PDU Session. Impact assessment towards the previous version of the specification (same release): This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the bearer context setup/modifiy procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN specification is not aligned with SA2 specification to support the reduandant transmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.2, 8.3.2, 8.3.3, 9.2.2.1, 9.2.2.2, 9.2.2.4, 9.2.2.7, 9.3.1.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 0146 TS 38.423 CR 0141 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

<unchanged texts omitted>

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in [20].

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

For each PDU session, if the Redundant UL NG-U UP TNL Information is included in BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it as the uplink termination point for the user plane data for this PDU session for the redundant transmission as described in TS 23.501 [9] and it shall include the Redundant NG DL UP Transport Layer Information IE in the BEARER CONTEXT SETUP RESPONSE message.

For each PDU Session Resource, if the Redundant Network Instance IE is included in the PDU Session Resource To Setup List IE in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in [20].

For each PDU session, if the Redundant QoS Flow Information IE is included in the QoS Flow Level QoS Parameters IE in the QoS Flow QoS Parameters List IE of the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may consider it for the redundant transmission.

<unchanged texts omitted>

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

<unchanged texts omitted>

If the S-NSSAI IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store the corresponding information and replace any existing information.

If the Redundant UL NG-U UP TNL Information is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported,  include the Redundant NG DL UP Transport Layer Information IE in the PDU Session Resource Setup List IE or the PDU Session Resource Modified List IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

if the Redundant Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in [20].

If the Redundant Tunnel Release Indicator IE is included in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP  shall, if supported, stop the redundant transmission and release the redundant tunnel for the concerned PDU Session.

<unchanged texts omitted>

### 8.3.3	Bearer Context Modification Required (gNB-CU-UP initiated)

#### 8.3.3.1	General

The purpose of the Bearer Context Modification Required procedure is to allow the gNB-CU-UP to modify a bearer context (e.g., due to local problems) and inform the gNB-CU-CP. The procedure uses UE-associated signalling.

#### 8.3.3.2	Successful Operation

Figure 8.3.3.2-1: Bearer Context Modification Required procedure: Successful Operation.

The gNB-CU-UP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUIRED message to the gNB-CU-CP. The gNB-CU-CP replies with the BEARER CONTEXT MODIFICATION CONFIRM message.

If the S1 DL UP Transport Layer Information IE or the NG DL UP Transport Layer Information IE or Redundant NG DL UP Transport Layer Information IE is contained in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall update the corresponding information.

If the gNB-CU-UP Cell Group Related Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUIRED message, the gNB-CU-CP shall try to change the cell group related configuration accordingly. If the gNB-CU-CP is not able to update the requested cell group related configuration, it shall include the Cell Group Information IE with the current cell group configuration in the DRB Modified List IE in the BEARER CONTEXT MODIFICATION CONFIRM message.

----------------------------------------------------------Next Change--------------------------------------------------------

9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| Security Information | M |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | M |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Serving PLMN | M |  | PLMN Identity 9.3.1.7 |  | YES | ignore |
| Activity Notification Level | M |  | ENUMERATED (DRB, PDU Session, UE, …) |  | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates the status of the Bearer Context | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List |  | 1 |  |  | YES | reject |
| >>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>E-UTRAN QoS | M |  | 9.3.1.17 |  | - | - |
| >>>>S1 UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Request | O |  | 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>>Existing Allocated S1 DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List |  | 1 |  |  | YES | reject |
| >>>PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >>>>S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >>>>Security Indication | M |  | 9.3.1.23 |  | - | - |
| >>>>PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when at least one Non-GBR QoS Flows is being setup. | - | - |
| >>>>NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 |  | - | - |
| >>>>PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >>>>Existing Allocated NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Network Instance | O |  | 9.3.1.62 |  | - | - |
| >>>>DRB To Setup List |  | 1 |  |  | - | - |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>>>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>>>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>>>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>>PDCP SN Status Information | O |  | 9.3.1.58 | Contains the PDCP SN Status at setup after Resume. | - | - |
| >>>>Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs for the redundant transmission. | - | - |
| >>>>Redundant Network Instance | O |  | Network Instance 9.3.1.62 |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

9.2.2.2	BEARER CONTEXT SETUP RESPONSE

This message is sent by the gNB-CU-UP to confirm the setup of the requested bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Setup List |  | 1 |  |  | YES | reject |
| >>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>S1 DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Response | O |  | 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |
| >>>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>S1 DL UP Unchanged | O |  | ENUMERATED (True, …) |  | - | - |
| >>DRB Failed List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Setup List |  | 1 |  |  | YES | reject |
| >>>PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Security Result | O |  | 9.3.1.52 |  | - | - |
| >>>>NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |  |  |
| >>>>PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |
| >>>>NG DL UP Unchanged | O |  | ENUMERATED (True, …) |  | - | - |
| >>>>DRB Setup List |  | 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>DRB Data forwarding information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Providing forwarding info from the target gNB-CU-UP. | - | - |
| >>>>>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >>>>DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>>> Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 | Providing the endpoint of the NG-U transport bearer, for delivery of DL PDUs for the redundant transmission. | - | - |
| >>PDU Session Resource Failed List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource Failed Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

----------------------------------------------------------Next Change--------------------------------------------------------

#### 9.2.2.4	BEARER CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Security Information | O |  | 9.3.1.10 |  | YES | reject |
| UE DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | YES | reject |
| UE DL Maximum Integrity Protected Data Rate | O |  | Bit Rate 9.3.1.20 | The Bit Rate is a portion of the UE’s Maximum Integrity Protected Data Rate, and is enforced by the gNB-CU-UP node. | YES | reject |
| Bearer Context Status Change | O |  | ENUMERATED (Suspend, Resume, …) | Indicates the status of the Bearer Context | YES | reject |
| New UL TNL Information Required | O |  | ENUMERATED (required, …) | Indicates that new UL TNL information has been requested to be provided. | YES | reject |
| UE Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to UE. | - | - |
| Data Discard Required | O |  | ENUMERATED (required, …) | Indicate to discard the DL user data in case of RAN paging failure. | YES | ignore |
| CHOICE System | O |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>E-UTRAN QoS | M |  | 9.3.1.17 |  | - | - |
| >>>>S1 UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Request | O |  | 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry | - | - |
| >>>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>DRB To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>>E-UTRAN QoS | O |  | 9.3.1.17 |  | - | - |
| >>>>S1 UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Response | O |  | 9.3.2.6 | Providing forwarding info to the source gNB-CU-UP. | - | - |
| >>>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. | - | - |
| >>>>PDCP SN Status Information | O |  | 9.3.1.58 | Providing SN Status information to the target gNB-CU-UP. | - | - |
| >>>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Setup List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>PDU Session Type | M |  | 9.3.1.22 |  | - | - |
| >>>>S-NSSAI | M |  | 9.3.1.9 |  | - | - |
| >>>>Security Indication | M |  | 9.3.1.23 |  | - | - |
| >>>>PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 | This IE shall be present when Non-GBR QoS Flows are setting up. | - | - |
| >>>>NG UL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>>PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >>>>Network Instance | O |  | 9.3.1.62 |  | - | - |
| >>>>DRB To Setup List |  | 1 |  |  | - | - |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 | The gNB-CU-UP shall provide one UL UP Transport Layer Information Item per cell group entry. | - | - |
| >>>>>>QoS Flows Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>>>>DRB Data forwarding information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding info from the target gNB-CU-UP. | - | - |
| >>>>>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>>Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs for the redundant transmission. | - | - |
| >>>>Redundant Network Instance | O |  | Network Instance 9.3.1.62 |  |  |  |
| >>PDU Session Resource To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Security Indication | O |  | 9.3.1.23 |  | - | - |
| >>>>PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | - | - |
| >>>>NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >>>>PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >>>>PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >>>>Network Instance | O |  | 9.3.1.62 |  |  |  |
| >>>>DRB To Setup List |  | 0..1 |  |  | - | - |
| >>>>>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>>>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>>>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>>>>Flow Mapping Information | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>>>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >>>>>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>>DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>SDAP Configuration | O |  | 9.3.1.39 |  | - | - |
| >>>>>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>>>>DRB Data forwarding information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >>>>>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. | - | - |
| >>>>>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. | - | - |
| >>>>>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>>>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>>>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>>>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. | - | - |
| >>>>>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>>DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>S-NSSAI | O |  | 9.3.1.9 |  | YES | reject |
| >>>>Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs for the redundant transmission. | - | - |
| >>>>Redundant Network Instance | O |  | Network Instance 9.3.1.62 |  |  |  |
| >>>>Redundant Tunnel Release Indicator | O |  | ENUMERATED (true, …) | Indicates to release the redundant NG-U tunnel. |  |  |
| >>PDU Session Resource To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Remove Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

#### 9.2.2.5	BEARER CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | O |  |  |  | YES | ignore |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Setup List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>S1 DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Response | O |  | 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >>>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>DRB Modified List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>S1 DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status from the source gNB-CU-UP. | - | - |
| >>>>UL UP Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters for newly added cell groups. | - | - |
| >>DRB Failed To Modify List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Setup List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Security Result | O |  | 9.3.1.52 |  | - | - |
| >>>>NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |  |  |
| >>>>PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >>>>DRB Setup List |  | 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>DRB Data forwarding information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >>>>>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >>>>DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>>>Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 | Provides the endpoint of the NG-U transport bearer, for delivery of DL PDUs for the redundant transmission. |  |  |
| >>PDU Session Resource Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Failed Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>PDU Session Resource Modified List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Modified Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Security Result | O |  | 9.3.1.52 |  | - | - |
| >>>>NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information Response 9.3.2.6 |  | - | - |
| >>>>DRB Setup List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>DRB Data forwarding information Response | O |  | Data Forwarding Information Response 9.3.2.6 |  | - | - |
| >>>>>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >>>>DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>>>DRB Modified List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides PDCP SN Status to the target gNB-CU-UP. |  |  |
| >>>>>>UL UP Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters for newly added cell groups. | - | - |
| >>>>>>Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >>>>DRB Failed To Modify List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>>>Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 | Provides the endpoint of the NG-U transport bearer, for delivery of DL PDUs for the redundant transmission | - | - |
| >>PDU Session Resource Failed To Modify List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Failed To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

----------------------------------------------------------Next Change--------------------------------------------------------

9.2.2.7	BEARER CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-CU-UP to inform the gNB-CU-CP that a modification of a bearer context is required (e.g., due to local problems at the gNB-CU-UP).

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | M |  |  |  | YES | reject |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>S1 DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>gNB-CU-UP Cell Group Related Configuration | O |  | 9.3.1.34 |  | - | - |
| >>>>Cause | O |  | 9.3.1.2 |  | - | - |
| >>DRB To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource To Modify List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>gNB-CU-UP Cell Group Related Configuration | O |  | 9.3.1.34 |  | - | - |
| >>>>>>Flow To Remove | O |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Cause | O |  | 9.3.1.2 |  | - | - |
| >>>>DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>>>Redundant NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>PDU Session Resource To Remove List |  | 0.. 1 |  |  | YES | reject |
| >>>PDU Session Resource To Remove Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

9.3.1.26	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  |
| >Non-dynamic 5QI |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.27 |  |
| >Dynamic 5QI |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.29 |  |
| GBR QoS Flow Information | O |  | 9.3.1.30 | This IE shall be present for GBR QoS Flows only. |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, …) | Details in TS 23.501 [20]. This IE applies to Non-GBR flows only and shall be ignored otherwise. |
| Additional QoS Flow Information | O |  | ENUMERATED (more likely, …) | This IE indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU Session. |
| RDI | O |  | ENUMERATED (enabled, …) | Indicates whether Reflective QoS flow to DRB mapping should be applied. |
| Redundant QoS Flow Information | O |  | ENUMERATED (true, …) | This IE indicates that this QoS flow is requested for the redundant transmission. |