3GPP TSG-RAN WG3 Meeting #104	R3-193084

Reno, Nevada, US, 13 - 17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0064 | rev |  | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.401 on in-order delivery for RAN initiated PDU Session Split |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For PDU Session Split at UPF (RAN initiated), the TS 37.340 states that "If in-order delivery is required, the SN buffers the first packets received from the UE until it receives end marker packets over the UL forwarding tunnel indicating that the MN has delivered all UL packets from the source side to UPF.". The action of SN node for PDU Session Split when in-order delivery is required needs to be clarified in TS 38.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Classify the action of SN-gNB-CU-UP in PDU Session Split scenarios when in-order delivery is required. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because this only classifies the action of SN-gNB-CU-UP in PDU Session Split scenarios when in-order delivery is required, This CR has an impact under functional point of view. The impact can be considered isolated because the change affects PDU Session Split scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect SN-gNB-CU-UP actions in the PDU Session Split scenarios when in-order delivery is required. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

8.9.2	Bearer context setup over F1-U

Figure 8.9.2-1 shows the procedure used to setup the bearer context in the gNB-CU-UP.

Figure 8.9.2-1: Bearer context setup over F1-U

0.	Bearer context setup (e.g., following an SGNB ADDITION REQUEST message from the MeNB) is triggered in gNB-CU-CP.

1.	The gNB-CU-CP sends a BEARER CONTEXT SETUP REQUEST message containing UL TNL address information for S1-U or NG-U, and if required, DL TNL address information for X2-U or Xn-U to setup the bearer context in the gNB-CU-UP. For NG-RAN, the gNB-CU-CP decides flow-to-DRB mapping and sends the generated SDAP and PDCP configuration to the gNB-CU-UP.

2.	The gNB-CU-UP responds with a BEARER CONTEXT SETUP RESPONSE message containing the UL TNL address information for F1-U, and DL TNL address information for S1-U or NG-U, and if required, UL TNL address information for X2-U or Xn-U.

NOTE:	The indirect data transmission for split bearer through the gNB-CU-UP is not precluded.

3.	F1 UE context setup procedure is performed to setup one or more bearers in the gNB-DU.

4.	The gNB-CU-CP sends a BEARER CONTEXT MODIFICATION REQUEST message containing the DL TNL address information for F1-U and PDCP status.

5.	The gNB-CU-UP responds with a BEARER CONTEXT MODIFICATION RESPONSE message.

NOTE: For PDU Session Split at UPF, if in-order delivery is required, the SN-gNB-CU-UP buffers the first packets received from the UE until it receives end marker packets over the UL forwarding tunnel indicating that the MN has delivered all UL packets from the source side to UPF.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>