3GPP TSG-RAN WG3 Meeting #104	R3-193201

# Reno, NV , USA, 13-17 May 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0096 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Retainability measurements for DRBs and QoS flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_16 |  |  |  |  |  | Date: |  |  | 2019-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN3 received an LS from SA5 in R3-192503, asking RAN3 to support measurements for DRB and QoS Flow retainability in case of 3-split scenario. The gNB-CU-CP needs to report if the DRB or the QoS flow was in session (active) while released: “For DRBs with bursty flow, a DRB is said to be active if any data (UL or DL) has been transferred during the last 100 ms”. The accumulated session time for each DRB or QoS Flow needs also to be reported in case of release. However, these informations can only be retrieved by the gNB-CU-UP, while the measurement itself is gNB-CU-CP related. Therefore, the gNB-CU-UP needs to inform the gNB-CU-CP if the released DRB or QoS Flow was in session, together with the accumulated session time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Retainability Measurements Information IE in BEARER CONTEXT MODIFICATION RESPONSE and BEARER CONTEXT RELEASE COMPLETE messages |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurements for DRB and QoS Flow retainability are not fully supported in case of 3-split scenario |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8.3.2.2, 8.3.4.2, 9.2.2.5, 9.2.2.10, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< 1st Change >>>>>>>>>>>>>>>>>>>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.401: "NG-RAN; Architecture Description".

[3]	3GPP TS 38.460: "NG-RAN; E1 general aspects and principles".

[4]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[5]	3GPP TR 25.921 (version.7.0.0): "Guidelines and principles for protocol description and error".

[6]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[7]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[8]	ITU-T Recommendation X.680 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[9]	ITU-T Recommendation X.681 (07/2002): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specificaiton”.

[11]	3GPP TS 23.401: “General Packet Radio Service (GPRS) Enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access”.

[12]	3GPP TS 23.203: “Policy and Charging Control Architecture”.

[13]	3GPP TS 33.501: “Security Architecture and Procedures for 5G System”.

[14]	IETF RFC 5905: “Network Time Protocol Version 4: Protocol and Algorithms Specification”.

[15]	3GPP TS 29.281: “General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U)”.

[16]	3GPP TS 38.414: “NG-RAN; NG Data Transport”.

[17]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[18]	3GPP TS 38.462: "NG-RAN; E1 Signalling Transport".

[19]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[20]	3GPP TS 23.501: "System Architecture for the 5G System".

[21]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

[x]	3GPP TS 32.425: “Performance measurements; Evolved Universal Terrestrial Radio Access Network (E-UTRAN)”.

[y]	3GPP TS 28.552: “Management and orchestration; 5G performance measurements”

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.5	BEARER CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-CU-UP to confirm the modification of the requested bearer context.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| CHOICE System | O |  |  |  | YES | ignore |
| >E-UTRAN |  |  |  |  |  |  |
| >>DRB Setup List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>S1 DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>Data Forwarding Information Response | O |  | 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >>>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>DRB Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>DRB Modified List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>S1 DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status from the source gNB-CU-UP. | - | - |
| >>>>UL UP Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters for newly added cell groups. | - | - |
| >>DRB Failed To Modify List |  | 0.. 1 |  |  | YES | ignore |
| >>>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>Retainability Measurements Information | O |  | 9.3.1.x | Provides information on all the removed DRB(s), needed for retainability measurements in the gNB-CU-CP | YES | ignore |
| >NG-RAN |  |  |  |  |  |  |
| >>PDU Session Resource Setup List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Setup Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Security Result | O |  | 9.3.1.52 |  | - | - |
| >>>>NG DL UP Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |  |  |
| >>>>PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >>>>DRB Setup List |  | 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>DRB Data forwarding information Response | O |  | Data Forwarding Information Response 9.3.2.6 | Provides forwarding information from the target gNB-CU-UP. | - | - |
| >>>>>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >>>>DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>PDU Session Resource Failed List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Failed Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>PDU Session Resource Modified List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Modified Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Security Result | O |  | 9.3.1.52 |  | - | - |
| >>>>NG DL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>>PDU Session Data Forwarding Information Response | O |  | Data Forwarding Information Response 9.3.2.6 |  | - | - |
| >>>>DRB Setup List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>DRB Data forwarding information Response | O |  | Data Forwarding Information Response 9.3.2.6 |  | - | - |
| >>>>>>UL UP Parameters | M |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>>>>Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >>>>DRB Failed List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Failed Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>>>DRB Modified List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Modified Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides PDCP SN Status to the target gNB-CU-UP. |  |  |
| >>>>>>UL UP Parameters | O |  | UP Parameters 9.3.1.13 | Carries the UL UP parameters for newly added cell groups. | - | - |
| >>>>>>Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  | - | - |
| >>>>>>Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  | - | - |
| >>>>DRB Failed To Modify List |  | 0.. 1 |  |  | - | - |
| >>>>>DRB Failed To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>>>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>PDU Session Resource Failed To Modify List |  | 0.. 1 |  |  | YES | ignore |
| >>>PDU Session Resource Failed To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >>>>PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >>>>Cause | M |  | 9.3.1.2 |  | - | - |
| >>Retainability Measurements Information | O |  | 9.3.1.x | Provides information on all the removed DRB(s) and QoS Flow(s), needed for retainability measurements in the gNB-CU-CP | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |

<<<<<<<<<<<<<<<<<<<< End of 2nd Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 3rd Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.10	BEARER CONTEXT RELEASE COMPLETE

This message is sent by the gNB-CU-UP to confirm the release of the UE-associated logical E1 connection.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-CU-UP UE E1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| Retainability Measurements Information | O |  | 9.3.1.x | Provides information on all the removed DRB(s) and QoS Flow(s), needed for retainability measurements in the gNB-CU-CP | YES | ignore |

<<<<<<<<<<<<<<<<<<<< End of 3rd Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 4th Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	Retainability Measurements Information

This IE contains information on removed DRB(s) and QoS Flow(s) which are needed to perform retainability measurements.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| DRB Removed List |  | 1 |  |  | - |  |
| >DRB Removed Item |  | 1..<maxnoofDRBs> |  |  | - |  |
| >>DRB ID | M |  | 9.3.1.16 |  | - |  |
| >>DRB Released In Session | O |  | ENUMERATED (released in session, not released in session, …) | Indicates if the DRB was “in session” or not (as defined in TS 32.425 [x]) when released | - |  |
| >>DRB Accumulated Session Time | O |  | OCTET STRING (5) | Accumulated “in session” time for the DRB, as defined in TS 32.425 [x], in milliseconds | - |  |
| >>QoS Flow Removed List |  | 0..1 |  |  | - |  |
| >>>QoS Flow Removed Item |  | 1..<maxnoofflows> |  |  | - |  |
| >>>>QoS Flow Identifier | M |  | 9.3.1.24 |  | - |  |
| >>>>QoS Flow Released In Session | O |  | ENUMERATED (released in session, not released in session, …) | Indicates if the QoS Flow was “in session” or not (as defined in TS 28.552 [y]), when released | - |  |
| >>>>QoS Flow Accumulated Session Time | O |  | OCTET STRING (5) | Accumulated “in session” time for the QoS Flow, as defined in TS 28.552 [y], in milliseconds | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofQoSFlows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< End of 4th Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 5th Change >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

#### -- PDU definitions for E1AP

--

-- **************************************************************

E1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules

--

-- **************************************************************

IMPORTS

Cause,

CriticalityDiagnostics,

GNB-CU-CP-UE-E1AP-ID,

GNB-CU-UP-UE-E1AP-ID,

UE-associatedLogicalE1-ConnectionItem,

GNB-CU-UP-ID,

GNB-CU-UP-Name,

GNB-CU-CP-Name,

CNSupport,

PLMN-Identity,

Slice-Support-List,

NR-CGI-Support-List,

QoS-Parameters-Support-List,

SecurityInformation,

BitRate,

BearerContextStatusChange,

DRB-To-Setup-List-EUTRAN,

DRB-Setup-List-EUTRAN,

DRB-Failed-List-EUTRAN,

DRB-To-Modify-List-EUTRAN,

DRB-Modified-List-EUTRAN,

DRB-Failed-To-Modify-List-EUTRAN,

DRB-To-Remove-List-EUTRAN,

DRB-Required-To-Remove-List-EUTRAN,

DRB-Required-To-Modify-List-EUTRAN,

DRB-Confirm-Modified-List-EUTRAN,

DRB-To-Setup-Mod-List-EUTRAN,

DRB-Setup-Mod-List-EUTRAN,

DRB-Failed-Mod-List-EUTRAN,

PDU-Session-Resource-To-Setup-List,

PDU-Session-Resource-Setup-List,

PDU-Session-Resource-Failed-List,

PDU-Session-Resource-To-Modify-List,

PDU-Session-Resource-Modified-List,

PDU-Session-Resource-Failed-To-Modify-List,

PDU-Session-Resource-To-Remove-List,

PDU-Session-Resource-Required-To-Modify-List,

PDU-Session-Resource-Confirm-Modified-List,

PDU-Session-Resource-To-Setup-Mod-List,

PDU-Session-Resource-Setup-Mod-List,

PDU-Session-Resource-Failed-Mod-List,

PDU-Session-To-Notify-List,

DRB-Status-Item,

DRB-Activity-Item,

Data-Usage-Report-List,

TimeToWait,

ActivityNotificationLevel,

ActivityInformation,

New-UL-TNL-Information-Required,

GNB-CU-CP-TNLA-Setup-Item,

GNB-CU-CP-TNLA-Failed-To-Setup-Item,

GNB-CU-CP-TNLA-To-Add-Item,

GNB-CU-CP-TNLA-To-Remove-Item,

GNB-CU-CP-TNLA-To-Update-Item,

TransactionID,

Inactivity-Timer,

DRBs-Subject-To-Counter-Check-List-EUTRAN,

DRBs-Subject-To-Counter-Check-List-NG-RAN,

PPI,

GNB-CU-UP-Capacity,

GNB-CU-UP-OverloadInformation,

DataDiscardRequired,

PDU-Session-Resource-Data-Usage-List,

RetainabilityMeasurementsInfo

FROM E1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-SingleContainer{},

E1AP-PRIVATE-IES,

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers

id-Cause,

id-CriticalityDiagnostics,

id-gNB-CU-CP-UE-E1AP-ID,

id-gNB-CU-UP-UE-E1AP-ID,

id-ResetType,

id-UE-associatedLogicalE1-ConnectionItem,

id-UE-associatedLogicalE1-ConnectionListResAck,

id-gNB-CU-UP-ID,

id-gNB-CU-UP-Name,

id-gNB-CU-CP-Name,

id-CNSupport,

id-SupportedPLMNs,

id-SecurityInformation,

id-UEDLAggregateMaximumBitRate,

id-BearerContextStatusChange,

id-System-BearerContextSetupRequest,

id-System-BearerContextSetupResponse,

id-System-BearerContextModificationRequest,

id-System-BearerContextModificationResponse,

id-System-BearerContextModificationConfirm,

id-System-BearerContextModificationRequired,

id-DRB-Status-List,

id-Data-Usage-Report-List,

id-TimeToWait,

id-ActivityNotificationLevel,

id-ActivityInformation,

id-New-UL-TNL-Information-Required,

id-GNB-CU-CP-TNLA-Setup-List,

id-GNB-CU-CP-TNLA-Failed-To-Setup-List,

id-GNB-CU-CP-TNLA-To-Add-List,

id-GNB-CU-CP-TNLA-To-Remove-List,

id-GNB-CU-CP-TNLA-To-Update-List,

id-DRB-To-Setup-List-EUTRAN,

id-DRB-To-Modify-List-EUTRAN,

id-DRB-To-Remove-List-EUTRAN,

id-DRB-Required-To-Modify-List-EUTRAN,

id-DRB-Required-To-Remove-List-EUTRAN,

id-DRB-Setup-List-EUTRAN,

id-DRB-Failed-List-EUTRAN,

id-DRB-Modified-List-EUTRAN,

id-DRB-Failed-To-Modify-List-EUTRAN,

id-DRB-Confirm-Modified-List-EUTRAN,

id-DRB-To-Setup-Mod-List-EUTRAN,

id-DRB-Setup-Mod-List-EUTRAN,

id-DRB-Failed-Mod-List-EUTRAN,

id-PDU-Session-Resource-To-Setup-List,

id-PDU-Session-Resource-To-Modify-List,

id-PDU-Session-Resource-To-Remove-List,

id-PDU-Session-Resource-Required-To-Modify-List,

id-PDU-Session-Resource-Setup-List,

id-PDU-Session-Resource-Failed-List,

id-PDU-Session-Resource-Modified-List,

id-PDU-Session-Resource-Failed-To-Modify-List,

id-PDU-Session-Resource-Confirm-Modified-List,

id-PDU-Session-Resource-Setup-Mod-List,

id-PDU-Session-Resource-Failed-Mod-List,

id-PDU-Session-Resource-To-Setup-Mod-List,

id-PDU-Session-To-Notify-List,

id-TransactionID,

id-Serving-PLMN,

id-UE-Inactivity-Timer,

id-System-GNB-CU-UP-CounterCheckRequest,

id-DRBs-Subject-To-Counter-Check-List-EUTRAN,

id-DRBs-Subject-To-Counter-Check-List-NG-RAN,

id-PPI,

id-gNB-CU-UP-Capacity,

id-GNB-CU-UP-OverloadInformation,

id-UEDLMaximumIntegrityProtectedDataRate,

id-DataDiscardRequired,

id-PDU-Session-Resource-Data-Usage-List,

id-RetainabilityMeasurementsInfo,

maxnoofErrors,

maxnoofSPLMNs,

maxnoofDRBs,

maxnoofTNLAssociations,

maxnoofIndividualE1ConnectionsToReset

FROM E1AP-Constants;

-- TEXT OMITTED –

-- **************************************************************

--

-- Bearer Context Modification Response

--

-- **************************************************************

BearerContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextModificationResponseIEs} },

...

}

BearerContextModificationResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID								PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID						CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID								PRESENCE mandatory }|

{ ID id-System-BearerContextModificationResponse	CRITICALITY ignore	TYPE System-BearerContextModificationResponse		PRESENCE optional  },

...

}

System-BearerContextModificationResponse	::=	CHOICE {

e-UTRAN-BearerContextModificationResponse			ProtocolIE-Container {{EUTRAN-BearerContextModificationResponse}},

nG-RAN-BearerContextModificationResponse			ProtocolIE-Container {{NG-RAN-BearerContextModificationResponse}},

choice-extension									ProtocolIE-SingleContainer {{System-BearerContextModificationResponse-ExtIEs}}

}

System-BearerContextModificationResponse-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

EUTRAN-BearerContextModificationResponse E1AP-PROTOCOL-IES ::= {

{ ID id-DRB-Setup-Mod-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Setup-Mod-List-EUTRAN					PRESENCE optional }|

{ ID id-DRB-Failed-Mod-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Failed-Mod-List-EUTRAN				PRESENCE optional }|

{ ID id-DRB-Modified-List-EUTRAN				CRITICALITY ignore	TYPE DRB-Modified-List-EUTRAN					PRESENCE optional }|

{ ID id-DRB-Failed-To-Modify-List-EUTRAN		CRITICALITY ignore	TYPE DRB-Failed-To-Modify-List-EUTRAN		PRESENCE optional }|

{ ID id-RetainabilityMeasurementsInfo			CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo				PRESENCE optional },

...

}

NG-RAN-BearerContextModificationResponse E1AP-PROTOCOL-IES ::= {

{ ID id-PDU-Session-Resource-Setup-Mod-List 		CRITICALITY reject	TYPE PDU-Session-Resource-Setup-Mod-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Failed-Mod-List		CRITICALITY reject	TYPE PDU-Session-Resource-Failed-Mod-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Modified-List			CRITICALITY reject	TYPE PDU-Session-Resource-Modified-List				PRESENCE optional }|

{ ID id-PDU-Session-Resource-Failed-To-Modify-List	CRITICALITY reject	TYPE PDU-Session-Resource-Failed-To-Modify-List		PRESENCE optional }|

{ ID id-RetainabilityMeasurementsInfo				CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo						PRESENCE optional },

...

}

-- TEXT OMITTED –

-- **************************************************************

--

-- Bearer Context Release Complete

--

-- **************************************************************

BearerContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BearerContextReleaseCompleteIEs} },

...

}

BearerContextReleaseCompleteIEs E1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-CP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-CP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-gNB-CU-UP-UE-E1AP-ID		CRITICALITY reject	TYPE GNB-CU-UP-UE-E1AP-ID			PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	}|

{ ID id-RetainabilityMeasurementsInfo	CRITICALITY ignore	TYPE RetainabilityMeasurementsInfo	PRESENCE optional },

...

}

-- TEXT OMITTED –

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TEXT OMITTED –

#### DRB-Removed-Item	::=	SEQUENCE {

dRB-ID									DRB-ID,

dRB-Released-In-Session					ENUMERATED {released-in-session, not-released-in-session, …}		OPTIONAL,

dRB-Accumulated-Session-Time			OCTET STRING (SIZE(5))													OPTIONAL,

qoS-Flow-Removed-List					SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Removed-Item		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-Removed-Item-ExtIEs } }			OPTIONAL,

...

}

DRB-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

#### }

-- TEXT OMITTED –

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

additional-QoS-Information				ENUMERATED {more-likely, ...}				OPTIONAL,

paging-Policy-Indicator					INTEGER	(1..8,	...)						OPTIONAL,

reflective-QoS-Indicator				ENUMERATED {enabled, ...}					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

...

}

#### QoS-Flow-Removed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoS-Flow-Released-In-Session			ENUMERATED {released-in-session, not-released-in-session, …}			OPTIONAL,

qoS-Flow-Accumulated-Session-Time		OCTET STRING (SIZE(5))														OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoS-Flow-Removed-Item-ExtIEs } }		OPTIONAL,

...

}

QoS-Flow-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

#### }

#### -- R

RAT-Type	::=	ENUMERATED	{

e-UTRA,

nR,

...

}

RetainabilityMeasurementsInfo	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-Removed-Item

-- TEXT OMITTED –

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired										ProtocolIE-ID ::= 70

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= xx

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>