3GPP TSG-RAN WG3 #105							                                                     R3-193522

# Ljubljana, Slovenia, 26th –30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | <CR#> | rev | <Rev#> | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR38423 for Providing TSC Assistance Information in XnAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIoT |  |  |  |  |  | Date: |  |  | 2019-08-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As specified in TS 23.501, The knowledge of TSN traffic pattern is useful for the gNB to allow it to more efficiently schedule periodic. To support this, TSC Assistance Information, Maximum Data Burst Volume Downlink and Maximum Data Burst Volume Uplink should be provided to the target NG-RAN via Xn. Furthermore, for gNB to decide whether the service is TSC with delay deterministic and whether to deliver the accurate reference timing to UE, the TSC Assistance Information should be transferred to the target NG-RAN via Xn. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of TSC Assistance Information including: - Add TSC Assistance Information in QoS Flow Level QoS Parameters IE. - Add Periodicity Downlink, Periodicity Uplink, Burst Arrival Time Downlink, Burst Arrival Time Uplink, Maximum Data Burst Volume Downlink and Maximum Data Burst Volume Uplink in TSC Assistance Information IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TSC deterministic QoS is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.5, 9.2.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR XXXX TS 38.463 CR XXXX TS 38.473 CR XXXX |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.2.3.5	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  |
| >Non Dynamic 5QI |  |  |  |  |
| >>Non dynamic 5QI Descriptor | M |  | 9.2.3.8 |  |
| >Dynamic 5QI |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.2.3.9 |  |
| Allocation and Retention Priority | M |  | 9.2.3.7 |  |
| GBR QoS Flow Information | O |  | 9.2.3.6 | This IE shall be present for GBR QoS Flows only. |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Reflective QoS is specified in TS 23.501 [7]. This IE applies to Non-GBR bearers only and shall be ignored otherwise. |
| Additional QoS flow Information | O |  | ENUMERATED (more likely, …) | If this IE is set to "more likely", this indicates that traffic for this QoS flow is likely to appear more often than traffic for other flows established for the PDU session. This IE may be present in case of Non-GBR flows only and shall be ignored otherwise. |
| TSC Assistance Information | O |  | 9.2.3.x | Details in TS 23.501 [9]. This IE shall be present for TSC traffic only. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.2.3.x TSC Assistance Information

This IE describes TSC traffic characteristics associated with the QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity Downlink | O |  | [FFS] | Periodicity in DL for TSC traffic. Details in TS 23.501 [7]. |
| Periodicity Uplink | O |  | [FFS] | Periodicity in UL for TSC traffic. Details in TS 23.501 [7]. |
| Burst Arrival Time Downlink | C-ifMDBVDL |  | [FFS] | Burst Arrival Time in DL for Periodic deterministic TSC traffic. Details in TS 23.501 [7]. |
| Burst Arrival Time Uplink | C-ifMDBVUL |  | [FFS] | Burst Arrival Time in UL for Periodic deterministic TSC traffic. Details in TS 23.501 [7]. |
| Maximum Data Burst Volume Downlink | O |  | 9.2.3.15 | Maximum Data Burst Volume Downlink is specified in TS 23.501 [7]. This IE shall be included for downlink traffic pattern deterministic TSC flow if the Delay Critical IE is set to “delay critical”. Otherwise, it is absent. The Maximum Data Burst Volume IE in Dynamic 5QI Descriptor and Non Dynamic 5QI Descriptor shall be ignored if this IE is included. |
| Maximum Data Burst Volume Uplink | O |  | 9.2.3.15 | Maximum Data Burst Volume Uplink is specified in TS 23.501 [7]. This IE shall be included for uplink traffic pattern deterministic TSC flow if the Delay Critical IE is set to “delay critical”. Otherwise, it is absent. The Maximum Data Burst Volume IE in Dynamic 5QI Descriptor and Non Dynamic 5QI Descriptor shall be ignored if this IE is included. |

| Condition | Explanation |
| --- | --- |
| C-ifMDBVDL | This IE can be present if the Maximum Data Burst Volume Downlink is present. Otherwise, it is absent. |
| C-ifMDBVUL | This IE should be present if the Maximum Data Burst Volume Uplink is present. Otherwise, it is absent. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>