3GPP TSG-RAN WG3 #105		R3-193679

Ljubljana, Slovenia, 26th – 30th August 2019

Source:	CATT

Title:	Discussion on PRACH coordination

Agenda Item:	10.2.3

Document for:	Discussion and decision

Introduction

In recent RAN WG3 meetings, most companies agreed that RACH optimisation should at least follow the current LTE method, including that neighbour RAN nodes may exchange their own PRACH configuration respectively in order to mitigate interference. As specified in TS 38.211, however, the PRACH configuration in NR does not exactly follow the same mechanism as in E-UTRA, which makes it improper or even impossible to copy the E-UTRA IEs into NR-related specifications. In this contribution we analyse what adaption should (or could) be performed for NR.

Discussion

Multiple sets of PRACH configuration per cell

At first we would analyse how many sets of PRACH configuration need to be exchanged between RAN nodes.

In E-UTRA, RA resource is usually completely in a per-cell granularity, which means all UEs connecting to a given E-UTRA cell share the same PRACH configuration. This makes the eNB only need to monitor one set of RA resource. As a result, one eNB only needs to inform its neighbour about one set of PRACH configuration currently in use:

### 9.2.8 Served Cell Information This IE contains cell configuration information of a cell that a neighbour eNB may need for the X2 AP interface. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. ///////////// Skip irrelevant fields //////////////. PRACH Configuration; O; PRACH Configuration 9.2.50; YES; ignore. ///////////// Skip irrelevant fields //////////////. ### 9.2.50 PRACH Configuration This IE indicates the PRACH resources used in neighbor cell. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. RootSequenceIndex; M; INTEGER (0..837); See section 5.7.2. in TS 36.211 [10]; –; –. ZeroCorrelationZoneConfiguration; M; INTEGER (0..15); See section 5.7.2. in TS 36.211 [10]; –; –. HighSpeedFlag; M; BOOLEAN; TRUE corresponds to Restricted set and FALSE to Unrestricted set. See section 5.7.2 in TS 36.211 [10]; –; –. PRACH-FrequencyOffset; M; INTEGER (0..94); See section 5.7.1 of TS 36.211 [10]; –; –. PRACH-ConfigurationIndex; O; INTEGER (0..63); Mandatory for TDD, shall not be present for FDD. See section 5.7.1. in TS 36.211 [10]; –; –.

In NR, however, the introduction of some new features (especially BWP and BFR) makes it quite common to configure multiple sets of RA resource per cell. Even though these “additional” sets of PRACH resource can only be provided in per-UE RRC messages in various forms (relative to carrier and BWP configuration), for efficiency and ease of management, one cell tends to reserve only a few semi-static blocks of time-frequency resource dedicated for RA, and determines their formats respectively prior to configuring them to UEs. It would be beneficial if we allow a gNB to exchange not only the set of PRACH configuration it broadcasts but also the “candidate” sets of PRACH configuration potentially used in per-UE dedicated configuration.

Proposal 1: One gNB may inform its neighbour about multiple sets of PRACH configuration currently in use or reserved as candidates for dedicated signalling.

Content of each set of PRACH configuration

Now we would discuss the content of each set. Before focusing the detail, we would review what we did in LTE when working on Rel-9 SON.

In RAN2#67bis meeting, RAN2 determined what PRACH parameters should be exchanged between eNBs. Two types of parameters were identified according to their usage:

- Preamble sequence generation: rootSequenceIndex, zeroCorrelationZoneConfiguration, and highSpeedFlag [1].

- Frequency resource to send RA MSG1: prach-FreqOffset for FDD cells, while both prach-FreqOffset and prach-ConfigIndex for TDD cells [2].

Although (Rel-9) TS 38.211 also specified the time resource to send RA preambles (the starting point and the length as well), RAN2 did not figure out the benefit to coordinate it between eNBs. Following we would analyse these two types separately.

Preamble sequence generation

The method of preamble sequence generation does not change much. The major difference comes from the RRC layer.

In (Rel-9) E-UTRA, three types of physical preamble generation method are defined: “unrestricted set”, “restricted set type A” and “restricted set type B”. Which method should be used by a UE to generated its preamble sequence should be selected according to whether the UE should use PRACH preamble Set 1 or PRACH preamble Set 2, as well as whether the RRC Boolean field highSpeedFlag is configured as “true” or “false”:

|  | highSpeedFlag = false | highSpeedFlag = true |
| --- | --- | --- |
| PRACH preamble Set 1 | unrestricted set | restricted set type A |
| PRACH preamble Set 2 | restricted set type B | restricted set type B |

In NR, these three types of restriction set in E-UTRA are still followed, while the indication is simplified. Now it is indicated by the RRC enumerated field restrictedSetConfig directly (there is no longer any highSpeedFlag field in NR):

RACH-ConfigCommon ::=        SEQUENCE {

///////////////////////////////////irrelevant fields omitted///////////////////////////////////////

restrictedSetConfig      ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB},

///////////////////////////////////irrelevant fields omitted///////////////////////////////////////

...

}

Therefore we could simply exchange the field restrictedSetConfig instead.

In addition, restricted sets are only applicable for the case of ΔfRA=1.25kHz or 5kHz (i.e. LRA=839 as of Table 6.3.3.2-1 of TS 38.211) as indicated in Table 6.3.3.1-1 and Table 6.3.3.1-2 of TS 38.211. Therefore it is not needed to exchange restrictedSetConfig for the case of LRA=139.

Proposal 2: The following 3 fields should be included within each set of PRACH configuration: prach-RootSequenceIndex, zeroCorrelationZoneConfig, for any case, and restrictedSetConfig for the case of LRA=839.

Frequency to use

The method to indicate the frequency to send random access MSG1 does differ much from (Rel-9) E-UTRA. In (Rel-9) E-UTRA there is only one band per cell so that the frequency can be clearly identified by a PRB offset. In NR, however, the frequency is deduced by a much more complex method as specified in §5.3.2 of TS 38.211:

The time-continuous signal on antenna port p for PRACH is defined by sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA K=ΔfΔfRA k1=k0μ+NBWP,istart-Ngridstart,μNscRB+nRAstartNscRB+nRANRBRANscRB-Ngridsize,μNscRB2 k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ

There are many fields which affect the frequency to send the RA MSG1, i.e. ΔfRA and Kk1+kΔfRA:

- LRA as implied in the RRC field prach-RootSequenceIndex or prach-ConfigurationIndex;

- ΔfRA as either implied in the RRC field prach-ConfigurationIndex for the case of LRA=839, or indicated by the RRC field msg1-SubcarrierSpacing for the case of LRA=139;

- Δf, NBWP,istart as indicated in the configuration of the relevant BWP (see in the annex for detail);

- nRAstart as indicated by the RRC field msg1-FrequencyStart;

- nRA as selected by the UE, with its upper limit indicated in the RRC field msg1-FDM; and

- NRBRA and k as deduced from LRA, ΔfRA and Δf, as specified in Table 6.3.3.2-1 of TS 38.211.

What makes the situation even worse is that BWPs are configured in a per-UE fashion, e.g. different UE can be configured with different first active BWPs with different Δf and/or NBWP,istart in order for different bandwidth, while still share the same PRACH resources.

In order to avoid getting involved deep into so many physical parameters, here we suggest a “break out” solution, i.e. to add up these parameters into a much cleaner version:

- the subcarrier spacing for RA ΔfRA;

- an absolute frequency point FRA,0, which is completely determined by the network, corresponding to Kk1-nRANRBRANscRBΔfRA in TS 38.211, or synonymously, the frequency of the lowest subcarrier which can be used to send MSG1 minus kΔfRA;

- KnRANRBRANscRBΔfRA, which depends on the UE selection; and

- kΔfRA, which can deduced from the network configuration.

Following is the analysis on how to encode these four items.

The subcarrier spacing for RA ΔfRA: As provided above, this IE is either implied in the RRC field prach-ConfigurationIndex for the case of LRA=839, or indicated by the RRC field msg1-SubcarrierSpacing for the case of LRA=139.

The absolute frequency point FRA,0: This item is always possible to be encoded directly as FREF (NR-AFRCN) and an optional Δshift (similar to RRC field frequencyShift7p5khz) as defined in §5.4.2.1 of TS 38.104 (see in the annex for explanation on why it is always possible).

KnRANRBRANscRBΔfRA: We should examine its factor one by one. nRA depends on UE selection and the network can only define its value range with the RRC field msg1-FDM, NscRB is now a constant “12” as defined in §4.4.4.1 of TS 38.211, ΔfRA is already given above, and the left factor is KNRBRA=NRBRAΔfΔfRA. After checking with Table 6.3.3.2-1 of TS 38.211, the value of KNRBRA can be summarised as following:

| LRA | 839 | 839 | 839 | 839 | 839 | 839 | 139 | 139 | 139 | 139 | 139 | 139 | 139 | 139 | 139 | 139 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfRA (kHz) | 1.25 | 1.25 | 1.25 | 5 | 5 | 5 | 15 | 15 | 15 | 30 | 30 | 30 | 60 | 60 | 120 | 120 |
| Δf (kHz) | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 30 | 60 | 15 | 30 | 60 | 60 | 120 | 60 | 120 |
| NRBRA | 6 | 3 | 2 | 24 | 12 | 6 | 12 | 6 | 3 | 24 | 12 | 6 | 12 | 6 | 24 | 12 |
| KNRBRA | 72 | 72 | 96 | 72 | 72 | 72 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |

We can find that the value of KNRBRA can be either “72” or “96” for LRA=839, and a constant “12” for LRA=139. Hence we propose:

For the case of LRA=839, the value of KNRBRA can be deduced from the RRC field subcarrierSpacing of the corresponding BWP (i.e. Δf) along with the ΔfRA provided above; while for the case of LRA=139, we need not introduce any new field at present as its value is currently a constant “12”.

kΔfRA: The analysis is similar. For the case of LRA=839, the value of k can be deduced from the RRC field subcarrierSpacing of the corresponding BWP (i.e. Δf) along with the ΔfRA provided above; while for the case of LRA=139, we need not introduce any new field at present as its value is currently a constant “2” as specified in Table 6.3.3.2-1 of TS 38.211.

Proposal 3: Following information should also be included, in order to clearly indicate the available frequencies to send random access MSG1 for each given set of PRACH configuration:

For both the case of LRA=839 and the case of LRA=139:

- an NR-ARFCN and an optional Boolean field frequencyShift7p5khz, used together to indicate the frequency of the lowest subcarrier which can be used to send MSG1 minus kΔfRA; and

- msg1-FDM as already defined in TS 38.331, used to indicate the number of PRACH transmission occasions FDMed in one time instance.

For the case of LRA=839:

- prach-ConfigurationIndex as already defined in TS 38.331, used to deduce ΔfRA according to Table 6.3.3.2-2 or Table 6.3.3.2-3 of TS 38.211; and

- subcarrierSpacing of the corresponding BWP, used to deduce KNRBRA and k according to Table 6.3.3.2-1 of TS 38.211.

For the case of LRA=139:

- msg1-SubcarrierSpacing as already defined in TS 38.331.

Proposed IE structure

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| ///////////// Skip irrelevant fields ////////////// |  |  |  |  |
| NR PRACH Configuration List | O |  | 9.2.2.x |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |
| maxnoofPRACHconfigs | Maximum no. of broadcast PRACH configuration per cell. Value is 256. |

#### 9.2.2.x	NR PRACH Configuration List

This IE indicates the PRACH resources used or reserved in the current cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NR PRACH Configuration Item |  | 1..<maxnoofPRACHconfigs> |  |  |
| >Reference ARFCN | M |  | INTEGER (0.. maxNRARFCN) | Absolute frequency of the lowest subcarrier which can be used to send MSG1 minus kΔfRA in Section 5.3.2 in TS 38.211 [x]. |
| >FrequencyShift7p5khz | O |  | ENUMERATED (TRUE) | Enable the NR UL transmission with a 7.5 kHz shift to the LTE raster. If the field is absent, the frequency shift is disabled. |
| >MSG1-FDM | M |  | ENUMERATED (one, two, four, eight, …) | M in Section 6.3.3.2 in TS 38.211 [x]. |
| >CHOICE Frequency-Domain-Length | M |  |  |  |
| >>L839 |  |  |  |  |
| >>>L839 Info |  | 1 |  |  |
| >>>>BWP Subcarrier Spacing | M |  | ENUMERATED (khz15, khz30, khz60, khz120, …) | Subcarrier Spacing used in the relative BWP, i.e. Δf in Section 5.3.2 in TS 38.211 [x]. |
| >>>>PRACH Configuration Index | M |  | INTEGER (0.. 255, …) | See Section 6.3.3.2 in TS 38.211 [x]. |
| >>>>PRACH Root Sequence Index | M |  | INTEGER (0.. 837) | See Section 6.3.3.1 in TS 38.211 [x]. |
| >>>>Zero Correlation Zone Config | M |  | INTEGER (0.. 15) | See Section 6.3.3.1 in TS 38.211 [x]. |
| >>>>Restricted Set Config | M |  | ENUMERATED (unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB, …) | See Section 6.3.3.1 in TS 38.211 [x]. |
| >>L139 |  |  |  |  |
| >>>L139 Info |  | 1 |  |  |
| >>>>MSG1 Subcarrier Spacing | M |  | ENUMERATED (khz15, khz30, khz60, khz120, …) | Subcarrier Spacing used in sending MSG1, i.e. ΔfRA in Section 5.3.2 in TS 38.211 [x]. |
| >>>>PRACH Root Sequence Index | M |  | INTEGER (0.. 137) | See Section 6.3.3.1 in TS 38.211 [x]. |
| >>>>Zero Correlation Zone Config | M |  | INTEGER (0.. 15) | See Section 6.3.3.1 in TS 38.211 [x]. |

Conclusion

Proposal 1: One gNB may inform its neighbour about multiple sets of PRACH configuration currently in use or reserved as candidates for dedicated signalling.

Proposal 2: The following 3 fields should be included within each set of PRACH configuration: prach-RootSequenceIndex, zeroCorrelationZoneConfig, for any case, and restrictedSetConfig for the case of LRA=839.

Proposal 3: Following information should also be included, in order to clearly indicate the available frequencies to send random access MSG1 for each given set of PRACH configuration:

For both the case of LRA=839 and the case of LRA=139:

- an NR-ARFCN and an optional Boolean field frequencyShift7p5khz, used together to indicate the frequency of the lowest subcarrier which can be used to send MSG1 minus kΔfRA; and

- msg1-FDM as already defined in TS 38.331, used to indicate the number of PRACH transmission occasions FDMed in one time instance.

For the case of LRA=839:

- prach-ConfigurationIndex as already defined in TS 38.331, used to deduce ΔfRA according to Table 6.3.3.2-2 or Table 6.3.3.2-3 of TS 38.211; and

- subcarrierSpacing of the corresponding BWP, used to deduce KNRBRA and k according to Table 6.3.3.2-1 of TS 38.211.

For the case of LRA=139:

- msg1-SubcarrierSpacing as already defined in TS 38.331.

Based on the proposals above, we proposed 4 CRs [3–6]. For EN-DC, in order to facilitate PRACH coordination between en-gNBs, we also add the PRACH information within the “NR Neighbour Information” IE of an E-UTRA cell.

If RAN3 considers the analysis above generally RAN1/RAN2 issues and cannot make the final decision, we are also happy to send an LS toward RAN1/RAN2 for consultant [7].

Reference

[1] R2-096208; Selection of the proper root sequence index for RACHO; Samsung, Motorola, CATT.

[2] R2-096209; Detection of the neighboring cells’ frequency resources for PRACH; Samsung, Motorola, CATT.

[3] R3-193680; CR on PRACH coordination (38.300), CATT.

[4] R3-193681; CR on PRACH coordination (X2), CATT.

[5] R3-193682; CR on PRACH coordination (Xn), CATT.

[6] R3-193683; CR on PRACH coordination (F1), CATT.

[7] R3-193684; LS on PRACH coordination, CATT.

Annex: Explanation of the frequency part of the formula in §5.3.2 of TS 38.211

Signal of PRACH is specified in §5.3.2 of TS 38.211:

The time-continuous signal on antenna port p for PRACH is defined by sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA K=ΔfΔfRA k1=k0μ+NBWP,istart-Ngridstart,μNscRB+nRAstartNscRB+nRANRBRANscRB-Ngridsize,μNscRB2 k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ

Obviously, sl(p,μ)t is a simple sum of LRA harmonic waves, whose relative frequency spreads from Kk1+kΔfRA toward LRA-1+Kk1+kΔfRA. This frequency is provided assuming the centre frequency of the configured carrier with the largest μ (i.e. μ0) equals to zero.

Hence we can understand the value of Kk1+kΔfRA:

Kk1+kΔfRA=Kk1ΔfRA+kΔfRA
                              =k1Δf+kΔfRA
                              =k0μ+NBWP,istart-Ngridstart,μNscRB+nRAstartNscRB+nRANRBRANscRB-12Ngridsize,μNscRBΔf+kΔfRA
                              =k0μ-Ngridstart,μNscRB-12Ngridsize,μNscRB+NBWP,istartNscRB+nRAstartNscRB+nRANRBRANscRBΔf+kΔfRA
                              =k0μ-Ngridstart,μ+12Ngridsize,μNscRBΔf+NBWP,istartNscRBΔf+nRAstartNscRBΔf+nRANRBRANscRBΔf+kΔfRA
                              =-Ngridstart,μ0+12Ngridsize,μ0NscRB2μ0-μΔf+NBWP,istartNscRBΔf+nRAstartNscRBΔf+nRANRBRANscRBΔf+kΔfRA
                              =-Ngridstart,μ0+12Ngridsize,μ0NscRBΔfμ0+NBWP,istartNscRBΔf+nRAstartNscRBΔf+nRANRBRANscRBΔf+kΔfRA

Note: NscRB is a constant which equals to 12.

Here Δfμ0 equals to the subcarrier spacing of the configured carrier with the largest μ. In §3.2 of TS 38.211, the parameter μ is defined so that Δf=2μ∙15 [kHz]. For μ=μ0, we have Δfμ0=2μ0∙15 [kHz]. Therefore:

2μ0-μΔf=2μ0-μ·2μ∙15 kHz=2μ0∙15 kHz=Δfμ0

Following we will explain the five items in the formula above one by one.

Centre of carrier → Point A

The first item, i.e. -Ngridstart,μ0+12Ngridsize,μ0NscRBΔfμ0, moves the start point toward the frequency of the “Point A”.

As specified in §4.4.4.2 of TS 38.211, the “Point A” is indicated directly with an NR-ARFCN for any case other than the initial cell selection, which de facto require that the “Point A” should always be possible to be defined with an NR-ARFCN. The detail definition of Point A is within §5.4.2.1 of TS 38.104:

#### 5.4.2.1 NR-ARFCN and channel raster The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements. The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal. RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [0…3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN. FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs) Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster Frequency range (MHz); ΔFGlobal (kHz); FREF-Offs (MHz); NREF-Offs; Range of NREF. 0 – 3000; 5; 0; 0; 0 – 599999. 3000 – 24250; 15; 3000; 600000; 600000 – 2016666. 24250 – 100000; 60; 24250.08; 2016667; 2016667 – 3279165. ///////////////////////////////////////////////////////////////////// skip irrelevant text ///////////////////////////////////////////////////////////////////////// For SUL bands and for the uplink of all FDD bands defined in table 5.2-1, FREF,shift = FREF + Δshift, Δshift = 0 kHz or 7.5 kHz where Δshift is signalled by the network in higher layer parameter frequencyShift7p5khz [11]. ///////////////////////////////////////////////////////////////////// skip irrelevant text /////////////////////////////////////////////////////////////////////////

For SUL bands or UL of FDD bands, the frequency of “Point A” is the exact value of FREF,shift. Otherwise the frequency of “Point A” is the exact value of FREF. Therefore the frequency of “Point A” can be indicated with an NR-ARFCN and an optional frequencyShift7p5khz. In TS 38.331 These two fields are both delivered within either UplinkConfigCommon or UplinkConfigCommonSIB, which means that they should be configured common to all UEs connecting to this cell.

The NR-ARFCN(s) mentioned here is also identical to the NR-ARFCN(s) delivered within “Served Cell Information NR” in TS 38.423 or other RAN WG3 specifications, however current RAN WG3 specifications lack of any frequencyShift7p5khz IEs.

Point A → subcarrier 0 of virtual resource block 0

The second item, i.e. NBWP,istartNscRBΔf, moves the start point toward the frequency of “subcarrier 0” of “virtual resource block 0”.

Ordinarily, the value of NBWP,istart—and the value of NBWP,isize, which we will face later—are deduced from a RIV value, namely the locationAndBandwidth field within BWP in TS 38.331. The range of NBWP,istart and NBWP,isize are both limited in §6.1.2.2.2 of TS 38.214:

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by if then else where 1 and shall not exceed.

Note: TS 38.214 uses different symbols for the same meaning compared with other specifications such as TS 38.211:

- LRBs in TS 38.214 means the number of resource blocks within the BWP, identical to NBWP,isize in TS 38.211;

- NBWPsize in TS 38.214 means the maximum LRBs allowed to configured, which value is defined as 275 as in §12 of TS 38.213;

- RBstart in TS 38.214 means the frequency offset from “Point A” toward the frequency of “subcarrier 0” of “virtual resource block 0” of this BWP, counting in length of resource blocks, identical to NBWP,istart in TS 38.211.

Therefore we can deduce that:

NBWP,isize≥1

NBWP,istart≥0

NBWP,istart+NBWP,isize≤275

Although there do exist some other ways to indicate the starting point and the bandwidth of a BWP, their limit on NBWP,isize and NBWP,istart≥0 is even narrower.

Within a BWP, resource blocks are numbered from 0 to NBWP,isize-1 as specified in §4.4.4.5 of TS 38.211.

Subcarrier 0 of virtual resource block 0 → reference frequency of a set of PRACH configuration

The third item, i.e. nRAstartNscRBΔf, moves the start point toward a frequency value here defined as a “reference frequency” of the considered set of PRACH configuration.

The value of  nRAstart is provided directly with the RRC field msg1-FrequencyStart as of TS 38.331. In addition, it is specified that:

msg1-FrequencyStart Offset of lowest PRACH transmission occasion in frequency domain with respective to PRB 0. The value is configured so that the corresponding RACH resource is entirely within the bandwidth of the UL BWP. (see TS 38.211 [16], clause 6.3.3.2).

Since the RRC ASN.1 limits that the nRAstart is non-negative, we can deduce that:

NBWP,istart+nRAstart≥0

Since the fourth item nRANRBRANscRBΔf and the fifth item kΔfRA are both non-negative, and the frequency length of MSG1 is either 839·ΔfRA or 139·ΔfRA, in order to guarantee the MSG1 is sent within the relative UL BWP, there should be:

nRAstart<NBWP,isize

And hence:

NBWP,istart+nRAstart<275

Reference frequency of a set of PRACH configuration → reference frequency to send MSG1

The fourth item, i.e. nRANRBRANscRBΔf, moves the start point toward a frequency value here defined as a “reference frequency” to send MSG1.

In fact, the value of NRBRA is determined as the smallest integer which makes two FDM MSG1s not to overlap with each other. Hence NRBRANscRBΔf tends to be slightly larger than LRAΔfRA. This explains why KNRBRA tends to be a constant.

Reference frequency to send MSG1 → frequency of the lowest subcarrier to send MSG1

The fourth item, i.e. kΔfRA, moves the start point toward the frequency of the subcarrier for which k=0 to send MSG1.