3GPP TSG-RAN3 Meeting #105	R3-193770

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0177 | rev |  | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Load Sharing and Load Balancing Optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-06-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current version of specification does not include any contents to support the function of load sharing and load balancing optimization. In order to support the load sharing and load balancing optimization, the corresponding signaling and IEs should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding resource status reporting initiation and resource status reporting procedure for supporting load sharing and load balancing optimization function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of load sharing and load balancing optimization can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

# XnAP procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |

Start of the next change

### 8.4.X	Resource Status Reporting Initiation

#### 8.4.X.1	General

This procedure is used by an NG-RAN node1 to request the reporting of load measurements to another NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.4.X.2	Successful Operation

Figure 8.2.X.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from NG-RAN node1 to NG-RAN node2 to start a measurment, stop a measurements, add cells to report for a measurement or remove certain cells from a measurement.

When starting a measurement, the Report Characteristics IE indicates the type of objects NG-RAN node2 shall perform measurements on. For each cell, the NG-RAN node2 shall include in the RESOURCE STATUS UPDATE message:

-	the Cell level load  IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the TNL Load Indicator IE, if the second bit, "TNL Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Hardware Load Indicator IE, if the third bit, "HW Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the fourth bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE;

If the List of S-NSSAI IE is present the NG-RAN node2 shall include in the RESOURCE STATUS UPDATE message, the Composite Available Capacity Group IE for each of the S-NSSAI contained in the list of S-NSSAI(s).

If NG-RAN node2 is capable to provide all requested resource status information, it shall initiate the measurement as requested by NG-RAN node1, and respond with the RESOURCE STATUS RESPONSE message.

#### 8.4.X.3	Unsuccessful Operation

Figure 8.4.X.3-1: Resource Status Reporting Initiation, unsuccessful operation

If the requested measurements cannot be initiated, NG-RAN node2 shall send a RESOURCE STATUS FAILURE message.

#### 8.4.X.4	Abnormal Conditions

FFS

### 8.4.Y	Resource Status Reporting

#### 8.4.Y.1	General

This procedure is initiated by NG-RAN node2 to report the result of measurements admitted by NG-RAN node2 following a successful Resource Status Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.4.Y.2	Successful Operation

Figure 8.4.Y.2-1: Resource Status Reporting, successful operation

The NG-RAN node2 shall report the results of the admitted measurements in RESOURCE STATUS UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Resource Status Reporting Initiation procedure.

#### 8.4.Y.3	Unsuccessful Operation

Not applicable.

#### 8.4.Y.4	Abnormal Conditions

Void.

Start of the next change

#### 9.1.3.11	RESOURCE STATUS REQUEST

This message is sent by an NG-RAN node1 to neighbouring NG-RAN node2 to initiate the requested measurement according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestStoporRemoveorAdd |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| CHOICE Registration Request |  |  |  |  |  |  |
| >Start |  |  |  |  |  |  |
| >>List of cells |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  |  |  |
| >>>Cell ID | M |  | NR CGI 9.2.2.7 |  | – | – |
| >>List of S-NSSAI |  | 0 .. <maxnoofSliceItems> |  |  |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  | – | – |
| >>Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the receiving node is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1, | - | - |
| >Stop |  |  | NULL |  |  |  |
| >Remove |  |  |  |  |  |  |
| >>List of cells |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  |  |  |
| >>>Cell ID | M |  | NR CGI 9.2.2.7 |  | – | – |
| >Add |  |  |  |  |  |  |
| >>List of cells |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  |  |  |
| >>>Cell ID | M |  | NR CGI 9.2.2.7 |  | – | – |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

| Condition | Explanation |
| --- | --- |
| fRegistrationRequestStoporRemoveorAdd | This IE shall be present if the Registration Request IE is set to the value "Stop", "Remove" or "Add". |

#### 9.1.3.12	RESOURCE STATUS RESPONSE

This message is sent by the NG-RAN node2 to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.13	RESOURCE STATUS FAILURE

This message is sent by the NG-RAN node2 to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cause | M |  |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.2.3.3 |  | YES | ignore |

#### 9.1.3.14	RESOURCE STATUS UPDATE

This message is sent by NG-RAN node2 to neighbouring NG-RAN node1 to report the results of the requested measurements.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxnoofCellsinNG-RAN node> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.2.2.7 |  |  |  |
| >>Hardware Load Indicator | O |  | 9.2.2.X1 |  |  |  |
| >>TNL Load Indicator | O |  | 9.2.2.X2 |  |  |  |
| >>Cell level load | O |  | 9.2.2.X3 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.X4 |  | YES | ignore |
| >>Slice Measurement Result List |  | 0 |  |  | YES | ignore |
| >>>Slice Measurement Result Item |  | 1 .. 1..<maxnoofSliceItems> |  |  | EACH | ignore |
| >>>>S-NSSAI | M |  | 9.2.3.21 |  |  |  |
| >>>>Composite Available Capacity Group | M |  | 9.2.2.X4 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

Start of the next change

### 9.2.2.X1	Hardware Load Indicator

The Hardware Load Indicator IE indicates the status of the Hardware Load experienced by the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Hardware Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |
| UL Hardware Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |
| SUL Hardware Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |

### 9.2.2.X2   	TNL Load Indicator

The NG TNL Load Indicator IE indicates the status of the NG Transport Network Load experienced by the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL TNL Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |
| UL TNL Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |

### 9.2.2.X5	Load Indicator

The Load Indicator IE indicates the status of Load.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Load Indicator | M |  | ENUMERATED (LowLoad, MediumLoad, HighLoad, Overload, ...) |  |

### 9.2.2.X3	Cell level load

The Cell level load IE indicates the usage of the PRBs for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL GBR PRB usage | M |  | INTEGER (0..100) |  |
| UL GBR PRB usage | M |  | INTEGER (0..100) |  |
| SUL GBR PRB usage | M |  | INTEGER (0..100) |  |
| DL non-GBR PRB usage | M |  | INTEGER (0..100) |  |
| UL non-GBR PRB usage | M |  | INTEGER (0..100) |  |
| SUL non-GBR PRB usage | M |  | INTEGER (0..100) |  |
| DL Total PRB usage | M |  | INTEGER (0..100) |  |
| UL Total PRB usage | M |  | INTEGER (0..100) |  |
| SUL Total PRB usage | M |  | INTEGER (0..100) |  |
| DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

### 9.2.2.X4	Composite Available Capacity Group

The Composite Available Capacity Group IE indicates the overall available resource level in the cell in Downlink, Uplink and supplementary Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | FFS | For the Downlink | - | - |
| Composite Available Capacity Uplink | M |  | FFS | For the Uplink | - | - |
| Composite Available Capacity supplementary Uplink | M |  | FFS | For the supplementary Uplink | - | - |

End of the first change