3GPP TSG-RAN3 Meeting #105	R3-193771

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0394 | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Load Sharing and Load Balancing Optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-04-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current version of specification does not include any contents to support the function of load sharing and load balancing optimization. In order to support the load sharing and load balancing optimization, the corresponding IEs should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new procedures for supporting load sharing and load balancing optimization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of load sharing and load balancing optimization can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

# 8	F1AP procedures

## 8.1	List of F1AP Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Reset | RESET | RESET ACKNOWLEDGE |  |
| F1 Setup | F1 SETUP REQUEST | F1 SETUP RESPONSE | F1 SETUP FAILURE |
| gNB-DU Configuration Update | GNB-DU CONFIGURATION UPDATE | GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-DU CONFIGURATION UPDATE FAILURE |
| gNB-CU Configuration Update | GNB-CU CONFIGURATION UPDATE | GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE | GNB-CU CONFIGURATION UPDATE FAILURE |
| UE Context Setup | UE CONTEXT SETUP REQUEST | UE CONTEXT SETUP RESPONSE | UE CONTEXT SETUP FAILURE |
| UE Context Release (gNB-CU initiated) | UE CONTEXT RELEASE COMMAND | UE CONTEXT RELEASE COMPLETE |  |
| UE Context Modification (gNB-CU initiated) | UE CONTEXT MODIFICATION REQUEST | UE CONTEXT MODIFICATION RESPONSE | UE CONTEXT MODIFICATION FAILURE |
| UE Context Modification Required (gNB-DU initiated) | UE CONTEXT MODIFICATION REQUIRED | UE CONTEXT MODIFICATION CONFIRM | UE CONTEXT MODIFICATION REFUSE |
| Write-Replace Warning | WRITE-REPLACE WARNING REQUEST | WRITE-REPLACE WARNING RESPONSE |  |
| PWS Cancel | PWS CANCEL REQUEST | PWS CANCEL RESPONSE |  |
| GNB-DU RESOURCE COORDINATION | GNB-DU RESOURCE COORDINATION REQUEST | GNB-DU RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |
| UE Context Release Request (gNB-DU initiated) | UE CONTEXT RELEASE REQUEST |
| Initial UL RRC Message Transfer | INITIAL UL RRC MESSAGE TRANSFER |
| DL RRC Message Transfer | DL RRC MESSAGE TRANSFER |
| UL RRC Message Transfer | UL RRC MESSAGE TRANSFER |
| UE Inactivity Notification | UE INACTIVITY NOTIFICATION |
| System Information Delivery | SYSTEM INFORMATION DELIVERY COMMAND |
| Paging | PAGING |
| Notify | NOTIFY |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| gNB-DU Status Indication | GNB-DU STATUS INDICATION |
| RRC Delivery Report | RRC DELIVERY REPORT |
| Network Access Rate Reduction | NETWORK ACCESS RATE REDUCTION |
| Resource Status Reporting | RESOURCE STATUS UPDATE |

Start of the next change

### 8.2.X	Resource Status Reporting Initiation

#### 8.2.X.1	General

This procedure is used by an gNB-CU to request the reporting of load measurements to gNB-DU.

The procedure uses non UE-associated signalling.

#### 8.2.X.2	Successful Operation

Figure 8.2.X.2-1: Resource Status Reporting Initiation, successful operation

The procedure is initiated with a RESOURCE STATUS REQUEST message sent from gNB-CU to gNB-DU to start a measurment, stop a measurements, add cells to report for a measurement or remove certain cells from a measurement.

When starting a measurement, the Report Characteristics IE indicates the type of objects gNB-DU shall perform measurements on. For each cell, the gNB-DU shall include in the RESOURCE STATUS UPDATE message:

-	the Cell level load IE, if the first bit, "PRB Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Composite Available Capacity Group IE, if the fourth bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1. If Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE;

-	the TNL Load Indicator IE, if the second bit, "TNL Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

-	the Hardware Load Indicator IE, if the third bit, "HW Load Ind Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to 1;

If the List of S-NSSAI IE is present the gNB-DU shall include in the RESOURCE STATUS UPDATE message, the Composite Available Capacity Group IE for each of the S-NSSAI contained in the list of S-NSSAI(s).

If gNB-DU is capable to provide all requested resource status information, it shall initiate the measurement as requested by gNB-CU, and respond with the RESOURCE STATUS RESPONSE message.

#### 8.2.X.3	Unsuccessful Operation

Figure 8.2.X.3-1: Resource Status Reporting Initiation, unsuccessful operation

If none of the requested measurements can be initiated, gNB-DU shall send a RESOURCE STATUS FAILURE message.

#### 8.2.X.4	Abnormal Conditions

FFS

### 8.2.Y	Resource Status Reporting

#### 8.2.Y.1	General

This procedure is initiated by gNB-DU to report the result of measurements admitted by gNB-DU following a successful Resource Status Reporting Initiation procedure.

The procedure uses non UE-associated signalling.

#### 8.2.Y.2	Successful Operation

Figure 8.2.Y.2-1: Resource Status Reporting, successful operation

The gNB-DU shall report the results of the admitted measurements in RESOURCE STATUS UPDATE message. The admitted measurements are the measurements that were successfully initiated during the preceding Resource Status Reporting Initiation procedure.

#### 8.4.Y.3	Unsuccessful Operation

Not applicable.

#### 8.4.Y.4	Abnormal Conditions

Void.

Start of the next change

#### 9.1.3.11	RESOURCE STATUS REQUEST

This message is sent by an gNB-CU to gNB-DU to initiate the requested measurement according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporRemoveorAdd |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| CHOICE Registration Request |  |  |  |  |  |  |
| >Start |  |  |  |  |  |  |
| >>List of cells |  | 1.. <maxCellingNBDU> |  |  |  |  |
| >>>Cell ID | M |  | NR CGI 9.3.1.12 |  | – | – |
| >>List of S-NSSAI |  | 0 .. <maxnoofSliceItems> |  |  |  |  |
| >>>S-NSSAI | M |  | 9.3.1.38 |  | – | – |
| >>Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the receiving node is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1, | YES | reject |
| >Stop |  |  | NULL |  |  |  |
| > Remove |  |  |  |  |  |  |
| >>List of cells |  | 1.. <maxCellingNBDU> |  |  |  |  |
| >>>Cell ID | M |  | NR CGI 9.3.1.12 |  | – | – |
| >Add |  |  |  |  |  |  |
| >>List of cells |  | 1.. <maxCellingNBDU> |  |  |  |  |
| >>>Cell ID | M |  | NR CGI 9.3.1.12 |  | – | – |

#### 9.1.3.12	RESOURCE STATUS RESPONSE

This message is sent by the gNB-DU to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.1.3.13	RESOURCE STATUS FAILURE

This message is sent by the gNB-DU to indicate that for none of the requested measurement objects the measurement can be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence |  | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  |  | INTEGER (1..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestStoporRemoveorAdd |  |  | INTEGER (1..4095,...) | Allocated by gNB-DU | YES | ignore |
| Cause | M |  |  | 9.3.1.2 | Ignored by the receiver when the Complete Failure Cause Information IE is included | YES | ignore |
| Criticality Diagnostics |  | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.1.3.14	RESOURCE STATUS UPDATE

This message is sent by gNB-DU to gNB-CU to report the results of the requested measurements.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-DU Cell Measurement Result |  | 1 |  |  | YES | ignore |
| > gNB-DU Cell Measurement Result Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  |  |  |
| >>Cell level load | O |  | 9.2.2.X3 |  |  |  |
| >>Hardware Load Indicator | O |  | 9.2.2.X1 |  |  |  |
| >>TNL Load Indicator | O |  | 9.2.2.X2 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.X4 |  | YES | ignore |
| Slice Measurement Result |  | 1 |  |  | YES | ignore |
| >Slice Measurement Result Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>Cell ID |  |  | 9.3.1.12 |  |  |  |
| >>S-NSSAI | M |  | 9.3.1.38 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.2.X4 |  | YES | ignore |

Start of the next change

### 9.2.2.X1	Hardware Load Indicator

The Hardware Load Indicator IE indicates the status of the Hardware Load experienced by the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL Hardware Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |
| UL Hardware Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |
| SUL Hardware Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |

### 9.2.2.X2   	TNL Load Indicator

The NG TNL Load Indicator IE indicates the status of the NG Transport Network Load experienced by the cell.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL TNL Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |
| UL TNL Load Indicator | M |  | Load Indicator 9.2. 2.X5 |  |

### 9.2.2.X5	Load Indicator

The Load Indicator IE indicates the status of Load.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Load Indicator | M |  | ENUMERATED (LowLoad, MediumLoad, HighLoad, Overload, ...) |  |

### 9.2.2.X3	Cell level load

The Cell level load IE indicates the usage of the PRBs for all traffic in Downlink and Uplink and the usage of PDCCH CCEs for Downlink and Uplink scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL GBR PRB usage | M |  | INTEGER (0..100) |  |
| UL GBR PRB usage | M |  | INTEGER (0..100) |  |
| SUL GBR PRB usage | M |  | INTEGER (0..100) |  |
| DL non-GBR PRB usage | M |  | INTEGER (0..100) |  |
| UL non-GBR PRB usage | M |  | INTEGER (0..100) |  |
| SUL non-GBR PRB usage | M |  | INTEGER (0..100) |  |
| DL Total PRB usage | M |  | INTEGER (0..100) |  |
| UL Total PRB usage | M |  | INTEGER (0..100) |  |
| SUL Total PRB usage | M |  | INTEGER (0..100) |  |
| DL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |
| UL scheduling PDCCH CCE usage | O |  | INTEGER (0..100) |  |

### 9.2.2.X4	Composite Available Capacity Group

The Composite Available Capacity Group IE indicates the overall available resource level in the cell in Downlink, Uplink and supplementary Uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | FFS | For the Downlink | - | - |
| Composite Available Capacity Uplink | M |  | FFS | For the Uplink | - | - |
| Composite Available Capacity supplementary Uplink | M |  | FFS | For the supplementary Uplink | - | - |

End of the next change