3GPP TSG-RAN WG3 Meeting #105	R3-194141

# Ljubljana, Slovenien, 26th – 30th August 2019

Agenda Item:	9.2

Source:	Ericsson

Title: 	ERROR INDICATION Message in S1AP

Document for:	Discussion and Approval

# 1	Discussion

The Error Indication procedure is used for one node receiving the message to indicate to the node sending the message when there is an error related to unknown, protocol data, when there is no appropriate response message to feedback the error.

When the message is sent for a particular UE, but without any UE APID, the receiving node cannot feedback the error ping point to the right UE. For example, in RETRIEVE UE INFORMATUION message, only S-TMSI is used. If S-TMSI is unknown by MME, it would be benefit to feedback to eNB what went wrong. One possible scenario of the S-TMSI is unknow by MME is during MME restart. In TS 23.007, it is specified that after a MME restart, the MME shall delete all MM Bearer contexts affected by the restart that it may have stored. In this case MME does not have any S-TMSI.

At the last meeting, a proposal is to handle this abnormal case with a successful operation signalling. In our view the unknow protocol data should be handled as we have already specified, in Error Indication procedure.

We propose to add S-TMSI in ERROR INDICATION.

#### 9.1.4.21	RETRIEVE UE INFORMATION

The message is sent by the eNB to request UE information over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| S-TMSI | M |  | 9.2.3.6 |  | YES | reject |

#### 9.1.8.3	ERROR INDICATION

This message is sent by both the MME and the eNB and is used to indicate that some error has been detected in the node.

Direction: MME  eNB and eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| MME UE S1AP ID | O |  | 9.2.3.3 |  | YES | ignore |
| eNB UE S1AP ID | O |  | 9.2.3.4 |  | YES | ignore |
| Cause | O |  | 9.2.1.3 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |

Proposal 1:   RAN3 to agree to add S-TMSI in ERROR INDICATION

# 2	Proposal

Proposal 1:   RAN3 to agree to add S-TMSI in ERROR INDICATION