3GPP TSG-RAN WG3 #105	R3-194286

Ljubljana, SI, 26th August – 30th August 2019

Agenda Item:	10.2.2

Source:	Ericsson, NTT DOCOMO, INC.

Title:	Limitation of cell-level load reporting in load balancing

Document for:	Discussion, Decision

# Introduction

The TR 37.816 [1] has concluded that:

“load sharing and load balancing functions over X2, Xn, F1 and E1 interfaces should be specified in Release 16 normative phase, and at least the following load related information should be specified:

- Composite Available Capacity per cell (DL/UL), reported via X2, Xn and F1 interfaces;

- Cell Level Load (DL/UL/SUL) reported via X2, Xn, and F1 interfaces;

- TNL Load reported via X2, Xn, F1, and E1 interfaces.

Further refinement on the above metrics, as well as an evaluation on whether these metrics are sufficient for the correct functioning of the load management function and whether they should be complemented with additional enhancements during Release 16 normative phase. “

In addition, the TR concluded that the “introduction of additional load information over X2, Xn, F1 and E1 interfaces is seen as potential enhancement on several areas and may be considered for specification in Release 16”, including beam level load indication.

This contribution discusses limitation of exploiting only cell-level load reporting in load balancing as part of the task of assessing “whether the listed metrics in the conclusions are sufficient for the correct functioning of the load management function and whether they should be complemented with additional enhancements during Release 16 normative phase”.

# Discussion

## Cell-specific load information in LTE and NR

Cell-specific load information, such as Composite Available Capacity per cell, Cell Level Load and TNL Load, has been introduced in early releases of LTE to enable load management between neighbouring eNBs. Due to their limited MIMO capabilities and the available downlink reference signals, these early LTE releases could not support a finer characterisation of the cell load distribution, e.g. in the spatial domain. Despite recent advances in active antenna technology have enabled to increase the number of antenna available at the base station and improve the MIMO capability of the cell by means, for instance of new CSI-RS signals and beamformed CSI-RS transmission (i.e., class B FD-MIMO), load information reports in LTE has remained cell-specific. It should be noted that 3GPP did not analyse the load balancing function in light of progress in active antenna systems, which is the reason the load balancing function remained unchanged across the releases.

To better capitalize the MIMO capabilities of large antenna arrays, a 3GPP NR cell can be configured to transmit multiple Synchronization Signals and PBCH Blocks (SSB) for cell search and synchronization. An SSB consists of a primary and secondary synchronization signals (PSS, SSS) and physical broadcast channel (PBCH). While for a given frequency location the PSS/SSS signals carry the physical cell ID (PCI), up to 64 SSB can be transmitted within a NR half-frame in different spatial directions (i.e. using different spatial beams, spanning the coverage area of a cell), each identified by a unique SSB beam index.

A user equipment (UE) can be configured to measure and report the strength of different SSB signals. Since SSB beams can be transmitted to cover different parts of the cell’s coverage area, and given that, from a UE point of view, measurement reports are based on detection of such SSBs, it is possible to divide the cell in SSB beam coverage areas and to determine parameters such as load, composite capacity, resource status information for each SSB beam coverage area. With this it is worth noticing that while the reference signal structure in LTE did not allow for a spatial characterization of a cell (because the cell had only one RS for its all coverage), in NR a cell can have different subareas covered by a different RS. This allows for load characterization in the special domain.

- NR already supports downlink reference signals that can be used to characterize  the cell load distribution in the spatial domain, e.g. on a per SSB beam coverage area.

### Limitations of cell-specific load information

The TR 37.816 [1] has concluded that for load sharing and load balancing in NR, the reported information over X2, Xn and F1 should contain at least cell-specific load.  Cell-specific load information, however, is unable to capture uneven load distribution within the cell area and can lead, by itself, to harmful mobility decisions.

Figure 1 shows two examples wherein using only cell-specific load information is not enough for the correct operation of the load management function. Both examples consider a service cell highly loaded at least in some local area, here defined by the coverage area of different SSB beams. A target UE in the loaded area may detect and report a neighbor target cell as a good mobility target. The UE reports measurements per SSB, i.e. per SSB beam area. Given its loaded status, the serving cell can request resource status report to the target cell.

In the example in Figure 1(a), the target cell reports high cell load, although in the SSB beam coverage area where the UE is moving, the target cell has enough available capacity for the UE thanks to its spatial multiplexing capability. The serving node is then led to believe that the target cell is also high load and chooses not to handover the UE to the target cell.

In the example in Figure 1(b), the target cell reports medium-low cell load. While cell-wise the reported load is not high, the load is all concentrated in the SSB beams coverage area where the UE is moving. The loaded serving cell is thus led to believe that the target cell has enough available capacity for the target UE and initiates a handover request, whilst it should have kept the target UE.

- Cell-specific load information is not enough for the correct operation of the load management function in NR.

| (a) Fail to move target UE regardless the neighboring beam has enough available capacity (due to high cell load report). | (b) Fail to keep target UE regardless the neighboring beam has NOT enough available capacity (due to medium-low cell load report) |
| --- | --- |

Figure 1: Examples where using only cell-specific load information may lead to wrong mobility decisions.

Therefore, introducing in NR beam level load indication, e.g. on a per SSB beam level, as opposed to only cell-specific, can be beneficial to enhance the mobility load balance functionality. In the example in Figure 1(a), using beam level load information, the serving cell would be aware that in the beam coverage area where the UE is moving, the target cell has enough available capacity to accept the UE, and would therefore be able to correctly initiate a handover request. On the other hand, in the example in Figure 1(b), using beam level load information, the serving cell would be aware that in the beam coverage area where the UE is moving, the target cell does not have enough available capacity to accept the UE, and would therefore avoid to initiate a handover request which would be likely to lead to suboptimal performance.

Therefore, when one consider the parameters:

- Composite Available Capacity per cell (DL/UL), reported via X2, Xn and F1 interfaces;

- Cell Level Load (DL/UL/SUL) reported via X2, Xn, and F1 interfaces;

- TNL Load reported via X2, Xn, F1, and E1 interfaces.

and asks the question of whether they are sufficient for the correct functioning of the load management function, the answer is that they are not sufficient and that they need to be complemented with some extra information that would make the load information accurate from a spatial point of view. Considering this, the following is proposed.

- To introduce beam level composite available capacity and/or load on a per SSB beam level as part of the load per cell exchanged over RAN interfaces.

# Conclusions

In this contribution, the following observations are captured:

- NR already supports downlink reference signals that can be used to characterize  the cell load distribution in the spatial domain, e.g on a per SSB beam coverage area.

- Cell-specific load information is not enough for the correct operation of the load management function in NR.

In this contribution, the following proposals are captured:

- To introduce beam level composite available capacity and/or load on a per SSB beam level as part of the load per cell exchanged over RAN interfaces.

# References

- 3GPP TR 37.816 v1.0.0 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network;Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Study on RAN-centric data collection and utilization for LTE and NR”, June 2019.