3GPP TSG-RAN3 Meeting #105	R3-194430

Ljubljana, Slovenia, 26-30 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0083 | rev |  | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.401 on MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT |  |  |  |  |  | Date: |  |  | 2019-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document captures the stage 2 agreements on MDT in split RAN as discussed in R3-194101. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of immediate MDT procedures in split RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MDT of NR is not supported in split RAN scenario |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begin

7.x	MDT

7.x.1	Immediate MDT support in CU/DU

The MDT function in non-split gNB case is specified in [x].

The gNB-CU, gNB-DU and gNB-CU-UP supports immediate MDT function. The gNB-DU and gNB-CU-UP receives MDT task for UEs from gNB-CU or from OAM. The gNB-CU-CP, gNB-CU-UP and gNB-DU sends the collected MDT measurement results to the TCE directly.

The gNB-CU-DU supports the following measuremments for immediate MDT:

- M2: Power Headroom.

- M3: Received Interference Power.

- M5: Scheduled IP Throughput for DL and UL.

- M6: Packet Delay for UL/DL.

- M7: Packet Loss rate for UL(FFS).

The gNB-CU-UP supports the following measuremments for immediate MDT:

- M4: Data Volume for UL/DL.

- M6: Packet Delay for UL/DL.

- M7: Packet Loss rate for UL(FFS).

Change End