3GPP TSG-RAN WG3 #105								R3-194510

Ljubljana, Slovenia, 26-30 August 2019

Agenda Item:	31.3.12

Source:	China Telecom

Title: 	Support of inter-system HO from SA to EN-DC in Rel-16

Document for:	Discussion

Introduction

The most operators will deploy EN-DC to support eMBB service in their first stage of 5G deployment, and after that state, part EN-DC base station may be upgraded to NR SA or NSA/SA dual mode via software update. So when NSA/SA dual mode UE moves out of NR SA cell coverage, direct configuration of EN-DC during inter-system handover can reduce the DC configuration latency and quickly recover user’s data rate after HO procedure. From the operator’s point of view, it would be beneficial to support SA to EN-DC in NR practical network. However, the NR SA to EN-DC HO is not supported in Rel-15 for time limitations [1]. The intention of this contribution is to discuss the scenario and gives some analysis on the RAN3 impact.

Discussion

Scenarios and Requirements

The main driver for employing EN-DC, is to utilize the extended bandwidth provided by NR spectrum, while keeping the control signalling on LTE. So EN-DC is a good solution to provide high date rate for 2C service in first stage of 5G deployment. In order to provide 2B service, such as uRLLC service, in certain areas, the operators will consider to upgrade part of en-gNBs to support EN-DC and SA alone access simultaneously. So there is a boundary between SA and NSA region for the NSA/SA dual mode UE. As per Rel-15 specification, when the NSA/SA dual mode UE moves out of NR SA cell coverage or triggers EPS fallback, it would be handover to LTE firstly and then perform subsequent SCG reconfigurations. This will result in more control plane messages and a larger service interruption than direct HO from NR SA to EN-DC. Specially for the scenario of EPS fallback, the eMBB service has to be also handover to a LTE cell with the voice service. We must say that this is not a perfect solution. In principle, the eMBB service shall be kept in the source NR cell during the EPS fallback procedure to achieve better experience.

In Rel-15, the HO from NR SA to EN-DC is not supported due to time limitations. Now the NR to LTE Handover has been fully supported in Rel-15. From the technical point of view, there are no obstacles for supporting NR SA to EN-DC HO in Rel-16. We also have a RAN2 contribution to suggest the support of NR SA to EN-DC in Rel-16 [2].

Proposal 1: The NR SA to EN-DC handover shall be supported in Rel-16.

Figure 1 UE moves out of SA coverage to EN-DC

RAN3 Impact Analysis

UE capability: The feature of NR SA to EN-DC HO may be not support for partial commercial NR UE. So this function may need to introduce a new UE capability indication in RAN2. If RAN2 agree with the introduction of this new UE capability, it will be contained in HO required message to inform the target eNB to perform SCG addition during HO procedure.

Cause value for EPS fallback: Currently, the VoNR is not supported in most NR base station. For voice service, the NR SA cell shall trigger EPS fallback to handover this UE to the LTE cell. The LTE eNB need to know this UE is still in the NR coverage to configure the source gNB as the target SN because there may be no measurement report for the source gNB contained in HO request message. After the voice call is over, the LTE eNB can trigger the UE HO back to NR gNB. In order to solve this issue, the HO cause for EPS fallback should be added in HO required message on NG interface and HO request message on S1 interface.

Proposal 2: The new UE capability for supporting SA to EN-DC HO shall be contained in in HO required message on NG interface.

Proposal 3: the HO cause for EPS fallback should be added in HO required message on NG interface and HO request message on S1 interface

3	Proposal

The intention of this contribution is to discuss the scenario and requirement for the support of NR SA to EN-DC in Rel-16, and gives some analysis on the RAN3 impact.

Proposal 1: The NR SA to EN-DC handover shall be supported in Rel-16.

Proposal 2: The new UE capability for supporting SA to EN-DC HO shall be contained in in HO required message on NG interface.

Proposal 3: the HO cause for EPS fallback should be added in HO required message on NG interface and HO request message on S1 interface

4	Reference

- R2-1907085, ZTE Corporation, Sanechips, Inter-RAT HO support

- R2-1911230, China Telecom, Support of inter-RAT HO from SA to EN-DC in Rel-16