# 3GPP TSG-RAN WG3 #105	Tdoc R3-194672

Ljubljana, Slovenia, 26th – 30th August

Agenda Item:	9.3

Source:	Ericsson, CATT, Google, Samsung

Title:	Summary of offline discussion on clarification on the handling of CG-ConfigInfo over F1

Document for:	Discussion, Decision

# Introduction

An offline discussion was carried out on the topic of handling the CG-ConfigInfo at SN in DC scenarios, which is based on contributions [1] and [2]. This paper provides a summary of the outcomes of the discussion.

# Discussion

The group discussed handling of the CG-ConfigInfo in DC scenarios involving an MN and SN connected via X2/Xn.

In the discussion it was acknowledged that:

- The MN signals a CG-ConfigInfo to the SN

- If the SN consist of a split gNB, the SN gNB-CU receiving the CG-ConfigInfo may remove from the CG-ConfigInfo the source CellGroupConfig and signal the remaining content of the CG-ConfigInfo to the SN gNB-DU. The latter is done to trigger full configuration at the SN gNB-DU.

All companies but one agreed to the following principle:

In case the MN signals the CGConfigInfo to SN for DC scenarios such as SN addition, the content of the CGConfigInfo should be treated as follows:

Except for the source CellGoupConfig, which can be removed by the SN gNB-CU from the CG-ConfigInfo, the remaining information contained in the CG-ConfigInfo signalled by the SN gNB-CU to SN gNB-DU are as-received from the MN

It is proposed to minute the principle above as a working assumption in RAN3

# References

[1] R3-194142, “Clarification on the handling of CG-ConfigInfo over F1” (Ericsson)

[2] R3-194526, “Response to R3-194142 and R3-194143” (Google)